Particulate matter removal by precipitation under normal and wildfire conditions across the United States using double machine learning: Supplement Material

Dinesh Neupane1, Viviana Maggioni2, Lucas Henneman1*,

1Department of Civil, Environmental and Infrastructure Engineering, George Mason University, Fairfax Virginia 22030

2Department of Civil and Environmental Engineering, Politecnico di Milano, Milano, Italy

*Corresponding author: Lucas Henneman

Email: lhennem@gmu.edu

Table SI-1 Definitions, symbols, units, and brief descriptions for the meteorological variables included in the study. The table lists near-surface temperature, humidity, pressure, wind components, radiation terms, convective indicators, evaporation, precipitation, and derived wind speed used in the modeling framework.

| Var | Symbol | Units | Description |
| --- | --- | --- | --- |
| TMP | T | °C | Temperature at 2 meters |
| SPFH | q | Mass fraction | Specific humidity at 2 meters |
| PRES | P | Pa | Surface pressure |
| UGRD | u | m/s | U wind component at 10 meters |
| VGRD | v | m/s | V wind component at 10 meters |
| DLWRF | L↓ | W/m² | Downward longwave radiation |
| CONVfrac | f_conv | fraction | Convective precipitation fraction |
| CAPE | CAPE | J/kg | Convective available potential energy |
| PEVAP | PEvap | kg/m² | Potential evaporation |
| APCP | P_tot | kg/m² | Total hourly precipitation |
| DSWRF | S↓ | W/m² | Downward shortwave radiation |
| Wind Speed | ¦V¦ | m/s | Wind speed (sqrt(u²+v²)) |

Table SI-2 PM2.5 quartiles and percent increases across wildfire smoke density categories relative to no-smoke conditions.

| Smoke Density | Q1 | Q1 % Increase | Median | Median % Increase | Q3 | Q3 % Increase |
| --- | --- | --- | --- | --- | --- | --- |
| NS | 4 | 0 | 6.5 | 0 | 10.2 | 0 |
| Light | 5.5 | 37.5 | 8.3 | 27.69 | 12.2 | 19.61 |
| Medium | 7.2 | 80 | 11.7 | 80 | 17.9 | 75.49 |
| Heavy | 9 | 125 | 16.9 | 160 | 34.1 | 234.31 |

Table SI-3 PM10 quartiles and percent increases across wildfire smoke density categories relative to no-smoke conditions.

| Smoke Density | Q1 | Q1 % Increase | Median | Median % Increase | Q3 | Q3 % Increase |
| --- | --- | --- | --- | --- | --- | --- |
| NS | 9 | 0 | 15.9 | 0 | 26 | 0 |
| Light | 14 | 55.56 | 21.5 | 35.22 | 33.8 | 30 |
| Medium | 18.3 | 103.33 | 28 | 76.1 | 45 | 73.08 |
| Heavy | 24.4 | 171.11 | 42 | 164.15 | 73 | 180.77 |

Figure SI-1 Distribution of hourly PM2.5 concentrations across wildfire smoke categories from 2014–2023.

Figure SI-2 Matched baseline PM distributions across smoke categories for PM2.5 and PM10.

Figure SI-3 Distributions of estimated removal rates for PM2.5 and PM10 across wildfire smoke density categories. Boxplots show removal rates under the full precipitation distribution (Total) and within fixed precipitation ranges (1-3 mm, 2-4 mm, and 3-5 mm).

Figure SI-4 Distribution of outcome residuals across five cross-fitting folds for PM2.5 and PM10. Quartiles and medians changed little from fold to fold. Median residuals were negative in every fold for both particle fractions. This indicates stable estimates and consistently negative removal-efficiency residuals.

Figure SI-5 Regression estimates across the five folds for PM2.5 and PM10. Estimates were negative in every fold for both particle fractions. Fold-to-fold variation was small. PM10 estimates were consistently more negative than PM2.5. Error bars show 95% confidence intervals.

Figure SI- 6 Expected removal of PM2.5 (left) and PM10 (right) as a function of precipitation intensity APCP (For APCP > 2mm). Shaded bands show 95% CI. Dashed curves show the fitted power-law washout model with fitted a and b reported in each panel from Equation (10)