Supplementary Materials (Tables: S1 to S7)

| Table S1. Detailed physicochemical profile and multi-ion concentrations for each of the 17 surface wells in Al-Musaymir District (at 25 °C) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name of well | Village | Parameter | Turbidity | pH | EC | TDS | NO3- | Fluoride | ICBE |
|  |  | Unit | NTU | - | µg/cm | mg / L | mg / L | mg / L | (%) |
|  |  | WHO guidelines | 1-15 | 6.5-8.5 | 450-1000 | 1000 | 50 | 0.5-1.5 | - |
| Fadhal Saeed | Al-Sarahn'a | Sample-1 | 2 | 7.5 | 2030 | 1218 | 41 | 2.1 | +0.68 |
| Faraa | Fara'a | Sample-2 | 0.5 | 7.38 | 2110 | 1266 | 39 | 4.4 | +1.36 |
| Nakhelaa | Nakhelaa | Sample-3 | 0.2 | 7.76 | 1409 | 926 | 24.8 | 1.1 | +0.45 |
| Mutein | Nakhelaa | Sample-4 | 2.3 | 7.15 | 1833.33 | 1210 | 60 | 2.24 | +0.11 |
| Qayif | Qaaf | Sample-5 | 2 | 7.6 | 3800 | 2280 | 24 | 2.17 | +2.52 |
| Qaaf | Qaaf | Sample-6 | 2.69 | 7.35 | 2000 | 1200 | 59 | 4.3 | +1.82 |
| Bashria | Bashria | Sample-7 | 1.14 | 7.06 | 2690 | 1779 | 35.6 | 2.52 | +0.40 |
| Alhaqaf | Al-Musaymir Center | Sample-8 | 2 | 7.6 | 2298.48 | 1517 | 68 | 1.4 | -0.06 |
| Majwer | Goal Madram | Sample-9 | 2 | 7.7 | 2400 | 1380 | 50.1 | 1.7 | +2.86 |
| Mareeb | Mareeb | Sample-10 | 2 | 7.4 | 1800 | 1080 | 15.2 | 2.4 | +3.93 |
| Dagran | Dagran | Sample-11 | 2 | 7.6 | 1700 | 1020 | 11.9 | 2 | +2.35 |
| Ali Nusorah | Qaaf | Sample-12 | 2 | 7.6 | 1480 | 950 | 20 | 5.7 | +0.30 |
| Yaseen | Mucadium | Sample-13 | 0.2 | 7.6 | 1696.97 | 1120 | 28 | 1.47 | +0.25 |
| Ahmed | Allugmaa | Sample-14 | 0.3 | 7.42 | 1695 | 1122 | 16.4 | 3.1 | +0.74 |
| Alsaeed | Top asel | Sample-15 | 0.3 | 7.33 | 1766.67 | 1166 | 40 | 2.8 | +0.20 |
| Abdulkareem | Habeel Alaf | Sample-16 | 1.3 | 7.7 | 1598.48 | 1055 | 35 | 1.5 | +0.15 |
| Shathaa | Shathaa | Sample-17 | 0.3 | 7.43 | 1639.39 | 1082 | 33 | 2.8 | +0.12 |
| Average |  |  | 1.37 | 7.48 | 1996.9 | 1257 | 35.4 | 2.57 | 2.15 |
| Standard deviation |  |  | 0.85 | 0.19 | 574.8 | 336.98 | 16.5 | 1.19 | 1.35 |
| Minimum |  |  | 0.2 | 7.33 | 1409 | 950 | 11.9 | 1.1 | 0.15 |
| Maximum |  |  | 2.69 | 7.76 | 3800 | 2280 | 68 | 5.7 | 4.92 |
| Average ± SD |  |  | 1.37±0.85 | 7.48±0.19 | 1996.9±574.8 | 1257±336.98 | 35.4±16.5 | 2.57±1.19 | 1.35±2.15 |
| Failure (%) |  |  | 0% | 0% | 100% | 88.2% | 23.53% | 82.35% | Pass 0% |

| Table S2. Relative weight of physicochemical parameters for WQI calculation Based on the framework of Saleh et al., (2022a) and regional literature |  |  |  |
| --- | --- | --- | --- |
| Parameter | WHO Standard (Si)13 | Weight (wi) | Relative Weight (Wi) |
| Fluoride (F⁻) | 1.5 | 5* | 0.1389 |
| Nitrate (NO₃⁻) | 50 | 5 | 0.1389 |
| Total Dissolved Solids (TDS) | 1000 | 5 | 0.1389 |
| pH | 8.5 | 3 | 0.0833 |
| Chloride (Cl⁻) | 250 | 3 | 0.0833 |
| Sulfate (SO₄²⁻) | 250 | 3 | 0.0833 |
| Sodium (Na⁺) | 200 | 3 | 0.0833 |
| Total Hardness (TH) | 500 | 2 | 0.0556 |
| Calcium (Ca²⁺) | 75 | 2 | 0.0556 |
| Magnesium (Mg²⁺) | 50 | 2 | 0.0556 |
| Potassium (K⁺) | 12 | 2 | 0.0556 |
| Bicarbonate (HCO₃⁻) | 120** | 1 | 0.0278 |
| Total |  | ∑wi=36 | ∑Wi=1 |
| * Chalise et al., 2023 ; **USPHS Standard, 1962 Note: Parameters were weighted according to their relative importance in drinking water quality and potential health impacts, following the specialized framework for the Yemeni hydrogeological context (Saleh et al., 2022a) |  |  |  |

| Table S3 Water quality classification based on WQI (*) |  |  |
| --- | --- | --- |
| Water Class | Type of water |  |
| Ⅰ | Excellence | < 50 |
| Ⅱ | Good water | 50-100 |
| Ⅲ | Poor water | 100-200 |
| Ⅳ | Very poor water | 200-300 |
| Ⅴ | Water unsuitable for drinking | > 300 |
| Note:(*) (Ramakrishnaiah, et al., 2009; Lateef, 2011; Ketata-Rokbani, et al., 2011) |  |  |

| Table S4. The classification of irrigation waters according (Richards1954; Wilcox 1955) |  |  |
| --- | --- | --- |
| Index method | Range | Classification |
| EC | ˂ 250 | (C1) low salinity |
|  | 250 – 750 | (C2) medium salinity |
|  | ˃ 750 – 2250 | (C3) high salinity |
|  | ˃2250 | (C4) very high salinity |
| SAR | ˂ 10 | (S1) low sodium |
|  | 10 – 18 | (S2) medium sodium |
|  | ˃ 18 – 26 | (S3) high sodium |
|  | ˃ 26 | (S4) very high sodium |

| Table S5. Linear regression parameters and sensitivity metrics (LOD/LOQ) for the fluoride determination method |  |
| --- | --- |
| Parameters | Value |
| Standard Deviation (SD) | 0.370 |
| Standard Error (SE) | 0.247 |
| Standard Error of the Intercept | 0.168 |
| Limit of Detection (LOD) | 0.02 mg/L |
| Limit of Quantification (LOQ) | 0.07 mg/L |
| Slope of the Calibration Curve | -51.579 |
| Intercept of the Curve | 315.02 mV |
| R-Square (R2) | 0.9998 |
| Note: The exceptional linearity (CaCO3) and low Limit of Detection (LOD = 0.02 mg/L) provide the analytical backbone for this study, ensuring that even trace-level geogenic fluoride fluctuations are quantified with high precision. |  |

| Table S6 (a & b). Analytical performance and cross-validation metrics (ISE vs. Spectrophotometry) for fluoride quantification |  |  |  |
| --- | --- | --- | --- |
| Table S6 (a). Potentiometric Method Validation (Ion-Selective Electrode) |  |  |  |
| Taken (mg/L) | Metric* | Within-Day (n=3) | Between-Day (n=3) |
| 0.19 | Measured (mg/L ± SD) | 0.1897±0.0006 | 0.187±0.0058 |
|  | Precision (RSD %) | 8.96E-05 0.0000896 | 0.000896 |
|  | Accuracy (RE %) | -0.158 | -1.579 |
|  | Recovery (RCV %) | 99.84 | 98.42 |
| 2 | Measured (mg/L ± SD) | 1.98±0.0764 | 1.99 ±0.1 |
|  | Precision (RSD %) | 0.12472 | 0.163299 |
|  | Accuracy (RE %) | -1.00 | -0.5 |
|  | Recovery (RCV %) | 99.00 | 99.5 |
| 19 | Measured (mg/L ± SD) | 18.99±0.0321 | 19.02±0.0763 |
|  | Precision (RSD %) | 0.49869 | 1.184858 |
|  | Accuracy (RE %) | -0.053 | 0.105 |
|  | Recovery (RCV %) | 99.95 | 100.11 |
| Table S6 (b). Spectrophotometric Method Validation (HACH DR 3900) |  |  |  |
| Taken (mg/L) | Metric* | Within-Day (n=3) | Between-Day (n=3) |
| 0.3 | Measured (mg/L ± SD) | 0.257±0.0252 | 0.253±0.0153 |
|  | Precision (RSD %) | 0.005342 | 0.003367 |
|  | Accuracy (RE %) | -14.33 | -15.67 |
|  | Recovery (RCV %) | 85.67 | 84.33 |
| 1.4 | Measured (mg/L ± SD) | 1.35±0.1323 | 1.367±0.2082 |
|  | Precision (RSD %) | 0.162019 | 0.271948 |
|  | Accuracy (RE %) | -3.57 | -2.36 |
|  | Recovery (RCV %) | 96.43 | 97.64 |
| 1.8 | Measured (mg/L ± SD) | 1.87 ±0.1528 | 1.7±0.20 |
|  | Precision (RSD %) | 0.236972 | 0.277609 |
|  | Accuracy (RE %) | 3.89 | -5.56 |
|  | Recovery (RCV %) | 103.89 | 94.44 |
| *SD: Standard Deviation, RSD: Relative Standard Deviation, RE: Relative Error, RCV% Recovery, *Average of three replicate determinations |  |  |  |

| Table S7. Synergistic inter-elemental relationships: Pearson correlation analysis (r) of the 15-parameter suite in Al-Musaymir’s surface wells (n=17) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | pH | EC | TDS | Ca²⁺ | Mg²⁺ | TH | Na⁺ | K⁺ | Cl⁻ | SO₄²⁻ | HCO₃⁻ | NO₃⁻ | F⁻ |
| pH | 1.00 |  |  |  |  |  |  |  |  |  |  |  |  |
| EC | 0.71** | 1.00 |  |  |  |  |  |  |  |  |  |  |  |
| TDS | 0.70** | 0.99** | 1.00 |  |  |  |  |  |  |  |  |  |  |
| Ca²⁺ | 0.45 | 0.65** | 0.46 | 1.00 |  |  |  |  |  |  |  |  |  |
| Mg²⁺ | 0.42 | 0.61** | 0.62** | 0.81** | 1.00 |  |  |  |  |  |  |  |  |
| TH | 0.48 | 0.68** | 0.67** | 0.93** | 0.91** | 1.00 |  |  |  |  |  |  |  |
| Na⁺ | 0.75** | 0.88** | 0.91** | 0.26 | 0.60* | 0.68** | 1.00 |  |  |  |  |  |  |
| K⁺ | 0.31 | 0.45 | 0.43 | 0.34 | 0.26 | 0.31 | 0.25 | 1.00 |  |  |  |  |  |
| Cl⁻ | 0.31 | 0.78** | 0.76** | 0.88** | 0.84** | 0.90** | 0.74** | 0.31 | 1.00 |  |  |  |  |
| SO₄²⁻ | 0.33 | 0.72** | 0.75** | 0.60* | 0.54 | 0.62** | 0.75** | 0.49 | 0.86** | 1.00 |  |  |  |
| HCO₃⁻ | 0.19 | 0.44 | 0.48 | 0.12 | 0.11 | 0.15 | 0.23 | 0.65** | 0.32 | 0.31 | 1.00 |  |  |
| NO₃⁻ | 0.20 | 0.18 | 0.17 | 0.31 | 0.34 | 0.32 | 0.20 | 0.25 | 0.31 | 0.37 | 0.37 | 1.00 |  |
| F⁻ | 0.71** | 0.86** | 0.82** | 0.65** | 0.61** | 0.68** | 0.65** | 0.29 | 0.78** | 0.72** | 0.37 | 0.25 | 1.00 |
| * p value < 0.05; ** p value < 0.01 Table S7. displays the Pearson correlation coefficients (r) for 15 physicochemical parameters, revealing the synergistic relationships driving the hydrogeochemistry of Al-Musaymir. Significant positive correlations are observed between Fluoride and pH (r=0.71), and Fluoride and TDS (r=0.82) mathematically confirming the alkaline leaching mechanism and geogenic mineralization. Conversely, the negligible correlation between Fluoride and Nitrate (r=0.25) serves as a diagnostic indicator, effectively isolating the primary geogenic crisis from secondary anthropogenic inputs. |  |  |  |  |  |  |  |  |  |  |  |  |  |