Tables

Manuscript: A Unified Multi-Scale Simulation Framework for High-Bandwidth Electrohydraulic Actuators: Regime-Aware Discharge Physics, Transient Fluid Inertia, and Triple-Domain Experimental Validation

All eight manuscript tables are reproduced here in full, with captions, values, and notes, one table per page. Tables 1–4 and 6 appear in the main text; Tables 5, 7, and 8 appear in Appendices A and B of the manuscript. Numbering follows the order of first citation in the text.

Table 1. Comparison of existing electrohydraulic actuator modeling approaches. (Main text, Section 2.4)

| Reference | Modeling method | Constant Cd? | CFD integration | Transient fluid inertia | Reduced-order model | Validation method | Main limitation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Merritt [32] | Classical lumped-parameter hydraulic control model | Yes | No | No | Yes | Classical experimental benchmark | Assumes fully turbulent orifice flow; no regime-dependent Cd(Re) |
| Blackburn et al. [33] | Quasi-steady orifice and flow-force model | Yes | No | No | Yes | Experimental / analytical | Neglects laminar–transitional–turbulent transition and unsteady inertia |
| Ashoori et al. [9] | Empirical / semi-empirical Cd correlation | No; variable Cd(Re) | Partial | No | Partial | Experimental / correlation fitting | Not embedded in a full actuator dynamic model |
| Lisowski and Rajda [37] | CFD-based valve flow-force analysis | N/A; field-resolved | Yes | No | No | CFD; limited experimental comparison | Mostly steady-state; no real-time ROM or closed-loop dynamics |
| Vacca et al. [39] | Numerical flow-force evaluation for spool valves | Partial | Yes | Partial | No | Numerical / experimental | Does not couple regime-aware metering to full system dynamics |
| Rossi and Vacca [43] | Multi-scale CFD-to-system modeling | Partial | Yes | Partial | Yes | Experimental | Lacks a continuous closed-form Cd(Re) closure and explicit transient-inertia propagation |
| Garcia et al. [44] | Predictive commissioning for high-bandwidth hydraulic systems | No explicit Cd(Re) closure | Partial | Partial | Yes | HIL / experimental | Limited micro-scale CFD physics |
| Liang et al. [45] | Physics-informed digital twin for valve prognostics | Partial | Partial | Partial | Yes | Experimental / prognostic demonstration | Targets health monitoring rather than high-bandwidth dynamic prediction |
| Sitte et al. [46] | Physics-informed machine-learning identification | No explicit Cd(Re) physics | No | No | Yes | Experimental | Extrapolation limited by the training domain; weaker physical interpretability |
| This work | Unified micro–meso–macro physics-informed framework | No; CFD-derived Cd(Re) | Yes | Yes | Yes | Laboratory test, HIL, operating-condition emulation | Validated on a laboratory NG10 sharp-edged geometry; isothermal, non-cavitating assumptions |

Note. The table establishes that the contribution of this work is the unified coupling of regime-aware micro-scale metering physics, meso-scale flow forces, and macro-scale reduced-order dynamics, rather than the isolated invention of CFD or variable discharge-coefficient modeling.

Table 2. Geometric and physical parameters of the representative electrohydraulic actuator. (Main text, Section 3.3)

| Category | Parameter | Symbol | Value | Unit | Source / note |
| --- | --- | --- | --- | --- | --- |
| Valve architecture | Valve type | – | NG10 four-way, three-position direct-drive spool valve | – | ISO 4401-05 manifold |
| Valve architecture | Spool diameter | ds | 10 | mm | Laboratory valve geometry |
| Valve architecture | Spool travel | xv,max | 0–3.5 | mm | Direct-drive stroke |
| Valve architecture | Pressure-sensitive spool area | As | 78.5 | mm2 | NG10 spool geometry |
| Valve architecture | Prototype envelope | – | 180 × 120 × 95 | mm | Measured |
| Valve architecture | Prototype mass | – | 2.4 | kg | Measured |
| Metering orifice | Orifice type | – | Sharp-edged rectangular slot | – | Representative servo-valve metering edge |
| Metering orifice | Orifice length | L | 1.0 | mm | CFD domain |
| Metering orifice | Orifice depth | d | 0.5 | mm | CFD domain |
| Metering orifice | Slot width | w | 10 | mm | CFD / experiment |
| Metering orifice | Orifice opening range | h | 0.3–3.5 | mm | CFD / experiment sweep |
| Mechanical spool | Spool mass | ms | 0.045 | kg | CAD and scale measurement |
| Mechanical spool | Identified effective spring stiffness | Ks | 1000 | N/m | Static deflection test |
| Mechanical spool | Mechanical / fluid viscous damping | Bs | 8 | N·s/m | Step-response log-decay fit |
| Hydraulic fluid | Fluid type | – | ISO VG 46 hydraulic oil | – | Laboratory test fluid |
| Hydraulic fluid | Density | ρ | 860 | kg/m3 | Datasheet / model assumption |
| Hydraulic fluid | Dynamic viscosity | μ | 0.03 | Pa·s | ≈ 25 °C assumption |
| Hydraulic fluid | Cleanliness level | – | ISO 4406:18/16/13 | – | Laboratory filtration |
| Supply and flow | Rated manifold pressure | – | 21 | MPa | Manifold rating |
| Supply and flow | Pilot relief pressure | – | 18 | MPa | Manifold specification |
| Supply and flow | Tested supply pressure range | – | 15–21 | MPa | Experimental matrix |
| Supply and flow | Maximum flow capacity | Qmax | 55 | L/min | Full-open condition |
| Load emulator | Load cylinder bore | – | 32 | mm | Double-acting cylinder |
| Load emulator | Load cylinder rod diameter | – | 16 | mm | Double-acting cylinder |
| Load emulator | Load stroke | – | 100 | mm | Load cylinder |
| Load emulator | Effective load mass | mL | 1.5–2.5 | kg | Adjustable inertial load |
| Load emulator | Load spring stiffness | – | ≈ 5 | N/mm | Spring–damper fixture |
| Motor | Motor type | – | 12 V DC servo motor | – | Direct spool drive |
| Motor | Rated voltage | Va | 12 | V | Nominal |
| Motor | Armature resistance | Ra | 3.5 | Ω | Bench measurement |
| Motor | Armature inductance | La | 1.2 × 10−3 | H | AC impedance test |
| Motor | Torque constant | Km | 0.086 | N·m/A | Bench calibration |
| Motor | Back-EMF constant | Kb | 0.086 | V·s/rad | Equal to Km |
| Motor | Rotor inertia | Jm | 2 × 10−6 | kg·m2 | CAD / coast-down |
| Motor | Motor viscous friction | Bm | 5 × 10−4 | N·m·s/rad | No-load test |
| Motor | Coupling radius | r | 5 | mm | Mechanical coupling |
| Motor | Continuous torque (manufacturer rating) | Tcont | 3.7 | mN·m | Manufacturer data |
| Motor | Peak dynamic torque inferred from step tests | Tpeak | 0.45 | N·m | Experimental inference |
| Controller | Controller | – | PIC16F877A, 4 MHz | – | Supervisory control |
| Controller | Drive PWM | – | 500 | Hz | H-bridge drive |
| Data acquisition | DAQ | – | NI CompactDAQ, 16-bit, 10 kHz | – | Phase-coherent dynamic identification |

Note. The motor continuous torque (3.7 mN·m) is the manufacturer-rated continuous specification, while the peak dynamic torque (0.45 N·m) is the experimentally inferred maximum under transient step excitation. The identified effective spring stiffness Ks = 1000 N/m supersedes the nominal 0.5 N/mm catalogue value and is used throughout the model.

Table 3. Mesh independence, Richardson extrapolation, and Grid Convergence Index analysis. (Main text, Section 4.2.1)

| Opening h | Mesh level | Elements | Cd | ΔP (kPa) | Deviation from fine mesh |
| --- | --- | --- | --- | --- | --- |
| 0.3 mm | Coarse | 8,000 | 0.486 | 142.8 | 4.1% |
| 0.3 mm | Medium | 18,000 | 0.512 | 141.2 | 0.4% |
| 0.3 mm | Fine | 35,000 | 0.512 | 141.5 | Reference |
| 1.0 mm | Coarse | 8,000 | 0.621 | 58.3 | 2.9% |
| 1.0 mm | Medium | 18,000 | 0.638 | 58.1 | 0.2% |
| 1.0 mm | Fine | 35,000 | 0.638 | 58.2 | Reference |
| 3.0 mm | Coarse | 8,000 | 0.656 | 21.5 | 1.8% |
| 3.0 mm | Medium | 18,000 | 0.662 | 21.4 | 0.1% |
| 3.0 mm | Fine | 35,000 | 0.662 | 21.3 | Reference |

Note. For the discharge coefficient, the medium and fine meshes agree to the reported precision, indicating asymptotic convergence. The medium mesh was selected for the 112-case production campaign because it reproduced the fine-mesh discharge coefficient within 0.4% at a fraction of the cost; the full campaign required approximately 48 hours on a 12-core workstation.

Table 4. Experimental instrumentation and measurement uncertainty. (Main text, Section 5.3)

| Instrument | Measured variable | Range | Resolution / key specification | Accuracy | Sampling rate | Approx. standard uncertainty | Role in validation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WIKA PS-23 pressure transducer | Supply / load pressure | 0–25 MPa | Transducer-grade output | ±0.5% FS | 1 kHz | ±0.07 MPa (Type B, rectangular assumption) | Pressure drop, flow force, load pressure |
| Micro-Epsilon ILD1700 laser sensor | Spool displacement | Short-range laser triangulation | 0.01 mm resolution; sub-μm repeatability reported | Manufacturer specification | 1 kHz | ≈ ±0.003 mm (resolution-limited) | Spool position, static metering, dynamic response |
| Turbine flow meter | Return-line flow | 5–80 L/min | ±1% accuracy | ±1% of reading | 100 Hz | ≈ ±0.32 L/min at 55 L/min | Steady-state flow calibration |
| Shunt resistor + instrumentation amplifier | Motor current | Laboratory drive range | 10 mΩ shunt, 0.1% tolerance | ±0.1% shunt tolerance | 1 kHz | ≈ ±0.1% of reading | Torque inference |
| Precision voltage divider | Supply voltage | 0–15 V class | Precision resistive network | High stability | 100 Hz | ≈ ±0.1% of reading | Power-rail monitoring |
| NI CompactDAQ (NI 9172) | Multi-channel acquisition | 16-bit analog inputs | 1 μs hardware synchronization | 16-bit quantization | Up to 10 kHz | ±0.5 LSB quantization | Phase-coherent Bode and transient identification |

Note. Standard uncertainties are indicative Type B estimates based on manufacturer specifications and should be finalized according to ISO GUM propagation. All dynamic channels used for bandwidth identification were sampled at or above 1 kHz, with hardware synchronization minimizing inter-channel phase drift.

Table 5. Dynamic model parameters and identified constants. (Appendix A; first cited in Section 4.4.4)

| Parameter group | Parameter | Symbol | Value | Unit | Identification method |
| --- | --- | --- | --- | --- | --- |
| Spool mechanics | Spool mass | ms | 0.045 | kg | CAD and scale |
| Spool mechanics | Effective spring stiffness | Ks | 1000 | N/m | Static deflection test |
| Spool mechanics | Mechanical / fluid viscous damping | Bs | 8 | N·s/m | Step-response log-decay |
| Spool mechanics | Pressure-sensitive area | As | 78.5 × 10−6 | m2 | NG10 geometry |
| Flow-force model | Fluid density | ρ | 860 | kg/m3 | ISO VG 46 datasheet |
| Flow-force model | Effective fluid-column length | Le | 0.015 | m | Geometry and transient pressure fit |
| Flow-force model | Jet deflection angle | θ | 18 | deg | CFD exit-plane velocity field |
| Flow-force model | Flow-force damping coefficient | Kf | 1.2 | N·s/m | Step-response fitting |
| Flow-force model | Distributed transient-inertia coefficient | ρLe | 12.9 | kg/m2 | Reynolds-transport closure |
| Flow-force model | Equivalent lumped transient inertia | Mt | 0.013 | kg | Frequency-response identification |
| Motor electromechanics | Armature resistance | Ra | 3.5 | Ω | DC measurement |
| Motor electromechanics | Armature inductance | La | 1.2 × 10−3 | H | AC impedance at 1 kHz |
| Motor electromechanics | Torque constant | Km | 0.086 | N·m/A | Bench calibration |
| Motor electromechanics | Back-EMF constant | Kb | 0.086 | V·s/rad | No-load speed test |
| Motor electromechanics | Rotor inertia | Jm | 2 × 10−6 | kg·m2 | CAD / coast-down |
| Motor electromechanics | Motor friction coefficient | Bm | 5 × 10−4 | N·m·s/rad | No-load rotation test |
| Motor electromechanics | Coupling radius | r | 0.005 | m | CAD measurement |
| Hydraulic line impedance | Line resistance | Rline | 0.5 | bar/(L/s) | Steady-flow pressure drop |
| Hydraulic line impedance | Line inertance | Lline | 2 × 107 | Pa·s2/m5 | Step-response oscillation frequency |
| Hydraulic line impedance | Line compliance | Cline | 0.5 × 10−9 | m3/Pa | Hose volume and elasticity estimate |
| Reduced-order effective | Effective moving mass (lumped ROM) | Meff | ms + Mt = 0.058 | kg | Lumped reduced-order model |
| Reduced-order effective | Effective damping | Beff | Bs + Kf = 9.2 | N·s/m | Linearized ROM |
| Reduced-order effective | Hydraulic stiffness | Kh | 3.5 × 107 | N/m | Line-impedance coupling / identified ROM |
| Reduced-order effective | Effective stiffness | Keff | Ks + Kh ≈ 3.5001 × 107 | N/m | Linearized ROM |
| Reduced-order effective | Natural frequency | fn | 950 | Hz | Closed-loop frequency response |
| Reduced-order effective | Closed-loop bandwidth | fBW | 650 | Hz | −3 dB point |
| Reduced-order effective | Damping ratio | ζ | 0.65–0.75 | – | Frequency-response fit |
| Reduced-order effective | Phase margin | PM | 55 | deg | Bode crossover |

Note. The distributed transient-fluid-inertia coefficient ρLe = 12.9 kg/m2 appears in the Reynolds-transport flow-force expression Fx,trans = ρLe dQ/dt. The equivalent lumped inertia Mt = 0.013 kg is obtained after projection of the distributed fluid inertia into the reduced-order spool coordinate and subsequent frequency-response identification. Accordingly, ρLe must not be interpreted as a lumped mass, and Mt must not be obtained by direct numerical substitution of ρLe into the second-order mechanical equation without coordinate projection.

Table 6. Ablation study: influence of physics simplifications. (Main text, Section 6.5)

| Model variant | Displacement RMSE | Pressure RMSE | Maximum overshoot | Relative RMSE increase | Phase-margin error | Bandwidth / natural-frequency error |
| --- | --- | --- | --- | --- | --- | --- |
| Full model: Cd(Re) + transient inertia | 0.032 mm | 1.2 bar | 2.1% | Baseline | < 2° | < 3.2% |
| Constant Cd + transient inertia | 0.048 mm | 1.8 bar | 5.4% | +50% | Up to 18° near Re ≈ 1500 | Exceeds full-model bound |
| Cd(Re) only, no transient inertia | 0.041 mm | 2.1 bar | 8.6% | +28% | Not separately archived | 10–15% natural-frequency error |
| Baseline: constant Cd, no transient inertia | 0.072 mm | 3.4 bar | 15.3% | +125% | Up to 18° degradation | Largest deviation; > 15% inferred |

Note. RMSE and overshoot values are measured from 500 ms step-response tests with N = 5 repeats. The phase-margin degradation of approximately 18° is associated primarily with constant-Cd modeling in the transitional regime, Re ≈ 500–2500.

Table 7. Model validation summary. (Appendix B; cited in Section 6.7)

| Validation metric | Experimental result | Model prediction | Error / deviation | Acceptance criterion | Status |
| --- | --- | --- | --- | --- | --- |
| Cd(Re) correlation accuracy | 112 CFD / experimental comparison points | Eq. (5) fit | R2 = 0.978, RMSE = 0.022 | R2 > 0.95 | Pass |
| Mesh independence | Medium / fine mesh comparison | Medium mesh selected | < 0.4% deviation; GCI32 for Cd ≤ 0.02% | < 1% deviation | Pass |
| Static metering agreement | Measured Cd,exp | CFD-derived Cd(Re) | ≈ ±2% | ±5% | Pass |
| Current–displacement linearity | R2 = 0.998 | Linear small-signal assumption | < 1% deviation | R2 > 0.99 | Pass |
| Displacement uncertainty | ±0.08 mm | – | – | Consistent with sensor resolution | Pass |
| Rise time (10–90%) | 0.18 s | ≈ 0.18 s | < 5% | ≤ 10% | Pass |
| Settling time (±2%) | 0.45 s | ≈ 0.45 s | < 5% | ≤ 10% | Pass |
| Step-response overshoot | 2.1% | 2.1% | < 1 percentage point | ≤ 5 percentage points | Pass |
| Natural frequency | 950 Hz | 955 Hz | ≈ 0.5% | ≤ 5% | Pass |
| Closed-loop bandwidth | 650 Hz | 650 Hz | < 3.2% | ≤ 5% | Pass |
| Phase margin | 55° | 55° | < 2° | PM ≥ 45° | Pass |
| Magnitude error, 1–800 Hz | Measured Bode | Predicted Bode | < 5% | ≤ 5% | Pass |
| Phase error, up to 600 Hz | Measured Bode | Predicted Bode | < 10° | ≤ 10° | Pass |
| Robustness across pressure / load | 15–21 MPa, 1.5–2.5 kg | Same operating matrix | 96.8% of predictions within ±3.2% | ≥ 95% within ±5% | Pass |

Note. Acceptance criteria are selected to be consistent with high-bandwidth electrohydraulic validation practice. The phase-margin criterion is intentionally conservative relative to typical aerospace-inspired stability-margin expectations [56], without claiming certification compliance.

Table 8. Comparison with published electrohydraulic actuator models. (Appendix B; cited in Section 6.7)

| Study | CFD used | Variable Cd(Re) | Transient inertia | Multi-scale coupling | Experimental validation | Maximum reported prediction error |
| --- | --- | --- | --- | --- | --- | --- |
| Merritt [32] | No | No | No | No | Yes | Not reported; constant-Cd assumption |
| Blackburn et al. [33] | No | No | No | No | Yes | Not reported; quasi-steady model |
| Ashoori et al. [9] | Partial | Yes | No | No | Yes (correlation level) | RMSE ≈ 0.022 for Cd fit |
| Lisowski and Rajda [37] | Yes | Field-resolved | No | No | Partial | Not reported at system level |
| Vacca et al. [39] | Yes | Partial | Partial | No | Partial | Not reported at closed-loop level |
| Rossi and Vacca [43] | Yes | Partial | Partial | Yes | Yes | Not reported in unified bandwidth / phase-margin form |
| Sitte et al. [46] | No | No explicit physics closure | No | Partial | Yes | Data-driven; extrapolation not fully quantified |
| This work | Yes | Yes; closed-form Cd(Re) | Yes | Yes; micro–meso–macro | Laboratory, HIL, operating-condition emulation | < 3.2% bandwidth error; < 10° phase error up to 600 Hz |

Note. The comparison emphasizes modeling capability rather than hardware certification. The present work is validated on a representative laboratory-scale direct-drive electrohydraulic actuator whose operating physics overlap with aerospace-relevant servo-valve conditions, particularly the transitional Reynolds-number regime.