Supplementary Material

A Unified Multi-Scale Simulation Framework for High-Bandwidth Electrohydraulic Actuators: Regime-Aware Discharge Physics, Transient Fluid Inertia, and Triple-Domain Experimental Validation

[Author names]

Department of Mechanical Engineering, Bapuji Institute of Engineering and Technology (BIET), Davangere 577004, Karnataka, India

This file contains four Supplementary Notes (S1–S4) providing derivations and methodological detail supporting the main article, and three Supplementary Tables (S1–S3). Supplementary Tables S1–S3 correspond to Tables 5, 7, and 8 of the integrated (appendix-based) manuscript version; a submission should include either Appendices A and B or this file, not both. Citation numbers in square brackets refer to the reference list of the main article. Equation numbers of the form (Sx.y) are internal to this file; plain equation numbers, for example Eq. (18), refer to the main article.

Contents

- Supplementary Note S1. Derivation of the voltage-to-spool-displacement transfer function, Eq. (18)

- Supplementary Note S2. Linearization and coordinate projection of the Reynolds-transport flow force

- Supplementary Note S3. CFD campaign design, numerical settings, and grid-convergence methodology

- Supplementary Note S4. Measurement-uncertainty budget methodology (ISO GUM)

- Supplementary Table S1. Dynamic model parameters and identified constants

- Supplementary Table S2. Model validation summary

- Supplementary Table S3. Comparison with published electrohydraulic actuator models

- Edits required in the main text when this file is used in place of the appendices

# Supplementary Note S1. Derivation of the voltage-to-spool-displacement transfer function, Eq. (18)

The derivation combines the motor electromechanics, Eqs. (14)–(16) of the main article, with the reduced-order spool dynamics, Eq. (12), through the rigid coupling of radius r. The coupling kinematics relate the rotor angle θm to the spool displacement:

xv = rθm,    Ωm(s) = sXv(s)/r	(S1.1)

Taking the Laplace transform of the armature circuit, Eq. (14), at zero initial conditions gives

(Las + Ra) Ia(s) = Va(s) − KbΩm(s)	(S1.2)

Let Fc denote the force transmitted through the coupling to the spool. The rotor balance, Eq. (16), with Tm = Kmia from Eq. (15), yields

Fc(s) = [KmIa(s) − (Jms + Bm) Ωm(s)] / r	(S1.3)

For the plant transfer function with respect to armature voltage, the hydraulic disturbance input in Eq. (12) is set to zero and the coupling force drives the spool:

(Meffs2 + Beffs + Keff) Xv(s) = Fc(s)	(S1.4)

Substituting Eqs. (S1.1) and (S1.3) into Eq. (S1.4) and collecting terms in Xv reflects the rotor inertia and friction into the translational coordinate:

(M′s2 + B′s + Keff) Xv(s) = (Km/r) Ia(s),    M′ = Meff + Jm/r2,    B′ = Beff + Bm/r2	(S1.5)

Solving Eq. (S1.2) for the armature current with the back-EMF expressed through Eq. (S1.1),

Ia(s) = [Va(s) − (Kb/r) sXv(s)] / (Las + Ra)	(S1.6)

and substituting into Eq. (S1.5) gives, after rearrangement, the third-order plant of Eq. (18) of the main article:

xv(s)/Va(s) = (Km/r) / {(Las + Ra)(M′s2 + B′s + Keff) + (KmKb/r2)s}	(S1.7)

Two structural observations follow. First, the term (KmKb/r2)s is the back-EMF coupling; at frequencies well below the electrical corner Ra/La it acts as an additional electrically induced viscous damping of magnitude KmKb/(r2Ra) added to B′, which is why the direct-drive stage is well damped without a dedicated dashpot. Second, the polynomial is third order: a fast electrical pole near Ra/La multiplies the dominant mechanical–hydraulic pair governed by M′, B′, and Keff. The numerical response characteristics quoted in the main article (DC gain, natural frequency, damping ratio, and bandwidth) are experimentally identified values obtained by closed-loop system identification as described in Sections 4.4.4 and 6.4; Eq. (S1.7) provides the model structure used for that identification.

# Supplementary Note S2. Linearization and coordinate projection of the Reynolds-transport flow force

The total axial fluid force on the spool, Eq. (6) of the main article, comprises a steady jet-momentum term and a transient fluid-column term. Writing the jet velocity as vjet = Q/Ac, with Ac the contracted (vena contracta) jet area, the steady term is Fsteady = ρQ2 cos θ / Ac. Perturbing about the operating point (Q0, θ0, Ac,0), with the jet angle and contraction area frozen at their operating values (their perturbations enter only at second order for small signals), gives

δFsteady = KFQ δQ,    KFQ = 2ρQ0 cos θ0 / Ac,0	(S2.1)

which is Eq. (8) of the main article. The transient term is already linear in the flow perturbation:

δFtrans = ρLe d(δQ)/dt	(S2.2)

The flow perturbation itself follows the small-signal metering relation, Eqs. (9)–(10):

δQ = Kq δxv + Kc δΔP,    Kq = ∂Q/∂xv,    Kc = ∂Q/∂ΔP	(S2.3)

Substituting Eq. (S2.3) into Eqs. (S2.1)–(S2.2) decomposes the flow force into four channels:

δFx = KFQKq δxv + ρLeKq δẋv + KFQKc δΔP + ρLeKc d(δΔP)/dt	(S2.4)

The four channels map onto the reduced-order coefficients as follows. The first term is proportional to displacement and is stiffness-like; in the identified model it is absorbed into the effective stiffness together with the dominant hydraulic stiffness Kh. The second term is proportional to spool velocity and is the direct flow-force contribution to the damping coefficient Kf. The third and fourth terms act through the pressure perturbation: when the chamber continuity equation and the line impedance, Eq. (17), are appended, δΔP itself contains components proportional to δẋv and δẍv, so these channels return additional velocity-proportional damping and, importantly, an acceleration-proportional contribution. The acceleration-proportional part is what the reduced-order model lumps as the equivalent transient inertia Mt.

Because the coefficients of these pressure-mediated channels depend on installed quantities (trapped chamber volumes, line inertance and compliance, and the operating-point metering gradients), Kf and Mt are identified experimentally as described in Section 4.3.3 rather than evaluated by substitution. In particular, the distributed transient-inertia coefficient ρLe = 12.9 kg/m2 multiplies dQ/dt and carries units of kg/m2; it is not numerically interchangeable with the lumped inertia Mt = 0.013 kg, which is defined in the spool coordinate after the projection above and identified from the measured frequency response.

# Supplementary Note S3. CFD campaign design, numerical settings, and grid-convergence methodology

## S3.1 Campaign design and solver settings

The production campaign comprises 112 converged steady-state cases spanning orifice openings h = 0.3–3.5 mm and Reynolds numbers Re = 100–5000, obtained by sweeping the pressure-inlet boundary condition for each opening. The domain is the sharp-edged rectangular orifice of Section 4.2.1 (L = 1.0 mm, d = 0.5 mm, w = 10 mm) with upstream and downstream chambers whose reference planes lie at least four hydraulic diameters from the orifice. All cases use the medium mesh (18,000 elements, y+ ≤ 1 inflation layers, fifteen divisions across the throat) selected by the grid-convergence study of Table 3 of the main article. The working fluid is ISO VG 46 oil (ρ = 860 kg/m3, μ = 0.03 Pa·s, isothermal at approximately 25 °C).

Turbulence closure is regime-adaptive: a laminar solution for Re < 500, the γ–Reθ transition model for 500 ≤ Re ≤ 2000, and the k–ε model for Re > 2000, so that each case is solved with the closure appropriate to its regime and no case is solved with overlapping or conflicting closures. Convergence is enforced with all scaled residuals below 10−5, mass-flow-rate stability below 0.1% over the final 500 iterations, and mass imbalance below 0.01%. The full campaign required approximately 48 hours on a 12-core workstation.

## S3.2 Richardson extrapolation and Grid Convergence Index

Grid convergence follows the ASME V&V 20 procedure. With f1, f2, and f3 the fine-, medium-, and coarse-mesh values of the observed quantity (here Cd or ΔP) and r the effective grid refinement ratio between successive meshes, the observed order of accuracy is

p = ln |(f3 − f2) / (f2 − f1)| / ln r	(S3.1)

the Richardson-extrapolated estimate is

fext = f1 + (f1 − f2) / (rp − 1)	(S3.2)

and the fine-grid convergence index, with safety factor 1.25, is

GCIfine = 1.25 |(f1 − f2) / f1| / (rp − 1) × 100%	(S3.3)

At the representative transitional condition Re = 1500, the discharge coefficient converges monotonically (0.598 → 0.619) under successive refinement, the observed order is p = 1.87, the fine-grid index is GCIfine = 0.82%, and the asymptotic-range check yields a convergence ratio of 0.94, confirming that the solutions lie in the asymptotic range. The resulting numerical uncertainty is Unum < 1%, and the mesh-level agreement across openings is documented in Table 3 of the main article.

## S3.3 Correlation fitting procedure

The 112 (Re, Cd) pairs were fitted to the hyperbolic-tangent form of Eq. (4) by nonlinear least squares (Python SciPy curve_fit, trust-region reflective algorithm) with parameter bounds C0 ∈ [0.3, 0.7], C1 ∈ [−0.3, 0.3], Rec ∈ [500, 3000], and Δ ∈ [0.5, 2.0]. The converged fit is Eq. (5) of the main article, with R2 = 0.978 and RMSE = 0.022, corresponding to a relative error of 3.5% with respect to the mean discharge coefficient. The tanh form was selected because it bridges the laminar and turbulent plateaus while remaining monotonic and continuously differentiable, properties required for its use inside the metering gradients of Eq. (10) and for real-time embedding.

# Supplementary Note S4. Measurement-uncertainty budget methodology (ISO GUM)

Uncertainty quantification follows the ISO Guide to the Expression of Uncertainty in Measurement. Type A (statistical) standard uncertainties are evaluated from N = 5 repeated measurements of each quantity,

uA = s / √N	(S4.1)

where s is the sample standard deviation; for spool displacement the repeat-test standard deviation is σdisp ≈ 0.08 mm, and the main article conservatively quotes this repeat-test scatter itself as the displacement uncertainty. Type B (systematic) standard uncertainties are propagated from instrument specifications assuming rectangular distributions with half-width a:

uB = a / √3	(S4.2)

For example, the WIKA PS-23 pressure transducer specification of ±0.5% of full scale on the 25 MPa range gives a = 0.125 MPa and uB ≈ 0.07 MPa, the value listed in Table 4 of the main article. Quantization of the 16-bit acquisition system is treated as a rectangular contribution of ±0.5 LSB over the channel span. The combined standard uncertainty and the expanded uncertainty are

uc = √(Σ ui2),    U = kuc,    k = 2	(S4.3)

with the coverage factor k = 2 corresponding to approximately 95% confidence. Instrument-level standard uncertainties, sampling rates, and the role of each channel in the validation are collected in Table 4 of the main article; all dynamic channels used for bandwidth identification were sampled at or above 1 kHz with hardware synchronization to minimize inter-channel phase drift.

# Supplementary tables

Supplementary Tables S1–S3 reproduce, in full, the content of Appendix Tables 5, 7, and 8 of the integrated manuscript version. Citation numbers in square brackets refer to the reference list of the main article.

Table S1. Dynamic model parameters and identified constants.

| Parameter group | Parameter | Symbol | Value | Unit | Identification method |
| --- | --- | --- | --- | --- | --- |
| Spool mechanics | Spool mass | ms | 0.045 | kg | CAD and scale |
| Spool mechanics | Effective spring stiffness | Ks | 1000 | N/m | Static deflection test |
| Spool mechanics | Mechanical / fluid viscous damping | Bs | 8 | N·s/m | Step-response log-decay |
| Spool mechanics | Pressure-sensitive area | As | 78.5 × 10−6 | m2 | NG10 geometry |
| Flow-force model | Fluid density | ρ | 860 | kg/m3 | ISO VG 46 datasheet |
| Flow-force model | Effective fluid-column length | Le | 0.015 | m | Geometry and transient pressure fit |
| Flow-force model | Jet deflection angle | θ | 18 | deg | CFD exit-plane velocity field |
| Flow-force model | Flow-force damping coefficient | Kf | 1.2 | N·s/m | Step-response fitting |
| Flow-force model | Distributed transient-inertia coefficient | ρLe | 12.9 | kg/m2 | Reynolds-transport closure |
| Flow-force model | Equivalent lumped transient inertia | Mt | 0.013 | kg | Frequency-response identification |
| Motor electromechanics | Armature resistance | Ra | 3.5 | Ω | DC measurement |
| Motor electromechanics | Armature inductance | La | 1.2 × 10−3 | H | AC impedance at 1 kHz |
| Motor electromechanics | Torque constant | Km | 0.086 | N·m/A | Bench calibration |
| Motor electromechanics | Back-EMF constant | Kb | 0.086 | V·s/rad | No-load speed test |
| Motor electromechanics | Rotor inertia | Jm | 2 × 10−6 | kg·m2 | CAD / coast-down |
| Motor electromechanics | Motor friction coefficient | Bm | 5 × 10−4 | N·m·s/rad | No-load rotation test |
| Motor electromechanics | Coupling radius | r | 0.005 | m | CAD measurement |
| Hydraulic line impedance | Line resistance | Rline | 0.5 | bar/(L/s) | Steady-flow pressure drop |
| Hydraulic line impedance | Line inertance | Lline | 2 × 107 | Pa·s2/m5 | Step-response oscillation frequency |
| Hydraulic line impedance | Line compliance | Cline | 0.5 × 10−9 | m3/Pa | Hose volume and elasticity estimate |
| Reduced-order effective | Effective moving mass (lumped ROM) | Meff | ms + Mt = 0.058 | kg | Lumped reduced-order model |
| Reduced-order effective | Effective damping | Beff | Bs + Kf = 9.2 | N·s/m | Linearized ROM |
| Reduced-order effective | Hydraulic stiffness | Kh | 3.5 × 107 | N/m | Line-impedance coupling / identified ROM |
| Reduced-order effective | Effective stiffness | Keff | Ks + Kh ≈ 3.5001 × 107 | N/m | Linearized ROM |
| Reduced-order effective | Natural frequency | fn | 950 | Hz | Closed-loop frequency response |
| Reduced-order effective | Closed-loop bandwidth | fBW | 650 | Hz | −3 dB point |
| Reduced-order effective | Damping ratio | ζ | 0.65–0.75 | – | Frequency-response fit |
| Reduced-order effective | Phase margin | PM | 55 | deg | Bode crossover |

Note. The distributed transient-fluid-inertia coefficient ρLe = 12.9 kg/m2 appears in the Reynolds-transport flow-force expression Fx,trans = ρLe dQ/dt. The equivalent lumped inertia Mt = 0.013 kg is obtained after projection of the distributed fluid inertia into the reduced-order spool coordinate and subsequent frequency-response identification. Accordingly, ρLe must not be interpreted as a lumped mass, and Mt must not be obtained by direct numerical substitution of ρLe into the second-order mechanical equation without coordinate projection.

Table S2. Model validation summary.

| Validation metric | Experimental result | Model prediction | Error / deviation | Acceptance criterion | Status |
| --- | --- | --- | --- | --- | --- |
| Cd(Re) correlation accuracy | 112 CFD / experimental comparison points | Eq. (5) fit | R2 = 0.978, RMSE = 0.022 | R2 > 0.95 | Pass |
| Mesh independence | Medium / fine mesh comparison | Medium mesh selected | < 0.4% deviation; GCI32 for Cd ≤ 0.02% | < 1% deviation | Pass |
| Static metering agreement | Measured Cd,exp | CFD-derived Cd(Re) | ≈ ±2% | ±5% | Pass |
| Current–displacement linearity | R2 = 0.998 | Linear small-signal assumption | < 1% deviation | R2 > 0.99 | Pass |
| Displacement uncertainty | ±0.08 mm | – | – | Consistent with sensor resolution | Pass |
| Rise time (10–90%) | 0.18 s | ≈ 0.18 s | < 5% | ≤ 10% | Pass |
| Settling time (±2%) | 0.45 s | ≈ 0.45 s | < 5% | ≤ 10% | Pass |
| Step-response overshoot | 2.1% | 2.1% | < 1 percentage point | ≤ 5 percentage points | Pass |
| Natural frequency | 950 Hz | 955 Hz | ≈ 0.5% | ≤ 5% | Pass |
| Closed-loop bandwidth | 650 Hz | 650 Hz | < 3.2% | ≤ 5% | Pass |
| Phase margin | 55° | 55° | < 2° | PM ≥ 45° | Pass |
| Magnitude error, 1–800 Hz | Measured Bode | Predicted Bode | < 5% | ≤ 5% | Pass |
| Phase error, up to 600 Hz | Measured Bode | Predicted Bode | < 10° | ≤ 10° | Pass |
| Robustness across pressure / load | 15–21 MPa, 1.5–2.5 kg | Same operating matrix | 96.8% of predictions within ±3.2% | ≥ 95% within ±5% | Pass |

Note. Acceptance criteria are selected to be consistent with high-bandwidth electrohydraulic validation practice. The phase-margin criterion is intentionally conservative relative to typical aerospace-inspired stability-margin expectations [56], without claiming certification compliance.

Table S3. Comparison with published electrohydraulic actuator models.

| Study | CFD used | Variable Cd(Re) | Transient inertia | Multi-scale coupling | Experimental validation | Maximum reported prediction error |
| --- | --- | --- | --- | --- | --- | --- |
| Merritt [32] | No | No | No | No | Yes | Not reported; constant-Cd assumption |
| Blackburn et al. [33] | No | No | No | No | Yes | Not reported; quasi-steady model |
| Ashoori et al. [9] | Partial | Yes | No | No | Yes (correlation level) | RMSE ≈ 0.022 for Cd fit |
| Lisowski and Rajda [37] | Yes | Field-resolved | No | No | Partial | Not reported at system level |
| Vacca et al. [39] | Yes | Partial | Partial | No | Partial | Not reported at closed-loop level |
| Rossi and Vacca [43] | Yes | Partial | Partial | Yes | Yes | Not reported in unified bandwidth / phase-margin form |
| Sitte et al. [46] | No | No explicit physics closure | No | Partial | Yes | Data-driven; extrapolation not fully quantified |
| This work | Yes | Yes; closed-form Cd(Re) | Yes | Yes; micro–meso–macro | Laboratory, HIL, operating-condition emulation | < 3.2% bandwidth error; < 10° phase error up to 600 Hz |

Note. The comparison emphasizes modeling capability rather than hardware certification. The present work is validated on a representative laboratory-scale direct-drive electrohydraulic actuator whose operating physics overlap with aerospace-relevant servo-valve conditions, particularly the transitional Reynolds-number regime.

# Edits required in the main text when this file is used in place of the appendices

The integrated manuscript version carries Appendices A and B. If the target journal requires this separate Supplementary Material file instead, the following edits apply to the main text.

- Delete Appendix A and Appendix B (including Tables 5, 7, and 8 and their notes).

- Section 4.4.4: replace the pointer to Appendix A (Table 5) with a citation of Supplementary Table S1.

- Section 6.7: replace the pointer to Appendix B (Tables 7 and 8) with citations of Supplementary Tables S2 and S3.

- Renumber main-text Table 6 as Table 5, updating its citations in Section 6.5 and in the captions of Figs. 8 and 9 so that main-text table numbering runs 1–5 without a gap.

- Add a Supplementary Material statement before the reference list, for example: Supplementary material associated with this article, comprising Supplementary Notes S1–S4 and Supplementary Tables S1–S3, is available in the online version.

The CFD dataset underlying Supplementary Note S3 (112 converged cases and the fitted correlation coefficients) is available from the corresponding author on reasonable request, consistent with the Data availability statement of the main article.

Appendix A. Dynamic model parameters and identified constants

Table 5. Dynamic model parameters and identified constants.

| Parameter group | Parameter | Symbol | Value | Unit | Identification method |
| --- | --- | --- | --- | --- | --- |
| Spool mechanics | Spool mass | ms | 0.045 | kg | CAD and scale |
| Spool mechanics | Effective spring stiffness | Ks | 1000 | N/m | Static deflection test |
| Spool mechanics | Mechanical / fluid viscous damping | Bs | 8 | N·s/m | Step-response log-decay |
| Spool mechanics | Pressure-sensitive area | As | 78.5 × 10−6 | m2 | NG10 geometry |
| Flow-force model | Fluid density | ρ | 860 | kg/m3 | ISO VG 46 datasheet |
| Flow-force model | Effective fluid-column length | Le | 0.015 | m | Geometry and transient pressure fit |
| Flow-force model | Jet deflection angle | θ | 18 | deg | CFD exit-plane velocity field |
| Flow-force model | Flow-force damping coefficient | Kf | 1.2 | N·s/m | Step-response fitting |
| Flow-force model | Distributed transient-inertia coefficient | ρLe | 12.9 | kg/m2 | Reynolds-transport closure |
| Flow-force model | Equivalent lumped transient inertia | Mt | 0.013 | kg | Frequency-response identification |
| Motor electromechanics | Armature resistance | Ra | 3.5 | Ω | DC measurement |
| Motor electromechanics | Armature inductance | La | 1.2 × 10−3 | H | AC impedance at 1 kHz |
| Motor electromechanics | Torque constant | Km | 0.086 | N·m/A | Bench calibration |
| Motor electromechanics | Back-EMF constant | Kb | 0.086 | V·s/rad | No-load speed test |
| Motor electromechanics | Rotor inertia | Jm | 2 × 10−6 | kg·m2 | CAD / coast-down |
| Motor electromechanics | Motor friction coefficient | Bm | 5 × 10−4 | N·m·s/rad | No-load rotation test |
| Motor electromechanics | Coupling radius | r | 0.005 | m | CAD measurement |
| Hydraulic line impedance | Line resistance | Rline | 0.5 | bar/(L/s) | Steady-flow pressure drop |
| Hydraulic line impedance | Line inertance | Lline | 2 × 107 | Pa·s2/m5 | Step-response oscillation frequency |
| Hydraulic line impedance | Line compliance | Cline | 0.5 × 10−9 | m3/Pa | Hose volume and elasticity estimate |
| Reduced-order effective | Effective moving mass (lumped ROM) | Meff | ms + Mt = 0.058 | kg | Lumped reduced-order model |
| Reduced-order effective | Effective damping | Beff | Bs + Kf = 9.2 | N·s/m | Linearized ROM |
| Reduced-order effective | Hydraulic stiffness | Kh | 3.5 × 107 | N/m | Line-impedance coupling / identified ROM |
| Reduced-order effective | Effective stiffness | Keff | Ks + Kh ≈ 3.5001 × 107 | N/m | Linearized ROM |
| Reduced-order effective | Natural frequency | fn | 950 | Hz | Closed-loop frequency response |
| Reduced-order effective | Closed-loop bandwidth | fBW | 650 | Hz | −3 dB point |
| Reduced-order effective | Damping ratio | ζ | 0.65–0.75 | – | Frequency-response fit |
| Reduced-order effective | Phase margin | PM | 55 | deg | Bode crossover |

Note. The distributed transient-fluid-inertia coefficient ρLe = 12.9 kg/m2 appears in the Reynolds-transport flow-force expression Fx,trans = ρLe dQ/dt. The equivalent lumped inertia Mt = 0.013 kg is obtained after projection of the distributed fluid inertia into the reduced-order spool coordinate and subsequent frequency-response identification. Accordingly, ρLe must not be interpreted as a lumped mass, and Mt must not be obtained by direct numerical substitution of ρLe into the second-order mechanical equation without coordinate projection.

Appendix B. Model validation summary and comparison with published models

Table 7. Model validation summary.

| Validation metric | Experimental result | Model prediction | Error / deviation | Acceptance criterion | Status |
| --- | --- | --- | --- | --- | --- |
| Cd(Re) correlation accuracy | 112 CFD / experimental comparison points | Eq. (5) fit | R2 = 0.978, RMSE = 0.022 | R2 > 0.95 | Pass |
| Mesh independence | Medium / fine mesh comparison | Medium mesh selected | < 0.4% deviation; GCI32 for Cd ≤ 0.02% | < 1% deviation | Pass |
| Static metering agreement | Measured Cd,exp | CFD-derived Cd(Re) | ≈ ±2% | ±5% | Pass |
| Current–displacement linearity | R2 = 0.998 | Linear small-signal assumption | < 1% deviation | R2 > 0.99 | Pass |
| Displacement uncertainty | ±0.08 mm | – | – | Consistent with sensor resolution | Pass |
| Rise time (10–90%) | 0.18 s | ≈ 0.18 s | < 5% | ≤ 10% | Pass |
| Settling time (±2%) | 0.45 s | ≈ 0.45 s | < 5% | ≤ 10% | Pass |
| Step-response overshoot | 2.1% | 2.1% | < 1 percentage point | ≤ 5 percentage points | Pass |
| Natural frequency | 950 Hz | 955 Hz | ≈ 0.5% | ≤ 5% | Pass |
| Closed-loop bandwidth | 650 Hz | 650 Hz | < 3.2% | ≤ 5% | Pass |
| Phase margin | 55° | 55° | < 2° | PM ≥ 45° | Pass |
| Magnitude error, 1–800 Hz | Measured Bode | Predicted Bode | < 5% | ≤ 5% | Pass |
| Phase error, up to 600 Hz | Measured Bode | Predicted Bode | < 10° | ≤ 10° | Pass |
| Robustness across pressure / load | 15–21 MPa, 1.5–2.5 kg | Same operating matrix | 96.8% of predictions within ±3.2% | ≥ 95% within ±5% | Pass |

Note. Acceptance criteria are selected to be consistent with high-bandwidth electrohydraulic validation practice. The phase-margin criterion is intentionally conservative relative to typical aerospace-inspired stability-margin expectations [56], without claiming certification compliance.

Table 8. Comparison with published electrohydraulic actuator models.

| Study | CFD used | Variable Cd(Re) | Transient inertia | Multi-scale coupling | Experimental validation | Maximum reported prediction error |
| --- | --- | --- | --- | --- | --- | --- |
| Merritt [32] | No | No | No | No | Yes | Not reported; constant-Cd assumption |
| Blackburn et al. [33] | No | No | No | No | Yes | Not reported; quasi-steady model |
| Ashoori et al. [9] | Partial | Yes | No | No | Yes (correlation level) | RMSE ≈ 0.022 for Cd fit |
| Lisowski and Rajda [37] | Yes | Field-resolved | No | No | Partial | Not reported at system level |
| Vacca et al. [39] | Yes | Partial | Partial | No | Partial | Not reported at closed-loop level |
| Rossi and Vacca [43] | Yes | Partial | Partial | Yes | Yes | Not reported in unified bandwidth / phase-margin form |
| Sitte et al. [46] | No | No explicit physics closure | No | Partial | Yes | Data-driven; extrapolation not fully quantified |
| This work | Yes | Yes; closed-form Cd(Re) | Yes | Yes; micro–meso–macro | Laboratory, HIL, operating-condition emulation | < 3.2% bandwidth error; < 10° phase error up to 600 Hz |

Note. The comparison emphasizes modeling capability rather than hardware certification. The present work is validated on a representative laboratory-scale direct-drive electrohydraulic actuator whose operating physics overlap with aerospace-relevant servo-valve conditions, particularly the transitional Reynolds-number regime.