Supplementary Information

Beyond water quality: Integrating antimicrobial resistance into urban river water health assessment for one health governance

Arun Kashyap¹, Vikas Sonkar¹, Shashidhar Thatikonda¹*

¹One Health Research Laboratory, Environmental Engineering Division, Department of Civil Engineering, IIT Hyderabad, Kandi, Sangareddy, Telangana 502284, India

*Corresponding author: Prof. Shashidhar Thatikonda, Email: shashidhar@ce.iith.ac.in

*Corresponding author’s address: B-319, Department of Civil Engineering, IIT Hyderabad, Kandi, Sangareddy, 502284, Telangana, India. Phone: +91 040 2301 6305

List of Contents

Supplementary Information

S1 on the membership functions and on Water Quality Criteria based on Central Pollution Control Board (CPCB);

S2 on the fuzzy rule based construction;

SA for Stage 1 Module;

SB for Stage 2 Module;

SC for Stage 3 Module;

SD on ABI Reference-Site Robustness.

Supplementary Figures

S1: Frequency distributions of local gradient magnitudes for (a) WQI and (b) ABI obtained from global finite-difference sensitivity analysis over the full WQI-ABI space;

S2: Distribution of CWHI values obtained from Monte Carlo uncertainty propagation analysis;

Supplementary Tables

S1: Fusion tables used in the hierarchical WQI framework and the corresponding WQP pairings;

S2: AMR Fuzzy Input Variable MF Ranges;

S3: Fusion tables used in the ABI fuzzy inference framework and their interpretation;

S4: Representative fuzzy IF-THEN rules used in ARG-level ABI inference;

S5: PNEC values used for measured-antibiotic pressure estimation;

S6: Representative ARG metadata and tier assignment;

S7: PEC values used for fallback antibiotic-pressure estimation when measured antibiotic concentrations are unavailable;

S8: Fuzzy fusion matrix used for the ideal AND CWHI obtained from each combination of WQI and ABI state;

S9: Representative fuzzy IF-THEN rules used in the CWHI fusion layer;

S10: Idealized 5×5 fusion-space validation matrix for the CWHI framework, showing the 25 combinations of WQI and ABI class-center inputs and the corresponding centroid-defuzzified CWHI scores and output classes;

S11: Single point one-at-a-time (OAT) perturbation for CWHI framework;

S12: Global grid Sensitivity Analysis for the WQI and ABI;

S13: Seasonal WQI class transition matrix for Musi River sampling sites, showing wet-season and dry-season WQI classes and indicating whether class changes occurred between seasons;

S14: Spearman rank correlations between ARG and WQPs used in the Musi River dataset;

S15: Reference-site sensitivity analysis of ABI across four baseline scenarios for Musi River samples, showing scenario-specific ABI values, maximum spread, and coefficient of variation for each site–season combination.

Supplementary information 1 on membership function

For WQPs variables, the bounds of the universe of discourse and the membership breakpoints were defined using ecologically relevant ranges and regulatory thresholds (Table 1). For AMR-related variables, individual modules were defined for AMR burden. This structure allowed different types of evidence to be harmonized within the same fuzzy logic framework. Trapezoidal membership functions were assigned to the boundary linguistic terms VP and VG, as these represent open-ended extreme conditions with a plateau of full membership at the extreme end of the parameter range (equation S1).

μtrapx:a,b,c,d=maxmin0, x-ab-a , 1, d-xd-c          S1

Where x is the crisp input variable: a and d are the lower and upper limit beyond MF is zero; b and c define the core region of full MF; μtrap∈ [0,1] represents the degree to which x belongs to the fuzzy set.

Triangular membership functions were assigned to the interior terms P, M, and G, as these represent transient quality states with a single peak and symmetric transition slopes (equation S2):

μtrix:a,b,c=maxminx-ab-a, 0 , c-xc-b                (S2)

Where a and c are the left (lower limit) and right (upper limit) side of the triangle beyond which MF is zero; b is the apex of the triangle at which full MF (µ = 1) is achieved.

MF architecture is well established for fuzzy interface in environmental systems 1. The trapezoidal form for extreme terms ensures that parameters clearly within excellent or critically poor ranges receive full membership without ambiguity, while triangular forms for intermediate terms maximize sensitivity to deterioration, particularly important in the polluted urban river settings.

Water Quality Criteria - CPCB

The Central Pollution Control Board (CPCB) has developed a concept of designated best use by classifying water resources of the country according to their uses for setting water quality objectives for different water bodies. According to this, out of the several uses of water of a particular body, the use which demands highest quality is termed as its designated best use. Five designated best uses have been identified. This classification helps the water quality managers and planners to set water quality targets and design suitable restoration programs for various water bodies 2.

| Table SA. WATER QUALITY CRITERIA & DESIGNATED BEST USES |  |  |
| --- | --- | --- |
| Designated Best Use | Class | Criteria |
| Drinking Water Source without conventional treatment but after disinfection | A | 1.Total Coliforms Organism, MPN/100ml shall be 50 or less 2. pH between 6.5 and 8.5 3. Dissolved Oxygen, 6mg/l or more 4. Biochemical Oxygen Demand 5 days 20 °C, 2mg/l or less |
| Outdoor bathing (Organized) | B | 1.Total Coliforms Organism MPN/100ml shall be 500 or less 2. pH between 6.5 and 8.5 3. Dissolved Oxygen 5mg/l or more 4. Biochemical Oxygen Demand 5 days 20 °C, 3mg/l or less |
| Drinking water source after conventional treatment and disinfection | C | 1. Total Coliforms Organism MPN/100ml shall be 5000 or less 2. pH between 6 and 9 3. Dissolved Oxygen 4mg/l or more 4. Biochemical Oxygen Demand 5 days 20 °C, 3mg/l or less |
| Propagation of Wildlife and Fisheries | D | 1. pH between 6.5 and 8.5 2. Dissolved Oxygen 4mg/l or more 3. Free Ammonia (as N) 4. Biochemical Oxygen Demand 5 days 20 °C, 2mg/l or less |
| Irrigation, Industrial Cooling, Controlled Waste disposal | E | 1. pH between 6.0 and 8.5 2. Electrical Conductivity at 25 °C micro mhos/cm, maximum 2250 3. Sodium absorption Ratio Max. 26 4. Boron Max. 2mg/l |
|  | Below-E | Not meeting any of the A, B, C, D & E criteria |

Supplementary information 2 on the fuzzy rule based construction

Compound antecedents were evaluated using the min-operator as “AND” in the developed MFIS equation (S3):

μA∩Bx=minμAx, μBx                 (S3)

Where, μA(x)  and μB(x) are the individual membership degrees of x in fuzzy sets A and B, respectively; and μA∩B(x)∈[0,1] represents the aggregated membership degree of x satisfying both conditions simultaneously. This formulation is environmentally suitable option for WQ assessment, where the most limiting parameter is comparable to Liebig's Law of the Minimum 3. Each rule’s activation strength is constrained by its weakest antecedent. Therefore, even if the other parameter shows less impairment, a significant decline in any one parameter is enough to move the output toward a lower quality class.

Supplementary information A for Stage 1 Module

A.1 Rule based construction: Each rule is composed of two primary components: the antecedent (the “IF” part) and the consequent (the “THEN” part). For any two-input WQP subsystem, the rule base can be written as:

IF A is ai AND B is bj THEN Grk is cij

where A and B are the input WQPs, ai and bj are their associated linguistic terms, Grk is the intermediate fuzzy output for the k-th group, and cij is the output linguistic class determined by the selected fusion table. For higher-level aggregation of intermediate group scores, the same structure was retained using quality states as inputs:

IF Grm is qr AND Grn is qs THEN Grt is qrs

and for the three-input stage,

IF Gr13 is qr AND Gr14 is qs AND Gr15 is qu THEN Gr17 is qrsu

where q denotes the WQ linguistic classes VP, P, M, G, and E.

A.2 Hierarchical organization: The script organizes the WQI into ten first-level WQP groups, followed by successive second- and third-level aggregations (Figure 1). ​ The first layer combines pH-Temp into Gr1, DO-COD into Gr2, , TDS-Conductivity into Gr3, Turbidity-TSS into Gr4, Nitrate-Nitrite into Gr5, Ammonium-TN into Gr6, TP-TOC into Gr7, TBX-OSS into Gr8, Cu-Ni into Gr9, and Pb-Cd into Gr10. ​ These are subsequently aggregated as Gr11 = f(Gr1, Gr2), Gr12 = f(Gr3, Gr4), Gr13 = f(Gr5, Gr6), Gr14 = f(Gr7, Gr8), Gr15 = f(Gr9, Gr10), Gr16 = f(Gr11, Gr12), Gr17 = f(Gr13, Gr14, Gr15), and finally WQI = f(Gr16, Gr17).

This hierarchical design 4 reduces the complexity of directly combining all raw inputs in a single rule base and preserves ecological interpretation at each stage of integration. ​ The script explicitly implements two-input rule generators for pollutant-to-quality inference, two-input quality-to-quality inference, and three-input quality fusion, resulting in 25 rules for each two-input block and 125 rules for the three-input block. ​ The script also reports total rule counts of 250 rules in level 1, 125 rules in level 2, and 25 rules in the final level.

A.3 Fusion table: Different WQP pairs were assigned to different fusion tables (Supplementary Table S1) so that the rule behavior matched the environmental meaning of each parameter combination. ​ The rule matrices implemented for the fusion tables are the oxygen table, the pH-temperature table, the pollution-dominating table, the microbial table, the heavy-metal-high table, and the conservative table.

A.4 Example: The pollution-dominating table is intentionally asymmetric in environmental effect, because the poorer of the two inputs largely controls the resulting quality status. By contrast, the conservative table is used when merging already-derived quality scores so that improvement in one branch does not unrealistically compensate for poor status in another branch. The three-input rule matrix for Gr17 was additionally constructed by combining the conservative integration of Gr13 and Gr14 with a pollution-oriented penalty derived from Gr15, thereby allowing heavy-metal burden to degrade the nutrient-organic-microbial branch during upper-level fusion. Representative rules from the implemented WQI rule base are shown below with examples directly reflecting the fusion matrices-

IF DO is VL and COD is VL, THEN Gr1 is VP,

IF pH is M, Temperature is M, THEN Gr2 is P,

IF Turbidity is VL AND TSS is VL, THEN Gr4 is G,

IF TBX is VL AND OSS is L, THEN Gr8 is G,

IF Pb is M AND Cd is L, THEN Gr10 is VP.

The same rule logic was propagated to upper hierarchical layers by treating intermediate scores as fuzzy quality inputs.  For example, under the conservative table, IF Gr1 is G AND Gr2 is M, THEN Gr11 is M, whereas IF Gr1 is E AND Gr2 is E, THEN Gr11 is E.  Similarly, Gr15 is formed by combining Gr9 and Gr10 with pollution-dominating logic, and Gr17 is obtained through a three-input rule base integrating Gr13, Gr14, and Gr15.

The hierarchical arrangement allows the model to incorporate contrasting behaviors across WQPs while avoiding an unmanageably large flat rule base. These rule bases enabled the framework to encode parameter-specific behavior while maintaining a transparent and reproducible inference structure. The use of standardized rule-table schemas also reduced subjectivity in rule construction and supported consistent propagation of risk information across the three stages of the integrated model.

The intermediate outputs from each level were then fused through higher-order subsystems until a final WQI score was generated. The conservative aggregation strategy ensures that the WQI penalizes multi-parameter co-contamination, a scenario ubiquitous in wastewater-impacted urban rivers 5. This approach diverges from compensatory arithmetic aggregation (e.g., weighted sum WQI) where a single excellent parameter can numerically offset multiple poor parameters, producing misleadingly favorable index scores 6.

| Supplementary Table S1. Fusion tables used in the hierarchical WQI framework and the corresponding WQP pairings. |  |  |
| --- | --- | --- |
| Fusion table | Functional interpretation | WQP pairs or hierarchy level |
| Oxygen | Prioritizes oxygen depletion; very low DO forces very poor quality regardless of COD. | DO + COD = Gr1. |
| pH-temperature | Bell-shaped response; near-neutral pH and moderate temperature give the best quality, while extremes degrade the score. | pH + Temp = Gr2. |
| Pollution-dominating | The more polluted input dominates the output, causing rapid degradation as either variable shifts toward H or VH. | TDS + Conductivity = Gr3; Turbidity + TSS = Gr4; Nitrate + Nitrite = Gr5; Ammonium + TN = Gr6; TP + TOC = Gr7; Cu + Ni = Gr9. |
| Microbial | Apply stricter penalties because microbial contamination is treated as an immediate health risk. | TBX + OSS = Gr8. |
| Heavy metal | Strongly precautionary matrix in which moderate to high Pb or Cd produces poor to very poor quality. | Pb + Cd = Gr10 |
| Conservative | Monotone quality-preserving aggregation is used for higher-level merging of intermediate quality groups. | Gr1 + Gr2 = Gr11; Gr3 + Gr4 = Gr12; Gr5 + Gr6 = Gr13; Gr7 + Gr8 = Gr14; Gr11 + Gr12 = Gr16; Gr16 + Gr17 = WQI. |

| Supplementary Table S2. AMR Fuzzy Input Variable MF Ranges |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Input Variable | Universe | VL | L | M | H | VH |
| Enrichment Ratio (EF) | 0–1000 | 0–1 | 1–3 | 3–10 | 10–100 | 100–1000 |
| Mobility Score (Mob) | 0–100 | 0–10 | 8–30 | 25–60 | 50–85 | 75–100 |
| WHO-informed Rank (WIR) | 0–100 | 0–30 | 25–50 | 45–65 | 60–80 | 75–100 |
| Antibiotic Pressure (AbxPress) | 0–100 | 0–15 | 10–30 | 25–50 | 45–75 | 70–100 |
| ABI Output | 0–100 | 0–20 | 18–40 | 38–60 | 58–80 | 80–100 |

Supplementary information B for Stage 2 Module

B.1 Rationale for selecting the target genes and other factors for AMR Burden: The antimicrobial resistance genes (ARGs) incorporated in the AMR Burden Index (ABI) were selected to represent major resistance classes frequently associated with wastewater-impacted aquatic environments and urban anthropogenic pollution 7,8. Selection was based on three primary considerations: (i) reported occurrence in sewage-contaminated river systems; (ii) clinical relevance of the associated resistance class; and (iii) suitability for quantitative molecular detection using qPCR-based approaches.

The β-lactam resistance genes blaNDM and blaCTX-M were included due to their association with carbapenem and extended-spectrum cephalosporin resistance, respectively. These genes are widely recognized indicators of clinically important antimicrobial resistance dissemination in wastewater-impacted environments and have been repeatedly detected in urban river systems receiving municipal sewage discharge 9,10.

The genes sul1, sul2, qnrS, and ermF were selected to represent sulfonamide, fluoroquinolone, and macrolide resistance classes commonly linked to anthropogenic wastewater contamination 11. These genes are frequently associated with mobile genetic elements, including class 1 integrons and conjugative plasmids, which facilitate environmental dissemination through horizontal gene transfer 12. Their occurrence in urban aquatic systems has been extensively reported in rivers receiving domestic sewage, pharmaceutical discharge, and mixed urban runoff.

Tetracycline resistance genes (tetA/B, tetM, tetQ, and tetW) were incorporated because tetracycline-associated resistance determinants are among the most persistent and frequently detected ARGs in environmental resistome studies. Their continued occurrence in riverine systems reflects both historical and ongoing antibiotic selection pressure from human and veterinary usage 13.

The aminoglycoside resistance marker aph3-ib was included as an indicator associated with hospital wastewater and clinically influenced anthropogenic contamination 14 . Aminoglycoside resistance determinants are commonly detected in wastewater systems influenced by healthcare-associated discharge and therefore provide additional information regarding clinically derived resistance dissemination.

The class 1 integron integrase gene intI1 was incorporated as a mobility-associated marker rather than as a resistance determinant itself. intI1 abundance has been consistently associated with anthropogenic pollution intensity, ARG accumulation, and the environmental potential for horizontal gene transfer 11,12. Accordingly, intI1 was integrated into the mobility component of the ABI framework to account for the ecological potential for resistance dissemination in addition to ARG abundance alone.

Antibiotic selection pressure was incorporated using the predicted no-effect concentration for resistance selection (PNECres) framework 15,16. The PNEC approach provides environmentally relevant threshold concentrations below which resistance selection pressure is considered unlikely to occur. Measured antibiotics and fallback predicted environmental concentration (PEC)-based estimates were used to represent antibiotic pressure associated with the corresponding ARG classes. Fluoroquinolones were considered particularly important because of their low resistance-selection thresholds and their frequent occurrence in wastewater-impacted aquatic environments. In the present framework, ciprofloxacin was assigned to a PNEC value of 0.003 µg/L, reflecting its high ecological relevance for resistance selection under environmentally realistic concentrations. Macrolides were additionally incorporated using reported PNEC values ranging from 0.014–0.049 µg/L due to their potential role in co-selection of ermF-associated resistance determinants. Trimethoprim was retained as the primary sulfonamide-associated pressure indicator following exclusion of sulfamethoxazole because concentrations remained below detection limits in all analyzed samples. Trimethoprim was assigned a PNEC value of 0.042 µg/L and was incorporated because of its reported association with co-selection mechanisms involving mobile resistance determinants such as sul1 and sul2. Altogether, the selected ARGs, mobility markers, and antibiotic pressure variables were intended to represent clinically relevant resistance dissemination, environmental persistence, wastewater influence, and ecological selection pressure within sewage-impacted urban river systems.

B.2 Rule based construction: For each ARG, the ABI module builds two parallel fuzzy subsystems: an environmental load subsystem and a clinical risk subsystem.

The environmental load component combines enrichment factor (EF) and mobility score (Mob), whereas the clinical risk component combines ARG priority score based on metadata-derived ranking scores adapted from environmental AMR risk frameworks 9,17 and antibiotic selection pressure (Abx) 15,16. Accordingly, the generic rule structures may be written as:

IF EF is ai AND Mob is bj THEN EnvLoad is cij

and

IF WHO is di AND Abx is ej THEN ClinRisk is fij
Where ai, bj, di, and ej are fuzzy input labels in the VL–VH scale and cij, fij are fuzzy ABI outputs in the same VL–VH vocabulary.

The framework defines ABI output membership functions on a 0–100 universe using VL, L, M, H, and VH risk states, with the same trapezoidal-triangular pattern used in the WQI output layer.

B.3 Fusion table: The environmental load branch uses the AMR ENV TABLE, which is asymmetric and allows high enrichment alone to elevate risk even when mobility is only moderate.

The clinical-risk branch uses one of two rule tables depending on ARG priority: AMR CLIN CRITICAL TABLE is used for ARGs with WHO-informed rank (WIR) at least 70, while AMR CLIN SELECTION TABLE is used for lower-priority selection-tier ARGs 7–9.

| Supplementary Table S3. Fusion tables used in the ABI fuzzy inference framework and their interpretation. |  |  |
| --- | --- | --- |
| Fusion table | Inputs | Function in ABI |
| AMR ENV | EF + Mob | Converts environmental abundance and mobility potential into ARG-specific environmental load. |
| AMR CLIN CRITICAL | WIR + AbxPress | Used for critical ARGs with WIR greater than or equal to 70; gives strong penalties when either priority or selection pressure is high. |
| AMR CLIN SELECTION | WIR + AbxPress | Used for non-critical ARGs; allows a more gradual escalation of clinical risk. |

| Supplementary Table S4. Representative fuzzy IF-THEN rules used in ARG-level ABI inference. |  |  |
| --- | --- | --- |
| Branch | Example rule | Interpretation |
| Environmental load | IF EF is VL AND Mob is VL, THEN EnvLoad is VL. | Low ARG enrichment with low mobility indicates negligible environmental burden. |
| Environmental load | IF EF is M AND Mob is VH, THEN EnvLoad is VH. | Moderate enrichment combined with very high mobility is treated as severe dispersal risk. |
| Environmental load | IF EF is VH AND Mob is L, THEN EnvLoad is H. | Very high enrichment alone is sufficient to produce high environmental risk. |
| Clinical risk, critical table | IF WIR is VH AND AbxPress is VL, THEN ClinRisk is VH. | A very high-priority ARG remains critical even under low measured antibiotic pressure. |
| Clinical risk, critical table | IF WIR is M AND AbxPress is H, THEN ClinRisk is VH. | Strong antibiotic selection can elevate clinical risk for a moderately ranked ARG within the critical table. |
| Clinical risk, selection table | IF WIR is VL AND AbxPress is VL, THEN ClinRisk is VL. | Low-priority ARGs under negligible selection pressure contribute minimally to ABI. |
| Clinical risk, selection table | IF WIR is VH AND AbxPress is VH, THEN ClinRisk is VH. | Combined high priority and high selection pressure produce the maximum clinical-risk state. |

| Supplementary Table S5. PNEC values used for measured-antibiotic pressure estimation. |  |
| --- | --- |
| Antibiotic | PNEC |
| Ciprofloxacin (CIP) | 0.0006 |
| Norfloxacin (NOR) | 0.005 |
| Erythromycin (ERY) | 0.003 |
| Azithromycin (AZM) | 0.008 |
| Trimethoprim (TMP) | 0.009 |
| Meropenem (MEM) | 0.0006 |
| Imipenem (IPM) | 0.0008 |
| Cefotaxime (CTX) | 0.0008 |
| Ceftriaxone (CRO) | 0.0017 |
| Tetracycline (TC) | 0.011 |
| Doxycycline (DOX) | 0.005 |
| Oxytetracycline (OTC) | 0.019 |
| Gentamicin (GEN) | 0.011 |
| Amikacin (AMK) | 0.074 |

| Supplementary Table S6. Representative ARG metadata and tier assignment. |  |  |  |  |
| --- | --- | --- | --- | --- |
| ARG | WIR | Antibiotic class | Measured antibiotic(s) | Pressure tier |
| blaNDM | 95 | Carbapenem | None | 2 |
| blaCTX-M | 75 | Cephalosporin | None | 2 |
| qnrS | 70 | Fluoroquinolone | CIP, NOR | 1 |
| ermF | 62 | Macrolide | ERY, AZM | 1 |
| tetM | 52 | Tetracycline | None | 2 |
| tetW | 47 | Tetracycline | None | 2 |
| sul1 | 42 | Sulfonamide | TMP | 1 |
| sul2 | 38 | Sulfonamide | TMP | 1 |
| tetA/B | 35 | Tetracycline | None | 2 |
| tetQ | 33 | Tetracycline | None | 2 |
| aph3ib | 36 | Aminoglycoside | None | 2 |

| Supplementary Table S7. PEC values used for fallback antibiotic-pressure estimation when measured antibiotic concentrations are unavailable. |  |  |  |
| --- | --- | --- | --- |
| Antibiotic class | Dry PEC | Wet PEC | Fallback PNEC |
| Carbapenem (CARB) | 0.168 | 0.017 | 0.0006 |
| Cephalosporin (CEP) | 0.600 | 0.060 | 0.0008 |
| Fluoroquinolone (FQ) | 3.500 | 0.350 | 0.0006 |
| Macrolide (MAC) | 0.600 | 0.060 | 0.003 |
| Sulfonamide (SXT) | 3.200 | 0.320 | 0.009 |
| Tetracycline (TC) | 2.750 | 0.275 | 0.005 |
| Aminoglycoside (AMG) | 0.560 | 0.056 | 0.011 |

Supplementary information C for Stage 3 Module

C.1 Rule based construction: The general fuzzy fusion rule used for CWHI can be written as follows:

IF WQI is wi AND ABI is aj THEN CWHI is cij
where wi∈{VP,P,M,G,E}, aj∈{VL,L,M,H,VH},

and cij∈{Critical,Poor,Moderate,Good,Safe}.
In the script, the final crisp score is obtained by fuzzifying the two inputs, applying the 25 fusion rules, aggregating the activated output sets, and defuzzifying the result by the centroid method. Thus, for a given sample k, the CWHI can be represented functionally as

CWHIk=FCWHI(WQIk, ABIk)
Where FCWHI denotes the fuzzy fusion mapping encoded by the 5 × 5 fusion table.
The corresponding implementation in the script clips both WQI and ABI to the range 0–100 before inference and returns the centroid-defuzzified CWHI score.

C.2 Fusion table: In this framework, WQI acts as a positive environmental-quality indicator, where higher values indicate better water quality, whereas ABI acts as a negative microbial-risk indicator, where higher values indicate greater antimicrobial resistance burden. The fusion layer therefore combines physicochemical water condition and AMR burden into a single decision surface using a Mamdani-type fuzzy inference system. The resulting output classes are Critical, Poor, Moderate, Good, and Safe, with output membership functions defined over 0–100 and class ranges of 0–20, 20–40, 40–60, 60–80, and 80–100, respectively.

| Supplementary Table S8. Fuzzy fusion matrix used for the ideal AND CWHI obtained from each combination of WQI and ABI state. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WQI \ ABI | VL | L | M | H | VH |
| VP | Critical | Critical | Critical | Critical | Critical |
| P | Poor | Poor | Critical | Critical | Critical |
| M | Moderate | Moderate | Poor | Critical | Critical |
| G | Good | Moderate | Poor | Poor | Critical |
| E | Safe | Good | Moderate | Poor | Critical |

C.3 Example: Representative fuzzy IF-THEN rules from the implemented CWHI fusion matrix are shown below. These rules illustrate how the fusion logic preserves the beneficial effect of high WQI only when ABI remains low, while penalizing combinations involving poor water quality or high AMR burden.

| Supplementary Table S9. Representative fuzzy IF-THEN rules used in the CWHI fusion layer. |  |
| --- | --- |
| Rule | Interpretation |
| IF WQI is VP AND ABI is VL, THEN CWHI is Critical. | Very poor physicochemical water quality is sufficient to produce a critical overall water-health score even when AMR burden is low. |
| IF WQI is P AND ABI is M, THEN CWHI is Critical. | Poor water quality combined with moderate AMR burden is treated as critically unsafe. |
| IF WQI is M AND ABI is VL, THEN CWHI is Moderate. | Moderate water quality with negligible AMR burden yields an intermediate health state. |
| IF WQI is G AND ABI is L, THEN CWHI is Moderate. | A low but non-negligible AMR burden downgrades otherwise good water quality. |
| IF WQI is E AND ABI is VL, THEN CWHI is Safe. | Excellent water quality and negligible AMR burden together define the safest class. |
| IF WQI is E AND ABI is H, THEN CWHI is Poor. | High AMR burden substantially reduces the composite water-health classification even under excellent water quality. |
| IF WQI is E AND ABI is VH, THEN CWHI is Critical. | Very high AMR burden overrides excellent physicochemical quality and forces the final class to Critical. |

| Supplementary Table S10. Idealized 5×5 fusion-space validation matrix for the CWHI framework, showing the 25 combinations of WQI and ABI class-center inputs and the corresponding centroid-defuzzified CWHI scores and output classes. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario | WQI Class | ABI Class | WQI | ABI | CWHI | CWHI Class |
| S1 | VP | VL | 10 | 10 | 8.38 | Critical |
| S2 | VP | L | 10 | 30 | 8.9 | Critical |
| S3 | VP | M | 10 | 50 | 8.39 | Critical |
| S4 | VP | H | 10 | 70 | 8.39 | Critical |
| S5 | VP | VH | 10 | 90 | 8.38 | Critical |
| S6 | P | VL | 30 | 10 | 30 | Poor |
| S7 | P | L | 30 | 30 | 30 | Poor |
| S8 | P | M | 30 | 50 | 8.9 | Critical |
| S9 | P | H | 30 | 70 | 8.9 | Critical |
| S10 | P | VH | 30 | 90 | 8.9 | Critical |
| S11 | M | VL | 50 | 10 | 50 | Moderate |
| S12 | M | L | 50 | 30 | 50 | Moderate |
| S13 | M | M | 50 | 50 | 30 | Poor |
| S14 | M | H | 50 | 70 | 8.39 | Critical |
| S15 | M | VH | 50 | 90 | 8.39 | Critical |
| S16 | G | VL | 70 | 10 | 70 | Good |
| S17 | G | L | 70 | 30 | 50 | Moderate |
| S18 | G | M | 70 | 50 | 30 | Poor |
| S19 | G | H | 70 | 70 | 30 | Poor |
| S20 | G | VH | 70 | 90 | 8.39 | Critical |
| S21 | VG | VL | 90 | 10 | 91.62 | Safe |
| S22 | VG | L | 90 | 30 | 70 | Good |
| S23 | VG | M | 90 | 50 | 50 | Moderate |
| S24 | VG | H | 90 | 70 | 30 | Poor |
| S25 | VG | VH | 90 | 90 | 8.38 | Critical |

| Supplementary Table S11. Single point one-at-a-time (OAT) perturbation for CWHI framework |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Factor | CWHI Base | CWHI New | ΔCWHI | Sensitivity Index |
| WQI | 0.9 | 30.000 | 30.000 | 0.000 | 0.000 |
| WQI | 1.1 | 30.000 | 30.000 | 0.000 | 0.000 |
| ABI | 0.9 | 30.000 | 30.000 | 0.000 | 0.000 |
| ABI | 1.1 | 30.000 | 29.884 | -0.116 | -0.039 |

| Supplementary Table S12. Global grid Sensitivity Analysis for the WQI and ABI |  |
| --- | --- |
| Metric | Value |
| Mean Gradient WQI | 0.436 |
| Mean Gradient ABI | 0.458 |
| Median Grad WQI | 2.13E-14 |
| Median Grad ABI | 2.84E-14 |

Supplementary Figure S1. Frequency distributions of local gradient magnitudes for (a) WQI and (b) ABI obtained from global finite-difference sensitivity analysis over the full WQI-ABI space. The pronounced right-skew and concentration near zero indicate that most of the response surface is locally stable, whereas only limited regions show stronger sensitivity.

Supplementary Figure S2. Distribution of CWHI values obtained from Monte Carlo uncertainty propagation analysis. Summary statistics of the simulated outputs were mean = 29.14, SD = 3.08, 95% CI = [19.72-30.00], and CV = 10.55%.

| Supplementary Table S13. Seasonal WQI class transition matrix for Musi River sampling sites, showing wet-season and dry-season WQI classes and indicating whether class changes occurred between seasons. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Location | Wet | Dry | Wet Class | Dry Class | WQI Class Change |
| OS | 27.334 | 27.334 | Poor | Poor | Poor -> Poor |
| BGB | 7.826 | 7.852 | Critical | Critical | Critical -> Critical |
| SRN | 7.852 | 7.852 | Critical | Critical | Critical -> Critical |
| PP | 7.826 | 7.852 | Critical | Critical | Critical -> Critical |
| GNB | 7.826 | 7.778 | Critical | Critical | Critical -> Critical |
| NGB | 7.852 | 7.904 | Critical | Critical | Critical -> Critical |
| PZ | 7.852 | 7.852 | Critical | Critical | Critical -> Critical |
| PSB | 7.852 | 7.904 | Critical | Critical | Critical -> Critical |
| NRB | 7.826 | 7.860 | Critical | Critical | Critical -> Critical |
| MR | 27.334 | 27.345 | Poor | Poor | Poor -> Poor |

| Supplementary Table S14. Spearman rank correlations between ARG and WQPs used in the Musi River dataset. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters | blaNDM | blaCTX-M | qnrS | ermF | tetM | tetW | sul1 | sul2 | tetQ |
| Temp | -0.039 | 0.157 | -0.038 | -0.230 | -0.553 | -0.396 | -0.139 | -0.008 | -0.425 |
| pH | 0.292 | 0.044 | -0.011 | -0.249 | 0.160 | -0.106 | -0.358 | -0.375 | -0.129 |
| TDS | -0.232 | -0.317 | 0.045 | 0.280 | 0.662 | 0.450 | 0.174 | 0.074 | 0.484 |
| DO | 0.624 | 0.479 | -0.001 | -0.466 | -0.514 | -0.624 | -0.556 | -0.484 | -0.630 |
| EC | -0.232 | -0.317 | 0.045 | 0.280 | 0.662 | 0.450 | 0.174 | 0.074 | 0.484 |
| Tur | -0.759 | -0.711 | -0.153 | 0.513 | 0.746 | 0.797 | 0.287 | 0.110 | 0.839 |
| TSS | -0.633 | -0.636 | -0.134 | 0.511 | 0.791 | 0.777 | 0.266 | 0.090 | 0.803 |
| COD | -0.626 | -0.630 | -0.141 | 0.496 | 0.689 | 0.675 | 0.278 | 0.153 | 0.688 |
| Nitrate | 0.350 | 0.403 | 0.153 | -0.227 | -0.782 | -0.629 | -0.065 | 0.075 | -0.609 |
| Ammonium | -0.260 | -0.372 | -0.099 | 0.162 | 0.693 | 0.460 | 0.057 | -0.028 | 0.489 |
| Nitrite | 0.261 | 0.500 | 0.226 | -0.077 | -0.721 | -0.547 | 0.159 | 0.338 | -0.567 |
| TN | -0.744 | -0.620 | -0.068 | 0.620 | 0.687 | 0.814 | 0.505 | 0.373 | 0.817 |
| TP | -0.537 | -0.606 | -0.263 | 0.382 | 0.827 | 0.642 | 0.096 | -0.036 | 0.684 |
| TOC | -0.665 | -0.657 | -0.077 | 0.547 | 0.725 | 0.735 | 0.317 | 0.177 | 0.768 |
| TBX | -0.303 | -0.147 | 0.215 | 0.162 | -0.096 | 0.158 | 0.309 | 0.353 | 0.112 |
| OSS | -0.148 | -0.032 | 0.405 | 0.086 | -0.272 | -0.023 | 0.347 | 0.380 | -0.029 |
| Cu | -0.507 | -0.478 | -0.125 | 0.417 | 0.875 | 0.738 | 0.259 | 0.095 | 0.806 |
| Ni | -0.371 | -0.006 | 0.072 | 0.373 | -0.051 | 0.212 | 0.389 | 0.423 | 0.238 |
| Pb | -0.519 | -0.277 | 0.048 | 0.236 | 0.074 | 0.271 | 0.295 | 0.245 | 0.349 |
| Cd | -0.477 | -0.324 | -0.014 | 0.460 | 0.499 | 0.464 | 0.098 | 0.069 | 0.497 |
| 16S rRNA | -0.674 | -0.717 | -0.134 | 0.478 | 0.756 | 0.753 | 0.239 | 0.063 | 0.812 |
| intI1 | -0.675 | -0.566 | -0.016 | 0.609 | 0.763 | 0.870 | 0.551 | 0.403 | 0.871 |

Supplementary Information D on the ABI Reference-Site Robustness: Robustness of ABI outputs to reference-site specification were assessed by comparing ABI scores across three baseline scenarios: Scenario A (OS-only), B (dry-season baseline), and C (pooled 10th percentile). Of 20 site-seasons, 16 (80.0%) yielded an ABI scenario spread (max − min across scenarios) below 5 ABI units and were classified as Robust; two (MR Wet, spread = 5.55; MR Dry, spread = 7.68) were classified as Moderate; and two (OS Dry, spread = 16.59; NRB Dry, spread = 15.75) were classified as Sensitive (Table 15; Fig. S6). The two Sensitive-class site–seasons are the upstream reference site (OS) and the furthest downstream site (NRB), both evaluated during the dry season. The elevated spread at these sites is driven by Scenario C, in which the pooled 10th-percentile baseline diverges from the site-specific OS-only reference. No CWHI class change resulted from reference-site variation at any site–season unit; all 20 outputs retained the Critical CWHI classification across all four scenarios.

| Supplementary Table S15. Reference-site sensitivity analysis of ABI across four baseline scenarios for Musi River samples, showing scenario-specific ABI values, maximum spread, and coefficient of variation for each site–season combination. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Location | Season | Stretch | Case A | Case B | Case C | ABI spread | CVABI |
| OS | Wet | Upstream | 37.414 | 37.421 | 42.156 | 4.741 | 0.061 |
| BGB | Wet | Urban | 56.639 | 55.760 | 57.393 | 1.633 | 0.012 |
| SRN | Wet | Urban | 64.996 | 64.574 | 68.856 | 4.282 | 0.031 |
| PP | Wet | Urban | 57.969 | 59.200 | 62.311 | 4.342 | 0.035 |
| GNB | Wet | Urban | 53.812 | 56.889 | 56.981 | 3.170 | 0.033 |
| NGB | Wet | Urban | 48.840 | 48.492 | 45.913 | 2.927 | 0.029 |
| PZ | Wet | Urban | 63.928 | 67.841 | 68.723 | 4.795 | 0.038 |
| PSB | Wet | Urban | 53.818 | 53.890 | 57.289 | 3.470 | 0.032 |
| NRB | Wet | Semi-urban | 64.538 | 62.633 | 64.152 | 1.905 | 0.014 |
| MR | Wet | Downstream | 37.170 | 37.031 | 42.581 | 5.550 | 0.071 |
| OS | Dry | Upstream | 45.428 | 44.432 | 61.021 | 16.589 | 0.163 |
| BGB | Dry | Urban | 65.891 | 66.392 | 69.203 | 3.313 | 0.024 |
| SRN | Dry | Urban | 61.024 | 64.923 | 63.083 | 3.899 | 0.030 |
| PP | Dry | Urban | 60.210 | 59.317 | 61.076 | 1.759 | 0.012 |
| GNB | Dry | Urban | 58.411 | 57.404 | 58.731 | 1.327 | 0.010 |
| NGB | Dry | Urban | 64.804 | 65.793 | 68.880 | 4.076 | 0.029 |
| PZ | Dry | Urban | 58.396 | 57.384 | 57.648 | 1.013 | 0.009 |
| PSB | Dry | Urban | 58.842 | 58.594 | 59.096 | 0.502 | 0.003 |
| NRB | Dry | Semi-urban | 54.974 | 49.161 | 64.915 | 15.753 | 0.117 |
| MR | Dry | Downstream | 46.521 | 45.749 | 53.432 | 7.683 | 0.075 |

References

(1)	Scannapieco, D.; Naddeo, V.; Zarra, T.; Belgiorno, V. River Water Quality Assessment: A Comparison of Binary- and Fuzzy Logic-Based Approaches. Ecol. Eng. 2012, 47, 132–140. https://doi.org/10.1016/j.ecoleng.2012.06.015.

(2)	CPCB. “Indicative Guidelines for Restoration of Water Bodies” Central Pollution Control Board (Ministry of Environment, Forest and Climate Change, Govt. of India); 2019. www.cpcb.nic.in (accessed 2026-05-19).

(3)	Ocampo-Duque, W.; Ferré-Huguet, N.; Domingo, J. L.; Schuhmacher, M. Assessing Water Quality in Rivers with Fuzzy Inference Systems: A Case Study. Environ. Int. 2006, 32 (6), 733–742. https://doi.org/10.1016/J.ENVINT.2006.03.009.

(4)	Gharibi, H.; Mahvi, A. H.; Nabizadeh, R.; Arabalibeik, H.; Yunesian, M.; Sowlat, M. H. A Novel Approach in Water Quality Assessment Based on Fuzzy Logic. J. Environ. Manage. 2012, 112, 87–95. https://doi.org/10.1016/j.jenvman.2012.07.007.

(5)	Fortes, A. C. C.; Barrocas, P. R. G.; Kligerman, D. C. Water Quality Indices: Construction, Potential, and Limitations. Ecol. Indic. 2023, 157, 111187. https://doi.org/10.1016/j.ecolind.2023.111187.

(6)	Chanapathi, T.; Thatikonda, S.; Pandey, V. P.; Shrestha, S. Fuzzy-Based Approach for Evaluating Groundwater Sustainability of Asian Cities. Sustain. Cities Soc. 2019, 44, 321–331. https://doi.org/10.1016/J.SCS.2018.09.027.

(7)	Shuai, X.; Zhou, Z.; Zhu, L.; Achi, C.; Lin, Z.; Liu, Z.; Yu, X.; Zhou, J.; Lin, Y.; Chen, H. Ranking the Risk of Antibiotic Resistance Genes by Metagenomic and Multifactorial Analysis in Hospital Wastewater Systems. J. Hazard. Mater. 2024, 468, 133790. https://doi.org/10.1016/J.JHAZMAT.2024.133790.

(8)	Zhang, A. N.; Gaston, J. M.; Dai, C. L.; Zhao, S.; Poyet, M.; Groussin, M.; Yin, X.; Li, L. G.; van Loosdrecht, M. C. M.; Topp, E.; Gillings, M. R.; Hanage, W. P.; Tiedje, J. M.; Moniz, K.; Alm, E. J.; Zhang, T. An Omics-Based Framework for Assessing the Health Risk of Antimicrobial Resistance Genes. Nature Communications 2021 12:1 2021, 12 (1), 4765-. https://doi.org/10.1038/s41467-021-25096-3.

(9)	Martínez, J. L.; Coque, T. M.; Baquero, F. What Is a Resistance Gene? Ranking Risk in Resistomes. Nature Reviews Microbiology 2014 13:2 2014, 13 (2), 116–123. https://doi.org/10.1038/nrmicro3399.

(10)	Ruiz, S. E.; Morandini, F. N.; Panzetta, M. E.; Lipari, F. G.; Irrazábal, M. G.; Toselli, R.; Der Ohannesian, M.; Amieva, C.; Valdes, M. E.; Giraudo, F. J.; Rollán, M. del R.; Amé, V.; Sola, C.; Saka, H. A. Urban Wastewater Overflows as Hotspots for Dissemination of Bacteria Producing Extended-Spectrum β-Lactamases and Carbapenemases in the Suquía River, Argentina. Front. Microbiol. 2025, 16, 1669531. https://doi.org/10.3389/FMICB.2025.1669531/TEXT.

(11)	Zhao, Z.; Li, C.; Jiang, L.; Wu, D.; Shi, H.; Xiao, G.; Guan, Y.; Kang, X. Occurrence and Distribution of Antibiotic Resistant Bacteria and Genes in the Fuhe Urban River and Its Driving Mechanism. Science of The Total Environment 2022, 825, 153950. https://doi.org/10.1016/J.SCITOTENV.2022.153950.

(12)	Zheng, Q.; Wu, R.; Chen, X.; Liu, P.; Li, K.; Ke, C.; Wu, Y.; Zhang, Y.; Xiao, S.; Huang, J. Co-occurrence of microbial source tracking markers and antibiotic resistance genes in Chishui River, China. Environmental Science: Processes and Impacts. https://doi.org/10.1039/d5em00070j.

(13)	Sabri, N. A.; Schmitt, H.; Van Der Zaan, B.; Gerritsen, H. W.; Zuidema, T.; Rijnaarts, H. H. M.; Langenhoff, A. A. M. Prevalence of Antibiotics and Antibiotic Resistance Genes in a Wastewater Effluent-Receiving River in the Netherlands. J. Environ. Chem. Eng. 2020, 8 (1), 102245. https://doi.org/10.1016/J.JECE.2018.03.004.

(14)	Devarajan, N.; Laffite, A.; Mulaji, C. K.; Otamonga, J. P.; Mpiana, P. T.; Mubedi, J. I.; Prabakar, K.; Ibelings, B. W.; Poté, J. Occurrence of Antibiotic Resistance Genes and Bacterial Markers in a Tropical River Receiving Hospital and Urban Wastewaters. PLoS One 2016, 11 (2), e0149211. https://doi.org/10.1371/JOURNAL.PONE.0149211.

(15)	Bengtsson-Palme, J.; Larsson, D. G. J. Concentrations of Antibiotics Predicted to Select for Resistant Bacteria: Proposed Limits for Environmental Regulation. Environ. Int. 2016, 86, 140–149. https://doi.org/10.1016/J.ENVINT.2015.10.015.

(16)	Kneis, D.; De La Cruz Barron, M.; Konyali, D.; Westphal, V.; Schröder, P.; Westphal-Settele, K.; Schönfeld, J.; Jungmann, D.; Berendonk, T. U.; Klümper, U. Ecology-Based Approach to Predict No-Effect Antibiotic Concentrations for Minimizing Environmental Selection of Resistance. ISME J. 2025, 19 (1). https://doi.org/10.1093/ISMEJO/WRAF172.

(17)	Sati, H.; Carrara, E.; Savoldi, A.; Hansen, P.; Garlasco, J.; Campagnaro, E.; Boccia, S.; Castillo-Polo, J. A.; Magrini, E.; Garcia-Vello, P.; Wool, E.; Gigante, V.; Duffy, E.; Cassini, A.; Huttner, B.; Pardo, P. R.; Naghavi, M.; Mirzayev, F.; Zignol, M.; Cameron, A.; Tacconelli, E.; Aboderin, A.; Al Ghoribi, M.; Al-Salman, J.; Amir, A.; Apisarnthanarak, A.; Blaser, M.; El-Sharif, A.; Essack, S.; Harbarth, S.; Huang, X.; Kapoor, G.; Knight, G.; Muhwa, J. C.; Monnet, D. L.; Ousassa, T.; Sacsaquispe, R.; Severin, J.; Sugai, M.; Taneja, N.; Umubyeyi Nyaruhirira, A. The WHO Bacterial Priority Pathogens List 2024: A Prioritisation Study to Guide Research, Development, and Public Health Strategies against Antimicrobial Resistance. Lancet Infect. Dis. 2025, 25 (9), 1033–1043. https://doi.org/10.1016/S1473-3099(25)00118-5.