Supplementary Information

for

Digital Scene Construction Method for Bridge Construction Based on Pan-Spatial Information System

Yongwei Wang1,2*, Yongtao Zhang1,2, Can Huang1,2, Yuan Chen1, Yakun Xie3, Xiankai Xue1

1. CCCC Second Harbor Engineering Co., Ltd., Wuhan 430040, China; 2. Key Laboratory of Large-Span Bridge Construction Technology, Wuhan 430040, China; 3. School of Earth Sciences and Engineering, Southwest Jiaotong University, Chengdu 611756, China

*Corresponding author: wangyongwei@ccccltd.cn

S1. Overview of the illustrative scenario

The illustrative scenario described in Section “Illustrative application” of the main text is a constructed example of the pier body construction stage of a medium-scale cable-stayed bridge, denoted as Bridge-Project-2024; it is designed to demonstrate the application of the proposed method and does not correspond to any specific real engineering project. The main construction activities in this stage include steel bar binding, formwork installation, concrete pouring and curing of the pier body segments; a tower crane (TC-01) and a concrete pump truck (CPT-01) serve as the main equipment, and anemometers and strain gauges are deployed on the pier for real-time monitoring. This supplementary file provides the data generated and analysed in this scenario: the registry of instantiated spatio-temporal objects (S2), the formalized description instances of the key objects (S3), a sample of the dynamic attribute data streams involved in the wind speed early warning event (S4), and the basis of the comparative analysis reported in Table 2 of the main text (S5).

All object identifiers follow the naming rules commonly used on bridge construction project data platforms. All parameter values are hypothetical but set within engineering-plausible ranges, and are consistent with the examples given in the main text.

S2. Registry of instantiated spatio-temporal objects

Table S1. Instantiation of spatio-temporal objects in the pier construction stage (extended from Table 1 of the main text).

| Object category | Instance ID | Instantiated object | Key features populated | Data source |
| --- | --- | --- | --- | --- |
| Bridge components | Pier-Abutment-1 | Pier body segment of Abutment-1 (cushion cap, pier body, bent cap) | SR, SP, CS, AT | Design drawings, BIM model |
| Workers | Worker-SB-01–06; Worker-FW-01–04; Engineer-SE-01 | Steel bar workers, formwork workers, site engineer | CI, BA, AT | Personnel management system |
| Machinery and equipment | TC-01; CPT-01 | Tower crane, concrete pump truck | SP, BA, AT | Equipment IoT platform |
| Bridge site environment | ENV-Site-01 | Wind speed, temperature and humidity at the site | SR, AT | Meteorological monitoring station |
| Construction measures | Formwork-Sys-01; CF-01 | Formwork system, climbing frame | CS, IR, AT | Construction scheme documents |
| Monitoring and sensing | Anemometer-01; SG-01–SG-04 | Anemometer, strain gauges on the pier | SP, BA, AT | Sensor data acquisition system |
| Quality inspection | QI-Conc-S5 | Concrete strength test of pier segment S5 | IR, AT | Quality inspection records |

S3. Formalized description instances of key objects

The following instances populate the symbolic expressions defined in Eqs. (1)–(7) of the main text. They are given in the same tuple notation and are directly machine-parsable.

S3.1 Spatio-temporal reference (SR) and spatio-temporal positioning (SP) of Pier-Abutment-1

SR_Pier = (T_Ref: "Gregorian calendar, UTC+8", S_Ref_Global: "CGCS2000", S_Ref_Parent: "Bridge-Project-2024 construction control network", S_Ref_Local: "Pier-Abutment-1 local frame")

SP_Pier = (T: "2024-05-27T10:30:00+08:00", P: (38432105.32, 3352018.47, 42.50), O: (0.0, 0.0, 90.0), V: (0, 0, 0), ω: (0, 0, 0))

Coordinates are planimetric coordinates (m) in the project control network with ellipsoidal height (m); orientation is given as (roll, pitch, azimuth) in degrees.

S3.2 Compositional structure (CS) of Pier-Abutment-1

CS_Pier = (Whole: "Pier-Abutment-1", Parts: {"CushionCap-1", "PierBody-1", "BentCap-1"}, Relation: "is-composed-of")

CS_PierBody = (Whole: "PierBody-1", Parts: {"RebarCage-PB1", "ConcreteBatch-PB1-01", "ConcreteBatch-PB1-02", "ConcreteBatch-PB1-03"}, Relation: "is-composed-of")

S3.3 Attributes (AT) of Pier-Abutment-1 and Anemometer-01

AT_Pier = (Static_Attributes: {"design_elevation": 46.50, "concrete_grade": "C40", "section_size": "8.0 × 4.5"}, Dynamic_Attributes: {"pouring_progress": (0.62, "2024-05-27T10:30:00+08:00"), "strain_SG-03": (18.4, "2024-05-27T10:30:00+08:00")}, Metadata: {"design_elevation": {"unit": "m", "data_type": "float"}, "strain_SG-03": {"unit": "με", "data_type": "float", "source": "SG-03"}})

AT_Anemometer-01 = (Static_Attributes: {"model": "WSS-501", "mount_height": 32.5}, Dynamic_Attributes: {"wind_speed": (15.8, "2024-05-27T10:30:00+08:00")}, Metadata: {"wind_speed": {"unit": "m/s", "data_type": "float", "source": "Anemometer-01"}, "mount_height": {"unit": "m"}})

S3.4 Cognitive intelligence (CI) of the wind speed monitoring and early warning system

CI_WindAlert = (Perceive: lambda: {"wind_speed": query_sensor("anemometer_01")}, Analyze: lambda p, kb: {"risk": "High" if p["wind_speed"] > kb["max_wind"] else "Low"}, Decide: lambda s, g, kb: "Issue_Alert" if s["risk"] == "High" else "Continue", Act: lambda d: send_alert(d), KnowledgeBase: {"max_wind": 15.0}, Goal: "Ensure_Lifting_Safety")

S3.5 Behavioral action (BA) of tower crane TC-01

Action_Lift = (Actor: "TC-01", Verb: "Lift", Target: "SteelBarBundle-S5-02", Parameters: {"lift_height": 30.0, "speed": 0.5}, Trigger: "On_Command(Command_ID: CMD-LIFT-20240527-003)", Precondition: {"self.status": "idle", "wind_speed": "< 15 m/s"}, Effect: {"Target.position": "at_installation_position", "Project.progress": "segment_S5_rebar_in_place"})

At 2024-05-27T10:30:00+08:00, the measured wind speed reached 15.8 m/s; the Decide function of CI_WindAlert output “Issue_Alert”, the Precondition of Action_Lift was no longer satisfied, and the lifting action was suspended (see the event sequence in S4).

S3.6 Interrelationship (IR) instances

IR_Attach = (ID: "IR-ATT-001", Subject: "Anemometer-01", Predicate: "attached-to", Object: "Pier-Abutment-1", Attributes: {"mount_height": 32.5}, T_Valid: ["2024-03-01", "2024-09-30"], S_Scope: "pier top platform")

IR_Precedence = (ID: "IR-PRC-002", Subject: "Formwork-Sys-01 installation (segment S5)", Predicate: "precedes", Object: "Concrete pouring (segment S5)", Attributes: {"min_interval": "12 h"}, T_Valid: ["2024-05-20", "2024-06-15"], S_Scope: "Pier-Abutment-1")

S4. Sample of dynamic attribute data streams in the wind speed early warning event

Table S2. Wind speed and pier strain records around the early warning event of 2024-05-27 (sampling interval 5 min).

| Timestamp | wind_speed (m/s) | strain_SG-03 (με) | System state |
| --- | --- | --- | --- |
| 2024-05-27T10:05:00+08:00 | 11.2 | 17.9 | Normal |
| 2024-05-27T10:10:00+08:00 | 12.6 | 18.0 | Normal |
| 2024-05-27T10:15:00+08:00 | 13.9 | 18.1 | Normal |
| 2024-05-27T10:20:00+08:00 | 14.7 | 18.2 | Normal |
| 2024-05-27T10:25:00+08:00 | 15.3 | 18.3 | Alert (risk = High) |
| 2024-05-27T10:30:00+08:00 | 15.8 | 18.4 | Alert; Action_Lift suspended |
| 2024-05-27T10:35:00+08:00 | 15.1 | 18.4 | Alert |
| 2024-05-27T10:40:00+08:00 | 14.2 | 18.3 | Normal; suspension lifted |
| 2024-05-27T10:45:00+08:00 | 13.0 | 18.2 | Normal |
| 2024-05-27T10:50:00+08:00 | 12.1 | 18.1 | Normal |

S5. Basis of the comparative analysis (Table 2 of the main text)

The comparison reported in Table 2 of the main text covers nine feature dimensions: core data unit, spatio-temporal expression, structural organization, relation expression, intelligent ability, behavior modeling, attribute management, granularity flexibility and knowledge expression. The assessment of the BIM method is based on the IFC standard documentation [18]; the assessment of digital twin methods is based on the characteristics reported in [19–20]; and the assessment of the proposed method follows the definitions of the seven-feature framework in Section 3 of the main text. The comparison is a qualitative feature analysis conducted by the authors and uses no external datasets beyond the references cited in the main text.

Note. All identifiers, coordinates and parameter values in this Supplementary Information belong to the constructed illustrative scenario and do not correspond to any real engineering project.