Supplementary document for:

Process simulation integrated life cycle net energy analysis and GHG assessment of fuel-grade bioethanol production from unutilized rice straw

Piyumali Mewanthika Jayasundaraa , Thisara Kaveendra Jayasinghea , Mahinsasa Rathnayakea*  aDepartment of Chemical and Process Engineering, University of Moratuwa, Sri Lanka

* Corresponding author

Email: mratnayake@uom.lk

| Component | CO2 | CH4 | N2O |
| --- | --- | --- | --- |
| Emission factor (mg/kg diesel) [1] | 2,640,000.00 | 93.00 | 21.00 |

Table S01: GHG emission factors for diesel combustion in farming machinery

| Raw Material | Consumption (kg / FU) | Enautical (MJ/FU) | Einland transport (MJ/FU) | ETotal (MJ/FU) |
| --- | --- | --- | --- | --- |
| H2SO4a | 636.02 | 305.29 | 256.12 | 561.41 |
| Ca(OH)2 b | 480.47 | 230.63 | 193.48 | 424.11 |
| (NH4)2HPO4 a | 15.41 | 7.40 | 6.21 | 13.61 |
| Enzyme a | 26.70 | 12.82 | 10.75 | 23.57 |
| Yeast a | 8.23 | 3.95 | 3.32 | 7.27 |
| Ethylene Glycol b | 0.28 | 0.13 | 0.11 | 0.25 |

Table S02: Energy consumption in transportation of raw materials

a Amounts of chemicals are adopted from previously published data [1, 2]

b Calculated values from process simulations

Table S03: GHG emission factors for fuel combustion in transportation [1]

| Emission source | GHG emission factors for fuel combustion (mg/tonne.km) |  |  |
| --- | --- | --- | --- |
|  | CO2 | CH4 | N2O |
| Diesel for the truck (10 tonnes capacity) | 74,000.00 | 1.00 | 1.60 |
| Heavy oil for shipping | 6,200.00 | 0.20 | 0.20 |

Table S04: Emission factors for biogenic fuel combustion [1]

| Emission source | Emission factors (mg/MJ) |  |
| --- | --- | --- |
|  | CH4 | N2O |
| Biogas as a fuel | 440.00 | 1.60 |
| Lignin contained solid residue as a fuel | 3.10 | 0.80 |

Table S05: Pre-treatment conditions for unutilized rice straw [3]

| Process operation | Chemicals used | Process conditions | Reactions |
| --- | --- | --- | --- |
| Dilute acid pre-treatment | H2SO4 | 1% (W/V) | - |
|  | Steam | 139°C at 3.5 bar |  |
| Enzyme hydrolysis | Cellulase | 162°C at 5 bar for 15 mins X=90% | C6H10O5(s)+H2O(aq)→C6H12O6(aq) C5H8O4(s)+H2O(aq)→C5H10O5 (aq) |
| Neutralization | Ca(OH)2 | 60°C at 1atm for 24 h | H2SO4(aq)+CaOH2(aq)→CaSO4∙2H2O(s) |

Table S06: Process conditions for ethanol fermentation [3]

| Process operation | Chemicals used | Process conditions | Reactions |
| --- | --- | --- | --- |
| SSF | Yeast (NH4)2HPO4 | 30°C at 1 atm for 48h Xglucose=90% Xxylose=68% ηyield=51% | C6H12O6(aq)+enzymes→C2H5OH(aq)+CO2(g) C5H10O5(aq)+enzymes→C2H5OH(aq)+CO2(g) |
| Distillation |  | 1 atm Reflux ratio = 2.75 | Number of stages = 30 |

Table S07: Process conditions for bioethanol dehydration [3]

| Process operation | Chemicals used | Process conditions |
| --- | --- | --- |
| Dehydration column (extractive distillation) | Makeup ethylene glycol (Recovered and recycled) | - atm Reflux ratio = 0.6 Number of stages = 20 |
| Ethylene glycol recovery column | - | 0.26 atm Reflux ratio = 1 Number of stages = 12 |

Table S08: Energy consumption for manufacturing of raw materials

| Chemical and raw material name | Average energy consumption for manufacturing (MJ/kg) | Consumption in the scaled-up plant (kg/FU) | Average energy consumption (MJ/FU) | Data sources |
| --- | --- | --- | --- | --- |
| (NH4)2HPO4 | 25.6 | 15.41 | 394.61 | [4] |
| H2SO4 | 5.8 | 636.02 | 3,688.91 | [4] |
| Ethylene glycol | 14.0 | 0.28 | 3.90 | [4] |
| Yeast | 5.8 | 8.23 | 47.76 | [5] |
| Enzymes | 5.8 | 26.70 | 154.85 | [5] |
| Ca(OH)2 | 4.8 | 480.47 | 2,286.91 | [6] |

Table S09: GHG emission factors for manufacturing of raw materials

| Component | GHG emission factors (kg CO2 eq./kg) | GHG emissions (kg CO2 eq./FU) | Data sources |
| --- | --- | --- | --- |
| (NH4)2HPO4 | 2.80 | 43.16 | [2] |
| H2SO4 | 0.20 | 127.20 | [2] |
| Ethylene glycol | 3.60 | 1.00 | [7] |
| Yeast | 3.20 | 26.35 | [2] |
| Enzyme | 5.50 | 146.84 | [2] |
| Ca(OH)2 | 1.07 | 515.06 | [2] |
| Total |  | 859.63 |  |

Table S10: Concentration of organic components in generated wastewater

| Component | Concentration (g/L) |
| --- | --- |
| Glucose | 9.60 |
| Xylose | 4.08 |
| Ethanol | 5.29 |
| Ethylene Glycol | 3.18 × 10-5 |

Table S11: Fuels, heating values, and efficiencies for calculation of process energy cogeneration

| Fuel type | LHV* (MJ/kg) | Combined Heat and Power unit efficiencies* |  |  | Energy generation (MJ/FU) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ηB | ηE | ηCHP | Heat | Electricity |
| Biogas | 22.40 | 0.90 | 0.35 | 0.80 | 1,141.9 | 888.2 |
| Solid residue | 19.00 | 0.80 | 0.13 | 0.70 | 43,922.3 | 10,017.4 |

* Values are adopted from [1]

Table S12: Emission factors for Sri Lankan electricity grid-mix [8]

| Emission type | Emission factors (kg/kWh) |
| --- | --- |
| CO2 | 5.49 × 10-1 |
| SO2 | 1.72 × 10-3 |
| NO | 1.39 × 10-3 |
| CO | - × 10-4 |

Table S13: Global Warming Potential values for CO2, CH4, N2O [9]

| Emission | GWP values for 100 - year time zone (IPCC fourth assessment report) |
| --- | --- |
| CO2 | 1 |
| CH4 | 25 |
| N2O | 298 |

References

1. 	Rathnayake, M., Chaireongsirikul, T., Svangariyaskul, A., Lawtrakul, L., Toochinda, P.: Process simulation based life cycle assessment for bioethanol production from cassava, cane molasses, and rice straw. J. Clean. Prod. 190, 24–35 (2018). https://doi.org/10.1016/j.jclepro.2018.04.152

2. 	Soam, S., Kapoor, M., Kumar, R., Borjesson, P., Gupta, R.P., Tuli, D.K.: Global warming potential and energy analysis of second generation ethanol production from rice straw in India. Appl. Energy. 184, 353–364 (2016). https://doi.org/10.1016/j.apenergy.2016.10.034

3. 	Yoswathana, N., Phuriphipat, P., Treyawutthiwat, P., Eshtiaghilications, M.N.: Bioethanol Production from Rice Straw. Energy. 1, 26–31 (2010)

4. 	Wernet, G., Bauer, C., Steubing, B., Reinhard, J., Moreno-Ruiz, E.and Weidema, B.: The ecoinvent database version 3 (part I): overview and methodology. Int. J. Life Cycle Assess. 21, 1218–1230 (2016)

5. 	Dunn, J.B., Mueller, S., Wang, M., Han, J.: Energy consumption and greenhouse gas emissions from enzyme and yeast manufacture for corn and cellulosic ethanol production. Biotechnol. Lett. 34, 2259–2263 (2012). https://doi.org/10.1007/s10529-012-1057-6

6. 	Sagastume Gutiérrez, A., Van Caneghem, J., Cogollos Martínez, J.B., Vandecasteele, C.: Evaluation of the environmental performance of lime production in Cuba. J. Clean. Prod. 31, 126–136 (2012). https://doi.org/10.1016/j.jclepro.2012.02.035

7. 	Claros Garcia, J.C., Von Sperling, E.: Greenhouse gas emissions from sugar cane ethanol: Estimate considering current different production scenarios in Minas Gerais, Brazil. Renew. Sustain. Energy Rev. 72, 1033–1049 (2017). https://doi.org/10.1016/j.rser.2017.01.046

8. 	Ceylon Electricity Board: Ceylon Electricity Board Annual Report, https://ceb.lk/publication-media/annual-reports/en, (2019)

9. 	Myhre, G., D., Shindell, F. M., Bréon, W., Collins, J., Fuglestvedt, J., Huang, D., Koch, J. ‐F., Lamarque, D., Lee, B., Mendoza, T., Nakajima, A., Robock, G., Stephens, T., Takemura, Zhang, H.: Global Warming Potential Values. Greenh. Gas Protoc. 2014, 2–5 (2015)