Supplementary Table S1.

Supplementary Table S1. Initial FDM Indicator Pool and Literature Sources

| Code | Indicator | Definition | References |
| --- | --- | --- | --- |
| I1 | Flood-prone space identification | Identifies flood-prone spaces such as building entrances, low-lying plazas, and underground-space entrances. | [1,4,9,13,22,51,75,76,79] |
| I2 | High-risk path identification | Identifies frequently used rainy-day routes vulnerable to ponding, slipperiness, detours, or interruption of rain-sheltered continuity. | [4,9,12,13,18,75,76] |
| I3 | Historical stormwater-event record | Records previous stormwater events, including ponding, backflow, slipping incidents, temporary closures, complaints, and maintenance actions. | [4,25,27,38,42,43,68,75] |
| I4 | Stormwater Risk Grading Representation | Represents stormwater risk points, risk routes, and risk grades through drawings, GIS, or risk maps. | [4,11,42,73,74] |
| I5 | Rainstorm warning and notification | Provides timely risk reminders and rainy-day mobility advice to students and staff before rainstorm events. | [13,18,25,38,42,75] |
| I6 | Alternative route guidance plan | Specifies recommended rainy-day routes, detour routes, restricted areas, and safety information. | [13,18,25,38,42,75,76] |
| I7 | Emergency plan for key nodes | Develops rainy-day evacuation and guidance plans for high-frequency nodes such as canteens, teaching buildings, dormitories, and libraries. | [4,11,13,20,27,38,42] |
| I8 | Coordinated response mechanism | Defines response responsibilities across logistics, security, property management, and dormitory management units. | [4,12,19,20,25,38,42,48,49] |
| I9 | Drainage facility maintenance | Ensures that inlets, drainage ditches, linear drains, and roof downpipes remain unobstructed before rainstorm events. | [42,44,45,48,49,50,51,76] |
| I10 | Entrance waterproofing and anti-slip measures | Reduces backflow and slipping risk at entrances through elevation control, interception, water barriers, anti-slip paving, and emergency facilities. | [4,11,12,19,42,45,74,76] |
| I11 | Rain-shelter connection provision | Provides continuous or semi-continuous rain-shelter connections between frequently used building nodes. | [11,13,18,19,20,76] |
| I12 | Water-sensitive facility provision | Provides rain gardens, sunken green spaces, permeable pavement, and related facilities in flood-prone and high-pedestrian-flow areas. | [42,44,45,48,49,50,51,76] |
| I13 | Ponding extent and depth | Evaluates whether the spatial extent and depth of ponding at entrances, plazas, and corridor nodes affect safe passage. | [4,9,28,42,45,48,74,76,79] |
| I14 | Runoff conveyance organization | Assesses whether runoff is organized away from pedestrian routes and building entrances and conveyed through drainage facilities or green spaces. | [4,9,18,28,42,45,48,49,74,79] |
| I15 | Drainage operation continuity | Assesses whether inlets, drainage ditches, and linear drainage facilities operate effectively during rainstorm events. | [4,9,18,28,42,45,48,49,74,79] |
| I16 | Control capacity for flood-prone locations | Evaluates whether low-lying areas, underground entrances, and roof-edge runoff points avoid backflow, concentrated discharge, or local water impact. | [4,9,13,42,45,74,76] |
| I17 | Rainy-day main-route accessibility | Assesses whether high-frequency routes connecting dormitories, teaching buildings, canteens, and libraries remain accessible during rainfall. | [12,13,18,76] |
| I18 | Continuity of rain-sheltered passage | Assesses whether corridors, canopies, pilotis, entrance porches, and related spaces form a continuous rain-sheltered passage system. | [4,11,12,13,18,19,20,22,33] |
| I19 | Controllability of detour cost | Evaluates whether detour distance, travel time, and route complexity remain acceptable when main paths are ponded or closed. | [12,13,18,76] |
| I20 | Peak-period path capacity | Assesses whether path capacity remains sufficient under umbrellas, bidirectional flows, carried objects, and concentrated peak-period pedestrian flow. | [4,13,76] |
| I21 | Continuity of barrier-free passage | Assesses whether barrier-free ramps and gently sloped routes remain continuous and usable during rainstorm events. | [4,13,76] |
| I22 | Pedestrian–vehicle conflict control | Evaluates whether conflicts among pedestrians, bicycles, electric bikes, and logistics vehicles remain controllable on rainy days. | [4,13,76] |
| I23 | Rain-shelter buffer capacity | Assesses whether entrances to teaching buildings, dormitories, canteens, and libraries are prone to ponding or backflow under rain-shelter conditions. | [4,11,12,18,19,20,22,33,74] |
| I24 | Sheltered waiting space at entrances | Assesses whether entrance areas provide sufficient sheltered space for umbrella handling, short waiting, queuing, and rain avoidance. | [4,11,12,18,19,20,22,33,74] |
| I25 | Ground anti-slip safety | Evaluates whether entrance ground surfaces can control slipperiness caused by rainwater, sediment, and dripping umbrellas. | [4,19,42,45,55,58,74,76] |
| I26 | Operational maintenance of core nodes | Assesses whether high-frequency nodes such as teaching buildings, canteens, dormitories, and libraries maintain use during rainy conditions. | [4,11,12,13,19] |
| I27 | Support for ancillary service nodes | Assesses whether shuttle stops, bus stops, bicycle parking points, delivery nodes, maintenance points, and cleaning-service nodes maintain basic operation and safe transfer. | [4,13,19,20,38,42] |
| I28 | Rainwater retention and peak attenuation | Assesses whether sunken green spaces, rain gardens, retention basins, and related facilities retain rainwater and attenuate runoff peaks. | [18,42,44,45,46,47,49,51,58,77] |
| I29 | Infiltration and conveyance efficiency | Assesses whether permeable pavement, grassed swales, shallow channels, and related measures promote infiltration, organize surface runoff, and reduce local ponding. | [18,42,44,45,46,47,49,51,58,77] |
| I30 | Facility-system coordination | Assesses whether green spaces, inlets, drainage ditches, rain gardens, and permeable pavement form continuous connections that prevent isolated facility failure. | [4,18,42,44,45,46,47,49,50] |
| I31 | Differentiated mobility support | Assesses whether users with mobility limitations, users carrying objects, cycling-transfer users, and night-returning students receive safe, continuous, and legible mobility support. | [4,13,25,27,38,42] |
| I32 | Drainage and functional recovery time | Measures the time required for main paths, entrances, plazas, and service nodes to recover to a condition without evident ponding. | [4,20,22,37,42] |
| I33 | Recovery of spatial accessibility | Assesses whether spaces temporarily closed or suspended because of ponding, backflow, or control measures are reopened in a timely manner. | [4,19,20,22,37,42,48,49,77] |
| I34 | Recovery of ground safety | Assesses whether post-rain safety hazards such as slippery surfaces, sediment, fallen leaves, and damaged pavement are addressed in a timely manner. | [4,19,22,42,48,49,58,77] |
| I35 | Post-rain inspection and sediment removal | Assesses whether post-rain inspections identify ponding points, blocked facilities, and damaged locations, and whether drainage sediment is removed. | [4,25,38,42,47,48,49,53,77] |
| I36 | Closed-loop maintenance and rectification | Assesses whether stormwater-related problems follow a closed-loop process of discovery, recording, dispatch, repair, review, and repeated-problem rectification. | [4,22,28,29,37,42,43,50,68] |
| I37 | Review feedback and data accumulation | Assesses whether post-event reviews summarize spatial failure, route congestion, and facility performance while collecting feedback, ponding locations, drainage recovery time, and maintenance records. | [11,19,25,27,42,43,53,68] |