E-mail: helensilva@ifsul.edu.br

Programa de Pós-graduação em Engenharia e Ciências Ambientais, Instituto Federal de Educação, Ciência e Tecnologia Sul-rio-grandense (IFSul), Pelotas, RS, Brazil

Corresponding author: Helen Tais da Rosa-Silva

Daniel Ricardo Arsand; Fabiane Araújo Caldasso; Érico Kunde Correa; Michel David Gerber; Helen Tais da Rosa-Silva

Environmental screening of dewatered fish-processing wastewater sludge for land application: species-specific phytotoxicity and exposure limitations

Online Resource 1

Water, Air, & Soil Pollution

# Supplementary Information

Environmental screening of dewatered fish-processing wastewater sludge for land application: species-specific phytotoxicity and exposure limitations

Scope note. Tables S1–S3 reproduce physicochemical values reported in the master's dissertation that generated the historical experiment (Anonymous, 2018). Because pot-level raw chemistry, analytical variance, detection limits, and complete unit metadata could not be recovered, the values are shown as reported and used only descriptively. They were not used for inferential tests, chemistry–plant correlations, regulatory-compliance claims, or land-application recommendations. Table S4 was generated from the reanalysis of the supplied plant-level raw workbooks. Table S5 reproduces the recoverable lime and mineral-fertilizer quantities reported in the source dissertation (Anonymous, 2018); it does not resolve the missing sludge doses for T2 and T4.

Treatments: T1 = lime-amended soil control; T2 = lime + sludge; T3 = lime + mineral NPK; T4 = sludge without lime. NR = not reported in the source. A recorded zero must not be interpreted as a non-detect unless an analytical detection limit is supplied.

# Table S1. Initial soil physicochemical characterization

| Parameter | Unit as reported | Value | Verification note |
| --- | --- | --- | --- |
| pH | unitless | 5.0 | Matrix described as initial soil in the methods. |
| Total phosphorus | mg kg⁻¹ P | 0.5 | Unit as reported. |
| Total Kjeldahl nitrogen | mg kg⁻¹ N | 4.8 | Unit as reported. |
| Electrical conductivity | µS cm⁻¹ | 22.8 | Unit as reported. |
| Organic matter | mg kg⁻¹ KMnO₄ | 35.0 | Unusual unit; verify against laboratory report. |
| Al | mg kg⁻¹ | 616.7 | As reported. |
| Cd | mg kg⁻¹ | 4.2 | The reported Cd concentration exceeds the 3.0 mg kg⁻¹ agricultural investigation value cited in the source table for CONAMA Resolution 420/2009. This was a pre-existing initial-soil condition and cannot be attributed to sludge application. |
| Ca | mg kg⁻¹ | 0.3 | As reported. |
| Pb | mg kg⁻¹ | NR | No result recovered. |
| Cu | mg kg⁻¹ | NR | No result recovered. |
| Cr | mg kg⁻¹ | NR | No result recovered. |
| Zn | mg kg⁻¹ | 70.3 | As reported. |
| Fe | mg kg⁻¹ | 373.3 | As reported. |
| Mn | mg kg⁻¹ | 13.3 | As reported. |
| Ni | mg kg⁻¹ | 22.6 | As reported. |
| Mg | mg kg⁻¹ | 0.2 | As reported. |
| Na | mg kg⁻¹ | 11.3 | As reported. |
| K | mg kg⁻¹ | 0.9 | As reported. |

Source: Caldasso (2018), Table 2, transcribed without reinterpretation. The original caption contains a matrix-label inconsistency, and complete laboratory metadata were not recovered; the values are presented only descriptively.

# Table S2. Post-harvest substrate chemistry for lettuce

| Parameter | Unit as reported | T1 | T2 | T3 | T4 |
| --- | --- | --- | --- | --- | --- |
| pH | unitless | 7.18 | 6.69 | 7.09 | 6.47 |
| Total phosphorus | mg kg⁻¹ P | 0.34 | 0.44 | 0.39 | 0.38 |
| Total Kjeldahl nitrogen | mg kg⁻¹ N | 34.85 | 46.53 | 36.67 | 45.00 |
| Electrical conductivity | µS cm⁻¹ | 115.03 | 152.82 | 225.43 | 288.30 |
| Organic matter | mg kg⁻¹ KMnO₄ | 58.52 | 40.68 | 28.23 | 59.10 |
| Al | mg kg⁻¹ | 0.00 | 0.00 | 0.00 | 0.00 |
| Cd | mg kg⁻¹ | 0.00 | 0.00 | 0.00 | 0.00 |
| Ca | mg kg⁻¹ | 38.50 | 57.15 | 31.50 | 46.00 |
| Cu | mg kg⁻¹ | 0.00 | 0.00 | 0.00 | 0.00 |
| Cr | mg kg⁻¹ | 0.07 | 0.13 | 0.03 | 0.14 |
| Zn | mg kg⁻¹ | 0.00 | 0.00 | 0.00 | 0.00 |
| Fe | mg kg⁻¹ | 0.00 | 0.00 | 0.00 | 0.00 |
| Mn | mg kg⁻¹ | 0.00 | 0.00 | 0.00 | 0.00 |
| Ni | mg kg⁻¹ | 0.00 | 0.00 | 0.00 | 0.00 |
| Mg | mg kg⁻¹ | 27.32 | 30.50 | 17.17 | 31.00 |
| Na | mg kg⁻¹ | 12.48 | 12.50 | 12.83 | 10.00 |
| K | plk | 1.46 | 1.20 | 1.28 | 1.00 |

Source: Caldasso (2018), Table 11. Values are treatment means as reported; n, analytical dispersion, and pot-level linkage were not available. The unit “plk” recorded for K could not be verified and is reproduced as reported.

# Table S3. Post-harvest substrate chemistry for arugula

| Parameter | Unit as reported | T1 | T2 | T3 | T4 |
| --- | --- | --- | --- | --- | --- |
| pH | unitless | 7.83 | 7.62 | 7.09 | 6.54 |
| Total phosphorus | mg kg⁻¹ P | 0.067 | 0.49 | 0.13 | 0.36 |
| Total Kjeldahl nitrogen | mg kg⁻¹ N | 3.77 | 5.53 | 2.70 | 21.50 |
| Electrical conductivity | µS cm⁻¹ | 150.47 | 445.50 | 393.33 | 153.53 |
| Organic matter | mg kg⁻¹ KMnO₄ | 38.58 | 22.40 | 31.50 | 62.33 |
| Al | mg kg⁻¹ | 0.00 | 0.00 | 0.00 | 0.00 |
| Cd | mg kg⁻¹ | 0.00 | 0.00 | NR | NR |
| Ca | mg kg⁻¹ | 11.02 | 73.00 | 50.00 | 98.67 |
| Cu | mg kg⁻¹ | NR | 0.00 | NR | 0.00 |
| Cr | mg kg⁻¹ | 0.006 | 0.010 | 0.010 | 0.010 |
| Zn | mg kg⁻¹ | 0.044 | 0.00 | 0.00 | 0.00 |
| Fe | mg kg⁻¹ | 0.12 | 0.00 | 0.00 | 0.00 |
| Mn | mg kg⁻¹ | 0.00 | 0.00 | 0.00 | 0.00 |
| Ni | mg kg⁻¹ | 0.00 | 0.00 | 0.00 | 0.00 |
| Mg | mg kg⁻¹ | 17.82 | 3.89 | 7.51 | 27.33 |
| Na | mg kg⁻¹ | 10.67 | 11.00 | 11.50 | 11.17 |
| K | mg kg⁻¹ | 1.03 | 1.18 | 1.23 | 1.08 |

Source: Caldasso (2018), Table 14. Values are treatment means as reported; NR denotes a blank or unrecovered entry, not a zero.

# Table S4. Complete greenhouse endpoint summary and treatment tests

## Panel S4A. Lactuca sativa

| Endpoint | T1 mean ± SD | T2 mean ± SD | T3 mean ± SD | T4 mean ± SD | p | BH p | Effect |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Leaf number | 49.33 ± 5.47 (a) | 38.83 ± 9.15 (a) | 48.00 ± 12.36 (a) | 40.17 ± 5.15 (a) | 0.0930 | 0.3254 | η² = 0.293 |
| Max. leaf length (cm) | 13.17 ± 1.47 (a) | 13.83 ± 1.72 (a) | 13.00 ± 3.92 (a) | 12.00 ± 4.15 (a) | 0.7594 | 0.8809 | η² = 0.062 |
| Shoot fresh mass (g) | 91.66 ± 15.37 (a) | 84.26 ± 42.77 (a) | 106.41 ± 73.46 (a) | 91.82 ± 34.14 (a) | 0.8809 | 0.8809 | η² = 0.035 |
| Shoot dry mass (g) | 8.70 ± 2.32 (a) | 5.81 ± 3.09 (a) | 7.10 ± 4.73 (a) | 5.71 ± 1.81 (a) | 0.2965 | 0.4150 | η² = 0.181 |
| Root length (cm) | 25.83 ± 6.05 (a) | 18.00 ± 7.10 (ab) | 20.50 ± 11.73 (ab) | 11.00 ± 2.90 (b) | 0.0151 | 0.1059 | η² = 0.432 |
| Root fresh mass (g) | 24.05 ± 5.69 (a) | 19.59 ± 8.74 (a) | 17.74 ± 14.03 (a) | 14.73 ± 3.75 (a) | 0.2908 | 0.4150 | η² = 0.183 |
| Root dry mass (g) | 6.02 ± 1.93 (a) | 4.11 ± 2.68 (a) | 3.85 ± 4.04 (a) | 2.71 ± 0.97 (a) | 0.1689 | 0.3941 | η² = 0.239 |

Letters in parentheses are endpoint-specific post-hoc groupings; treatments sharing a letter were not different in the selected pairwise procedure. BH p is the Benjamini–Hochberg-adjusted value across the seven endpoints within species. Global test: One-way ANOVA

## Panel S4B. Eruca sativa

| Endpoint | T1 mean ± SD | T2 mean ± SD | T3 mean ± SD | T4 mean ± SD | Global test | p | BH p | Effect |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Leaf number | 5.33 ± 1.75 (b) | 28.83 ± 9.50 (a) | 24.80 ± 12.93 (a) | 21.50 ± 10.71 (a) | One-way ANOVA; F = 7.00 | 0.0023 | 0.0027 | η² = 0.525 |
| Max. leaf length (cm) | 9.93 ± 1.60 (b) | 22.92 ± 1.96 (a) | 11.62 ± 2.12 (b) | 18.32 ± 4.79 (a) | One-way ANOVA; F = 24.45 | <0.0001 | <0.0001 | η² = 0.794 |
| Shoot fresh mass (g) | 3.49 ± 1.50 (b) | 61.67 ± 8.58 (a) | 13.94 ± 6.59 (ab) | 40.57 ± 27.17 (a) | Kruskal-Wallis; H = 17.64 | 0.0005 | 0.0012 | ε² = 0.770 |
| Shoot dry mass (g) | 0.65 ± 0.19 (c) | 6.35 ± 1.18 (a) | 1.49 ± 1.18 (bc) | 3.25 ± 2.13 (ab) | Kruskal-Wallis; H = 16.71 | 0.0008 | 0.0014 | ε² = 0.722 |
| Root length (cm) | 7.90 ± 1.60 (c) | 40.17 ± 12.51 (a) | 10.82 ± 3.89 (c) | 26.50 ± 8.02 (b) | One-way ANOVA; F = 21.08 | <0.0001 | <0.0001 | η² = 0.769 |
| Root fresh mass (g) | 2.26 ± 3.01 (a) | 8.76 ± 2.20 (b) | 17.25 ± 36.30 (ab) | 5.99 ± 5.59 (ab) | Kruskal-Wallis; H = 9.13 | 0.0277 | 0.0277 | ε² = 0.322 |
| Root dry mass (g) | 0.30 ± 0.25 (b) | 3.02 ± 1.15 (a) | 0.25 ± 0.10 (b) | 0.82 ± 0.43 (b) | Welch ANOVA; F = 12.16 | 0.0018 | 0.0026 | η² = 0.796 |

Letters in parentheses are endpoint-specific post-hoc groupings; treatments sharing a letter were not different in the selected pairwise procedure. BH p is the Benjamini–Hochberg-adjusted value across the seven endpoints within species.

Sample sizes: lettuce T1/T2/T4, n = 6 and T3, n = 4; arugula T1/T2/T4, n = 6 and T3, n = 5. Arugula root dry mass had n = 22 because one negative derived value was treated as missing. The primary arugula root-fresh-mass test retained the supplied 82.1866-g value; sensitivity analysis excluding it remained significant (Kruskal–Wallis H = 13.27, p = 0.0041, ε² = 0.571).

# Table S5. Recorded lime and mineral-fertilizer quantities in the greenhouse protocol

Values and units are reproduced from Caldasso (2018). Urea, triple superphosphate, and KCl correspond to T3. The lime quantities apply to the lime-amended treatments. These records improve protocol transparency but do not recover the sludge amounts applied in T2 or T4 and do not establish N-equivalence.

| Amendment | Arugula | Lettuce |
| --- | --- | --- |
| Urea (g L⁻¹) | 1.0 | 1.0 |
| Triple superphosphate (g L⁻¹) | 1.3 | 1.3 |
| KCl (g L⁻¹) | 1.0 | 1.0 |
| Lime, PRNT 60.39 (g L⁻¹) | 31.85 | 43.05 |

Reference

Caldasso, F. A. (2018). Avaliação da fitoxicidade de lodo de estação de tratamento de efluentes de indústria de pescado utilizando olerícolas como bioindicadores [Master’s dissertation, Universidade Federal de Pelotas]. Repositório Institucional da UFPel. http://guaiaca.ufpel.edu.br/xmlui/handle/prefix/18641