Table 1

| No. | Factors | Data Source | Resolution | Data Type |
| --- | --- | --- | --- | --- |
| I | Topographic and Hydrological Factors |  |  |  |
| 1 – (A) | Digital Elevation Model (DEM) | ASTER GDEM | 30 × 30 m grid | Continuous |
| 2 – (B) | Slope gradient | Derived from DEM | 30 × 30 m grid | Continuous |
| 3 – (C) | Horizontal dissection | Derived from DEM | 30 × 30 m grid | Continuous |
| 4 – (D) | Vertical dissection | Derived from DEM | 30 × 30 m grid | Continuous |
| 5 – (E) | Topographic Wetness Index (TWI) | Derived from DEM | 30 × 30 m grid | Continuous |
| 6 – (F) | Drainage density | Derived from DEM | 30 × 30 m grid | Continuous |
| II | Geological and Endogenous Factors |  |  |  |
| 7 – (G) | Geology | Geological Map | Map scale 1:50,000 | Categorical |
| 8 – (H) | Tectonic units | Tectonic maps of Bac Giang and vicinity | Map scale 1:50,000 | Categorical |
| 9 – (I) | Fault density | Geological Map | Map scale 1:50,000 | Continuous |
| 10 – (J) | Shear zone | Tectonic structure map of Bac Giang and vicinity | Map scale 1:50,000 | Categorical |
| 11 – (K) | Weathering crust | Tectonic structure map of Bac Giang and vicinity | Map scale 1:50,000 | Categorical |
| 12 – (L) | Soil type | Soil map of Bac Giang and vicinity | Map scale 1:50,000 | Categorical |
| III | Environmental and Climatic Conditioning Factors |  |  |  |
| 13 – (M) | Rainfall | Google Earth Engine (CHIRPS data) | 5,5 × 5,5 km grid | Continuous |
| 14 – (N) | NDVI | Sentinel-2 Satellite Imagery | 10 × 10 m grid | Continuous |
| 15 – (O) | Land use | Land use status map of Bac Giang and vicinity | Map scale 1:50,000 | Categorical |
| 16 – (P) | Road distance | OpenStreetMap | 30 × 30 m grid | Continuous |

### Table 2

| ### No | ### Factor | ### VIF | ### TOL |
| --- | --- | --- | --- |
| ### 1 | ### Shear zone partitioning | ### 4.224 | ### 0.237 |
| ### 2 | ### Digital Elevation Model (DEM) | ### 3.95 | ### 0.253 |
| ### 3 | ### Weathering crust | ### 3.658 | ### 0.273 |
| ### 4 | ### Rainfall | ### 2.699 | ### 0.371 |
| ### 5 | ### Soil type | ### 2.575 | ### 0.388 |
| ### 6 | ### Vertical dissection | ### 2.138 | ### 0.468 |
| ### 7 | ### Tectonic units | ### 1.852 | ### 0.54 |
| ### 8 | ### Geology | ### 1.682 | ### 0.595 |
| ### 9 | ### Road distance | ### 1.515 | ### 0.66 |
| ### 10 | ### Drainage density | ### 1.505 | ### 0.665 |
| ### 11 | ### Fault density | ### 1.247 | ### 0.802 |
| ### 12 | ### Horizontal dissection | ### 1.232 | ### 0.812 |
| ### 13 | ### Slope gradient | ### 1.229 | ### 0.814 |
| ### 14 | ### Topographic Wetness Index (TWI) | ### 1.149 | ### 0.871 |
| ### 15 | ### Land use | ### 1.141 | ### 0.877 |
| ### 16 | ### Normalized Difference Vegetation Index (NDVI) | ### 1.131 | ### 0.884 |

Table 3

| Parent Factor Layers | Number of Expanded Dummy Variables | Adjusted GVIF | Status |
| --- | --- | --- | --- |
| Lithology | 8 | 1.033 | Stable |
| Tectonic units | 3 | 1.135 | Stable |
| Shear zone | 3 | 1.544 | Stable |
| Strict Filtering Threshold | - | < 3.0 | Highly Robust System |

Table 4.

| Strategy | Model | Scenario | AUC | ACC | Kappa | Recall | Precision |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Random | RF | Random_RF | 0.935 | 0.867 | 0.735 | 0.876 | 0.861 |
| Random | XGBoost | Random_XGBoost | 0.923 | 0.857 | 0.713 | 0.876 | 0.843 |
| Random | LightGBM | Random_LightGBM | 0.926 | 0.859 | 0.717 | 0.867 | 0.854 |
| Random | DNN | Random_DNN | 0.838 | 0.740 | 0.479 | 0.814 | 0.721 |
| Random | Stacking_Full4 | Random_Stacking_Full4 | 0.934 | 0.867 | 0.735 | 0.892 | 0.851 |
| Random | Stacking_Trees3 | Random_Stacking_Trees3 | 0.936 | 0.866 | 0.733 | 0.893 | 0.848 |
| Random | Stacking_Hybrid3 | Random_Stacking_Hybrid3 | 0.934 | 0.867 | 0.735 | 0.889 | 0.853 |
| Slope-IOE | RF | Slope-IOE_RF | 0.971 | 0.913 | 0.826 | 0.900 | 0.925 |
| Slope-IOE | XGBoost | Slope-IOE_XGBoost | 0.968 | 0.906 | 0.812 | 0.903 | 0.908 |
| Slope-IOE | LightGBM | Slope-IOE_LightGBM | 0.967 | 0.904 | 0.809 | 0.896 | 0.911 |
| Slope-IOE | DNN | Slope-IOE_DNN | 0.848 | 0.750 | 0.500 | 0.831 | 0.721 |
| Slope-IOE | Stacking_Full4 | Slope-IOE_Stacking_Full4 | 0.972 | 0.906 | 0.813 | 0.912 | 0.902 |
| Slope-IOE | Stacking_Trees3 | Slope-IOE_Stacking_Trees3 | 0.972 | 0.907 | 0.814 | 0.912 | 0.903 |
| Slope-IOE | Stacking_Hybrid3 | Slope-IOE_Stacking_Hybrid3 | 0.971 | 0.910 | 0.819 | 0.918 | 0.903 |
| PU-Bagging | RF | PU-Bagging_RF | 0.979 | 0.928 | 0.857 | 0.938 | 0.920 |
| PU-Bagging | XGBoost | PU-Bagging_XGBoost | 0.976 | 0.925 | 0.849 | 0.922 | 0.927 |
| PU-Bagging | LightGBM | PU-Bagging_LightGBM | 0.978 | 0.930 | 0.861 | 0.931 | 0.930 |
| PU-Bagging | DNN | PU-Bagging_DNN | 0.884 | 0.778 | 0.555 | 0.743 | 0.824 |
| PU-Bagging | Stacking_Full4 | PU-Bagging_Stacking_Full4 | 0.980 | 0.931 | 0.862 | 0.943 | 0.921 |
| PU-Bagging | Stacking_Trees3 | PU-Bagging_Stacking_Trees3 | 0.981 | 0.932 | 0.864 | 0.947 | 0.920 |
| PU-Bagging | Stacking_Hybrid3 | PU-Bagging_Stacking_Hybrid3 | 0.980 | 0.931 | 0.863 | 0.943 | 0.922 |

Table 5

| Sampling Strategy | Machine Learning Models | Spatial AUC Mean | System Diagnostics |
| --- | --- | --- | --- |
| 1. Random | Random Forest (RF) | 0.9144 | This accurately reflects objective reality. Due to random sampling, the negative set is susceptible to contamination by unrecorded potential landslide locations (pseudo-negatives), slightly diminishing macro-level spatial classification performance |
|  | XGBoost | 0.9141 |  |
|  | LightGBM | 0.9155 |  |
|  | Deep Neural Network (DNN) | 0.8315 |  |
|  | Stacking_Trees3 (Proposed) | 0.9186 |  |
| 2. Slope-IOE | Random Forest (RF) | 0.9589 | Significant performance surge. Isolating negative samples within gentle-slope areas (<21.30) combined with low Index of Entropy (IOE) values effectively sharpens the decision boundary between safe and hazardous zones |
|  | XGBoost | 0.9570 |  |
|  | LightGBM | 0.9568 |  |
|  | Deep Neural Network (DNN) | 0.8524 |  |
|  | Stacking_Trees3 (Proposed) | 0.9625 |  |
| 3. PU-Bagging | Random Forest (RF) | 0.9976 | Peak and optimal performance. The semi-supervised iterative filtering algorithm successfully purified the negative sample pool, completely isolating noise to retain truly stable grid cells |
|  | XGBoost | 0.9981 |  |
|  | LightGBM | 0.9972 |  |
|  | Deep Neural Network (DNN) | 0.9932 |  |
|  | Stacking_Trees3 (Proposed) | 0.9982 |  |

### Table 6

| Slope Class | Number of Landslides (N) | Recall (Threshold >0.93) | Interpretation |
| --- | --- | --- | --- |
| Gentle (< 10°) | 280 | 97.86% | High recognition despite low topographic energy. |
| Moderate (10°-25°) | 413 | 99.52% | Optimal performance in the target midland terrain. |
| Steep (> 25°) | 269 | 99.63% | Expected high performance in high-risk zones. |

### Table 7

| Model Comparison Pair | AUC (Model 1) | AUC (Model 2) | Z-score | p-value | Statistical Conclusion |
| --- | --- | --- | --- | --- | --- |
| Stacking_Trees3 vs. Stacking_Full4 | 0.9859 | 0.9859 | -0.0198 | 0.9842 | No significant difference (p ≥ 0.05) |
| Stacking_Trees3 vs. Stacking_Hybrid3 | 0.9859 | 0.9856 | 1.6711 | 0.0947 | No significant difference (p ≥ 0.05) |

### Table 8

| Susceptibility Level | Number of Grid Cells | Number of Landslides | Landslide Density | Density Ratio |
| --- | --- | --- | --- | --- |
| Low / Moderate (P ≤ 0.93) | 2,897,055 | 119 | 0.000041 | 1.0 |
| High (0.93 < P ≤ 0.97) | 201,488 | 249 | 0.001236 | 30.1 |
| Very High (P > 0.97) | 72,841 | 831 | 0.011408 | 278.2 |