# Appendix A

Table A1 presents the full sensitivity analysis used to evaluate the ΔNDVI threshold and sequential terrain-based filtering parameters applied during development of the landslide mapping workflow.

Table A1 Sensitivity analysis of ΔNDVI thresholds and sequential terrain-based filtering parameters evaluated during development of the landslide mapping workflow.

| Filtering stage | Scenario / threshold | No. of polygons | Total area (km²) | Area metric (%) | Decision |
| --- | --- | --- | --- | --- | --- |
| ΔNDVI threshold | < −0.05 | 329,844 | 189.71 | 10.14 | Not selected – too permissive |
|  | < −0.10 | 124,932 | 72.97 | 3.90 | Selected |
|  | < −0.15 | 46,542 | 29.20 | 1.56 | Not selected – more restrictive |
|  | < −0.20 | 19,808 | 12.74 | 0.68 | Not selected – more restrictive |
|  | < −0.25 | 9,778 | 5.36 | 0.29 | Not selected – removed true landslides |
| River buffer | 0 m | 124,932 | 72.97 | 0.00 | Baseline |
|  | 25 m | 117,354 | 66.02 | 9.52 | Not selected – limited filtering |
|  | 50 m | 110,965 | 62.51 | 14.34 | Selected |
|  | 75 m | 106,518 | 59.49 | 18.48 | Not selected – more restrictive |
|  | 100 m | 102,955 | 56.79 | 22.18 | Not selected – removed true landslides |
| Elevation filter | >0 m | 110,965 | 62.51 | 0.00 | Baseline |
|  | >25 m | 99,592 | 46.35 | 25.85 | Not selected – limited filtering |
|  | >50 m | 88,475 | 37.66 | 39.80 | Not selected – limited filtering |
|  | >75 m | 77,961 | 31.07 | 50.30 | Not selected – limited filtering |
|  | >100 m | 66,731 | 25.31 | 59.50 | Selected |
|  | >125 m | 55,650 | 19.92 | 68.13 | Not selected – removed true landslides |
| Slope filter | River50 + Elev100 | 66,731 | 25.31 | 0.00 | Baseline |
|  | Slope >2° | 66,178 | 23.89 | 5.61 | Not selected – limited filtering |
|  | Slope >5° | 63,086 | 20.76 | 17.98 | Selected |
|  | Slope >7.5° | 58,442 | 17.31 | 31.61 | Not selected – more restrictive |
|  | Slope >10° | 52,372 | 13.75 | 45.67 | Not selected – removed true landslides |
| Minimum mapping unit (MMU) | 200 m² | 18,202 | 16.12 | 22.35 | Not selected – initial removal of many small, isolated patches |
|  | 400 m² | 7,510 | 13.31 | 35.89 | Not selected – high number of small, isolated features remained |
|  | 500 m² | 5,739 | 12.53 | 39.64 | Not selected – initial improvement removing small features |
|  | 600 m² | 4,597 | 11.91 | 42.63 | Selected |
|  | 800 m² | 3,219 | 10.97 | 47.16 | Not selected – removed true landslides |
| Manual validation | Before manual validation (MMU ≥600 m²) | 4,597 | 11.91 | — | Baseline |
|  | Final inventory after manual validation | 3,689 | 6.91 | 42.00 | Final inventory |

Note: For the ΔNDVI threshold analysis, area metric represents mapped area relative to the study area. For subsequent filtering stages, it represents area reduction relative to the baseline of each stage.