# Appendix

Table 7: Results from Prompt 1

| MAVIS |  |  | Copilot |  |  | DeepSeek |  |  | ChatGPT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ASIL | Safety Goals |  | ASIL | Safety Goals |  | ASIL | Safety Goals |  | ASIL | Safety Goals |
| M1 | D | The occupant protection system shall prevent any unintended deployment of restraint devices (airbags, curtain airbags, pretensioners) during vehicle operation. Faults that could lead to inadvertent actuation shall be detected; affected firing loops shall be inhibited, the crash message shall not be sent, and the driver shall be warned | CP1 | D | Ensure airbag deployment within 30 ms of crash detection | DS1 | D | Deploy frontal restraints | C1 | D | The IOP system shall reliably detect crashes and deploy the required airbags in a timely and correct manner to mitigate life-threatening or fatal injuries |
| M2 | C | The system shall detect relevant crash conditions and command timely deployment of the required restraint devices to provide occupant protection. | CP2 | C | Activate pretensioner before peak crash force | DS2 | C | Deploy side restraints | C2 | D | The IOP system shall activate belt pretensioners according to crash severity and direction to restrain occupants during crash events |
| M3 | C | The system shall ensure sufficient energy reserve and firing circuit integrity to complete all required deployments, even with supply disturbances caused by a crash | CP3 | B | Prevent unintended deployment through sensor plausibility checks | DS3 | B | Deploy rollover restraints | C3 | B | The IOP system shall prevent unintended or incorrect deployment of airbags and belt pretensioners outside defined crash conditions. |
| M4 | C | The system shall ensure that passenger airbag enabling/suppression is correct for the actual occupant status. Faults in occupancy/buckle sensors or classification that could lead to incorrect suppression for an adult shall be detected and mitigated | CP4 | D | Crash detection algorithm must meet timing constraints (<10 ms) | DS4 | B | Prevent unintended deployment | C4 | B | The IOP system shall reliably provide crash signals to other vehicle systems to enable post-crash functions |
| M5 | C | The system shall prevent unintended transmission of “crash” messages to other ECUs in the absence of a detected crash |  | - | - | DS5 | A | Provide post-crash signals | C5 | - | - |
|  | - | - |  | - | - | DS6 | D | Indicate deployment faults |  | - | - |

Table 8: Results from Prompt 2

| MAVIS |  |  | Copilot |  |  | Deep Seek |  |  | ChatGPT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ASIL | Safety Goals |  | ASIL | Safety Goals |  | ASIL | Safety Goals |  | ASIL | Safety Goals |
| M1 | D | Ensure timely and sufficient deployment of required restraints (airbags, pretensioners) for relevant crash types and severities to reduce injury risk to occupants (driver and passengers). | CP1 | D | Ensure timely and appropriate deployment of driver and passenger airbags (including stage/vent strategies, curtain, side, knee) for relevant crash directions and severities. | DS1 | D | Ensure deployment of required airbag(s) in case of a crash | C1 | D | The system shall correctly detect crash severity and direction within the required timing. |
| M2 | D | Prevent unintended deployment of any restraint during normal driving and non-deployment crash types (e.g., rear crash airbags; see item policy). | CP2 | D | Prevent inadvertent deployment of any airbag during non-crash conditions and ensure suppression where required (e.g., infant seat presence). | DS2 | C | Prevent unintended deployment of airbag(s) | C2 | D | The system shall command deployment of all relevant airbags when required and only when required. |
| M3 | D | Determine crash direction and severity correctly and within the required time to support SG1/SG2 decisions. | CP3 | D | Ensure timely activation of belt pretensioners and correct operation of load limiters for all occupied seats under relevant crash conditions. | DS3 | A | Ensure activation of belt pretensioner(s) in case of a crash | C3 | D | The system shall command activation of all relevant belt pretensioners when required and only when required. |
| M4 | D | Provide firing decisions and execute firing within the fault-tolerant time interval for each crash mode (front, side, rollover) so that deployment meets performance thresholds. | CP4 | C/B | Ensure accurate crash classification (direction and severity) and reliable provision of crash signals via CAN and redundant PWM for post-crash functions. | DS4 | D | Ensure correct crash classification and direction detection | C4 | C/D | Passenger airbag deactivation logic shall reliably detect child seats and adult occupants and enable/disable the airbag appropriately. |
| M5 | D | Ensure correct activation/deactivation of passenger front airbag based on occupant classification and/or manual deactivation, with correct status indication. | CP5 | B | Ensure correct operation of Passenger Airbag ON/OFF (deactivation) logic and OCS to avoid harm to infants and ensure protection for adults; provide clear HMI indication. | DS5 | C | Ensure passenger airbag is deactivated when child seat is detected | C5 | B/C | The crash signal (CAN + PWM redundant) shall be sent correctly and only when a real crash is detected. |
| M6 | B | Provide crash classification via CAN and redundant PWM crash yes/no robustly and in time to enable other items’ post-crash functions. | CP6 | B/A | Ensure that faults in sensors, power, or communication are detected and that the system transitions to a defined safe state (with driver warning) before or during operation. | DS6 | QM | Ensure crash signals are sent for post-crash functions | C6 | D | The system shall ensure no single point failure leads to loss of deployment capability. |
| M7 | B | Provide accurate airbag warning lamp and passenger airbag ON/OFF indication in case of detected malfunction or deactivation status. |  |  |  |  |  |  | C7 | B | The system shall detect internal faults and inform driver via airbag warning light. |
| M8 | D | Ensure that no single point failure can cause an unintended firing or prevent required firing for any single channel, and detect/latch critical faults with appropriate driver warning. |  |  |  |  |  |  |  |  |  |