# Supplementary material

Table S1  Electroanatomical mesh and graph characteristics per patient. Projection error is the distance from a mapping site to its nearest mesh vertex. The geodesic-to-Euclidean ratio is computed over retained graph edges.

| Patient | Mesh vertices | Faces | Nodes | Edges | Mean degree | Median edge (mm) | Geo/Eucl ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4268 | 8742 | 157 | 790 | 10.06 | 6.31 | 1.046 |
| 2 | 5121 | 10404 | 104 | 518 | 9.96 | 9.52 | 1.049 |
| 3 | 5670 | 11536 | 90 | 427 | 9.49 | 10.80 | 1.069 |
| 4 | 4084 | 8377 | 471 | 2380 | 10.11 | 2.92 | 1.000 |
| 5 | 3795 | 7779 | 46 | 232 | 10.09 | 12.23 | 1.087 |
| 6 | 5285 | 10566 | 839 | 4183 | 9.97 | 3.15 | 1.000 |
| 7 | 7193 | 14771 | 76 | 377 | 9.92 | 8.80 | 1.061 |
| 8 | 4020 | 8270 | 129 | 642 | 9.95 | 6.35 | 1.058 |
| 9 | 4014 | 8158 | 50 | 260 | 10.40 | 8.84 | 1.051 |
| Total / median | - | - | 1962 | 9809 | 9.97 | 8.80 | 1.005 |

Table S2  Ten most discriminative expert descriptors by univariate AUC (n = 1716 labelled electrograms). All 38 descriptors were significant after Benjamini-Hochberg correction. Direction indicates whether the descriptor increases or decreases in AVPs.

| Descriptor | AVP (median) | Physiol. (median) | AUC | Direction | Cliff's delta |
| --- | --- | --- | --- | --- | --- |
| Bipolar RMS (mV) | 0.045 | 0.176 | 0.973 | decrease | -0.946 |
| Peak-to-peak amplitude (mV) | 0.489 | 1.958 | 0.970 | decrease | -0.939 |
| log peak-to-peak amplitude | 0.398 | 1.085 | 0.970 | decrease | -0.939 |
| log CARTO voltage | 0.449 | 1.132 | 0.968 | decrease | -0.936 |
| CARTO bipolar voltage (mV) | 0.566 | 2.102 | 0.968 | decrease | -0.936 |
| Absolute maximum (mV) | 0.306 | 1.352 | 0.968 | decrease | -0.935 |
| Multiscale energy, level 3 | 0.083 | 0.808 | 0.941 | decrease | -0.883 |
| Peak-to-peak, 30-400 Hz (mV) | 0.307 | 1.345 | 0.938 | decrease | -0.876 |
| Multiscale energy, level 2 | 0.066 | 0.858 | 0.930 | decrease | -0.859 |
| Unipolar maximum negative slope | -300.3 | -844.7 | 0.918 | increase | 0.837 |

Table S3  AUC-ROC per held-out patient, averaged over three seeds. Patient 6 combines the highest mapping density with the highest AVP prevalence and is the hardest case for every deep model.

| Patient | Voltage rule | XGBoost | 1-D CNN | BiLSTM | Transformer | ST-GCN no volt. | ST-GCN |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.916 | 0.957 | 0.950 | 0.831 | 0.877 | 0.714 | 0.923 |
| 2 | 0.993 | 0.987 | 0.977 | 0.938 | 0.935 | 0.944 | 0.971 |
| 3 | 0.993 | 0.999 | 0.913 | 0.828 | 0.915 | 0.972 | 0.985 |
| 4 | 0.976 | 0.988 | 0.948 | 0.978 | 0.973 | 0.885 | 0.973 |
| 5 | 0.857 | 0.906 | 0.915 | 0.816 | 0.814 | 0.943 | 0.923 |
| 6 | 0.940 | 0.937 | 0.760 | 0.553 | 0.791 | 0.708 | 0.858 |
| 7 | 0.893 | 0.992 | 0.886 | 0.898 | 0.897 | 0.852 | 0.867 |
| 8 | 0.999 | 1.000 | 0.950 | 0.946 | 0.963 | 0.982 | 0.997 |
| 9 | 1.000 | 0.989 | 0.936 | 0.918 | 0.942 | 0.947 | 0.983 |

Fig. S1  Electroanatomical graph characteristics. a Mesh resolution and mapping density per patient on a logarithmic scale. b Median ratio of geodesic to Euclidean distance over retained graph edges; the two metrics nearly coincide at the scale of a k-nearest-neighbour neighbourhood, which explains why the corresponding ablation produced no measurable difference.

Fig. S2  Ten most discriminative expert descriptors ranked by univariate AUC. Descriptors related to signal amplitude are shown in orange. The dashed line marks the AUC of the proposed model, which a single descriptor already matches.

Fig. S3  AUC-ROC by held-out patient and model, averaged over three seeds. Patient 6 combines the highest mapping density with a high abnormal-potential prevalence and is the hardest case for every deep model, while the voltage rule handles it well.