Supplementary Material

Potentially Toxic Elements in Drainage Sediments of the Zang-Tabi Area, Northwest Cameroon and their Implications for Pollution, Ecotoxicological and Human Health Risk

Tibuck Joshua Ndang1 · Ngambu Aloysius1 Afahnwie · Olivier Anoh Njoh1 · Lemnyuy Prosper Yiika² · Bertrand Kehding Fomekong² · Gus Djibril Kouankap Nono²

Corresponding author: Tibuck Joshua Ndang, Department of Geology, Mining and Environmental Science, Faculty of Science, The University of Bamenda, P.O. Box 39, Bambili, Cameroon.

Contents

Tables: S1 Exposure parameter values used in the human health risk model; S2 Reference doses (RfD) and cancer slope factors (SF); S3 Contamination factor (CF), contamination degree (Cdeg), modified contamination degree (mCdeg) and pollution load index (PLI); S4 Enrichment factor (EF); S5 Geo-accumulation index (Igeo); S6 Ecological risk factor (ERF) and potential ecological risk index (PERI); S7 Toxic risk index (TRI); S8 Toxic units (TU). Figures: S1–S6 Sediment quality guideline comparison diagrams for the TEL, PEL, TEC, PEC, ERL and ERM benchmarks.

Table S1. Exposure parameter values used in the human health risk assessment (children and adults).

| Parameter | Definition | Unit | Children | Adults | Reference |
| --- | --- | --- | --- | --- | --- |
| EF | Exposure frequency | days year⁻¹ | 350 | 350 | USEPA (2025) |
| ED | Exposure duration | year | 6 | 30 | USEPA (2025) |
| BW | Body weight | kg | 15 | 70 | Liu et al. (2022) |
| IRⁱⁿᵍ | Soil ingestion rate | mg day⁻¹ | 200 | 100 | USEPA (2025) |
| SA | Exposed skin surface area | cm² | 1600 | 4350 | Liu et al. (2022) |
| SAAF | Skin adherence factor | mg cm⁻² | 0.2 | 0.7 | Liu et al. (2022) |
| PM₁₀ | Particulate matter concentration | mg m⁻³ | 0.11 | 0.11 | Liu et al. (2022) |
| DAIR | Daily air inhalation rate | m³ day⁻¹ | 7.63 | 20 | Liu et al. (2022) |
| PLAF | Pulmonary alveolar fraction | – | 0.9 | 0.9 | USEPA (2025) |
| FSPO | Fraction of soil-borne particles < 10 μm | – | 0.8 | 0.8 | USEPA (2025) |
| CF | Unit conversion factor | kg mg⁻¹ | 10⁻⁶ | 10⁻⁶ | USEPA (2025) |
| AT (non-carcinogenic) | Averaging time | days | 2190 | 10950 | ED × 365 |
| AT (carcinogenic) | Averaging time | days | 25550 | 25550 | 70 × 365 |
| ABS | Dermal absorption factor (As) | – | 0.03 | 0.03 | USEPA (2025) |
| ABS | Dermal absorption factor (all other metals) | – | 0.001 | 0.001 | USEPA (2025) |
| ABS | Dermal absorption factor (Cr) | – | 0.042 | 0.042 | USEPA (2025) |

Exposure parameter values after the exposure factor manual and soil guideline framework [1,2].

Table S2. Reference doses (RfD, mg kg⁻¹ day⁻¹) and cancer slope factors (SF, (mg kg⁻¹ day⁻¹)⁻¹) for the studied potentially hazardous metals.

| Metal | RfDⁱⁿᵍ | RfDᵈᵉʳᵐ | RfDⁱⁿʰ | SFⁱⁿᵍ | SFᵈᵉʳᵐ | SFⁱⁿʰ |
| --- | --- | --- | --- | --- | --- | --- |
| As | 3.00E–04 | 1.23E–04 | 3.01E–04 | 1.5 | 3.66 | 15.1 |
| Cd | 1.00E–03 | 1.00E–05 | 1.00E–03 | 6.1 | 20.0 | 6.3 |
| Cr | 3.00E–03 | 6.00E–05 | 2.86E–05 | 0.5 | 20.0 | 42.0 |
| Cu | 4.00E–02 | 1.20E–02 | 1.42E–02 | – | – | – |
| Hg | 3.00E–04 | 2.10E–05 | 8.60E–05 | – | – | – |
| Ni | 2.00E–02 | 5.40E–03 | 2.06E–02 | 1.7 | 2.1 | 0.84 |
| Pb | 3.50E–03 | 5.25E–04 | 3.52E–03 | 8.5E–03 | 8.5E–03 | – |
| Zn | 3.00E–01 | 6.00E–02 | 3.00E–01 | – | – | – |

“–” = not classified as carcinogenic by the USEPA. Pb inhalation slope factor unavailable. Values after the USEPA risk assessment framework [1,2].

Table S3. Contamination factor (CF) of potentially hazardous metals, contamination degree (Cdeg), modified contamination degree (mCdeg) and pollution load index (PLI) of the Zang-Tabi stream sediments.

| Sample | As | Cd | Cr | Cu | Hg | Ni | Pb | Th | Zn | Cdeg | mCdeg | PLI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SB01 | 0.096 | 0.033 | 6.39 | 0.181 | 0.18 | 0.282 | 0.52 | 3.38 | 0.5 | 11.56 | 1.28 | 0.38 |
| SN02 | 0.046 | 1.42 | 5.62 | 0.346 | 0.12 | 0.504 | 0.442 | 3.65 | 0.491 | 12.64 | 1.4 | 0.567 |
| SZM01 | 0.065 | 0.006 | 10.98 | 0.087 | 0.08 | 0.162 | 0.308 | 3.66 | 0.242 | 15.58 | 1.73 | 0.22 |
| STD01 | 0.073 | 0.022 | 7.64 | 0.14 | 0.14 | 0.217 | 0.268 | 2.26 | 0.343 | 11.1 | 1.23 | 0.281 |
| SBA01 | 0.037 | 0.033 | 8.2 | 0.125 | 0.12 | 0.217 | 0.439 | 5.02 | 0.349 | 14.54 | 1.62 | 0.309 |
| SAN01 | 0.054 | 0.006 | 4.64 | 0.193 | 0.14 | 0.398 | 0.85 | 6.37 | 0.543 | 13.2 | 1.47 | 0.328 |
| SKB01 | 0.05 | 0.222 | 5.45 | 1.29 | 0.16 | 0.796 | 0.518 | 3.26 | 1.69 | 13.43 | 1.49 | 0.673 |
| SZP01 | 0.044 | 0.006 | 4.92 | 0.287 | 0.12 | 0.312 | 1.32 | 8.21 | 0.59 | 15.81 | 1.76 | 0.352 |
| SOM01 | 0.054 | 0.1 | 6.67 | 0.115 | 0.14 | 0.26 | 0.621 | 18 | 0.648 | 26.61 | 2.96 | 0.469 |
| SG01 | 0.06 | 0.489 | 5.51 | 0.532 | 0.16 | 0.334 | 2.18 | 10.81 | 1.5 | 21.57 | 2.4 | 0.818 |
| SN03 | 0.031 | 0.006 | 4.99 | 0.084 | 0.1 | 0.405 | 0.446 | 3.02 | 0.594 | 9.67 | 1.07 | 0.237 |
| SAT01 | 0.071 | 0.044 | 13.21 | 1.88 | 0.08 | 0.326 | 3.46 | 1.48 | 0.203 | 20.74 | 2.3 | 0.504 |
| SK01 | 0.037 | 0.089 | 5.13 | 0.857 | 0.22 | 0.191 | 0.588 | 3.47 | 0.867 | 11.45 | 1.27 | 0.469 |
| SN01 | 0.035 | 0.006 | 5.98 | 0.225 | 0.1 | 0.228 | 0.709 | 6.23 | 0.554 | 14.06 | 1.56 | 0.29 |
| SO01 | 0.069 | 0.044 | 6.87 | 0.964 | 0.12 | 0.485 | 0.462 | 9.35 | 0.655 | 19.02 | 2.11 | 0.531 |
| Average | 0.055 | 0.169 | 6.81 | 0.487 | 0.132 | 0.341 | 0.875 | 5.88 | 0.652 | 15.4 | 1.71 | 0.428 |

Background values: UCC of Rudnick and Gao (2014) [3]. CF = Cᵢ/Cᵇ; Cdeg = ΣCF; mCdeg = Cdeg/9; PLI = (ΠCF)^(1/9).

Table S4. Enrichment factor (EF) of potentially hazardous metals in the Zang-Tabi stream sediments (Al-normalised).

| Sample | As | Cd | Cr | Cu | Hg | Ni | Pb | Th | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SB01 | 3.99 | 1.39 | 266 | 7.54 | 7.5 | 11.74 | 21.66 | 141 | 20.82 |
| SN02 | 3.72 | 115 | 456 | 28.07 | 9.73 | 40.9 | 35.83 | 296 | 39.83 |
| SZM01 | 1.78 | 0.15 | 302 | 2.39 | 2.2 | 4.47 | 8.47 | 101 | 6.65 |
| STD01 | 2.44 | 0.74 | 256 | 4.68 | 4.69 | 7.27 | 8.99 | 75.61 | 11.5 |
| SBA01 | 1.31 | 1.17 | 287 | 4.38 | 4.2 | 7.6 | 15.37 | 176 | 12.23 |
| SAN01 | 2.69 | 0.28 | 231 | 9.6 | 6.96 | 19.78 | 42.25 | 317 | 27.01 |
| SKB01 | 1.71 | 7.61 | 186 | 44.03 | 5.48 | 27.25 | 17.75 | 112 | 58.01 |
| SZP01 | 1.4 | 0.18 | 158 | 9.23 | 3.85 | 10.01 | 42.3 | 264 | 18.93 |
| SOM01 | 1.52 | 2.8 | 187 | 3.23 | 3.92 | 7.27 | 17.39 | 504 | 18.15 |
| SG01 | 1.79 | 14.49 | 163 | 15.77 | 4.74 | 9.9 | 64.5 | 320 | 44.45 |
| SN03 | 0.78 | 0.14 | 124 | 2.09 | 2.49 | 10.07 | 11.08 | 75.04 | 14.76 |
| SAT01 | 2.87 | 1.8 | 536 | 76.18 | 3.24 | 13.2 | 140 | 59.86 | 8.23 |
| SK01 | 1.2 | 2.85 | 165 | 27.52 | 7.06 | 6.12 | 18.88 | 111 | 27.84 |
| SN01 | 1.16 | 0.18 | 196 | 7.38 | 3.28 | 7.46 | 23.24 | 204 | 18.16 |
| SO01 | 1.51 | 0.98 | 151 | 21.23 | 2.64 | 10.68 | 10.17 | 206 | 14.42 |
| Average | 1.99 | 10.01 | 244 | 17.55 | 4.8 | 12.91 | 31.88 | 197 | 22.73 |

EF = (Cᵢ/Al)sample / (Cᵢ/Al)UCC [3,4].

Table S5. Geo-accumulation index (Igeo) of potentially hazardous metals in the Zang-Tabi stream sediments.

| Sample | As | Cd | Cr | Cu | Hg | Ni | Pb | Th | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SB01 | -3.97 | -5.49 | 2.09 | -3.05 | -3.06 | -2.41 | -1.53 | 1.17 | -1.58 |
| SN02 | -5.03 | -0.08 | 1.91 | -2.12 | -3.64 | -1.57 | -1.76 | 1.28 | -1.61 |
| SZM01 | -4.54 | -8.08 | 2.87 | -4.11 | -4.23 | -3.21 | -2.29 | 1.29 | -2.63 |
| STD01 | -4.36 | -6.08 | 2.35 | -3.42 | -3.42 | -2.79 | -2.48 | 0.59 | -2.13 |
| SBA01 | -5.32 | -5.49 | 2.45 | -3.58 | -3.64 | -2.79 | -1.77 | 1.74 | -2.10 |
| SAN01 | -4.79 | -8.08 | 1.63 | -2.96 | -3.42 | -1.92 | -0.82 | 2.09 | -1.47 |
| SKB01 | -4.91 | -2.75 | 1.86 | -0.22 | -3.23 | -0.92 | -1.53 | 1.12 | 0.17 |
| SZP01 | -5.10 | -8.08 | 1.72 | -2.38 | -3.64 | -2.27 | -0.19 | 2.45 | -1.35 |
| SOM01 | -4.79 | -3.91 | 2.15 | -3.70 | -3.42 | -2.53 | -1.27 | 3.58 | -1.21 |
| SG01 | -4.63 | -1.62 | 1.88 | -1.50 | -3.23 | -2.17 | 0.54 | 2.85 | 0.00 |
| SN03 | -5.58 | -8.08 | 1.73 | -4.15 | -3.91 | -1.89 | -1.75 | 1.01 | -1.34 |
| SAT01 | -4.40 | -5.08 | 3.14 | 0.33 | -4.23 | -2.20 | 1.21 | -0.02 | -2.89 |
| SK01 | -5.32 | -4.08 | 1.77 | -0.81 | -2.77 | -2.98 | -1.35 | 1.21 | -0.79 |
| SN01 | -5.40 | -8.08 | 2.00 | -2.74 | -3.91 | -2.72 | -1.08 | 2.05 | -1.44 |
| SO01 | -4.45 | -5.08 | 2.19 | -0.64 | -3.64 | -1.63 | -1.70 | 2.64 | -1.20 |
| Average | -4.84 | -5.34 | 2.12 | -2.34 | -3.56 | -2.27 | -1.19 | 1.67 | -1.44 |

Igeo = log₂[Cᵢ/(1.5 Bᵢ)] [3,5].

Table S6. Ecological risk factor (ERF) and potential ecological risk index (PERI) of the Zang-Tabi stream sediments.

| Sample | As | Cd | Cr | Cu | Hg | Ni | Pb | Th | Zn | PERI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SB01 | 0.96 | 1 | 12.78 | 0.91 | 7.2 | 1.41 | 2.6 | 3.38 | 0.5 | 30.74 |
| SN02 | 0.46 | 42.67 | 11.24 | 1.73 | 4.8 | 2.52 | 2.21 | 3.65 | 0.49 | 69.76 |
| SZM01 | 0.65 | 0.17 | 21.96 | 0.43 | 3.2 | 0.81 | 1.54 | 3.66 | 0.24 | 32.65 |
| STD01 | 0.73 | 0.67 | 15.28 | 0.7 | 5.6 | 1.09 | 1.34 | 2.26 | 0.34 | 28 |
| SBA01 | 0.38 | 1 | 16.39 | 0.62 | 4.8 | 1.09 | 2.19 | 5.02 | 0.35 | 31.84 |
| SAN01 | 0.54 | 0.17 | 9.28 | 0.97 | 5.6 | 1.99 | 4.25 | 6.37 | 0.54 | 29.71 |
| SKB01 | 0.5 | 6.67 | 10.89 | 6.43 | 6.4 | 3.98 | 2.59 | 3.26 | 1.69 | 42.41 |
| SZP01 | 0.44 | 0.17 | 9.85 | 1.44 | 4.8 | 1.56 | 6.59 | 8.21 | 0.59 | 33.64 |
| SOM01 | 0.54 | 3 | 13.35 | 0.58 | 5.6 | 1.3 | 3.1 | 18 | 0.65 | 46.11 |
| SG01 | 0.6 | 14.67 | 11.02 | 2.66 | 6.4 | 1.67 | 10.88 | 10.81 | 1.5 | 60.22 |
| SN03 | 0.31 | 0.17 | 9.98 | 0.42 | 4 | 2.03 | 2.23 | 3.02 | 0.59 | 22.75 |
| SAT01 | 0.71 | 1.33 | 26.41 | 9.39 | 3.2 | 1.63 | 17.29 | 1.48 | 0.2 | 61.65 |
| SK01 | 0.38 | 2.67 | 10.26 | 4.29 | 8.8 | 0.95 | 2.94 | 3.47 | 0.87 | 34.62 |
| SN01 | 0.35 | 0.17 | 11.96 | 1.12 | 4 | 1.14 | 3.54 | 6.23 | 0.55 | 29.07 |
| SO01 | 0.69 | 1.33 | 13.74 | 4.82 | 4.8 | 2.43 | 2.31 | 9.35 | 0.66 | 40.12 |
| Average | 0.549 | 5.06 | 13.63 | 2.43 | 5.28 | 1.71 | 4.37 | 5.88 | 0.651 | 39.55 |

ERF = Tᵣ × CF, with Tᵣ: As = 10, Cd = 30, Cr = 2, Cu = 5, Hg = 40, Ni = 5, Pb = 5, Th = 1, Zn = 1; PERI = ΣERF [6].

Table S7. Toxic risk index (TRI) of potentially hazardous metals in the Zang-Tabi stream sediments.

| Sample | As | Cd | Cr | Cu | Hg | Ni | Pb | Zn | TRI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SB01 | 0.08 | 0.01 | 17.06 | 0.14 | 0.06 | 0.82 | 0.27 | 0.29 | 18.74 |
| SN02 | 0.04 | 0.22 | 15.00 | 0.28 | 0.04 | 1.47 | 0.23 | 0.29 | 17.56 |
| SZM01 | 0.06 | 0.00 | 29.31 | 0.07 | 0.02 | 0.47 | 0.16 | 0.14 | 30.24 |
| STD01 | 0.06 | 0.00 | 20.40 | 0.11 | 0.04 | 0.63 | 0.14 | 0.20 | 21.60 |
| SBA01 | 0.03 | 0.01 | 21.88 | 0.10 | 0.04 | 0.63 | 0.23 | 0.20 | 23.12 |
| SAN01 | 0.05 | 0.00 | 12.39 | 0.15 | 0.04 | 1.16 | 0.44 | 0.32 | 14.56 |
| SKB01 | 0.04 | 0.03 | 14.54 | 1.02 | 0.05 | 2.32 | 0.27 | 0.99 | 19.27 |
| SZP01 | 0.04 | 0.00 | 13.15 | 0.23 | 0.04 | 0.91 | 0.69 | 0.34 | 15.39 |
| SOM01 | 0.05 | 0.01 | 17.82 | 0.09 | 0.04 | 0.76 | 0.32 | 0.38 | 19.47 |
| SG01 | 0.05 | 0.07 | 14.71 | 0.42 | 0.05 | 0.97 | 1.13 | 0.88 | 18.30 |
| SN03 | 0.03 | 0.00 | 13.32 | 0.07 | 0.03 | 1.18 | 0.23 | 0.35 | 15.21 |
| SAT01 | 0.06 | 0.01 | 35.26 | 1.50 | 0.02 | 0.95 | 1.80 | 0.12 | 39.72 |
| SK01 | 0.03 | 0.01 | 13.70 | 0.68 | 0.07 | 0.56 | 0.31 | 0.51 | 15.86 |
| SN01 | 0.03 | 0.00 | 15.96 | 0.18 | 0.03 | 0.67 | 0.37 | 0.32 | 17.56 |
| SO01 | 0.06 | 0.01 | 18.34 | 0.77 | 0.04 | 1.42 | 0.24 | 0.38 | 21.25 |
| Average | 0.05 | 0.03 | 18.19 | 0.39 | 0.04 | 1.00 | 0.46 | 0.38 | 20.52 |

TRIᵢ = √[(Cᵢ/TELᵢ)² + (Cᵢ/PELᵢ)²]; TRI = ΣTRIᵢ [7–9]. Th excluded (no TEL/PEL).

Table S8. Toxic units (TU) of potentially hazardous metals in the Zang-Tabi stream sediments.

| Sample | As | Cd | Cr | Cu | Hg | Ni | Pb | Zn | TU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SB01 | 0.027 | 0.001 | 6.533 | 0.026 | 0.018 | 0.368 | 0.097 | 0.106 | 7.18 |
| SN02 | 0.013 | 0.036 | 5.744 | 0.049 | 0.012 | 0.658 | 0.082 | 0.104 | 6.70 |
| SZM01 | 0.018 | 0.000 | 11.222 | 0.012 | 0.008 | 0.212 | 0.057 | 0.051 | 11.58 |
| STD01 | 0.021 | 0.001 | 7.811 | 0.020 | 0.014 | 0.283 | 0.050 | 0.073 | 8.27 |
| SBA01 | 0.011 | 0.001 | 8.378 | 0.018 | 0.012 | 0.283 | 0.082 | 0.074 | 8.86 |
| SAN01 | 0.015 | 0.000 | 4.744 | 0.028 | 0.014 | 0.519 | 0.158 | 0.116 | 5.60 |
| SKB01 | 0.014 | 0.006 | 5.567 | 0.183 | 0.017 | 1.039 | 0.097 | 0.360 | 7.28 |
| SZP01 | 0.012 | 0.000 | 5.033 | 0.041 | 0.012 | 0.407 | 0.245 | 0.125 | 5.88 |
| SOM01 | 0.015 | 0.003 | 6.822 | 0.016 | 0.014 | 0.339 | 0.116 | 0.138 | 7.46 |
| SG01 | 0.017 | 0.013 | 5.633 | 0.076 | 0.017 | 0.436 | 0.405 | 0.319 | 6.92 |
| SN03 | 0.009 | 0.000 | 5.100 | 0.012 | 0.010 | 0.529 | 0.083 | 0.126 | 5.87 |
| SAT01 | 0.020 | 0.001 | 13.500 | 0.267 | 0.008 | 0.425 | 0.644 | 0.043 | 14.91 |
| SK01 | 0.011 | 0.002 | 5.244 | 0.122 | 0.023 | 0.249 | 0.110 | 0.184 | 5.94 |
| SN01 | 0.010 | 0.000 | 6.111 | 0.032 | 0.010 | 0.297 | 0.132 | 0.118 | 6.71 |
| SO01 | 0.019 | 0.001 | 7.022 | 0.137 | 0.012 | 0.633 | 0.086 | 0.139 | 8.05 |
| Average | 0.015 | 0.004 | 6.964 | 0.069 | 0.014 | 0.445 | 0.163 | 0.139 | 7.81 |

TUᵢ = Cᵢ/PELᵢ; TU = ΣTUᵢ [8,9]. Th excluded (no PEL).

Fig. S1. Comparison of potentially hazardous metal concentrations in the Zang-Tabi stream sediments with the threshold effect level (TEL) benchmark (red columns = above guideline): (a) As, (b) Cd, (c) Cr, (d) Cu, (e) Pb, (f) Zn, (g) Hg and (h) Ni. SQG values after MacDonald et al. (2000) [9].

Fig. S2. Comparison of potentially hazardous metal concentrations in the Zang-Tabi stream sediments with the probable effect level (PEL) benchmark (red columns = above guideline): (a) As, (b) Cd, (c) Cr, (d) Cu, (e) Pb, (f) Zn, (g) Hg and (h) Ni. SQG values after MacDonald et al. (2000) [9].

Fig. S3. Comparison of potentially hazardous metal concentrations in the Zang-Tabi stream sediments with the threshold effect concentration (TEC) benchmark (red columns = above guideline): (a) As, (b) Cd, (c) Cr, (d) Cu, (e) Pb, (f) Zn, (g) Hg and (h) Ni. SQG values after MacDonald et al. (2000) [9].

Fig. S4. Comparison of potentially hazardous metal concentrations in the Zang-Tabi stream sediments with the probable effect concentration (PEC) benchmark (red columns = above guideline): (a) As, (b) Cd, (c) Cr, (d) Cu, (e) Pb, (f) Zn, (g) Hg and (h) Ni. SQG values after MacDonald et al. (2000) [9].

Fig. S5. Comparison of potentially hazardous metal concentrations in the Zang-Tabi stream sediments with the effect range-low (ERL) benchmark (red columns = above guideline): (a) As, (b) Cd, (c) Cr, (d) Cu, (e) Pb, (f) Zn, (g) Hg and (h) Ni. SQG values after MacDonald et al. (2000) [9].

Fig. S6. Comparison of potentially hazardous metal concentrations in the Zang-Tabi stream sediments with the effect range-median (ERM) benchmark (red columns = above guideline): (a) As, (b) Cd, (c) Cr, (d) Cu, (e) Pb, (f) Zn, (g) Hg and (h) Ni. SQG values after MacDonald et al. (2000) [9].

References (cited in the Supplementary Material)

[1] T. Liu, C.P. Liu, J. Deng, P.Y. Kang, K.K. Wang, Y.Y. Zhao, Ecological health risk assessment of soil heavy metals in eastern Yinan County, Shandong Province, Geol. China 49 (2022) 1497–1508. https://doi.org/10.12029/gc20220509.

[2] U.S. Environmental Protection Agency (USEPA), Conducting a Human Health Risk Assessment, USEPA, Washington, DC, 2025. https://www.epa.gov/risk/human-health-risk-assessment.

[3] R.L. Rudnick, S. Gao, Composition of the continental crust, in: H.D. Holland, K.K. Turekian (Eds.), Treatise on Geochemistry, second ed., vol. 4, Elsevier, Oxford, 2014, pp. 1–51. https://doi.org/10.1016/B978-0-08-095975-7.00301-6.

[4] H. Yongming, D. Peixuan, C. Junji, E.S. Posmentier, Multivariate analysis of heavy metal contamination in urban dusts of Xi'an, Central China, Sci. Total Environ. 355 (2006) 176–186. https://doi.org/10.1016/j.scitotenv.2005.02.026.

[5] G. Müller, Die Schwermetallbelastung der Sedimente des Neckars und seiner Nebenflüsse: eine Bestandsaufnahme, Chem. Ztg. 105 (1981) 157–164.

[6] L. Håkanson, An ecological risk index for aquatic pollution control. A sedimentological approach, Water Res. 14 (1980) 975–1001. https://doi.org/10.1016/0043-1354(80)90143-8.

[7] A. Pejman, G. Nabi Bidhendi, M. Ardestani, M. Saeedi, A. Baghvand, A new index for assessing toxicity risk of sediment metals, Environ. Geochem. Health 37 (2015) 755–764. https://doi.org/10.1007/s10653-015-9683-9.

[8] G. Zhang, J. Bai, Q. Zhao, Q. Lu, J. Jia, X. Wen, Heavy metals in wetland soils along a wetland-forming chronosequence in the Yellow River Delta of China: levels, sources and toxic risks, Ecol. Indic. 69 (2016) 331–339. https://doi.org/10.1016/j.ecolind.2016.04.042.

[9] D.D. MacDonald, C.G. Ingersoll, T.A. Berger, Development and evaluation of consensus-based sediment quality guidelines for freshwater ecosystems, Arch. Environ. Contam. Toxicol. 39 (2000) 20–31. https://doi.org/10.1007/s002440010075.