Supplementary Materials

Source apportionment and source-specific health risk of heavy metals in urban road dust of Jinan City, East China

Shengwei Zhanga,b, Ge Maa, Shuzhen Donga,c, Xang-Zhou Mengd, Lijun Wanga,*

aDepartment of Environmental Science and Engineering, School of Geography and Tourism, Shaanxi Normal University, Xi’an 710119, China

bState Key Laboratory of Water Environment Simulation, School of Environment, Beijing Normal University, Beijing 100875, China

cShangluo City Meteorological Bureau, Shangluo 726000, China

dState Key Laboratory of Pollution Control and Resources Reuse, College of Environmental Science and Engineering, Tongji University, Shanghai 200092, China

*Corresponding author E-mail: wanglijun@snnu.edu.cn (L. Wang)

Text 1. Sample digestion

The digestion of road dust samples was described in our previous work (Wang et al. 2018). In briefly, a total of 0.50 g road dust sample was weighted, placed into a 50-mL teflon crucible, and wetted using a small amount of ultra-pure water. After concentrated hydrogen nitrate (HNO3, 5 mL), hydrofluoric acid (HF, 5 mL) and perchloric acid (HClO4, 3 mL) were added, it was heated with an electric heating plate until near dryness. It was digested with HNO3 (3 mL), HF (3 mL) and HClO4 (1 mL) again. The residual was dissolved and transferred into a 25-mL volumetric flask and diluted to 25 mL with the solution of 25% HNO3 (v : v). The prepared solution was stored at 4 °C before analysis.

Text 2. BCR sequential extraction

The modified BCR sequential extraction was described in our previous studies (Wang et al. 2015). The detailed processes were as follows:

Step 1 (Acid soluble state): A total of 0.80 g road dust sample was weighted and placed into a 100-mL erlenmeyer flask. After A 32 mL solution of 0.11 mol/L CH3COOH was added, it was shaken for 16 h at room temperature. Next, it was transferred to a 50-mL glass centrifuge tube and centrifugated for 10 min at 7000 r/min. The supernatant was transferred into a 100-mL beaker and steamed to near dryness using a hot plate, then dissolved by adding 5 mL solution of 1.0 mol/L HNO3, and finally transferred into a 25-mL volumetric flask. After 20 mL ultra-pure water was added into the solid residue, it was shaken for 15 min and centrifuged, then the supernatant was discarded.

Step 2 (Reducible state): A 32 mL solution of 0.5 mol/L HONH2HCl was added into to the residue of Step 1 and the pH value of solution was adjusted to 2 with 2 mol/L HNO3. It was shaken for 16 h at room temperature, then centrifuged with the method above. The treatment of the supernatant and residual was same to the description above.

Step 3 (Oxidizable state): A 10 mL solution of 30% H2O2 was added into the residue of Step 2 and the pH value of solution was adjusted to 2-3 with HNO3. It was digested for 1 h at room temperature, occasionally shaken, then heated to 85 °C for 1 h in water bath. After a 50 mL solution of 1.0 mol/L CH3COONH4 (the pH of solution was adjusted to 2 with HNO3) was added, it was shaken for 16 h at room temperature, then centrifuged with the method above. The process was repeated. The treatment of the supernatant and residual was same to the description above.

Step 4 (Residual state): The residue of Step 3 was treated by concentrated HF, HNO3 and HClO4. The treatment process was same to the sample digestion above.

Text 3. Instrument parameters

The concentration and speciation of heavy metals were measured by ICP-AES. High purity (99.9%) argon was used as carrier gas, the partial pressure was maintained at 0.7~1.0 MPa, and the optical chamber temperature was stable to 15 ± 0.1 ℃. The standard curve was constructed with the standard gradients 0, 0.1, 0.5, 1, 2, 4, 8, 10 and 20 mg/L (external standards dissolve into 5% HNO3). Each sample is measured three times, the relative standard deviation (RSD) was less than 2%, and the regression coefficient R2 was greater than 0.999. The spectral lines of heavy metals were Co (230.786), Cr (283.565), Cu (324.700), Cd (228.802), Mn (257.611), Ni (231.604), Pb (168.215), V (292.402), Zn (202.613) and Ba (455.404), with corresponding instrumental limits of detection (LLDs) 0.003, 0.004, 0.002, 0.001, 0.013, 0.003, 0.008, 0.001, 0.001, 0.001 mg/L, respectively.

Text 4. PMF receptor model

PMF receptor model is a good tool in apportioning the composition and contribution of heavy metal sources in urban road dust (Men et al. 2018). The PMF model can be described by equation 1 (Hopke 2003; Manousakas et al. 2017).

(1)

where, xij is the concentration of the ith species measured in the jth sample, fkj is the contribution of the kth source to the jth sample, gik is the concentration of the ith species from the kth source, and eij is the error. In the PMF model, factor matrix is calculated under non-negative constraint condition, which means that all the results are positive. The composition and contribution of each source are derived from the PMF model by minimizing the sum of an ‘object function’ Q calculated by equation 2 (Norris and Henry, 2014; Kim and Hopke, 2007).

(2)

where, uij is the uncertainty; if the concentration is less than or equal to method detection limit (MDL), the uncertainty is equal to 5/6 MDL; otherwise, the uncertainty is calculated by equation 3.

(3)

The number of specific factors is not directly given, therefore, multiple debugging is required. Based on the ratios of signal-to-noise (S/N), the species is categorized as strong, weak or bad. If the scale of residuals for most of species is between -3 and +3, as well as Qtrue, Qrobust and Qtheoretical are close, the base run may be regarded reasonable. The value of Qtheoretical is calculated by equation 4.

(4)

where, n is the number of species, m is the number of receptor sites, and p is the number of factors (Henry 2003; Norris and Henry, 2014).

Text 5. Health risk assessment model

The health risk assessment model of US EPA has been widely used to assess the carcinogenic and non-carcinogenic risks of human exposure to heavy metals (Khairy et al. 2011; US EPA, 2011). The average daily doses (ADDs) of human exposure to heavy metals in urban road dust through ingestion, inhalation and dermal adsorption are calculated by equation 5-7.

(5)

(6)

(7)

where, C is the concentration of heavy metals in road dust, being 95% upper confidence limit (95% UCL) and modified as the product of C and Fbio (bio-availability); IngR is the ingestion rate of road dust; EF is the exposure frequency; ED is the exposure duration; BW is the average body weight; AT is the average exposure time; InhR is the inhalation rate; PEF is the particle emission factor; SA is the surface area of skin exposure; and AF is the skin adherence factor. The specific values of parameters are shown in Table S3.

The non-carcinogenic risk of human exposure to heavy metals in urban road dust can be described by equation 8 and 9.

(8)

(9)

where, HQi is the hazard quotient of exposure pathway i for certain metal, RfDi is the reference dose of corresponding exposure pathway for certain metal (Table S4), and HI is the hazard index of all exposure pathway for certain metal. HI > 1 indicates a significant non-carcinogenic risk, while HI ≤ 1 indicates that the non-carcinogenic risk is insignificant.

For carcinogenic effects, the lifetime average daily dose (LADD) is calculated by equation 10.

(10)

where, AR is absorption rate; AR = IngR for the ingestion, AR = InhR/PEF for the inhalation, and AR = SA×SL×ABS for dermal contact.

The carcinogenic risk of human exposure to heavy metals in urban road dust can be expressed by equation 11 and 12.

(11)

(12)

where, CRi is the carcinogenic risk of exposure route i, SFi is the carcinogenic slope factor of exposure route i (Table S4), and CR is the sum of cancer risk for all the exposure routes. The value of CR is between 1×10−6 and 1×10−4, suggesting that the carcinogenic risk is acceptable; the CR value is greater than 1 × 10−4, indicating a significant carcinogenic risk; and the CR is less than 1×10−6, implying that the carcinogenic risk is low or negligible (Shi et al. 2011; US EPA, 2019).

Fig. S1 Size fractions of road dust.

Fig. S2 Compositions of TOC (total organic carbon), IC (inorganic carbon) and TC (total carbon, TOC+IC), of road dust.

Fig. S3 Concentration of heavy metal in road dust of Jinan City.

Fig. S4 Spatial distribution of heavy metals in the road dust of Jinan City

Fig. S5 Speciation characteristics of heavy metals in the road dust of Jinan City

Table S1

Physicochemical properties (pH, EC, LF, HF and FD) of road dust

| Items | pH | EC | LF | HF | FD |
| --- | --- | --- | --- | --- | --- |
| Unit | Unitless | μS/cm | ×108 m3/kg | ×108 m3/kg | % |
| Min | 7.06 | 214 | 337 | 294 | 2.07 |
| Max | 9.64 | 6.23×103 | 1.69×103 | 1.66×103 | 12.8 |
| Mean | 8.02 | 1.81×103 | 608 | 561 | 8.40 |
| Median | 8.04 | 1.48×103 | 556 | 509 | 8.73 |

EC: Electrical conductivity; LF: Low frequency susceptibility; HF: High frequency susceptibility; FD: frequency susceptibility.

Table S2

Quality control of standard soils of GSS1 and GSD5

| Elements | Co | Cr | Cu | Cd | Mn | Ni | Pb | Zn | V | Ba |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSS1 | 12.36 | 63.51 | 20.07 | 4.35 | 1669.27 | 19.32 | 104.40 | 570.27 | 75.18 | 530.48 |
| Reference value | 14.2 | 62 | 21 | 4.3 | 1760 | 20.4 | 98 | 680 | 86 | 590 |
| Absolute error | 1.84 | 1.51 | 0.93 | 0.05 | 90.73 | 1.08 | 6.40 | 109.73 | 10.82 | 59.52 |
| Relative error | 12.94 | 2.44 | 4.43 | 1.26 | 5.16 | 5.30 | 6.54 | 16.14 | 12.59 | 10.09 |
| GSD5 | 16.89 | 70.81 | 129.87 | 1.00 | 1099.69 | 33.28 | 118.63 | 208.08 | 98.44 | 394.96 |
| Reference value | 18.9 | 70 | 137 | 0.82 | 1160 | 34 | 112 | 243 | 109 | 440 |
| Absolute error | 2.01 | 0.81 | 7.13 | 0.18 | 60.31 | 0.72 | 6.63 | 34.92 | 10.56 | 45.04 |
| Relative error | 10.64 | 1.15 | 5.20 | 21.88 | 5.20 | 2.12 | 5.92 | 14.37 | 9.69 | 10.24 |

Table S3

Parameter values of US EPA health risk assessment model

| Parameters | Parameters | Unit | Children | Adults |
| --- | --- | --- | --- | --- |
|  | C | mg/kg | 95%UCL | 95%UCL |
| Exposure behavior parameter | ED a | a | 6 | 24 |
|  | BW b | kg | 15 | 58.6 |
|  | EF c | d/a | 180 | 180 |
|  | AT-NCR a, c | d | 365 × ED | 365 × ED |
|  | AT-CR a, c | d | 365 ×70 | 365 ×70 |
| Ingestion | lngR a | mg/d | 200 | 100 |
| Inhalation | lnhR a, b | m3/d | 7.6 | 12.8 |
|  | PEF a | m3/kg | 1.36 × 109 | 1.36 × 109 |
| Adhesion | AF a | mg/cm2 | 0.2 | 0.07 |
|  | ABS a | unitless | 0.001 | 0.001 |
|  | SA b | cm2 | 1150 | 2145 |

a: adopted from US EPA (2001, 2019); b: adopted from Li et al. (2011); c: adopted from Li et al. (2017)

Table S4

Reference dose (RfD) and slope factor (SF) of heavy metals

| Elements | RfD (mg/kg/d) |  |  | SF (1/(mg/kg/d)) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RfDing | RfDinh | RfDdermal | SFing | SFinh | SFdermal |
| Co | 2.00E-02 | 5.71E-06 | 1.60E-02 |  | 9.80E+00 |  |
| Cr | 3.00E-03 | 2.86E-05 | 6.00E-05 |  | 4.20E+01 |  |
| Cu | 4.00E-02 | 4.00E-02 | 1.20E-02 |  |  |  |
| Cd | 1.00E-03 | 1.00E-03 | 1.00E-05 | 6.10E+00 | 1.80E-03 | 6.10E+00 |
| Mn | 4.60E-02 | 1.43E-05 | 1.84E-03 |  |  |  |
| Ni | 2.00E-02 | 2.06E-02 | 5.40E-03 |  | 8.40E-01 |  |
| Pb | 3.50E-03 | 3.52E-03 | 5.25E-04 |  |  |  |
| Zn | 3.00E-01 | 3.00E-01 | 6.00E-02 |  |  |  |
| V | 7.00E-03 |  | 7.00E-05 |  |  |  |
| Ba | 7.00E-02 | 1.43E-04 | 4.90E-03 |  |  |  |

Table S5

Mean concentrations of heavy metals in the study area and reference area (mg/kg)

| City | Co | Cr | Cu | Cd | Mn | Ni | Pb | Zn | V | Ba | References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Jinan | 8.25 | 114.09 | 87.71 | 1.08 | 517.04 | 30.29 | 85.71 | 497.64 | 51.76 | 642.77 | This study |
| Beijing | - | 92.1 | 83.12 | 0.59 | 553.73 | 32.47 | 60.88 | 280.65 | - | - | Mehr et al. 2017 |
| Guangzhou | - | - | 176 | 2.41 | - | - | 240 | 586 | - | - | Duzgoren-Aydin et al. 2006 |
| Hangzhou | - | - | 116 | 1.6 | - | - | 202 | 321 | - | - | Zhang et al. 2009 |
| Shanghai | - | - | 132.1 | 0.46 | - | - | 105.6 | 473.5 | - | - | Bi et al. 2018 |
| Nanchang | 7.9 | - | 102 | 0.58 | - | - | 62 | 208 | 40 | - | Tang et al. 2017 |
| Xi’an | 30.9 | 145 | 54.7 | - | - | 30.8 | 124.5 | 268.6 | 69.6 | - | Pan et al. 2017 |
| Tianjing | 9.3 | 71.85 | 55.47 | 0.45 | - | - | 60.11 | - | - | - | Yu et al. 2018 |
| Huai’an | 6.62 | 61.1 | 36.3 | 0.252 | - | - | 42.6 | - | - | - | Zhao et al. 2019 |
| Delhi | 23 | - | 169 | - | - | - | 129 | 264 | - | - | Suryawanshi et al. 2016 |
| Turin | - | - | 181 | 0.8 | - | - | 74 | 200 | - | - | Mohman et al. 2015 |
| Thessaloniki | - | 104.9 | 662.3 | 1.76 | 336.4 | 89.43 | 209 | 452.8 | - | - | Doabi et al. 2018 |
| Dhaka | - | 144.34 | 49.68 | 11.64 | 261.53 | 37.01 | 18.99 | 239.16 | - | - | Valottoet al. 2015 |
| Kermanshah | - | 73.74 | 47.63 | - | - | 119.53 | - | 210.29 | - | - | Bourliva et al. 2017 |

Note: “-” no available data.

Table S6

Source-specific health risk of heavy metals in the road dust of Jinan City

| Elements | Traffic emissions |  |  | Industrial discharges |  |  | Natural and coal combustion sources |  |  | Building material and manufacture sources |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NCR |  | CR | NCR |  | CR | NCR |  | CR | NCR |  | CR |
|  | Children | Adults |  | Children | Adults |  | Children | Adults |  | Children | Adults |  |
| Co | 1.00E-04 | 1.56E-05 | 1.17E-10 | 2.06E-04 | 3.20E-05 | 2.40E-10 | 5.46E-04 | 8.46E-05 | 6.35E-10 | 6.99E-05 | 1.08E-05 | 8.13E-11 |
| Cr | 1.21E-02 | 1.59E-03 | 9.42E-09 | 1.52E-02 | 1.99E-03 | 1.18E-08 | 2.37E-02 | 3.10E-03 | 1.84E-08 | 4.85E-03 | 6.35E-04 | 3.76E-09 |
| Cu | 3.50E-04 | 4.48E-05 |  | 2.87E-03 | 3.68E-04 |  |  |  |  | 6.08E-03 | 7.79E-04 |  |
| Cd | 3.84E-03 | 5.07E-04 | 2.72E-06 | 1.70E-03 | 2.24E-04 | 1.21E-06 |  |  |  | 7.55E-04 | 9.97E-05 | 5.36E-07 |
| Mn | 6.05E-03 | 9.28E-04 |  | 9.30E-03 | 1.43E-03 |  | 2.40E-02 | 3.68E-03 |  | 2.85E-03 | 4.37E-04 |  |
| Ni | 6.90E-04 | 8.84E-05 | 7.53E-11 | 2.90E-04 | 3.72E-05 | 3.17E-11 | 1.61E-03 | 2.06E-04 | 1.76E-10 | 2.37E-04 | 3.03E-05 | 2.58E-11 |
| Pb | 2.27E-02 | 2.91E-03 |  | 3.49E-02 | 4.48E-03 |  | 8.44E-03 | 1.08E-03 |  | 1.34E-02 | 1.72E-03 |  |
| Zn | 1.44E-03 | 1.85E-04 |  | 1.05E-03 | 1.35E-04 |  | 1.63E-03 | 2.09E-04 |  | 4.70E-04 | 6.02E-05 |  |
| V | 1.10E-03 | 1.46E-04 |  | 3.43E-03 | 4.53E-04 |  | 8.35E-03 | 1.10E-03 |  | 9.75E-04 | 1.29E-04 |  |
| Ba | 6.95E-03 | 9.22E-04 |  | 1.68E-02 | 2.23E-03 |  |  |  |  | 8.66E-06 | 1.15E-06 |  |
| ΣHeavy metals | 5.53E-02 | 7.33E-03 | 2.73E-06 | 8.58E-02 | 1.14E-02 | 1.22E-06 | 6.82E-02 | 9.46E-03 | 1.92E-08 | 2.97E-02 | 3.90E-03 | 5.40E-07 |

NCR: Non-carcinogenic risk; CR: Lifetime carcinogenic risk.

References

Bi C, Zhou Y, Chen Z, Jia, JP, Bao XI (2018) Heavy metals and lead isotopes in soils, road dust and leafy vegetables and health risks via vegetable consumption in the industrial areas of Shanghai, China. Science of the Total Environment 619-620: 1349-1357.

Bourliva A, Christophoridis C, Papadopoulou L, Giouri K, Papadopoulos A, Mitsika E, Fytianos K (2017) Characterization, heavy metal content and health risk assessment of urban road dusts from the historic center of the city of Thessaloniki, Greece. Environmental Geochemistry Health 39: 611-634.

Doabi SA, Karami M, Afyuni M, Yeganeh M (2018) Pollution and health risk assessment of heavy metals in agricultural soil, atmospheric dust and major food crops in Kermanshah province, Iran. Ecotoxicology and Environmental Safety 163: 153-164.

Duzgoren-Aydin NS, Wong CSC, Aydin A, Song Z, You M, Li XD (2006) Heavy metal contamination and distribution in the urban environment of Guangzhou, SE China. Environmental Geochemistry and Health 28: 375-391.

Henry RC, Christensen ER (2010) Selecting an appropriate multivariate source apportionment model result. Environmental Science and Technology 44: 2474-2481.

Hopke PK (2003) Recent developments in receptor modeling. Journal of Chemistry 17, 255-265.

Khairy MA, Barakat AO, Mostafa AR, Wade TL (2011) Multielement determination by flame ,atomic absorption of road dust samples in Delta Region, Egypt. Microchemistry Journal 97: 234-242.

Kim E, Hopke PK (2007) Comparison between sample-species specific uncertainties and estimated uncertainties for the source apportionment of the speciation trends network data. Atmospheric Environment 41: 567-575.

Li HH, Chen LJ, Yu L, Guo Z, Shan C, Lin J, Gu Y, Yang Z, Yang Y, Shao J (2017) Pollution characteristics and risk assessment of human exposure to oral bioaccessibility of heavy metals via urban street dusts from different functional areas in Chengdu, China. Science of the Total Environment 586: 1076-1084.

Li RZ, Zhou AJ, Dong F, Wu Y, Zhang P, Yu J (2011) Distribution of metals in urban dusts of Hefei and health risk assessment. Environmental Science 32: 2261-2268. (In Chinese)

Manousakas M, Papaefthymiou H, Diapouli E, Migliori A, Karydas AG, Bogdanovic-Radovic I, Eleftheriadis K (2017) Assessment of PM2.5 sources and their corresponding level of uncertainty in a coastal urban area using EPA PMF 5.0 enhanced diagnostics. Science of the Total Environment 574: 155-164.

Mehr MR, Keshavarzi B, Moore F, Sharifi R, Lahijanzadeh A, Kermanic M (2017) Distribution, source identification and health risk assessment of soil heavy metals in urban areas of Isfahan province, Iran. Journal of African Earth Sciences 132: 16-26.

Men C, Liu RM, Xu F, Wang Q, Guo LJ, Shen ZY (2018) Pollution characteristics, risk assessment, and source apportionment of heavy metals in road dust in Beijing, China. Science of the Total Environment 612: 138-147.

Mohmand J, Eqani SAMAS, Fasola M, Alamdar A, Mustafa I, Ali N, Liu LP, Peng SY, Shen HQ (2015) Human exposure to toxic metals via contaminated dust: bio-accumulation trends and their potential risk estimation. Chemosphere 132: 142-151.

Norris G. Duvall R (2014) EPA Positive Matrix Factorization (PMF) 5.0 Fundamentals and User Guide (EPA/600/R-14/108). US EPA.

Pan H, Lu X, Lei K (2017) A comprehensive analysis of heavy metals in urban road dust of Xi'an, China: Contamination, source apportionment and spatial distribution. Science of the Total Environment 609: 1361-1369.

Shi G, Chen Z, Bi C, Wang L, Teng J, Li Y, Xu S (2011) A comparative study of health risk of potentially toxic metals in urban and suburban road dust in the most populated city of China. Atmospheric Environment 45: 764-771.

Suryawanshi PV, Rajaram BS, Bjanarkar AD, Rao CVC (2016) Determining heavy metal contamination of road dust in Delhi, India. Atmósfera 29: 221-234.

Tang Z, Chai M, Cheng J, Jin J, Yang Y, Nie Z, Huang Q, Li Y (2017) Contamination and health risks of heavy metals in street dust from a coalmining city in eastern China. Ecotoxicology and Environmental Safety 138: 83-91.

US EPA (United State Environmental Protection Agency), 2011. Exposure Factors Handbook. 2011 Edition. National Center for Environmental Assessment. Washington, DC. Office of Research and Development 20460. (EPA/600/R-09/052F)

US EPA (United States Environmental Protection Agency), 2001. Supplemental Guidance for Developing Soil Screening Level for Superfund Sites. OSWER 9355, 4-24. Office of Solid Waste and Emergency Response; U.S. Environmental Protection Agency, Washington, DC, USA.

US EPA (United States Environmental Protection Agency), 2019. Regional Screening Levels (RSLs), Generic Tables.

Valotto G, Rampazzo G, Visin F, Gonella F, Cattaruzza E, Glisenti A, Formenton G, Tieppo P (2015) Environmental and traffic-related parameters affecting road dust composition: A multi-technique approach applied to Venice area (Italy). Atmospheric Environment 122: 596-608.

Wang LJ, Lu XW, Li LY, Ren XH, Luo DC, Chen JH (2015) Content, speciation and pollution assessment of Cu, Pb and Zn in soil around the lead–zinc smelting plant of Baoji, NW China. Environmental Earth Sciences 73: 5281–5288.

Wang LJ, Tao WD, Smardon RC, Xu X, Lu XW (2018) Speciation, sources and risk assessment of heavy metals in suburban vegetable garden soil of Xianyang City, Northwest China. Frontiers of Earth Science 12: 397–407.

Yu B, Wang Y, Zhou Q (2014) Human health risk assessment based on toxicity characteristic leaching procedure and simple bioaccessibility extraction test of toxic metals in urban street dust of Tianjin, China. PLOS One 9: e92459.

Zhang M, Wang, H (2009) Concentrations and chemical forms of potentially toxic metals in road-deposited sediments from different zones of Hangzhou, China. Journal of Environmental Science 21: 625-631.

Zhao L, Hu G, Yan Y, Yu R, Cui J, Wang X, Yan Y (2019) Source apportionment of heavy metals in urban road dust in a continental city of eastern China: Using Pb and Sr isotopes combined with multivariate statistical analysis. Atmospheric Environment 201: 201-211.