- Supplementary Material

Table S1 Boundary conditions for static mesh.

| Position | Boundary Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Front | Velocity inlet | Velocity | Temperature | Component |  |  |  |
|  |  | 0.0 m/s at 0 km/h | 300 K | 79% N2, 21% O2 |  |  |  |
|  |  | 8.3 m/s at 30 km/h |  |  |  |  |  |
|  |  | 25.0 m/s at 90 km/h |  |  |  |  |  |
| Rear | Pressure outlet |  |  |  |  |  |  |
| Left | Periodic boundary |  |  |  |  |  |  |
| Right | Periodic boundary |  |  |  |  |  |  |
| Upper | Pressure outlet |  |  |  |  |  |  |
| Bottom | No-slip wall |  |  |  |  |  |  |
| Vehicle surface | No-slip wall |  |  |  |  |  |  |
| Exhaust tailpipe | Velocity inlet | Velocity | Temperature | O2 | CO2 | H2O | NO |
|  |  | 2.0 m/s at 0 km/h | 384 K | 18.0% | 2.3% | 2.3% | 115 ppm |
|  |  | 15.6 m/s at 30 km/h | 548 K | 8.1% | 9.0% | 9.0% | 450 ppm |
|  |  | 40.4 m/s at 90 km/h | 613 K | 11.2% | 7.2% | 7.2% | 360 ppm |

Table S2 Boundary condition for dynamic mesh.

|  | Region A | Region B | Region C |
| --- | --- | --- | --- |
| Front | Velocity inlet | Interface A-B | Interface B-C |
| Rear | Interface A-B | Interface B-C | Pressure outlet |
| Left | Periodic boundary | Periodic boundary | Periodic boundary |
| Right | Periodic boundary | Periodic boundary | Periodic boundary |
| Upper | Pressure outlet | Pressure outlet | Pressure outlet |
| Bottom | No-slip wall | No-slip wall | No-slip wall |

Table S3 Fitting function for the mass diffusivity coefficient

| Type of gas | Second-order fitting function |
| --- | --- |
| CO2-H2O | D = -3.681×10-6+1.973×10-8T+1.520×10-10T2 |
| CO2-O2 | D = -5.564×10-6+4.349×10-8T+1.008×10-10T2 |
| CO2-NO | D = -5.632×10-6+4.325×10-8T+1.016×10-10T2 |
| CO2-N2 | D = -5.179×10-6+4.381×10-8T+9.812×10-11T2 |
| H2O-O2 | D = -7.106×10-6+4.319×10-8T+1.861×10-10T2 |
| H2O-NO | D = -6.755×10-6+4.039×10-8T+1.868×10-10T2 |
| H2O-N2 | D = -7.940×10-6+5.166×10-8T+1.701×10-10T2 |
| O2-NO | D = -6.665×10-6+5.755×10-8T+1.281×10-10T2 |
| O2-N2 | D = -6.138×10-6+5.699×10-8T+1.232×10-10T2 |
| NO-N2 | D = -6.182×10-6+5.677×10-8T+1.235×10-10T2 |