Supplemental Data

Improved tumor discrimination and shortened administration-to-imaging times in fluorescence guided surgery through paired-agent protocols

This file includes:

Supplementary Figure 1

Supplementary Figure 2

Supplementary Figure 3

Supplementary Figure 4

Supplementary Figure 5

Supplementary Table 1

Supplementary References

S1. Determination of paired-agent dose based on autofluorescence signal

In order to determine the effect of autofluorescence on the observed fluorescence emission intensity, mice (n = 3, per cell line) bearing tongue tumors were sacrificed without injection of fluorescent agents. The tongues were bisected along the raphe and imaged on the Odyssey CLx scanner, as described for paired-agent injected mice in the Materials and Methods. The average fluorescence intensity of the tumor and surrounding normal tongue were calculated and compared to mice with a human-equivalent microdose (30 nanomoles) of each agent (n = 3). Using the method of Reagan-Shaw (2007) (1), the mouse equivalent dose was determined to be 48.8 g/kg and an average mass of 22 g was used for a final dose of 1.07 g per mouse. So 10 x dose is 10.7 g per mouse. At a micro-dose, the ABY-029 signal was >15 times the autofluorescence signal in the 800-channel; however, the IRDye 700DX signal was <10 times the 700-channel autofluorescence. Therefore, 10 times human-equivalent microdose (300 nanomoles) of IRDye 700DX was tested and at 3-hours post-administration it was found that the fluorescence signal was ~8-10 times that of the autofluorescence. Therefore, it was determined that ABY-029 and IRDye 700DX will be administered in a 1:10 molar ratio (30:300 nanomoles). This discrepancy in the dose of the PAI agents is accounted for in the normalization factor (Equation 2).

In addition to determining the dose of paired-agent administration, the autofluorescence values of each tissue affects the observed binding potential values. Typically, autofluorescence is subtracted off of every post-injection image - on a pixel-by-pixel basis if images are taken without moving the animal, or on average if the animal is moved between pre- and post-administration images. Here, it was found that pre-injection the BPs were -0.79 ± 0.04, -0.88 ± 0.02, -0.83 ± 0.01, and -0.83 ± 0.01 for normal tongue, FaDu, Detroit 562, and A431, respectively. The BP values for tumor are lower than that for normal tongue because the autofluorescence in the 700 channel is stronger than that in the 800 channel. This indicates that the BP values we report here are actually underestimating the real BP when autofluorescence is considered, and the difference between normal and tumor tissue would actually be greater if the autofluorescence was considered.

Figure S1. Determining doses of paired-agent imaging using auto-fluorescence. A, For each tumor type a representative example of the autofluorescence in a tongue tumor from FaDu, Detroit 562 and A431 cell lines are shown for the 700 and 800 nm channels of the Odyssey CLx scanner and compared to standard H&E and epidermal growth factor receptor (EGFR) immunohistochemistry (IHC). B, The binding potential (BP) calculated for each naïve tongue is shown in the corresponding box plot.

Figure S2. A, Fluorescence measurements in tumor and normal tissue are shown in the 700 and 800 channels for auto-fluorescence, 1X dose of ABY-029 and IRDye 700DX (mouse equivalent to the human microdose, 30 nanomoles), and a 1X dose of ABY-029 and 10X dose of IRDye 700DX. B, The naïve and 10X dose values of IRDye 700DX are shown for each tumor type.

S2. Comparison of tumor-to-background ratio and contrast-to-variance ratio to measure image contrast

The normalization factor (NF) is an import part of calculating the binding potential (BP) as is corrects the fluorescence emission intensities of the targeted and non-targeted agent to remove effects of fluorescence emission efficiency and instrument excitation and collection efficiencies (Equation 2). Typically, the NF is calculated from tissue that lacks the receptor of interest (i.e. muscle) on the first image post-administration of the paired-agents after autofluorescence subtraction. However, in this case and during surgery it is nearly impossible to collect meaningful pre-injection image. To test the effect of the NF on the BP, the NF was varied from 0.2-0.4 and the resulting tumor-to-background ratio (TBR = average fluorescence in tumor divided by the average fluorescence in the normal tissue) and the contrast-to-variance ratio (CVR, Equation 3) was determined for each NF. As can be seen in Figure S2, varying the NF had an effect similar to window-leveling an image. The CVR remained stable at 1.7, while the TBR varied from 3.3 -19.0 over the range of NFs. As the NF increases in value, the fluorescence signal in the normal tissue decreases toward zero, causing an artificially high TBR. Although TBR is commonly used when analyzing contrast in FGS, it is an unstable measure as demonstrated here where the PAI determined BP could be artificially made to have high TBR values by simply altering the NF. CVR is a more robust measure that allows accurate comparisons between different imaging methods (2). Note that the NF here is less than one because the IRDye 700DX is 10 times more concentrated than the ABY-029 for administration and therefore is brighter. The effect on TBR while changing the NF would be opposite if the targeted agent was brighter than the non-targeted agent.

For this study, the NF was determined in the surrounding normal tissue such that the resultant BP was equal to 0.5 (Equation 1). A BP of 0.5 was used instead of 0 in order to minimize pixels with a negative BP value in the normal tongue. For use during surgery, it is hypothesized that the surgeon could select a region of known normal tissue in order to determine the NF ratio for that patient. This would optimize visualization by creating a “zero background” signal in the normal surrounding tissues.

Figure S3. The effect of the normalization factor (NF) on image contrast. As the normalization ratio increases, the overall signal decreases. This results in a constant contrast-to-variance ratio (CVR) that is stable when NF is varied. However, tumor-to-background ratio (TBR) increases as the signal in the normal tissue (the denominator) decreases. These results indicate that TBR is an unstable measure for binding potential and con not be used to compare to single agent fluorescence.

S3. Spatial alignment of fresh tissue sections with pathological sections

Assessment of the spatial alignment performed between the IHC and the fluorescence images was performed by creating image overlays with the mshowpair function in Matlab (Figure S3) (3).  “Checkerboard” creates an image with alternating rectangular regions from fluorescence   images and IHC images.

Figure S4. Image overlays created using imshowpair in Matlab allow visual assessment of spatial alignment between IHC and BP images. In the Checkerboard overlay, IHC is in RGB (brown pixels) while the BP map is a grey scale image.

S4. Determination of EGFR per cell using flow cytometry

The methodology for the quantification of the number of EGFR per cell using flow cytomtery has been described previously in detail(4). Briefly, cells were purchased from and cultured as per instructions from ATCC. Trypsinized and washed cells (5105) were labeled with either 4 mg/ml of EGF Biotin (Molecular Probes, Invitrogen, Camarillo, CA, USA) or PBS in triplicate for each trial. All cells were secondarily labelled with 1:25 dilution of Cy-5 Streptavidin (Invitrogen, Camarillo, CA, USA. Flow cytometric data was acquired with a FACSCalibur analysis system equipped with a FACStation, and Cell Quest Acquisition software (Becton Dickinson, San Jose, CA, USA). The results compared to Cy5 Quantum Cy5 MESF beads by creating a standard fluorescence concentration curve using QuickCal v2.3 software as provided by the manufacturer (Bangs Laboratory, Fishers, IN, USA). The geometic mean of each sample was used to determine the concentration of fluorophore from the concentration curve and the average value of EGF stained cells and control cells are reported in Table S1. The average control signal was subtracted from the average EGF stained cells to remove the contribution of autofluorescence and non-specific staining. Finally, the receptors per cell was calculated assuming one molecule of biotin per EGF and three Cy5 fluorophores per streptavidin molecule, as specified by Invitrogen. The entire procedure was repeated three times for each cell line, with the exception of A431, which was repeated four times due to large variation. The values for the EGF stained and control cells for each cell line are summarized in Table S1 and the average value plotted in Figure S5.

The cell lines selected for the study were A431, Detroit 562, and FaDu based on a combination of the EGFR expression (high, moderate, low-moderate) and the ability to successfully grow in orthotopic tongue tumors in mice.

Table S1. Summary of individual trial and average values of EGFR per cell determined by quantitative flow cytometry

|  |  | Squamous Cell Carcinoma Cell Lines |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A431 | SCC-15 | SCC-25 | Detroit 562 | FaDu | SCC-9 |
| Trial 1 | EGF Stained | 4440540 | 1787549 | 1113122 | 593638 | 307525 | 210564 |
|  | Control | 46448 | 99511 | 135942 | 8875 | 76920 | 115359 |
|  | EGF Stained - Control | 4394092 | 1688039 | 977181 | 584763 | 230604 | 95205 |
|  | EGFR/cell | 1464697 | 562680 | 325727 | 194921 | 76868 | 31735 |
| Trial 2 | EGF Stained | 4224933 | 1625443 | 1251095 | 571489 | 231411 | 95078 |
|  | Control | 53591 | 9653 | 48738 | 10691 | 5562 | 8774 |
|  | EGF Stained - Control | 41713412 | 1615790 | 1202357 | 560798 | 225849 | 86304 |
|  | EGFR/cell | 1390447 | 538597 | 400786 | 186933 | 75283 | 28768 |
| Trial 3 | EGF Stained | 2898956 | 1652385 | 1071946 | 268314 | 212193 | 92306 |
|  | Control | 57527 | 7181 | 9252 | 4332 | 2791 | 7400 |
|  | EGF Stained - Control | 2841429 | 1645204 | 1062694 | 263982 | 209402 | 84906 |
|  | EGFR/cell | 947143 | 548401 | 354231 | 87994 | 69801 | 28302 |
| Trial 4 | EGF Stained | 3040764 | - | - | - | - | - |
|  | Control | 44117 | - | - | - | - | - |
|  | EGF Stained - Control | 2996647 | - | - | - | - | - |
|  | EGFR/cell | 998882 | - | - | - | - | - |
| EGFR /Cell | Average | 120104 | 55104 | 36104 | 16104 | 7.4104 | 3.0104 |
|  | Std. Dev. | 30104 | 1104 | 4104 | 6104 | 0.4104 | 0.2104 |

Figure S5. EGFR molecules per cell line for squamous cell carcinomas determined by quantitative flow cytometry. Cell lines used for this study are indicated in green.

Supplementary References

1. 	Reagan-Shaw S, Nihal M, Ahmad N. Dose translation from animal to human studies revisited. FASEB J. 2008 Mar;22(3):659–61.

2. 	Tichauer KM, Wang C, Xu X, Samkoe KS. Task-based evaluation of fluorescent-guided cancer surgery as a means of identifying optimal imaging agent properties in the context of variability in tumor- and healthy-tissue physiology. In: Molecular-Guided Surgery: Molecules, Devices, and Applications VI [Internet]. International Society for Optics and Photonics; 2020 [cited 2020 Apr 21]. p. 112220O. Available from: https://www.spiedigitallibrary.org/conference-proceedings-of-spie/11222/112220O/Task-based-evaluation-of-fluorescent-guided-cancer-surgery-as-a/10.1117/12.2546700.short

3. 	Muthukumaran D, Sivakumar M. Medical Image Registration: A Matlab Based Approach. 2017;

4. 	Tichauer KM, Samkoe KS, Sexton KJ, Hextrum SK, Yang HH, Klubben WS, et al. In vivo quantification of tumor receptor binding potential with dual-reporter molecular imaging. Molecular Imaging and Biology. 2012;14(5):584–592.