Supplementary material

Optimal design of a green ammonia export supply chain from Saudi Arabia under demand, price and route uncertainty: a two-stage stochastic programming approach

# S1. Nomenclature

Table S1. Sets, indices, decision variables and parameters.

| Symbol | Description | Unit |
| --- | --- | --- |
| Sets and indices |  |  |
| i ∈ I | candidate export sites | - |
| m ∈ M | import markets (Rotterdam, Japan, Korea) | - |
| r ∈ R(i,m) | sea routes from site i to market m | - |
| k ∈ K | renewable supply blocks (PV, wind, hybrid) | - |
| s ∈ S | scenarios | - |
| First-stage variables |  |  |
| yi | 1 if site i is opened, 0 otherwise | binary |
| Bik | electrolyzer capacity of block k at site i | MW |
| PiPV | installed PV capacity at site i | MW |
| PiW | installed onshore wind capacity at site i | MW |
| PiEL | installed electrolyzer capacity at site i | MW |
| Ci | ammonia production capacity at site i | t NH3/yr |
| Second-stage variables |  |  |
| ximrs | ammonia delivered from i to m by route r in scenario s | t NH3/yr |
| η, zs | auxiliary variables of the CVaR formulation | USD/yr |
| Parameters |  |  |
| ps | probability of scenario s | - |
| πms | delivered (CFR) ammonia price in market m, scenario s | USD/t NH3 |
| Dms | addressable demand in market m, scenario s | t NH3/yr |
| cimrsship | shipping cost per tonne delivered | USD/t NH3 |
| cvar | variable production cost | USD/t NH3 |
| δimr | fraction of loaded cargo delivered after boil-off | - |
| fi | site development and export jetty cost at site i | USD |
| Cimax | maximum ammonia capacity at site i | t NH3/yr |
| a | capital recovery factor | 1/yr |
| aPV, aW, aEL | annualized capital and fixed O&M per MW | USD/MW/yr |
| aHB, aST, aH2 | annualized capital and fixed O&M per t/yr of capacity | USD/(t/yr)/yr |
| ρkPV, ρkW | PV and wind capacity per MW of electrolyzer in block k | MW/MW |
| γk | curtailment fraction of block k | - |
| CFiPV, CFiW | PV and wind capacity factors at site i | - |
| uik | achievable electrolyzer utilization of block k at site i | - |
| eEL | electricity for electrolysis per tonne of ammonia | MWh/t NH3 |
| eHB | electricity for synthesis and air separation per tonne | MWh/t NH3 |
| λ, α | risk-aversion weight and CVaR confidence level | - |
| F, Πs | annual fixed cost and scenario profit | USD/yr |

# S2. Complete input data

Table S2 lists every parameter used in the model, its value, unit and source. Entries marked “assumed” are modeling choices and not published values. Of these, the curtailment of the hybrid block, the hydrogen buffer cost, the site capacity limit, the bunker price, the Saudi market shares (through the demand scale) and the route-state probability are varied in the sensitivity analysis (Table S7); the other assumed values are held fixed. Sources refer to the reference list of the main paper.

Table S2. Model input parameters, values and sources.

| Parameter | Value | Unit | Source |
| --- | --- | --- | --- |
| Renewable generation |  |  |  |
| PV capital cost | 555 | USD/kW | Alhadhrami et al. (2024) |
| PV fixed O&M | 10 | USD/kW/yr | assumed (1.8 % of capital) |
| Onshore wind capital cost | 992 | USD/kW | Alhadhrami et al. (2024) |
| Onshore wind fixed O&M | 30 | USD/kW/yr | assumed (3.0 % of capital) |
| PV capacity factor scaling | 1.25 | - | assumed (tracking and bifacial) |
| Electrolysis |  |  |  |
| Alkaline electrolyzer capital cost | 700 | USD/kW | Alhadhrami et al. (2024); range 450-1820 in IEA (2025) |
| Electrolyzer fixed O&M | 3.0 | % of capital/yr | IEA (2025) |
| Specific electricity use | 52.1 | kWh/kg H2 | IEA (2025) (64 % LHV, 2030) |
| Ammonia synthesis |  |  |  |
| Haber-Bosch and ASU capital cost | 770 | USD per t/yr | Kriegsmann et al. (2025); range 560-980 in IEA (2025) |
| Haber-Bosch and ASU fixed O&M | 2.9 | % of capital/yr | IEA (2025) |
| Haber-Bosch and ASU electricity | 0.6 | MWh/t NH3 | IEA (2025) |
| Balance-of-plant electricity factor | 1.05 | - | assumed |
| Hydrogen per tonne of ammonia | 0.1765 | t H2/t NH3 | stoichiometric |
| Storage and water |  |  |  |
| Ammonia storage capital cost | 800 | USD/t capacity | within 560-1060 of Cao et al. (2025) |
| Ammonia storage sizing | 15 | days of production | Kriegsmann et al. (2025) |
| Ammonia storage fixed O&M | 3.0 | % of capital/yr | assumed |
| Hydrogen buffer capital cost | 100 | USD per t/yr NH3 | assumed; adds 12 USD/t (IRENA 2022: 35-150 USD/t) |
| Water use | 9 | L/kg H2 | stoichiometric minimum, rounded |
| Desalinated water price | 0.413 | USD/m3 | Knott (2020) |
| Other variable cost | 5 | USD/t NH3 | assumed |
| Finance and site limits |  |  |  |
| Cost of capital (real) | 8.0 | % | Alturki (2026); IRENA (2020) uses 6 and 10 |
| Plant life | 25 | years | Alturki (2026) |
| Capital recovery factor | 0.0937 | 1/yr | derived |
| Site development and jetty, sheltered | 140 | M USD | IEA (2025) plus 50 M USD site works (assumed) |
| Site development and jetty, open coast (Rabigh) | 240 | M USD | IEA (2025) plus 50 M USD site works (assumed) |
| Maximum capacity per site | 2.0 | Mt NH3/yr | assumed; 1.5 and 3.0 tested |
| Shipping |  |  |  |
| Carrier size | 84,000 | m3 | Seo et al. (2024) |
| Sea distances | Table 1 of the main paper | nm | Halili (2026); Eurostat (2022) |
| Cargo per voyage | 56,000 | t NH3 | derived (0.682 t/m3, 98 % fill); Engineering ToolBox (2005) |
| Service speed | 15 | knots | BW LPG (n.d.): 14.8 knots average |
| Time charter rate | 45,000 | USD/day | BW LPG (2026); Dorian LPG (2026); 30,000-90,000 tested |
| Bunker consumption at sea | 45 | t/day | BW LPG (n.d.): about 42 t/day |
| Bunker price (VLSFO) | 600 | USD/t | assumed long-run; 925 (2026) tested |
| Port time | 2 | days per round trip | Seo et al. (2024) |
| Port fee | 50,000 | USD per call | assumed |
| Suez toll, laden transit | 380,000 | USD | derived from Suez Canal Authority (2024) |
| Suez toll, ballast transit | 323,000 | USD (85 % of laden) | derived from Suez Canal Authority (2024) |
| Boil-off rate | 0.03 | %/day | assumed; IEA (2023) carrier 0.02 |
| Hull value for war-risk premium | 125 | M USD | assumed |
| War-risk premium, disrupted state | 0.5 | % of hull per transit | UNCTAD (2024a); Schuler (2026); 0.2-1.0 tested |
| Renewable supply blocks |  |  |  |
| PV block: PV per MW electrolyzer, curtailment | 1.5 / 0.18 | MW/MW, - | assumed |
| Wind block: wind per MW electrolyzer, curtailment | 1.2 / 0.10 | MW/MW, - | assumed |
| Hybrid block: PV, wind, curtailment | 1.2 / 0.8 / 0.08 | MW/MW, - | assumed; checked against NGHC (2026); hybrid curtailment 0.04-0.12 tested |
| Scenario structure |  |  |  |
| Demand and price levels | 5 each | - | similar to Ochoa Bique et al. (2023); Efthymiadou et al. (2025) |
| Level probabilities | 0.1 / 0.2 / 0.4 / 0.2 / 0.1 | - | assumed |
| Route states, probabilities | normal 0.75, disrupted 0.25 | - | assumed; 0-0.5 tested |
| Ammonia share of EU hydrogen imports | 40 | % | assumed |
| Saudi market share, EU | 25 | % | assumed; demand scale 0.5-2 tested |
| Saudi market share, Japan and Korea | 30 | % | assumed; demand scale 0.5-2 tested |

Sources refer to the reference list of the main paper.

# S3. Gate cost build-up by site and supply block

Table S3 gives the levelized gate cost of ammonia for every combination of site and renewable supply block, at full utilization of a 1 Mt/yr plant. The block selected in Fig. 1 of the main paper is the cheapest row for each site.

Table S3. Levelized gate cost of ammonia by site and supply block (USD/t NH3).

| Site | Block | Electrolyzer utilization | t NH3 per MW electrolyzer | PV | Wind | Electrolyzer | HB + ASU | Storage | Site and variable | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NEOM | PV only | 0.322 | 292 | 318.2 | 0.0 | 296.2 | 94.5 | 16.4 | 18.8 | 744.0 |
| NEOM | Wind only | 0.375 | 340 | 0.0 | 433.4 | 254.4 | 94.5 | 16.4 | 18.8 | 817.5 |
| NEOM | Hybrid | 0.545 | 494 | 150.5 | 199.0 | 175.2 | 94.5 | 16.4 | 18.8 | 654.3 |
| Yanbu | PV only | 0.316 | 287 | 324.0 | 0.0 | 301.6 | 94.5 | 16.4 | 18.8 | 755.2 |
| Yanbu | Wind only | 0.335 | 304 | 0.0 | 486.0 | 285.2 | 94.5 | 16.4 | 18.8 | 900.9 |
| Yanbu | Hybrid | 0.512 | 464 | 160.2 | 211.7 | 186.4 | 94.5 | 16.4 | 18.8 | 687.9 |
| Rabigh | PV only | 0.305 | 277 | 336.2 | 0.0 | 313.0 | 94.5 | 16.4 | 28.1 | 788.3 |
| Rabigh | Wind only | 0.223 | 202 | 0.0 | 729.0 | 427.8 | 94.5 | 16.4 | 28.1 | 1295.8 |
| Rabigh | Hybrid | 0.426 | 386 | 192.7 | 254.7 | 224.2 | 94.5 | 16.4 | 28.1 | 810.6 |
| Jubail | PV only | 0.290 | 263 | 353.6 | 0.0 | 329.3 | 94.5 | 16.4 | 18.8 | 812.6 |
| Jubail | Wind only | 0.253 | 230 | 0.0 | 641.5 | 376.5 | 94.5 | 16.4 | 18.8 | 1147.6 |
| Jubail | Hybrid | 0.433 | 393 | 189.4 | 250.4 | 220.5 | 94.5 | 16.4 | 18.8 | 790.0 |
| Ras Al-Khair | PV only | 0.293 | 266 | 349.5 | 0.0 | 325.4 | 94.5 | 16.4 | 18.8 | 804.5 |
| Ras Al-Khair | Wind only | 0.253 | 230 | 0.0 | 641.5 | 376.5 | 94.5 | 16.4 | 18.8 | 1147.6 |
| Ras Al-Khair | Hybrid | 0.436 | 396 | 188.1 | 248.6 | 218.9 | 94.5 | 16.4 | 18.8 | 785.3 |

# S4. Complete shipping cost table

Table S4 gives the shipping cost per tonne delivered for every combination of site, market, route and route state. “Standard” is the Suez route for Rotterdam and the Malacca route for East Asia, and “Cape” is the Cape of Good Hope route to Rotterdam. In the disrupted state the Suez route to Rotterdam is unavailable and a war-risk premium applies to every Bab el-Mandeb transit; the rows that the model cannot use in a given state are shown for completeness.

Table S4. Shipping cost by site, market, route and route state (USD per tonne delivered).

| Site | Market | Route | Route state | Shipping cost |
| --- | --- | --- | --- | --- |
| NEOM | Rotterdam | standard | Normal | 41.7 |
| NEOM | Rotterdam | standard | Disrupted | 41.7 |
| NEOM | Rotterdam | Cape | Normal | 85.4 |
| NEOM | Rotterdam | Cape | Disrupted | 107.9 |
| NEOM | Japan | standard | Normal | 58.9 |
| NEOM | Japan | standard | Disrupted | 81.3 |
| NEOM | Korea | standard | Normal | 56.2 |
| NEOM | Korea | standard | Disrupted | 78.7 |
| Yanbu | Rotterdam | standard | Normal | 43.9 |
| Yanbu | Rotterdam | standard | Disrupted | 43.9 |
| Yanbu | Rotterdam | Cape | Normal | 84.0 |
| Yanbu | Rotterdam | Cape | Disrupted | 106.5 |
| Yanbu | Japan | standard | Normal | 57.5 |
| Yanbu | Japan | standard | Disrupted | 80.0 |
| Yanbu | Korea | standard | Normal | 54.9 |
| Yanbu | Korea | standard | Disrupted | 77.3 |
| Rabigh | Rotterdam | standard | Normal | 44.7 |
| Rabigh | Rotterdam | standard | Disrupted | 44.7 |
| Rabigh | Rotterdam | Cape | Normal | 82.6 |
| Rabigh | Rotterdam | Cape | Disrupted | 105.1 |
| Rabigh | Japan | standard | Normal | 56.0 |
| Rabigh | Japan | standard | Disrupted | 78.5 |
| Rabigh | Korea | standard | Normal | 53.4 |
| Rabigh | Korea | standard | Disrupted | 75.9 |
| Jubail | Rotterdam | standard | Normal | 62.9 |
| Jubail | Rotterdam | standard | Disrupted | 85.4 |
| Jubail | Rotterdam | Cape | Normal | 85.0 |
| Jubail | Rotterdam | Cape | Disrupted | 85.0 |
| Jubail | Japan | standard | Normal | 51.6 |
| Jubail | Japan | standard | Disrupted | 51.6 |
| Jubail | Korea | standard | Normal | 48.9 |
| Jubail | Korea | standard | Disrupted | 48.9 |
| Ras Al-Khair | Rotterdam | standard | Normal | 63.6 |
| Ras Al-Khair | Rotterdam | standard | Disrupted | 86.0 |
| Ras Al-Khair | Rotterdam | Cape | Normal | 85.6 |
| Ras Al-Khair | Rotterdam | Cape | Disrupted | 85.6 |
| Ras Al-Khair | Japan | standard | Normal | 52.2 |
| Ras Al-Khair | Japan | standard | Disrupted | 52.2 |
| Ras Al-Khair | Korea | standard | Normal | 49.6 |
| Ras Al-Khair | Korea | standard | Disrupted | 49.6 |

# S5. Complete scenario set

Table S5 lists all 50 scenarios. Demand, price and route state are treated as independent, so the scenario probability is the product of the three level probabilities. Demand is the total addressable demand across the three markets.

Table S5. Scenario definitions and probabilities.

| Scenario | Demand level | Price level | Route state | Probability | Total demand (Mt/yr) | Rotterdam price (USD/t) | Japan price (USD/t) | Korea price (USD/t) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L-L-N | Low | Low | Normal | 0.0075 | 0.76 | 650 | 600 | 590 |
| L-L-D | Low | Low | Disrupted | 0.0025 | 0.76 | 650 | 600 | 590 |
| L-LM-N | Low | Low-mid | Normal | 0.0150 | 0.76 | 750 | 700 | 685 |
| L-LM-D | Low | Low-mid | Disrupted | 0.0050 | 0.76 | 750 | 700 | 685 |
| L-M-N | Low | Mid | Normal | 0.0300 | 0.76 | 850 | 800 | 780 |
| L-M-D | Low | Mid | Disrupted | 0.0100 | 0.76 | 850 | 800 | 780 |
| L-MH-N | Low | Mid-high | Normal | 0.0150 | 0.76 | 950 | 900 | 880 |
| L-MH-D | Low | Mid-high | Disrupted | 0.0050 | 0.76 | 950 | 900 | 880 |
| L-H-N | Low | High | Normal | 0.0075 | 0.76 | 1050 | 1000 | 980 |
| L-H-D | Low | High | Disrupted | 0.0025 | 0.76 | 1050 | 1000 | 980 |
| LM-L-N | Low-mid | Low | Normal | 0.0150 | 1.79 | 650 | 600 | 590 |
| LM-L-D | Low-mid | Low | Disrupted | 0.0050 | 1.79 | 650 | 600 | 590 |
| LM-LM-N | Low-mid | Low-mid | Normal | 0.0300 | 1.79 | 750 | 700 | 685 |
| LM-LM-D | Low-mid | Low-mid | Disrupted | 0.0100 | 1.79 | 750 | 700 | 685 |
| LM-M-N | Low-mid | Mid | Normal | 0.0600 | 1.79 | 850 | 800 | 780 |
| LM-M-D | Low-mid | Mid | Disrupted | 0.0200 | 1.79 | 850 | 800 | 780 |
| LM-MH-N | Low-mid | Mid-high | Normal | 0.0300 | 1.79 | 950 | 900 | 880 |
| LM-MH-D | Low-mid | Mid-high | Disrupted | 0.0100 | 1.79 | 950 | 900 | 880 |
| LM-H-N | Low-mid | High | Normal | 0.0150 | 1.79 | 1050 | 1000 | 980 |
| LM-H-D | Low-mid | High | Disrupted | 0.0050 | 1.79 | 1050 | 1000 | 980 |
| M-L-N | Mid | Low | Normal | 0.0300 | 2.82 | 650 | 600 | 590 |
| M-L-D | Mid | Low | Disrupted | 0.0100 | 2.82 | 650 | 600 | 590 |
| M-LM-N | Mid | Low-mid | Normal | 0.0600 | 2.82 | 750 | 700 | 685 |
| M-LM-D | Mid | Low-mid | Disrupted | 0.0200 | 2.82 | 750 | 700 | 685 |
| M-M-N | Mid | Mid | Normal | 0.1200 | 2.82 | 850 | 800 | 780 |
| M-M-D | Mid | Mid | Disrupted | 0.0400 | 2.82 | 850 | 800 | 780 |
| M-MH-N | Mid | Mid-high | Normal | 0.0600 | 2.82 | 950 | 900 | 880 |
| M-MH-D | Mid | Mid-high | Disrupted | 0.0200 | 2.82 | 950 | 900 | 880 |
| M-H-N | Mid | High | Normal | 0.0300 | 2.82 | 1050 | 1000 | 980 |
| M-H-D | Mid | High | Disrupted | 0.0100 | 2.82 | 1050 | 1000 | 980 |
| MH-L-N | Mid-high | Low | Normal | 0.0150 | 4.71 | 650 | 600 | 590 |
| MH-L-D | Mid-high | Low | Disrupted | 0.0050 | 4.71 | 650 | 600 | 590 |
| MH-LM-N | Mid-high | Low-mid | Normal | 0.0300 | 4.71 | 750 | 700 | 685 |
| MH-LM-D | Mid-high | Low-mid | Disrupted | 0.0100 | 4.71 | 750 | 700 | 685 |
| MH-M-N | Mid-high | Mid | Normal | 0.0600 | 4.71 | 850 | 800 | 780 |
| MH-M-D | Mid-high | Mid | Disrupted | 0.0200 | 4.71 | 850 | 800 | 780 |
| MH-MH-N | Mid-high | Mid-high | Normal | 0.0300 | 4.71 | 950 | 900 | 880 |
| MH-MH-D | Mid-high | Mid-high | Disrupted | 0.0100 | 4.71 | 950 | 900 | 880 |
| MH-H-N | Mid-high | High | Normal | 0.0150 | 4.71 | 1050 | 1000 | 980 |
| MH-H-D | Mid-high | High | Disrupted | 0.0050 | 4.71 | 1050 | 1000 | 980 |
| H-L-N | High | Low | Normal | 0.0075 | 6.58 | 650 | 600 | 590 |
| H-L-D | High | Low | Disrupted | 0.0025 | 6.58 | 650 | 600 | 590 |
| H-LM-N | High | Low-mid | Normal | 0.0150 | 6.58 | 750 | 700 | 685 |
| H-LM-D | High | Low-mid | Disrupted | 0.0050 | 6.58 | 750 | 700 | 685 |
| H-M-N | High | Mid | Normal | 0.0300 | 6.58 | 850 | 800 | 780 |
| H-M-D | High | Mid | Disrupted | 0.0100 | 6.58 | 850 | 800 | 780 |
| H-MH-N | High | Mid-high | Normal | 0.0150 | 6.58 | 950 | 900 | 880 |
| H-MH-D | High | Mid-high | Disrupted | 0.0050 | 6.58 | 950 | 900 | 880 |
| H-H-N | High | High | Normal | 0.0075 | 6.58 | 1050 | 1000 | 980 |
| H-H-D | High | High | Disrupted | 0.0025 | 6.58 | 1050 | 1000 | 980 |

# S6. Profit by scenario and wait-and-see solutions

Table S6 gives, for each scenario, the annual profit of the stochastic solution (RP), of the expected-value solution evaluated over all scenarios (EEV), and of the wait-and-see solution designed with knowledge of that scenario, together with the capacity and sites the wait-and-see solution would build. Probability-weighted averages of the three profit columns are 160.4, -140.2 and 359.2 million USD per year, which give the values reported in Table 6 of the main paper.

Table S6. Annual profit by scenario for the stochastic, expected-value and wait-and-see solutions.

| Scenario | Probability | RP profit (M USD/yr) | EEV profit (M USD/yr) | WS profit (M USD/yr) | WS capacity (Mt/yr) | WS sites opened |
| --- | --- | --- | --- | --- | --- | --- |
| L-L-N | 0.0075 | -722.5 | -1649.8 | 0.0 | 0.00 | none |
| L-L-D | 0.0025 | -757.1 | -1683.8 | 0.0 | 0.00 | none |
| L-LM-N | 0.0150 | -647.1 | -1574.4 | 13.0 | 0.40 | NEOM |
| L-LM-D | 0.0050 | -681.7 | -1608.4 | 0.0 | 0.00 | none |
| L-M-N | 0.0300 | -571.7 | -1499.0 | 85.4 | 0.76 | NEOM |
| L-M-D | 0.0100 | -606.3 | -1533.0 | 49.1 | 0.77 | NEOM |
| L-MH-N | 0.0150 | -495.7 | -1423.0 | 161.4 | 0.76 | NEOM |
| L-MH-D | 0.0050 | -530.3 | -1457.0 | 125.1 | 0.77 | NEOM |
| L-H-N | 0.0075 | -419.7 | -1347.0 | 237.4 | 0.76 | NEOM |
| L-H-D | 0.0025 | -454.3 | -1381.0 | 201.1 | 0.77 | NEOM |
| LM-L-N | 0.0150 | -118.6 | -1045.5 | 0.0 | 0.00 | none |
| LM-L-D | 0.0050 | -214.8 | -1137.1 | 0.0 | 0.00 | none |
| LM-LM-N | 0.0300 | 59.4 | -867.6 | 64.5 | 1.19 | NEOM |
| LM-LM-D | 0.0100 | -37.6 | -959.1 | 0.0 | 0.00 | none |
| LM-M-N | 0.0600 | 237.3 | -689.6 | 237.3 | 1.80 | NEOM |
| LM-M-D | 0.0200 | 139.6 | -781.2 | 140.1 | 1.81 | NEOM |
| LM-MH-N | 0.0300 | 416.3 | -510.6 | 416.3 | 1.80 | NEOM |
| LM-MH-D | 0.0100 | 317.8 | -602.2 | 319.1 | 1.81 | NEOM |
| LM-H-N | 0.0150 | 595.3 | -331.6 | 595.3 | 1.80 | NEOM |
| LM-H-D | 0.0050 | 496.1 | -423.2 | 498.1 | 1.81 | NEOM |
| M-L-N | 0.0300 | -75.5 | -441.3 | 0.0 | 0.00 | none |
| M-L-D | 0.0100 | -200.5 | -590.9 | 0.0 | 0.00 | none |
| M-LM-N | 0.0600 | 103.7 | -160.8 | 116.0 | 1.99 | NEOM |
| M-LM-D | 0.0200 | -22.4 | -310.4 | 0.0 | 0.00 | none |
| M-M-N | 0.1200 | 282.9 | 119.7 | 349.4 | 2.83 | NEOM, Yanbu |
| M-M-D | 0.0400 | 155.6 | -29.9 | 190.7 | 2.84 | NEOM, Yanbu |
| M-MH-N | 0.0600 | 462.1 | 401.7 | 631.4 | 2.83 | NEOM, Yanbu |
| M-MH-D | 0.0200 | 333.6 | 252.1 | 472.7 | 2.84 | NEOM, Yanbu |
| M-H-N | 0.0300 | 641.3 | 683.7 | 913.4 | 2.83 | NEOM, Yanbu |
| M-H-D | 0.0100 | 511.7 | 534.1 | 754.7 | 2.84 | NEOM, Yanbu |
| MH-L-N | 0.0150 | -75.5 | -178.6 | 0.0 | 0.00 | none |
| MH-L-D | 0.0050 | -200.5 | -394.9 | 0.0 | 0.00 | none |
| MH-LM-N | 0.0300 | 103.7 | 137.8 | 146.4 | 3.47 | NEOM, Yanbu |
| MH-LM-D | 0.0100 | -22.4 | -80.5 | 0.0 | 0.00 | none |
| MH-M-N | 0.0600 | 282.9 | 454.3 | 525.8 | 4.00 | NEOM, Yanbu |
| MH-M-D | 0.0200 | 155.6 | 234.0 | 274.3 | 4.00 | NEOM, Yanbu |
| MH-MH-N | 0.0300 | 462.1 | 770.8 | 953.8 | 4.73 | NEOM, Yanbu, Ras Al-Khair |
| MH-MH-D | 0.0100 | 333.6 | 548.5 | 701.6 | 4.75 | NEOM, Yanbu, Ras Al-Khair |
| MH-H-N | 0.0150 | 641.3 | 1087.2 | 1424.8 | 4.73 | NEOM, Yanbu, Ras Al-Khair |
| MH-H-D | 0.0050 | 511.7 | 862.9 | 1172.6 | 4.75 | NEOM, Yanbu, Ras Al-Khair |
| H-L-N | 0.0075 | -75.5 | -178.6 | 0.0 | 0.00 | none |
| H-L-D | 0.0025 | -200.5 | -394.9 | 0.0 | 0.00 | none |
| H-LM-N | 0.0150 | 103.7 | 137.8 | 161.8 | 4.00 | NEOM, Yanbu |
| H-LM-D | 0.0050 | -22.4 | -80.5 | 0.0 | 0.00 | none |
| H-M-N | 0.0300 | 282.9 | 454.3 | 572.8 | 4.95 | NEOM, Yanbu, Rabigh |
| H-M-D | 0.0100 | 155.6 | 234.0 | 284.9 | 4.00 | NEOM, Yanbu |
| H-MH-N | 0.0150 | 462.1 | 770.8 | 1159.7 | 6.61 | NEOM, Yanbu, Rabigh, Ras Al-Khair |
| H-MH-D | 0.0050 | 333.6 | 548.5 | 839.2 | 6.64 | NEOM, Yanbu, Jubail, Ras Al-Khair |
| H-H-N | 0.0075 | 641.3 | 1087.2 | 1817.7 | 6.61 | NEOM, Yanbu, Rabigh, Ras Al-Khair |
| H-H-D | 0.0025 | 511.7 | 862.9 | 1497.2 | 6.64 | NEOM, Yanbu, Jubail, Ras Al-Khair |

# S7. Complete sensitivity and risk analysis results

Table S7 gives every one-at-a-time sensitivity case run for the main paper, including the cases summarized in Fig. 5, Fig. 6 and Table 7. The base case is the row with a price shift of zero.

Table S7. One-at-a-time sensitivity results.

| Parameter | Value | Expected profit (M USD/yr) | Capacity (Mt/yr) | Sites opened | Minimum gate cost (USD/t) | Electrolyzer (GW) |
| --- | --- | --- | --- | --- | --- | --- |
| Electrolyzer capital cost (USD/kW) | 450 | 272.9 | 1.81 | NEOM | 585.9 | 3.65 |
| Electrolyzer capital cost (USD/kW) | 700 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Electrolyzer capital cost (USD/kW) | 1000 | 39.6 | 0.77 | NEOM | 733.4 | 1.55 |
| Electrolyzer capital cost (USD/kW) | 1400 | 0.0 | 0.00 | none | - | 0.00 |
| Cost of capital (%) | 5 | 385.4 | 2.00 | NEOM | 526.3 | 4.05 |
| Cost of capital (%) | 6 | 309.2 | 1.81 | NEOM | 565.8 | 3.65 |
| Cost of capital (%) | 8 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Cost of capital (%) | 10 | 28.2 | 0.76 | NEOM | 748.3 | 1.54 |
| PV capital cost (USD/kW) | 400 | 223.8 | 1.80 | NEOM | 613.2 | 3.64 |
| PV capital cost (USD/kW) | 555 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| PV capital cost (USD/kW) | 700 | 101.1 | 1.80 | NEOM | 681.5 | 3.64 |
| Wind capital cost (USD/kW) | 650 | 253.6 | 1.80 | NEOM | 596.6 | 3.64 |
| Wind capital cost (USD/kW) | 992 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Wind capital cost (USD/kW) | 1250 | 90.1 | 1.80 | NEOM | 687.6 | 3.64 |
| HB+ASU capital cost (USD per t/yr) | 560 | 206.7 | 1.80 | NEOM | 622.7 | 3.64 |
| HB+ASU capital cost (USD per t/yr) | 770 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| HB+ASU capital cost (USD per t/yr) | 980 | 114.1 | 1.80 | NEOM | 674.2 | 3.64 |
| Charter rate (USD/day) | 30,000 | 175.8 | 1.80 | NEOM | 648.5 | 3.64 |
| Charter rate (USD/day) | 45,000 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Charter rate (USD/day) | 60,000 | 145.1 | 1.80 | NEOM | 648.5 | 3.64 |
| Charter rate (USD/day) | 90,000 | 114.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Bunker price (USD/t) | 400 | 169.1 | 1.80 | NEOM | 648.5 | 3.64 |
| Bunker price (USD/t) | 600 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Bunker price (USD/t) | 925 | 146.3 | 1.80 | NEOM | 648.5 | 3.64 |
| Price shift (USD/t) | -150 | 0.0 | 0.00 | none | - | 0.00 |
| Price shift (USD/t) | -100 | 20.9 | 0.76 | NEOM | 658.4 | 1.54 |
| Price shift (USD/t) | -50 | 76.1 | 1.80 | NEOM | 648.5 | 3.64 |
| Price shift (USD/t) | 0 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Price shift (USD/t) | +50 | 244.7 | 1.80 | NEOM | 648.5 | 3.64 |
| Price shift (USD/t) | +100 | 329.2 | 1.81 | NEOM | 648.5 | 3.65 |
| Price shift (USD/t) | +150 | 416.8 | 1.99 | NEOM | 647.8 | 4.02 |
| Demand scale | 0.5 | 73.7 | 0.90 | NEOM | 655.8 | 1.82 |
| Demand scale | 0.75 | 117.0 | 1.35 | NEOM | 650.9 | 2.73 |
| Demand scale | 1 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Demand scale | 1.5 | 210.2 | 2.70 | NEOM, Yanbu | 647.7 | 5.54 |
| Demand scale | 2 | 266.0 | 3.59 | NEOM, Yanbu | 647.7 | 7.48 |
| Disruption probability | 0 | 188.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Disruption probability | 0.25 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Disruption probability | 0.5 | 132.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Price outlook | pessimistic | 59.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Price outlook | base | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Price outlook | optimistic | 261.5 | 1.80 | NEOM | 648.5 | 3.64 |
| NEOM excluded | - | 98.0 | 1.80 | Yanbu | 682.1 | 3.87 |
| NEOM and Yanbu excluded | - | 0.0 | 0.00 | none | - | 0.00 |
| Price floor (USD/t) | 650 | 161.8 | 1.80 | NEOM | 648.5 | 3.64 |
| Price floor (USD/t) | 700 | 170.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Price floor (USD/t) | 750 | 181.5 | 1.80 | NEOM | 648.5 | 3.64 |
| Price floor (USD/t) | 800 | 207.2 | 1.80 | NEOM | 648.5 | 3.64 |
| War-risk premium (% of hull per transit) | 0.2 | 166.1 | 1.80 | NEOM | 648.5 | 3.64 |
| War-risk premium (% of hull per transit) | 0.5 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| War-risk premium (% of hull per transit) | 1 | 151.0 | 1.80 | NEOM | 648.5 | 3.64 |
| Hydrogen buffer cost added (USD/t) | 12.4 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Hydrogen buffer cost added (USD/t) | 35 | 119.7 | 1.80 | NEOM | 671.1 | 3.64 |
| Hydrogen buffer cost added (USD/t) | 150 | 0.0 | 0.00 | none | - | 0.00 |
| Site capacity limit (Mt/yr) | 1.5 | 144.0 | 1.50 | NEOM | 649.9 | 3.03 |
| Site capacity limit (Mt/yr) | 2 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Site capacity limit (Mt/yr) | 3 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Hybrid-block curtailment (%) | 4 | 199.7 | 1.80 | NEOM | 626.6 | 3.48 |
| Hybrid-block curtailment (%) | 8 | 160.4 | 1.80 | NEOM | 648.5 | 3.64 |
| Hybrid-block curtailment (%) | 12 | 117.6 | 1.80 | NEOM | 672.3 | 3.80 |

Table S8 gives the complete risk-aversion frontier for the case without price support and for delivered price floors of 700 and 750 USD/t. CVaR is computed at the 90 percent level, so it is the probability-weighted profit in the worst 10 percent of scenarios. Each design is evaluated with its first-stage decisions fixed and the risk-neutral objective, so that the recourse is optimal in every scenario. Rows with zero capacity are cases in which every design that invests gives a negative weighted objective, and the model invests nothing.

Table S8. Expected profit, CVaR and capacity against the risk-aversion weight.

| Price floor (USD/t) | Risk-aversion weight | Expected profit (M USD/yr) | CVaR 0.9 (M USD/yr) | Worst scenario (M USD/yr) | Capacity (Mt/yr) | Sites opened |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.0 | 160.4 | -580.1 | -757.1 | 1.80 | NEOM |
| 0 | 0.1 | 126.9 | -212.9 | -378.3 | 1.20 | NEOM |
| 0 | 0.2 | 107.2 | -81.5 | -195.6 | 0.91 | NEOM |
| 0 | 0.3 | 102.6 | -65.8 | -152.5 | 0.85 | NEOM |
| 0 | 0.4 | 97.1 | -55.3 | -101.7 | 0.77 | NEOM |
| 0 | 0.5 | 96.9 | -55.0 | -101.3 | 0.76 | NEOM |
| 0 | 0.6 | 96.9 | -55.0 | -101.3 | 0.76 | NEOM |
| 0 | 0.7 | 0.0 | 0.0 | 0.0 | 0.00 | none |
| 0 | 0.8 | 0.0 | 0.0 | 0.0 | 0.00 | none |
| 0 | 0.9 | 0.0 | 0.0 | 0.0 | 0.00 | none |
| 0 | 1.0 | 0.0 | 0.0 | 0.0 | 0.00 | none |
| 700 | 0.0 | 170.4 | -574.0 | -699.9 | 1.80 | NEOM |
| 700 | 0.1 | 133.0 | -191.9 | -316.1 | 1.19 | NEOM |
| 700 | 0.2 | 113.4 | -52.9 | -146.1 | 0.93 | NEOM |
| 700 | 0.3 | 102.9 | -20.4 | -55.8 | 0.78 | NEOM |
| 700 | 0.4 | 102.9 | -20.4 | -55.8 | 0.78 | NEOM |
| 700 | 0.5 | 101.6 | -18.5 | -44.5 | 0.77 | NEOM |
| 700 | 0.6 | 101.6 | -18.5 | -44.5 | 0.77 | NEOM |
| 700 | 0.7 | 101.4 | -18.4 | -44.3 | 0.76 | NEOM |
| 700 | 0.8 | 101.4 | -18.4 | -44.3 | 0.76 | NEOM |
| 700 | 0.9 | 0.0 | 0.0 | 0.0 | 0.00 | none |
| 700 | 1.0 | 0.0 | 0.0 | 0.0 | 0.00 | none |
| 750 | 0.0 | 181.5 | -566.6 | -661.9 | 1.80 | NEOM |
| 750 | 0.1 | 140.3 | -182.9 | -278.1 | 1.19 | NEOM |
| 750 | 0.2 | 116.1 | -15.7 | -81.3 | 0.88 | NEOM |
| 750 | 0.3 | 108.2 | 6.8 | -17.8 | 0.78 | NEOM |
| 750 | 0.4 | 106.8 | 9.9 | -6.5 | 0.77 | NEOM |
| 750 | 0.5 | 106.5 | 10.3 | -6.5 | 0.76 | NEOM |
| 750 | 0.6 | 106.5 | 10.3 | -6.5 | 0.76 | NEOM |
| 750 | 0.7 | 106.5 | 10.3 | -6.5 | 0.76 | NEOM |
| 750 | 0.8 | 106.5 | 10.3 | -6.5 | 0.76 | NEOM |
| 750 | 0.9 | 106.5 | 10.3 | -6.5 | 0.76 | NEOM |
| 750 | 1.0 | 106.5 | 10.3 | -6.5 | 0.76 | NEOM |

# S8. Model verification

The checks below were run on the stochastic solution and are reproduced by the script verify.py in the code package.

## S8.1 Scenario set and balances

The 50 scenario probabilities sum to 1.000. At NEOM the stochastic solution builds 1.797 Mt/yr of ammonia capacity, which requires 317 kt/yr of hydrogen. The electrolyzer electricity requirement is 17.35 TWh/yr, exactly equal to the energy available from 3636 MW of electrolysis at the hybrid block utilization of 0.545. The total electricity requirement, including synthesis and air separation, is 18.49 TWh/yr, equal to the usable output of 4364 MW of PV and 2909 MW of wind after 8 percent curtailment. The resulting specific electricity consumption is 10.29 MWh per tonne of ammonia. The renewable-to-electrolyzer capacity ratio is 2.00 and the output is 0.494 Mt of ammonia per GW of electrolysis. The NEOM Green Hydrogen project plans up to 1.2 Mt/yr of ammonia from more than 2 GW of electrolysis fed by 2.2 GW of PV and 1.6 GW of wind (thyssenkrupp 2021; NGHC 2026), which is a ratio of up to 1.9 and up to about 0.6 Mt of ammonia per GW.

## S8.2 Cost build-up and benchmarks

Table S9. Gate cost build-up of the stochastic solution at NEOM.

| Component | USD/t NH3 | Share (%) |
| --- | --- | --- |
| Site development and jetty | 7.3 | 1.1 |
| PV | 150.5 | 23.2 |
| Onshore wind | 199.0 | 30.7 |
| Electrolyzers | 175.2 | 27.0 |
| Haber-Bosch and air separation | 94.5 | 14.6 |
| Ammonia storage | 4.1 | 0.6 |
| Hydrogen buffer | 12.4 | 1.9 |
| Variable cost including water | 5.7 | 0.9 |
| Total | 648.5 | 100.0 |

The gate cost of 648.5 USD/t lies within the 556 to 680 USD/t reported for NEOM and Duqm by Sadiq et al. (2025). The implied hydrogen cost of 2.9 USD/kg lies within the 2.34 to 3.08 USD/kg of Alhadhrami et al. (2024). The shipping cost of 42 USD/t from NEOM to Rotterdam through Suez is close to the 37 USD/t of KBR (2022) for Saudi Arabia to Rotterdam and below the 49 to 67 USD/t reported for Chile to Germany over a longer distance (Souissi 2024). The Jubail to Korea cost of 49 USD/t is above the 32.5 USD/t that Seo et al. (2024) computed for Dammam to Ulsan.

## S8.3 Structural checks

The following were verified for every scenario: sales never exceed the scenario demand in any market; production never exceeds the installed capacity at any site; no voyage uses the Suez route in a disrupted scenario; and the stochastic programming identities WS ≥ RP ≥ EEV hold, with EVPI and VSS both non-negative. Deliveries equal 42.3 percent of capacity in the low-demand scenarios and 99.1 to 99.7 percent in all others, where production is at full capacity and the difference is boil-off.

# S9. Model code and data files

The model is written in Python 3.11 with Pyomo 6 and solved with the open-source HiGHS solver; sea distances are computed with the searoute package, version 1.6.0. The package contains the following files.

Table S10. Contents of the model and data package.

| File | Contents |
| --- | --- |
| data.py | All input parameters, sites, distances, scenarios and the shipping cost function, with a source comment on every value |
| distances.py | Sea distances between the export ports and the three markets with the searoute package (Suez and Cape routes) |
| model.py | Pyomo formulation of the two-stage stochastic MILP, including the CVaR option and the price floor |
| run.py | Experiments: stochastic solution, wait-and-see, expected-value problem, EVPI, VSS, CVaR frontier, sensitivity analysis and site ranking |
| verify.py | The verification checks reported in Section S8 |
| figures.py | Generation of Figures 1 to 6 of the main paper |
| sources/01_production_costs.md | Production-side data with full source citations and URLs |
| sources/02_shipping_logistics.md | Shipping, route and disruption data with full source citations and URLs |
| sources/03_demand_prices.md | Demand, price, auction and policy data with full source citations and URLs |
| results/ | All result tables in CSV format, including those reproduced in this document |
| figures/ | Figures in PNG at 300 dpi, TIFF at 600 dpi and PDF |

The complete experiment set, about 180 optimization runs, is reproduced by running run.py, verify.py and figures.py in that order; distances.py regenerates the sea distances.