Supporting Information

Table. S1 MS/MS parameters for Mycotoxins

| Analyte | Precursor ion | Fragment ion | DP | CE |
| --- | --- | --- | --- | --- |
| ZEN | 317.1 (-H) | 131.2 | -140 | -38 |
|  |  | 175.1 | -140 | -38 |
| AFB1 | 313.1 (+H) | 270.1 | -153 | 51 |
|  |  | 285.1 | -153 | 51 |
| AOH | 257.0 (-H) | 212.1 | -130 | -39 |
|  |  | 215.0 | -130 | -39 |
| AME | 271.1 (-H) | 256.0 | -118 | -32 |
|  |  | 228.0 | -118 | -32 |

Note: “DP” declustering potential; “CE” collision energies

Table. S2-1 LC-MS test conditions

| Parameter | Value/Setting |
| --- | --- |
| ESI | Positive/Negative |
| Spray voltage(Positive) | 3.5 KV |
| Spray voltage (Negative) | -2.5 KV |
| Sheath Gas Flow Rate | 50 arb.units |
| Auxiliary Gas Flow Rate | 10 arb. units |
| Capillary Temperature | 320℃ |
| Scan Resolution(Full MS) | 17500 FWHM |
| MS/MS Resolution | 15,000 FWHM |
| Collision Energy | 30 / 50 / 70 |
| Scan Range | 70~1000 m/z |

| Time/min | 0.1%Aqueous formic acid/% | Acetonitrile /% |
| --- | --- | --- |
| 0 | 95 | 5 |
| 5 | 10 | 90 |
| 8 | 10 | 90 |
| 8.01 | 95 | 5 |
| 10 | 95 | 5 |

Table. S2-2 Gradient elution

- Table S3 Degradation of ZEN, AOH, and AME by rCotA in Herbal Samples

| Sample | Toxin | Control (μg/L, mean ± SD) | Treated (μg/L, mean ± SD) | Degradation (%) (mean ± SD) |
| --- | --- | --- | --- | --- |
| Sample 1 | ZEN | 8.145 ± 0.088 | 2.190 ± 0.152 | 72.0 ± 1.9 |
|  | AOH | 0.665 ± 0.015 | 0.615 ± 0.038 | 7.5 ± 2.3 |
|  | AME | 0.501 ± 0.029 | 0.013 ± 0.017 | 97.0 ± 2.3 |
| Sample 2 | ZEN | 0.661 ± 0.175 | 0.355 ± 0.009 | 45.5 ± 5.1 |
|  | AOH | 3.038 ± 0.094 | 1.918 ± 0.096 | 36.9 ± 1.0 |
|  | AME | 4.012 ± 1.676 | 1.667 ± 0.171 | 58.3 ± 3.2 |
| Sample 3 | ZEN | —— | —— | —— |
|  | AOH | 1.301 ± 0.334 | 0.997 ± 0.070 | 23.1 ± 1.6 |
|  | AME | 0.926 ± 0.255 | 0.544 ± 0.000 | 40.6 ± 3.0 |

- Note: Data are presented as mean ± standard deviation (SD) (n = 3). Control represents samples without rCotA treatment, while treated represents samples with rCotA. Degradation (%) = [(Control − Treated) / Control] × 100%