| # Study Category | # CFD-Based Analysis | # AI/ML Prediction | # Heat Loss | # Head Loss | # Power Loss | # Multi-Target Prediction | Uncertainty Quantification | Explainable AI (SHAP) | Novel Physics Informed Feature RHLI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # CFD Erosion and Corrosion Studies [1–3] | # ✓ | # ✗ | # Partial | # Partial | # ✗ | # ✗ | # ✗ | # ✗ | # ✗ |
| # Rheology and Heavy-Oil Transport Studies [4–6] | # ✓ | # ✗ | # Partial | # ✓ | # Partial | # ✗ | # ✗ | # ✗ | # ✗ |
| # ML-Based Pressure Prediction Studies [8–9] | # ✗ | # ✓ | # ✗ | # ✓ | # ✗ | # ✗ | # ✗ | # ✗ | # ✗ |
| # Pipeline Failure and Corrosion Prediction Studies[10–11] | # ✗ | # ✓ | # ✗ | # ✗ | # ✗ | # ✗ | # ✗ | # ✗ | # ✗ |
| # AutoML and ML Pipeline Studies [12–13] | # ✗ | # ✓ | # ✗ | # ✗ | # ✗ | # ✗ | # ✗ | # ✗ | # ✗ |
| # XAI-Based Pipeline Energy Studies [14–16] | # Partial | # ✓ | # Partial | # Partial | # Partial | # ✗ | # ✗ | # ✓ | # ✗ |
| # COMSOL Pareto Optimization Study [17] | # ✓ | # ✗ | # ✓ | # ✓ | # ✓ | # ✗ | # ✗ | # ✗ | # ✗ |
| # Proposed Physics-Informed AI Framework | # ✓ | # ✓ | # ✓ | # ✓ | # ✓ | # ✓ | # ✓ | # ✓ | # ✓ |

Table 1. Existing and Proposed Study

| Property | Symbol | Value (units) |
| --- | --- | --- |
| Density | ρ | 7870 kg/m3 |
| Thermal Conductivity | k | 51.9 W/m.K |
| Specific heat capacity | Cp | 434 J/Kg.K |

Table 2. Material Properties of Pipeline

Table 3. Parameters used for Omani crude oil

| Property | Symbol | Value / Range | Unit |
| --- | --- | --- | --- |
| Density |  | 935 | kg/m3 |
| Plastic Viscosity | (PV) | 0.0018-0.0046 | Pa.s |
| Yield Stress | (Ys) | 10-12 | Pa |
| Model Parameter | (m) | 6000 | s |
| Thermal expansion coefficient |  | 0.0007 | 1K |

Table 4. Geometric flow and thermophysical parameters in simulation

| Thickness of the pipe (dz) | Temperature (T) | Viscosity μ | Thermal Conductivity (k) | Specific heat capacity ( Cp ) |
| --- | --- | --- | --- | --- |
| 0.01 | 298.15 | 1.2046 | 0.13 | 2100 |
| 0.01 | 313.15 | 1.1028 | 0.135 | 2200 |
| 0.01 | 328.15 | 1.0018 | 0.14 | 2300 |
| 0.012 | 298.15 | 1.2046 | 0.13 | 2100 |
| 0.012 | 313.15 | 1.1028 | 0.135 | 2200 |
| 0.012 | 328.15 | 1.0018 | 0.14 | 2300 |
| 0.015 | 298.15 | 1.2046 | 0.13 | 2100 |
| 0.015 | 313.15 | 1.1028 | 0.135 | 2200 |
| 0.015 | 328.15 | 1.0018 | 0.14 | 2300 |

*Used only in the Papanastasiou regularized form (not in the ideal Bingham limit).

Table 5. Mesh refinement levels and computational statistics

| Mesh Level | Domain Elements | Boundary Elements | DOF | Internal DOF | Solution Time (s) |
| --- | --- | --- | --- | --- | --- |
| Normal | 41,978 | 3,635 | 91,201 | 3,667 | 14 |
| Fine | 69,332 | 4,672 | 147,973 | 4,709 | 19 |
| Finer | 104,040 | 5,839 | 219,713 | 5,881 | 47 |

Table 6. Actual RHLI Thresholds from the dataset

| RHLI Regime | RHLI Range |
| --- | --- |
| Low RHLI | RHLI < 0.008669 |
| Medium RHLI | 0.008669 ≤ RHLI < 0.014707 |
| High RHLI | RHLI ≥ 0.014707 |

Table 7. RHLI Regime and Energy Losses

| RHLI Regime | Heat Loss | HL_Avg | Power Loss | RHLI |
| --- | --- | --- | --- | --- |
| Low RHLI | 609.65 | 24.60 | 1812.46 | 0.00326 |
| Medium RHLI | 546.37 | 76.40 | 5933.46 | 0.01200 |
| High RHLI | 684.32 | 0.45 | 54.42 | 0.01781 |

Table 8. Model Performance Metrics for Heat Loss Prediction

| Model | Test R² | Test RMSE | Test MAE | Test MAPE (%) |
| --- | --- | --- | --- | --- |
| Ridge | 1 | 0.1031 | 0.0904 | 0.0152 |
| Lasso | 1 | 0.045 | 0.0265 | 0.0045 |
| Random Forest | 1 | 0.0421 | 0.0055 | 0.0009 |
| Extra Trees | 1 | 0.0018 | 0.0004 | 0.0001 |
| Gradient Boosting | 1 | 0.0113 | 0.0044 | 0.0008 |
| XGBoost | 1 | 0.0497 | 0.0167 | 0.0029 |
| LightGBM | 1 | 0.0598 | 0.0221 | 0.0035 |
| Voting Ensemble | 1 | 0.0243 | 0.0101 | 0.0017 |
| Stacking Ensemble | 1 | 0.0078 | 0.0035 | 0.0006 |

Table 9. Model Performance Metrics for Power Loss Prediction

| Model | Test R² | Test RMSE | Test MAE | Test MAPE (%) |
| --- | --- | --- | --- | --- |
| Random Forest | 1.0000 | 0.0981 | 0.0358 | 0.0944 |
| Extra Trees | 1.0000 | 0.0117 | 0.0000 | 0.0078 |
| Gradient Boosting | 0.9999 | 0.147 | 0.1255 | 0.4851 |
| XGBoost | 0.9999 | 0.0497 | 0.1954 | 0.6344 |
| Stacking | 1.0000 | 0.0133 | 0.0046 | 0.0109 |

Table 10. Model Performance Metrics for Head Loss Prediction

| Model | Test R² | Test RMSE | Test MAE | Test MAPE (%) |
| --- | --- | --- | --- | --- |
| Random Forest | 1.0000 | 0.0981 | 0.0358 | 0.0944 |
| Extra Trees | 1.0000 | 0.0117 | 0.0040 | 0.0078 |
| Gradient Boosting | 0.9999 | 0.1470 | 0.1255 | 0.4851 |
| XGBoost | 0.9999 | 0.1954 | 0.1554 | 0.6344 |
| SVR | 0.9987 | 0.6854 | 0.3028 | 1.9482 |

Table 10. Model Performance Metrics for Head loss Prediction

| Model | Test R² | Test RMSE | Test MAE | Test MAPE (%) |
| --- | --- | --- | --- | --- |
| Ridge | 0.8983 | 86.7048 | 7.8681 | 2.0218 |
| Lasso | 0.8645 | 100.0812 | 8.9835 | 1.7024 |
| Random Forest | 0.9998 | 3.9850 | 0.4010 | 0.0612 |
| Extra Trees | 1.0000 | 0.0010 | 0.0001 | 0.0088 |

Table 11. Comparison of Ensemble-Learning Models for Heat, Head and Power-Loss Prediction

| Target Variable | Ensemble Model | Train R² | Test R² | Ranking |
| --- | --- | --- | --- | --- |
| Heat Loss | Voting Ensemble | 1.0000 | 1.0000 | 1 |
| Heat Loss | Stacking Ensemble | 1.0000 | 1.0000 | 1 |
| Head Loss | Voting Ensemble | 0.9867 | 0.9754 | 2 |
| Head Loss | Stacking Ensemble | 0.9997 | 0.9995 | 1 |
| Power Loss | Stacking Ensemble | 1.0000 | 1.0000 | 1 |

| Target | Best Model | Preferred Deployable Framework |
| --- | --- | --- |
| Heat Loss | Extra Trees | Stacking Ensemble |
| Power Loss | Extra Trees | Stacking Ensemble |
| Head Loss | Extra Trees | Stacking Ensemble |

Table 12. Best Predictive Models and Recommended Deployment Frameworks