Supplementary Section 2:

Transport Properties of ML-TMDCs

In this section, we again employ the effective massive Dirac-like Hamiltonian governing ML-TMDC semiconductors (Eq. A1) and use the extended transfer-matrix method to investigate the carrier transmission probability through the structure in the presence of the photogating potential. As illustrated in Fig. B1, the monolayer is assumed to extend in the x-y plane, with carrier transport considered along the in-plane x direction. As shown schematically, carriers are incident on the structure at an angle θ0 and emerge from the structure at an angle θe.

Region III (x>L)

Pristine (Vpg=0)

θ

qj

ky

kj

x=xe

x=0

e-

θe

θ0

x<0

z

x

y

L (length of ML-TMDC)

Region I (x<0)

Pristine (Vpg=0)

Region II (0≤x≤L)

Photogating region (Vpg≠0) (Vpg=0)

Figure B1. Schematic illustration of carrier transport through a photogated ML-TMDC, showing the incident and transmitted carrier wave vectors and the corresponding angles θ0​ and θe​.

Η=ℏvf τkxσx+kyσy+∆2σz+λsτ2I2-σz+eVpgI2           (B1)

The wave vector components along the x- and y-directions are defined as kx=-i∂x and ky=-i∂y, respectively. This Hamiltonian acts on a state described by the two-component pseudospin wavefunction Ψ=(ψA,ψB), where ψA and ψB correspond to the two sublattices of ML-TMDCs. Since carrier propagation is considered only along the x-direction, the wavefunction can be written in the form ψA,B(x,y)=ψA,B(x)eikyy. Substituting this wavefunction into the above Hamiltonian, the resulting problem for ML-TMDCs can be solved as follows:

∆2+eVpgℏvfτkx-ikyℏvfτkx+ikyλτs-Δ2+eVpgψA(x)ψB(x)eikyy=EψA(x)ψB(x)eikyy         (B2)

E denotes the energy of the incident charge carriers, while k+=[E+Δ2-eVpg-λτsτℏvf] and k-=E-Δ2-eVpgτℏvf are defined as the wave vectors within the monolayer, and k0=Eτℏvf corresponds to the wave vector of the incident carrier in the x<0 region. By expressing the above equation as two coupled equations and differentiating with respect to x, a second-order differential equation for the A and B sublattices can be obtained by defining kj2=k+k- as:

d2dx2ψAx+kj2-ky2ψAx=0d2dx2ψBx+kj2-ky2ψBx=0               (B2)

If qj​ denotes the x-component of the wave vector within the monolayer, it is obtained as

qj=sgnk+kj2-ky2            , kj2>ky2  iky2-kj2                         ,otherwise          (B3)

For the wavefunctions, we assume solutions of the form ψAx=Aeiqjx+Be-iqjx and ψBx=Ceiqjx+De-iqjx. Here, A and C denote the forward-propagating spinor components, whereas B and D represent the backward-propagating spinor components. Substituting these wavefunctions into Eq. (B2) yields the relations C=ik-iqj+1τkyA and D=-ik-iqj-1τkyB. To establish the relationship between the wavefunctions ψA,Bx and ψA,Bx+∆x, we assume the basis functions φx=φ1(x)φ2(x) to be of the following form:

φ1x=Aeiqjx+Be-iqjxφ2x=Aeiqjx-Be-iqjx               (B4)

We rewrite the wavefunctions in terms of the basis states. Using the definitions given by Eqs. P=k-k, sin⁡(θ)=kyk, and cos⁡(θ)=qk, the wavefunctions can be expressed as follows:

ψAx=φ1x                                                   ψBx=iτPsin(θ)φ1x+Pcos(θ)φ2x         (B5)

If the matrix R(E,Vpg) is defined as the transformation matrix in the form ψAxψBx=R(E,Vpg)φ1xφ2x, then we obtain:

RE,Vpg=10iτPsin(θ)Pcos(θ)                (B6)

Next, we consider the relationship between the wavefunction ψAxψBx and the wavefunction ψAx+∆xψBx+∆x within the ML-TMDC plane:

φ1x+∆x=Aeiq(x+∆x)+Be-iq(x+∆x)φ2x+∆x=Aeiq(x+∆x)-Be-iq(x+∆x)            (B7)

Substituting these relations into ψAxψBx=R(E,Vpg)φ1xφ2x, and subsequently expressing the resulting equation in matrix form as ψAx+∆xψBx+∆x=T(∆x,E,Vpg)φ1xφ2x, we obtain:

T∆x,E,Vpg=cosq∆xisinq∆xiP1τsin(θ)cosq∆x+cos(θ)sinq∆x-P[1τsin(θ)sinq∆x-cos(θ)cosq∆x]      (B8)

Using the two transformation relations for the matrices R(E,Vpg) and T(∆x,E,Vpg), the relationship between the wavefunctions ψAxψBx and ψAx+∆xψBx+∆x can then be written as:

ψAx+∆xψBx+∆x=Tj(∆x,E,Vpg)Rj-1(E,Vpg)ψAxψBx           (B9)

Finally, the relationship between ψA,Bx and ψA,Bx+∆x is formulated in terms of the extended transfer matrix M∆x,E,Vpg=T(∆x,E,Vpg)R-1(E,Vpg). This matrix shifts the wavefunction ψAxψBx by a distance ∆x and transforms it into ψAx+∆xψBx+∆x. Accordingly, the individual elements of the matrix M∆x,E,Vpg are obtained as follows:

M∆x,E,Vpg=cos(θ)cosq∆x+1τsin(θ)sinq∆xcos(θ)isinq∆xPcos(θ)iPjsinq∆xcos(θ)cos(θ)cosq∆x-1τsin(θ)sinq∆xcos(θ)       (B10)

One of the most important quantities that should be considered in the study of both the transport and optical properties is the wavefunction of the structure. The two-component wavefunction at the position x0​, denoted by ψAx0ψBx0, can be determined from the boundary conditions, as illustrated in Fig. B1. We assume that a free electron with energy E is incident on the structure from the x0<0 region at an angle θ0. In this region, the total wavefunction is given by the superposition of the incident and reflected waves. Therefore, at x0=0, we have:

ψA0=ψiE+ψrE=(1+r)ψiE       (B11)

As shown in Fig. B1, ψiE represents the wave packet of the incident electron at x0​=0. In deriving the above relations, the relation ψrE=rψiE has been used, where r is the overall reflection amplitude of the structure. Based on the proposed definition of the wavefunction, ψAx=Aeiqx+Be-iqx, the corresponding relation at the point of incidence is given by ψA0=A+B. By combining Eq. (B11) with the above description, the relations A=ψiE and B=ψrE=rψiE are obtained. Accordingly, the basis functions can be rewritten in the following form:

φ10=(1+r)ψiEφ20=(1-r)ψiE             (B12)

On the other hand, we previously obtained the relation ψBx=iτPsin(θ)φ1x+Pcos(θ)φ2x. At the point x0​=0, and using the above relations, we obtain

ψB0=P0[iτsinθ0+cosθ0]ψiE,ky+P0r[iτsinθ0-cosθ0]ψiE,ky       (B13)

Equation ψA(B)0 can be expressed in a compact matrix form as ψA0ψB0=1+rP0[cosθ0+iτsinθ0-r(cosθ0-iτsinθ0)]ψiE.

We now consider the other end of the structure, where the carrier exits the structure at position x=xe after undergoing various interactions, with an exit angle of θe. Assuming that t denotes the transmission amplitude of the structure, the corresponding wavefunction can be written as ψAxe=tψiE,ky. Based on the definition of the wavefunctions in terms of the basis functions at position x=xe, and assuming the relation φ1,2xe=tψiE, the wavefunction at the end of the structure, i.e., at the carrier-exit position, can be expressed in the following compact matrix form ψAxeψBxe=ttPecosθe+iτsinθeψiE.

Using the extended transfer matrix obtained for this structure, the matrix M∆x,E,Vpg relates the wavefunction ψAx0ψBx0 to ψAxeψBxe, such that ∆x=xe-x0. Therefore, we obtain the relation ψAxeψBxe=M∆x,E,VpgψA0ψB0. By combining the relations obtained for ψAx0ψBx0 and ψAxeψBxe, we arrive at:

ttPecosθe+iτsinθe=M11M12M21M221+rP0[cosθ0+iτsinθ0-r(cosθ0-iτsinθ0)]           (B14)

Solving the above equation and using detM=1, the reflection coefficient r and transmission coefficient t of the structure, which are key quantities for characterizing its transport properties, are obtained for the K (K') valleys as follows:

rE,ky=M22P0e±iθ0-M11Pee±iθe-M12P0Pee±i(θe+θ0)+M21M22P0e∓iθ0+M11Pee±iθe-M12P0Pee±i(θe-θ0)-M21       (B15)

tE,ky=2P0cosθ0M22P0e∓iθ0+M11Pee±iθe-M12P0Pee±i(θe-θ0)-M21       (B16)

Finally, the wave function at an arbitrary position x can be written as ψAxψBx=M(∆x,E,Vpg)ψAx0=0ψBx0=0. Accordingly, the wavefunctions corresponding to the different A and B sublattices are obtained as follows:

ψAx=ψiE[(1+r)M11+P0[cosθ0+iτsinθ0-r(cosθ0-iτsinθ0)]M12]ψBx=ψiE[(1+r)M21+P0[cosθ0+iτsinθ0-r(cosθ0-iτsinθ0)]M22]       (B17)

Here, Mmn denote the elements of the matrix M(∆x,E,Vpg). These equations are used to describe the spatial evolution of the two-component pseudospin wave function in ML-TMDCs for a given incident electron wave packet ψiE.