Comparative performance analysis of direct-coupled DC and variable-frequency AC photovoltaic water pumping systems under tropical climatic conditions

Felix Ngangoum Welaji,1,2 Ariel Teyou Ngoupo,1,*, Bertrand Bodo1, Yaouba2, Venant Sorel Chara-Dackou3,*, Charles Bertrand Gubong Takam2, Lyse Vanessa Ngansop2, Antoine Fanyep Nana2, Elie Bertrand Kegne Signe2

1University of Yaoundé I, Faculty of Science, Department of Physics, P.O. Box: 812, Yaoundé, Cameroon

2Research Centre for Renewable Energy, Institute of Geological and Mining Research, P.O. Box 4110, Yaoundé, Cameroon

3Carnot Energy Laboratory (CEL), Department of Physics, Faculty of Science, University of Bangui, P.O. Box 1450, Bangui, Central African Republic

# *Corresponding author: A. Teyou Ngoupo (arielteyou@yahoo.fr), V. S. Chara-Dackou (chav7@yahoo.com)

Supplementary materials

Table S1: Simulation program modules and their role in the calculation chain.

| Module | Role in the simulation |
| --- | --- |
| mod_constantes | Physical constants and system parameters (PV module, DC/AC pumps, tank, motor, costs, emission factors) |
| mod_controle | Anti-bounce hysteresis filter for realistic engine start counting |
| mod_pv | Electrical model of the PV array (one diode, 5 parameters), cell temperature, efficiency |
| mod_mppt | Adaptive stepping MPPT Perturb & Observe algorithm for the AC branch |
| mod_pompe | Centrifugal pump flow laws and efficiencies, DC and AC branches |
| mod_hydraulique | Total dynamic head (TDH), reservoir update, hydraulic power |
| mod_dynamique | Electromechanical dynamics of the DC motor, integrated by Runge-Kutta 4 |
| mod_dimensionnement_moteur | Determining the electromechanical parameters of the motor from its nameplate |
| mod_vieillissement | PV array time degradation, motor/inverter replacement schedule |
| mod_economie | CAPEX, OPEX and levelized cost of pumped water (LCOW) |
| mod_empreinte_carbone | Carbon footprint: manufacturing, replacements, emissions avoided |
| mod_fiabilite | Hydraulic reliability indicators (LWSP, resilience, vulnerability), reservoir optimization |
| mod_postprocessing | Statistical sorting and selection of representative days |

Table S2: Electrical parameters of the photovoltaic field ( Suniva OPT270-60-4-100 module) under STC  (1000 W/m2, 25 °C).

| Setting | Value | Unit | Comment |
| --- | --- | --- | --- |
| Isc_ref | 9.15 | A | STC |
| Voc_ref | 38.50 | V | STC |
| Vmp_ref | 31.20 | V | STC |
| Pmax_ref | 270.82 | Wc | STC ( Vmp×Imp ) |
| N_cells | 60 | - | cells in series / module |
| NOCT | 49.20 | °C | manufacturer's data |
| alpha_Isc | 0.000960 | /°C | coeff. temp. Isc |
| beta_Voc | -0.003650 | /°C | coeff. temp. Voc |
| Rs_mod | 0.200 | Ω | resistance new module series |
| Rp_mod | 1200.0 | Ω | new shunt resistor module |
| A_module | 1,576 | m2 | Surface |
| n1 | 1.3 | - | postman of ideality |
| A_total | 18,912 | m2 | = N_series×N_parallel×A_module |
| P_max_champ | 3250 | W | Reference power for normalization of the MPPT step size (=12×270.82) |

Figure S1: Comparative architectures of direct-coupled DC pumping and variable-frequency AC pumping with inverter/VFD and MPPT.

Table S3: Conversion parameters, yields and coefficients used in the simulation.

| Setting | Fortran symbol | Value | Unit | Source / justification |
| --- | --- | --- | --- | --- |
| Inverter/VFD efficiency (AC) | eta_onduleur | 0.96 | — | Typical value for a three-phase inverter: 1.5 kW |
| Inverter power threshold | P_seuil_onduleur | 50 | W | Minimum starting power |
| Rated power DC motor pump | P_nom_dc | 1,500 | W | Technical data sheet (1.5 kW, HMT = 40 m, Q = 8 m3/h) |
| Rated power AC motor pump | P_nom_ac | 1,500 | W | Equivalent to DC for controlled comparison |
| DC starting threshold | P_demarr_dc | 300 | W | 20% of P_nom_dc |
| AC starting threshold | P_demarr_ac | 350 | W | 23% of P_nom_ac |
| Rated efficiency of DC motor | eta_nom_motor | 0.85 | / | Sign plate |
| DC Armature Resistance | R_a | 1.44 | Ω | engine_sizing_mod |
| DC armature inductance | L_m | 7.2 × 10-3 | H | Idem |
| DC torque constant | K_m | 0.3247 | N·m/A | Idem |
| Moment of inertia | J_m | 2.58 × 10-3 | kg·m2 | Idem |
| Quadratic coefficient of friction | K_p | 4.11 × 10-5 | N·m·s2/rad2 | Idem |
| Static HMT | HMT_statique | 38.0 | m | / |
| Nominal pressure losses | HMT_pertes | 2.0 | m | Additional losses |
| Nominal flow rate of motor pumps | Q_nom | 8.0 | m3/h | Common Nominal Point |
| Consumption rate | Q_conso | 49 | m3/jour | Methodological Note S1 |
| Tank volume | V_reservoir | 50 | m3 | Battery life ≈ 1 day |
| Nominal efficiency of PV module | eta_STC | 0.172 | / | Suniva OPT270-60-4-100 |
| Diode ideality factor | n1 | 1.3 | / | Adjusted according to manufacturer's specifications |
| Number of modules in series | N_serie | 4 | / | MPPT/inverter voltage window |
| Number of parallel channels | N_paral | 3 | / | Target peak power (3.25 kWp) |

Methodological Note A: Justification of the consumption rate Qconso

The consumption rate is set at a single value of 49 m3/day for the four sites (Meskine, Agola, Mbé, Soa), in accordance with the single-factor experimental design adopted in our study. The calculation of this value follows the FAO crop balance method [1, 2] :

- Reference evapotranspiration: ET0 = 6.0 mm/day (maximum value corresponding to the Sahelian climate of Meskine),

- Cultural coefficient: Kc = 1.0 (mid-season market gardening),

- Irrigation efficiency: Eg = 0.85 (drip irrigation),

- Net requirement: BN = ET0×Kc = 6.0 mm/day,

- Gross requirement: BB =BNEg= 7.06 mm/day = 70.6 m3/ha/day,

- Reference surface: 0,70 ha,

- Daily demand: Qconso= 70.6 ×0.70  = 49 m3/day

Figure S2: Volume deficit after irradiance ramps. (a) Annual cumulative deficit per site. (b) Mean deficit per ramp. (c) Ratio of deficit to annual pumped volume.

Figure S3: Distribution of volume deficit per ramp for DC and AC architectures, across all sites. The dashed line indicates the mean. The multimodal distribution reflects the variability of detected ramps (duration, amplitude, pump state prior to the ramp).

# References

| [1] | J. Doorenbos and W. O. Pruitt, "Les besoins en eau des cultures.," in FAO Irrigation and Drainage Paper No. 24., Rome, 1977. |
| --- | --- |
| [2] | R. G. Allen, L. S. Pereira, D. Raes and M. Smith, "Crop evapotranspiration — Guidelines for computing crop water requirements," in FAO Irrigation and Drainage Paper No. 56, Rome, 1998. |