Effects of biochar application on soil nitrogen conversion and microbial community composition

Pingnan Zhao, Shen Wang, Dong Liu, Hongxu Li, Song Han*，Ming Li**

College of Forestry，Northeast Forestry University，Harbin 150040，China

*Corresponding author:

Song Han, E-mail: songh77@126.com

Ming Li，E-mail：liming1986@nefu.edu.cn

The composition of the straw used in the experiments is analyzed in Table S1 and the elemental analysis is shown in Table S2

Table S1 Analysis of straw components

| Sample | Ash/% | Components analysis/% |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cellulose | Hemi-cellulose | Lignin | Extractable |
| Corn straw | 7.44 | 22.82 | 43.01 | 15.59 | 18.58 |

Table S2 Element analysis of straw

| Sample | Ultimate analysis/% |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C | H | O | N | S | Cl | H/C | O/C |
| Corn straw | 43.83 | 5.75 | 49.00 | 0.97 | 0.13 | 0.32 | 1.57 | 0.84 |

Thermogravimetric analysis of samples under nitrogen protection was carried out using a thermogravimetric balance. Prior to the experiment, nitrogen was injected into the furnace to exhaust the air from the furnace. The crushed straw was weighed to approximately 10 mg and loaded into an alumina crucible. The flow of nitrogen was adjusted to 10 mL/min and the final reaction temperatures were set to 300 °C, 400 °C and 500 °C for two variables. The heating rates were 10 K/min, 20 K/min and 30 K/min. The holding time was controlled for 10 min to allow the straw to react adequately at the controlled temperature.

The results of the TG analysis are shown in Table 3 below. It can be seen that the extrapolated initial temperature did not change much between 258 and 266°C, although the reaction termination temperature and the rate of temperature increase were constantly changing. The temperature of the inflection point did not vary much between 263 and 282°C. The extrapolation termination point temperature varied little, between 277 and 300°C. As the reaction termination temperature increased, the rate of weight loss increased significantly, but the rate of weight loss was not significantly related to the rate of temperature increase.

Table S3 Thermogravimetric analysis

| Number | The terminal temperature reaction/℃ | Heating rate/(K/min) | Extrapolate the starting temperature/℃ | The inflection point temperature/℃ | Extrapolate the termination point temperature/℃ | Percentage weightlessness/% |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 500 | 30 | 261.5 | 282.6 | 301.0 | 71.23 |
| 2 | 400 | 30 | 259.2 | 266.9 | 280.4 | 55.90 |
| 3 | 300 | 30 | 258.4 | 263.6 | 278.9 | 52.59 |
| 4 | 500 | 20 | 258.8 | 267.2 | 282.8 | 67.70 |
| 5 | 400 | 20 | 260.7 | 268.8 | 281.8 | 54.70 |
| 6 | 300 | 20 | 257.3 | 269.1 | 283.4 | 48.17 |
| 7 | 500 | 10 | 263.3 | 275.0 | 294.7 | 82.44 |
| 8 | 400 | 10 | 262.5 | 269.4 | 276.9 | 70.60 |
| 9 | 300 | 10 | 266.6 | 278.3 | 281.9 | 51.05 |

The main weight loss range is 200°C to 400°C. The pyrolysis process of straw consists of three stages:

The first stage is from room temperature to around 200°C and is a dehydration and drying stage. The free water in the straw evaporates rapidly, which is equivalent to a drying process. The higher the water content of the straw, the longer the processing time. At the same time, some of the free water in the biological macromolecules is stripped away.

The second stage is around 200°C to 400°C, which is a rapid pyrolysis stage, especially in the 260°C to 300°C stage. In this stage, the weight loss curve decreases sharply and the main stages of pyrolytic volatilization of biomass are analyzed. At this stage, the different component contents lead to large differences in the pyrolysis process. The higher temperatures in this process meet the energy requirements for the pyrolysis of cellulose and hemicellulose, which rapidly decompose into small molecules of hydrocarbon gas, releasing large amounts of heat.

The third stage, from 400°C to the final temperature, is the final stage of residual pyrolytic carbonization, a process that mainly involves the pyrolysis of difficult-to-degrade lignin. The thermal decomposition of lignin needs to occur at a higher temperature, while the pyrolysis of lignin produces more coke, accompanied by the production of tar-like substances, wood vinegar liquids and other liquids, as well as ash and residues. 400°C later lignin continues to pyrolyze, but the pyrolysis process is slow, which is the reason for the structural stability of lignin.