Supplementary

Table 1. Factors and levels for response surface design of tea saponin extraction.

| Factor | Levels |  |  |
| --- | --- | --- | --- |
|  | -1 | 0 | 1 |
| A Trisodium citrate dosage (g/10 mL) | 0.8 | 1.1 | 1.4 |
| B Extraction temperature (℃) | 70 | 75 | 80 |
| C Extraction time (min) | 30 | 35 | 40 |

Table 2. Factors and levels for response surface design of purification using AB-8 resin.

| ##### Factor |  | Levels |  |
| --- | --- | --- | --- |
|  | -1 | 0 | ##### 1 |
| ##### A Loading pH | ##### 5 | ##### 6 | ##### 7 |
| ##### B Loading flow rate (BV/h) | ##### 1.0 | ##### 1.5 | ##### 2.0 |
| C Elution volume (BV) | ##### 3 | ##### 4 | ##### 5 |

Table 3. Analysis of variance for the extraction response surface model (after removing AB and BC).

| #### Source | #### Sum of Squares | #### Df | #### Mean square | #### F-value | #### P-value | #### Significance |
| --- | --- | --- | --- | --- | --- | --- |
| #### Model | #### 2.0030 | #### 7 | #### 0.2900 | #### 45.19 | #### < 0.0001 | #### *** |
| #### A | #### 0.4870 | #### 1 | #### 0.4870 | #### 75.93 | #### < 0.0001 | #### *** |
| #### B | #### 0.0134 | #### 1 | #### 0.0134 | #### 2.08 | #### 0.1830 | #### ns |
| #### C | #### 0.0008 | #### 1 | #### 0.0008 | #### 0.13 | #### 0.7290 | #### ns |
| #### AC | #### 0.0426 | #### 1 | #### 0.0426 | #### 6.65 | #### 0.0298 | #### * |
| #### A2 | #### 0.7820 | #### 1 | #### 0.7820 | #### 122.17 | #### ＜0.0001 | #### *** |
| #### B2 | #### 0.4590 | #### 1 | #### 0.4590 | #### 71.60 | #### ＜0.0001 | #### *** |
| #### C2 | #### 0.1110 | #### 1 | #### 0.1110 | #### 17.40 | #### 0.0024 | #### ** |
| #### Residual | #### 0.0577 | #### 9 | #### 0.0064 |  |  |  |
| #### Lack of fit | #### 0.0499 | #### 5 | #### 0.0100 | #### 5.10 | #### 0.0699 | #### ns |
| #### Pure error | #### 0.0078 | #### 4 | #### 0.0020 |  |  |  |
| #### Total | #### 2.0900 | #### 16 |  |  |  |  |

Note: * P<0.05,** P<0.01, *** P<0.001; ns = not significant. R²=0.9721, Adjusted R² = 0.951, Predicted R² = 0.846.

Table 4. Validation results of the optimized extraction process.

| Experiment No. | 1 | 2 | 3 | Mean | RSD/% |
| --- | --- | --- | --- | --- | --- |
| Tea saponin yield (%) | 22.834 | 22.903 | 22.949 | 22.895 | 0.253 |

Table 5. Static adsorption-desorption performance of AB-8 and D101 resins.

| Resin type | Average pore size (nm) | Polarity | Specific surface area | Adsorption rate (%) | Static desorption rate (%) |
| --- | --- | --- | --- | --- | --- |
| AB-8 | 13-14 | Weakly polar | 500 ± 20 | 85.3 ± 1.2 | 92.4 ± 1.5 |
| D101 | 20-22 | Non-polar | 650 ± 50 | 86.2 ± 1.1 | 87.5 ± 1.3 |

Note:Data are presented as mean ± SD (n = 3). The desorption rate of AB-8 was significantly higher than that of D101 (P < 0.05).

Table 6. Nonlinear regression parameters for the adsorption kinetics of tea saponin on AB-8 resin.

| Model | Qe,cal (mg/g) | k | R² | Adjusted R² | SSE | AIC | BIC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo-first-order | 43.47 | 0.5889 h⁻¹ | 0.9991 | 0.9987 | 1.45 | -9.66 | -9.50 |
| Pseudo-second-order | 54.22 | 0.01124 g/(mg·h) | 0.9927 | 0.9898 | 11.52 | 6.92 | 7.08 |

Note: Data were fitted by nonlinear regression (Origin 2021). Qe,cal, calculated equilibrium adsorption capacity; k₁, pseudo-first-order rate constant; k₂, pseudo-second-order rate constant; SSE, sum of squared errors; AIC, Akaike Information Criterion; BIC, Bayesian Information Criterion.

Table 7. Analysis of variance for the purification response surface model (after removing AB).

| #### Source | Sum of Squares | #### Df | #### Mean square | #### F-value | #### P-value | #### Significance |
| --- | --- | --- | --- | --- | --- | --- |
| #### Model | 11.8043 | #### 8 | 1.4689 | 221.14 | ＜0.001 | *** |
| #### A | 0.4823 | #### 1 | 0.4823 | 72.3 | ＜0.001 | *** |
| #### B | 0.1890 | #### 1 | 0.1890 | 28.5 | 0.0007 | *** |
| #### C | 0.1568 | #### 1 | 0.1568 | 14.2 | 0.0054 | ** |
| #### AC | 0.1605 | 1 | 0.1605 | 24.1 | 0.0012 | ** |
| #### BC | 0.1891 | #### 1 | 0.1891 | 28.5 | 0.0007 | *** |
| #### A2 | 5.7309 | #### 1 | 5.7309 | 863 | ＜0.001 | *** |
| #### B2 | 2.5914 | #### 1 | 2.5914 | 390 | ＜0.001 | *** |
| #### C2 | 1.3197 | #### 1 | 1.3197 | 198 | ＜0.001 | *** |
| #### Residual | 0.0531 | #### 8 | 0.0066 |  |  |  |
| #### Lack of fit | 0.0238 | #### 4 | 0.0060 | 0.813 | 0.5570 | ns |
| #### Pure error | 0.0293 | #### 4 | 0.0073 |  |  |  |
| #### Total | 11.8043 | #### 16 |  |  |  |  |

Note: * P<0.05, ** P<0.01, ***P<0.001; ns = not significant. R² = 0.9949, Adjusted R² = 0.9912 Predicted R² = 0.9720.

Table 8. Validation results of the optimized purification process.

| Experiment No. | 1 | 2 | 3 | Mean | RSD/% |
| --- | --- | --- | --- | --- | --- |
| Tea saponin purity (%) | 92.34 | 92.28 | 92.01 | 92.21 | 0.191 |

Table 9. Summary of antioxidant activity parameters of tea saponin and vitamin C.

| Antioxidant indicator | Sample | IC50/EC50 (mg/mL) | 95%CI (mg/mL) | R² |
| --- | --- | --- | --- | --- |
| DPPH radical scavenging | Tea saponin | 0.42 | 0.35~0.49 | 0.996 |
|  | Vitamin C | 0.09 | 0.07~0.11 | 0.998 |
| Hydroxyl radical scavenging | Tea saponin | 0.56 | 0.48~0.64 | 0.994 |
|  | Vitamin C | 0.21 | 0.18~0.24 | 0.997 |
| Total reducing power | Tea saponin | 0.22 | 0.18~0.25 | 0.994 |
|  | Vitamin C | 0.09 | 0.07~0.11 | 0.998 |

Fig. 1. Chemical structure of tea saponin.

Fig. 2. Static adsorption kinetics curve of AB-8 resin.

Fig. 5. Graphica bstract