Supplementary Information

Automated Underwater Delivery System for Coral Probiotics

Alexander Przybysz1*, Neus Garcias-Bonet2, Eslam O. Osman2, Francisca C. García2, Ahmed R. Aliyu3, José Ilton de Oliveira Filho4, Raquel Peixoto2*, Khaled Nabil Salama1*

Power Consumption

The optimal battery dimensions given the constraints of the enclosure was a 12V 12 Ah sealed lead acid (SLA) battery. To conserve energy, the device is placed in sleep mode where the dormant current of the device was found to be around 6 mA with 60 mA spikes lasting 3.5 seconds that occur every minute. These spikes are due to the device waking up interrogating the real-time clock and going back to sleep. The energy consumed during this minute cycle is 1.748 mWh. The energy consumption for the 10-second agitation and 30 mL pumping cycle was determined to be 23 mWh and 42.66 mWh, respectively. Given an hourly agitation frequency and 30 mL daily dispensation volumes, the battery has a projected life of 830.899 hours or 34 days. In practice, however, the results vary slightly (Figure 2d), which suggests that these variations due to manufacturing tolerances.

Agitation

Regarding agitation, there are two types of failure, namely: slip and stall. A slip condition is where the magnetic coupling is lost and the driver spins freely while the impeller is stationary. A stall condition is where the driver motor does not have an adequate amount of torque to drive the impeller and both the driver and the impeller remain stationary. It was determined that the impellers slip if connected directly to 12 volts. Therefore, the driver motors are controlled with a 1 kHz 12 V pulse width modulation (PWM) signal, which can be used to tune the speed of the motors to avoid slippage.

As the relationship between magnetic force and distance is not linear, an experiment was conducted to determine the strength of the coupling of the magnetic driver and the impeller as a function of distance. It was experimentally determined that for the agitation of distilled water, the driver pole-face to impeller pole-face distances where the stall condition occurs are beyond 15 mm (Figure S1a).

Due to the physical constraints of the device the minimum pole-face to pole-face distance that could be achieved is 7 mm, which should mitigate stalling risks. With this fixed distance, the relationship between dynamic viscosity and maximum impeller appears to be logarithmic (Figure S1b). At a maximal solution dynamic viscosity of 0.67 Pa.s, the impeller speed was found to be 55 RPM and the duty cycle of the PWM is 13.73%. Exceeding this viscosity threshold results in a stall condition. The maximum speed that can be achieved without slippage was determined to be 973.5 RPM, which occurs at a maximum PWM duty cycle of 26.67 % and a dynamic viscosity is 0.0010049 Pa.s, which is the viscosity of distilled water at 20 oC.

Figure S1. Agitation experiment results. (a) Voltage signal PWM duty cycle with respect to minimum pole-face to pole-face distance. (b) Dynamic viscosity of solution with respect to impeller speed.

Mobile Application

To allow seamless interfacing with the controller board, a companion mobile application was implemented. The app connects to the device if programming mode is enabled. Once connected, the user can perform diagnostic tests for possible hardware faults (Figure S2a). A variety of parameters can be tuned to give the user more flexibility for scheduling, dispensation volume control and agitation settings (Figure S2b). Additionally, the application facilitates fleet management via storing data logs and settings of individual devices (Figure S2c).

Figure S2. Mobile application interface. (a) Fleet management menu for storing information of multiple dispensers. (b)  Parameter tuning menu for setting the dispensation and agitation parameters. (c) Diagnostic menu for testing the device.

Figure S3. Hydraulic circuit of the dispenser.

Figure S4. Circuit diagram of the controller board.

Figure S5. Hydraulic damper. (a) hydraulic damper assembly with delivery tube and nozzle detached. (b) pressure comparison of undamped continuous pumping and damped intermittent pumping.

Figure S6. Program flow diagram.

| Table S1 - Mechanical Components and Their Function in the System |  |  |  |
| --- | --- | --- | --- |
| Name | Brand | Model | Function |
| Delivery Nozzle | Sprycle | SPL11-048 | To disperse probiotic and to prevent ingress of contaminants |
| Delivery tube A | ANPTGHT | ‎aww-123100980 | Transports probiotics from device to coral |
| Deliver tube adapter A | ‎AIEX | ‎ B13246 | Couples to Delivery tube A |
| Deliver tube socket | ‎AIEX | B13193 | Couples to tube luer to damper tee |
| Damper tee | VALUE PLASTICS, INC. | FTLT-6 | Couples to Delivery tube socket, Damper syringe and damper inlet adapter |
| Damper syringe | Becton Dickinson | 302995 | Takes up pressure spikes and improves dispensation volume accurace |
| Silicone stopper | Made in house | - | To contain the compressible volume of air in the damper syringe |
| Deliver tube adapter B | ‎AIEX | B13193 | Couples delivery Tube B to damper tee |
| Flush tee | Orbit | ‎67401 | Couples delivery, pump and flush line |
| Check valve | Leinuosen | 4346798961 | prevents backflow into flush pump |
| 1/4" tubing | PRUNkuar | YY0001 | Used to transport liquid throughout the entire device |
| M3 Brass insert | ‎AIEX | B16155 | Used for fastening purposes |
| M3x20 Nylon | Electronics-Salon | ‎ SCW-NLB/M3x20-100 | Used for fastening purposes |
| Damper Bayonet | Made in house | - | Bracket that holds the Damper syringes |
| Compression rod nut | ‎ Brookview Bolt | 12CNFHN | Fastens components to the compression rod |
| Compression brace top | Made in house | - | Squeezes down on enclosure flanges and holds syringe and damper brackets |
| Compression brace bottom | Made in house | - | Squeezes down on enclosure flanges and holds syringe and damper brackets |
| Compression rod | Enpro Industries | Custom request | Takes up tension for the compression braces and provides a support structure for syringe and damper brackets |
| Stand bolt | McMaster-Carr | 94175A670 | Stand legs for the device |
| Stand nut | Vital-Parts | HN934-M10-NY-N | Lock nut for Stand bolt |
| M12 washer | Made in house | - | Maintains gap for the foot of the 500 mL syringe cartridge |
| Syringe Top Clip | Made in house | - | Holds the foot 500 mL syringe cartridge |
| Syringe Bottom Clip | Made in house | - | Holds the foot 500 mL syringe cartridge |
| Nozzle bracket | Made in house | - | Holds the nozzle of the 500ml syringe cartridge and the magnetic driver assembly |
| M3 spacer | Electronics-Salon | ‎ SPC-RB7x4x3-100 | Maintains gap for the foot of the 500 mL syringe cartridge |
| Three way valve | Tiantiankeji | EMT | Luer lok valve for syringe cartridge |
| Nozzle Luer | ‎AIEX | B13193 | For making the syringe cartridge luer compatible |
| Syringe Cartridge | Jiansu Nuohong Medical Technology | 500ML | Reservoir for the probiotic solution. |
| Probiotic Intake adapter A | MEETOOT | MLLPHBA-316-10 | Adapts the intake tube to syringe valve |
| Probiotic Intake adapter B | ‎ ITROLLE | FLJSASTJLL-T10 | Adapts the intake tube to syringe valve |
| Impeller frame | Made in house | - | Holds the magnetic impeller |
| Impeller thick spacer | Made in house | - | Spaces impeller from the frame |
| Impeller thin spacer | Made in house | - | Spaces impeller from the frame |
| Impeller | ABCRapid | Custom manufacture from design | Agitates the probiotic solution |
| Disk magnet | First4Magnets | F1025-20 | Used in magnetic agitation |
| Impeller grub screw | Accu-components | SFF-M10-30-N | Support axle for impeller |
| Impeller bearing | Bushing MFG | 6000 10x26x8 | Corrosion proof bearing for impeller |
| O-ring | Uxcell | a18041600ux0071 | Used for gripping the impeller frame with the syringe |
| Driver tube | Guangzhou Engineering Plastic Factory | Custom length | Forms part of enclosure for magnetic driver |
| Driver end cap | ABCRapid | Custom manufacture from design | Forms part of enclosure for magnetic driver, also used for sensor enclosure |
| Driver top cap | ABCRapid | Custom manufacture from design | Forms part of enclosure for magnetic driver, also used for sensor enclosure |
| Driver disk | ABCRapid | Custom manufacture from design | Forms part of enclosure for magnetic driver, also used for sensor enclosure |
| Driver motor | Ningbo Leison Motor Co., Limited | RS-555-SH-4033 | Driver motor for magnetic agitation driver |
| Driver platform | Made in house | - | support platform for driver motor |
| Driver support rod | Uxcell | a16071500ux0035 | Support pillar for motor in driver enclosure |
| Driver cable | GjQ | GJQ-0012 (2P) | For delivering power to the driver motor |
| Bolt | McMaster-Carr | 91239A110 | Fastening purposes |
| Wetlink penetrator 6.5mm | Bluerobotics | WLP-M10-6.5MM-LC | Compression gland form 1/4" tubing |
| Vent hole and plug | Bluerobotics | VENT-M10-R1-RP | Allows access for battery charging cable. |
| Pressure relief valve | Bluerobotics | PRV-M10-ASM-R1-RP | Releases any buildup of pressure in enclosure |
| End flange | Bluerobotics | WTE6-M-FLANGE-SEAL-R1 | Seals the end cap of the enclosure tube |
| Acrylic end cap | Bluerobotics | WTE6-P-END-CAP-R1 | Blanks off the end where the controller board is situated |
| 15-hole end cap | Bluerobotics | WTE6-M-END-CAP-15-HOLE-R1 | Provides the interface for the tubes and cables for the enclosure |
| 6" Acrylic tube | Bluerobotics | WTE6-P-TUBE-R1 | Enclosure tube |
| Brass hex stand | Augiimor | XYDA0276 | Supports the motor frame to the end flange |
| Pump Frame Top | Made in house | - | Supports the diaphragm pumps |
| Pump Frame Top | Made in house | - | Supports the diaphragm pumps |
| Encoder magnet | First4Magnets | F413-20 | Encodermagnet for the pump motor |
| Diaphragm pump | ET-Pump | R27-02 | Diaphragm pump |
| Battery top clip | Made in house | - | Supports the battery |
| Battery bottom clip | Made in house | - | Supports the battery |
| Battery | Yuasa | NP12-12 | Sealed Lead Acid Battery |
| Controller board frame | Made in house | - | Supports the controller board |
| Nylon board hex stand | XunLiu | ‎ XL-NY-CA | Pillar for controller board support |
| Tether cable | Bluerobotics | CAB-NBPUF-4UTP-26AWG-R2 SKU | For connecting the sensor board to the main device through RS-485 |
| Electronic Components |  |  |  |
| PCB | PCBway | Custom design | Circuit board of controller |
| Clock battery holder | Q&J | CR2032-BS-6-1 | Holder for RTC time keeping battery |
| Clock battery | Energizer | CR-2032 | Powering clock battery |
| Sensor board | PCBway | Custom design | Circuit board of sensor board |
| Conrtroller | Atmel | ATMEGA-328 | Microcontroller unit for sensors |
| Temperature probe | Dallas Semiconductor | DS18B20 | Temperature sensor |
| RS485 transceiver | Analog Devices | MAX485 | For communicating with |
| Capacitors | LCSC | 100 nF, 680 uF, 1 uF |  |
| Resistors | LCSC | 10k, 56k, 91k, 100k, 120 |  |
| TVS diodes | LCSC | SMBJ12A, ESD5V5U5ULC, SM712, P4FL3.3A | Protects circuit from voltage transients |
| Voltage regulators | LCSC | AMS1117-5.0,AMS1117-3.3 | Provides 3.3v and 5V buses |
| LEDs | MOUSER | OVLGC0C6B9, C503B-BAS-CZ0A0452 | Indicator LEDs |
| Watchdog timer | Texas Instruments | TPL5010 | To reboot microcontroller if program crashes for some reason |
| Real time clock | Maxim Integrated | DS3231 | To keep track of real time |
| SD card holder | Molex | 47219-2001 | For connected the SD card |
| SD card | Kioxia | Exceria 64 GB | For logging data |
| Latch power transistor | International Rectifier | IRF9540N | For latching/unlatching power from the battery to the rest of the circuit |
| Motor driver transistor | International Rectifier | IRF540N | For driving pumps and motors |
| Gate driver transistor | On-Semi | 2n7002 | For driving the gates of the Motor drive transistors |
| Buffer opamp | Texas Instruments | LMV321 | For measuring battery voltage |
| ESP32 | Hiletgo | ESP-WROOM-32 Dev | Microcontroller board for controller |