Supplement

Drug quantification in single 3D organoids reveals correlation to reported in vivo brain uptake and demonstrates translational relevance for prediction of blood-brain barrier permeability

Cindy Bay1, Meike Kästing1, Jiayi Chu1, Eric Mühlberg2,3, Camilo Scherkl1, Philipp Uhl3,4, Julia Carolin Stingl1, Jürgen Burhenne1, Gzona Bajraktari-Sylejmani1, Max Sauter1,†, Johanna Weiss1 ,†, *

Supplement Table S1: Mass spectrometry and chromatography parameters for analyzed drugs

| Parameter mass spectrometry | Fluoxetine ([2H6]-Fluoxetine) |
| --- | --- |
| MRM-transition [m/z] Capillary voltage Cone voltage Dwell time Collision energy Collision gas flow (Ar) | 309.9 43.8 (315.8 43.8) 0.2 kV 2 V 79 ms 6 V 0.05 mL/min |
| Chromatography |  |
| Column | ACQUITY® UPLC® BEH C18 column (1.7 µm, 2.1 × 50 mm) |
| Column temperature | 40 °C |
| Aqueous eluent A | H2O/ACN (95/5, v/v) + 0.1 % FA |
| Organic eluent B | ACN + 0.1% FA |
| Flow rate | 0.5 mL/min |
| Injection modus | Full loop (20 µL) |
| Gradient | 0-0.5 min: 10% eluent B |
|  | 0.5 – 2.0 min: increase to 90% eluent B |
|  | 2.0 – 2.5 min: 90% eluent B 2.5 – 3.0 min: decrease to 10% eluent B 3.0 – 3.5 min: 10% eluent B |
| Parameter mass spectrometry | Octreotide ([13C6]-Octreotide) |
| MRM-transition [m/z] Capillary voltage Cone voltage Dwell time Collision energy Collision gas flow (Ar) | 510.2 119.9 (513.2 119.9) 0.7 kV 10 V 25 ms 30 V 0.15 mL/min |
| Chromatography |  |
| Column | ACQUITY® Premier Peptide BEH C18 (1.7 µm, 2.1 × 50 mm) |
| Column temperature | 60 °C |
| Aqueous eluent A | H2O/ACN (95/5, v/v) + 0.1% FA |
| Organic eluent B | ACN + 0.1% FA |
| Flow rate | 0.5 mL/min |
| Injection modus | Full loop (20 µL) |
| Gradient | 0 – 0.1 min: 5% eluent B |
|  | 0.1 – 2.5 min: increase to 20% eluent B |
|  | 2.5 – 2.7 min: increase to 95% eluent B 2.7 – 3.2 min: 95% eluent B 3.2 – 3.4 min: decrease to 5% eluent B 3.4 – 3.7 min: 5% eluent B |
| Parameter mass spectrometry | Olaparib ([2H4]-Olaparib) |
| MRM-transition [m/z] Capillary voltage Cone voltage Dwell time Collision energy Collision gas flow (Ar) | 435.4 281.1 (439.4 367.2) 3 kV 28 V 63 ms 24 V (14 V) 0.15 mL/min |
| Chromatography |  |  |
| Column | ACQUITY® UPLC® BEH C18 column (1.7 µm, 2.1 × 50 mm) |  |
| Column temperature | 40 °C |  |
| Aqueous eluent A | H2O/ACN (95/5, v/v) + 0.1% FA |  |
| Organic eluent B | ACN + 0.1% FA |  |
| Flow rate | 0.5 mL/min |  |
| Injection modus | Full loop (20 µL) |  |
| Gradient | 0-0.5 min: 5% eluent B |  |
|  | 0.5 – 2.0 min: Increase to 95% eluent B |  |
|  | 2.0 – 2.5 min: 95% eluent B 2.5 – 3.0 min: decrease to 5% eluent B 3.0 – 3.5 min: 5% eluent B |  |
| Parameter mass spectrometry | Paroxetine (Diazepam) |  |
| MRM-transition [m/z] Capillary voltage Cone voltage Dwell time Collision energy Collision gas flow (Ar) | 330.16 70.13 (285.16 154.12) 0.5 kV 12 V (24 V) 25 ms 26 V (26 V) 0.15 mL/min |  |
| Chromatography |  |  |
| Column | ACQUITY® UPLC® BEH C18 column (1.7 µm, 2.1 × 50 mm) |  |
| Column temperature | 40 °C |  |
| Aqueous eluent A | H2O/ACN (95/5. v/v) + 0.1% FA |  |
| Organic eluent B | ACN + 0.1% FA |  |
| Flow rate | 0.5 mL/min |  |
| Injection modus | Full loop (20 µL) |  |
| Gradient | 0-0.1 min: 10% Eluent B |  |
|  | 0.1 . 2.5 min: increase to 50% eluent B |  |
|  | 2.5 – 2.6 min: increase to 95% eluent B 2.6 – 2.8 min: 95% eluent B 2.8 – 3.0 min: decrease to 10% eluent B 3.0 – 3.1 min: 10% eluent B |  |
| Parameter mass spectrometry | Selinexor ([2H6]-Ulixertinib) |  |
| MRM-transition [m/z] Capillary voltage Cone voltage Dwell time Collision energy Collision gas flow (Ar) | 444.1 334.1 (439.1 220.1) 3 kV 36 V 108 ms 18 V 0.15 mL/min |  |
| Chromatography |  |  |
| Column | ACQUITY® UPLC® BEH C18 column (1.7 µm, 2.1 × 50 mm) |  |
| Column temperature | 40 °C |  |
| Aqueous eluent A | H2O/ACN (95/5. v/v) + 0.1% FA |  |
| Organic eluent B | ACN + 0.1% FA |  |
| Flow rate | 0.5 mL/min |  |
| Injection modus | Full loop (20 µL) |  |
| Gradient | 0-0.5 min: 30% eluent B |  |
|  | 0.5 – 3.0 min: increase to 70% eluent B |  |
|  | 3.0 – 3.5 min: 95% eluent B 3.5 – 4.0 min: 30% eluent A |  |
| Parameter mass spectrometry | Senicapoc (Lorlatinib) |  |
| MRM-transition [m/z] Capillary voltage Cone voltage Dwell time Collision energy Collision gas flow (Ar) | 324.2 122.03 (407.3 149.0) 3 kV 14 V (28 V) 21 ms 36 V (20 V) 0.15 mL/min |  |

| Chromatographie |  |
| --- | --- |
| Column | ACQUITY® UPLC® BEH C18 column (1.7 µm, 2.1 × 50 mm) |
| Column temperature | 40 °C |
| Aqueous eluent A | H2O/ACN (95/5, v/v) + 0.1% FA |
| Organic eluent B | ACN + 0.1% FA |
| Flow rate | 0.5 mL/min |
| Injection modus | Full loop (20 µL) |
| Gradient | 0-0.5 min: 5% eluent B |
|  | 0.5 – 1.5 min: increase to 95% eluent B |
|  | 1.5 – 2.0 min: 95% eluent B 2.0 – 2.5 min: 5% eluent B 2.5 – 3.0 min: 5% eluent B |
| Parameter mass spectrometry | Temsirolismus ([13C3, 2H7]-Temsirolismus) |
| MRM-transition [m/z] Capillary voltage Cone voltage Dwell time Collision energy Collision gas flow (Ar) | 1052.4 461.1 (1062.5 471.2) 3 kV 62 V 80 ms 56 V 0.15 mL/min |
| Chromatography |  |
| Column | ACQUITY® UPLC® BEH C18 column (1.7 µm, 2.1 × 50 mm) |
| Column temperature | 60 °C |
| Aqueous eluent A | H2O/ACN (95/5, v/v) + 0.1% FA |
| Organic eluent B | ACN + 0.1% FA |
| Flow rate | 0.5 mL/min |
| Injection modus | Full loop (20 µL) |
| Gradient | 0-0.5 min: 50% eluent B |
|  | 0.5 – 3.0 min: increase to 95% eluent B |
|  | 3.0 – 3.5 min: 95% eluent B 3.5 – 4.0 min: 50% eluent B |
| Parameter mass spectrometry | Ulixertinib ([2H6]-Ulixertinib) |
| MRM-transition [m/z] Capillary voltage Cone voltage Dwell time Collision energy Collision gas flow (Ar) | 433.1 262.1 (439.1 268.2) 0.5 kV 40 V 80 ms 16 V 0.15 mL/min |
| Chromatography |  |
| Column | ACQUITY® UPLC® BEH C18 column (1.7 µm, 2.1 × 50 mm) |
| Column temperature | 40 °C |
| Aqueous eluent A | H2O/ACN (95/5, v/v) + 0.1% FA |
| Organic eluent B | ACN + 0.1% FA |
| Flow rate | 0.5 mL/min |
| Injection modus | Full loop (20 µL) |
| Gradient | 0-0.5 min: 5% eluent B |
|  | 0.5 – 2.0 min: increase to 98% eluent B |
|  | 2.0 – 2.5 min: 98% eluent B 2.5 – 3.0 min: 5% eluent B |

| Parameter mass spectrometry | drug (IS) | Collision energy [V] |  |
| --- | --- | --- | --- |
| MRM-transition [m/z] Repotrectinib Selitrectinib Futibatinib Lorlatinib Larotrectinib Capillary voltage Cone voltage Dwell time Collision gas flow (Ar) | 356 161 (425 302 [2H6]-Futibatinib) 381 202 (425 302 [2H6]- Futibatinib) 419 296 (425 302 [2H6]- Futibatinib) 407 228 (411 228 [2H3,13C1]-Lorlatinib) 429 342 (433 342 [2H4]-Larotrectinib) 1 kV 20 V 29 ms 0.15 mL/min | 22 (22) 32 (22) 22 (22) 15 (15) 29 (29) |  |
| Chromatography |  |  |  |
| Column | ACQUITY® UPLC® BEH C18 column (1.7 µm, 2.1 × 50 mm) |  |  |
| Column temperature | 60 °C |  |  |
| Aqeaus eluent A | H2O/ACN (95/5, v/v) + 0.1% FA |  |  |
| Organic eluent B | ACN + 0.1% FA |  |  |
| Flow rate | 0.5 mL/min |  |  |
| Injection modus | Full loop (20 µL) |  |  |
| Gradient | 0 - 0.1 min: 5% eluent B - 2.3 min: 45% eluent B |  |  |
|  | 2.3 – 2.5 min: increase to 95% eluent B 2.5 - 2.8 min: 95% eluent B |  |  |
|  | 2.8 – 3.0 min: decrease to 5% eluent B 3.0 – 3.3 min: 5% eluent B |  |  |

All drugs were quantified in positive ion mode. The cone gas flow was 150 L/h, the source temperature 150 °C, the desolvation gas flow (N2) 1000 L/h and the desolvation temperature 600 °C.

Supplementary Figure S1: Representative CLSM images over culture time on A day 2 B day 4 and C on day 5 after assembly. See main manuscript for selected analysis time point day 3. Either pericytes or astrocytes were stained with CellTrackerTM in red as indicated in the respective panel. hCMEC/D3 were stained with the CD31-antibody (green). Cell nuclei are labeled with DAPI (blue). The middle panels of each day depict the z-projections. Right and left side show the z-slices. Different channel combinations are depicted as indicated in the images. All images are representative for at least three independent organoid preparations. Scale bar = 100 µm. Cell illustrations taken and adapted from Smart Servier.

Supplementary Figure S2: Representative CLSM images of astrocytes located outside the organoid cover in some organoid preparations. All nuclei were labeled with DAPI (blue). Scale bar = 100 µm. Cell illustrations taken and adapted from Smart Servier.

Supplementary Table S2: Validation results for ᴅ-Tyr(Me)Orn in organoid matrix

| Organoid matrix |  | LLOQ | low QC | mid QC | high QC |
| --- | --- | --- | --- | --- | --- |
|  |  | 0.050 ng/mL | 0.150 ng/mL | 112.5 ng/mL | 225 ng/mL |
| Intraday |  |  |  |  |  |
| 1 | mean [ng/mL] | 0.047 | 0.144 | 100.6 | 195.76 |
|  | accuracy [%] / precision [% CV] | 93.7/11.3 | 95.9/4.92 | 89.4/2.89 | 87.0/9.48 |
| 2 | mean [ng/mL] | 0.049 | 0.14 | 99.2 | 195.7 |
|  | accuracy [%] / precision [% CV] | 97.0/8.32 | 93.3/8.42 | 88.2/3.35 | 87.0/5.28 |
| 3 | mean [ng/mL] | 0.050 | 0.147 | 107.0 | 198.4 |
|  | accuracy [%] / precision [% CV] | 99.5/13.9 | 97.9/4.07 | 95.1/3.56 | 88.2/4.89 |
| Interday |  |  |  |  |  |
|  | mean [ng/mL] | 0.048 | 0.144 | 102.3 | 196.5 |
|  | accuracy [%] / precision [% CV] | 96.4/10.5 | 95.7/6.01 | 90.9/4.62 | 87.3/6.56 |

Precision [% CV]; accuracy [% nominal value] LLOQ: lower limit of quantification; QC: quality control. n = at least four replicates.

|  | mid QC 112.5 ng/mL | high QC 225 ng/mL |
| --- | --- | --- |
| Matrix effect [%] | 111.8 | 114.5 |
| Recovery [%] | 85.6 | 85.4 |

IS: internal standard, QC: quality control. n = three replicates per QC-Level.

Supplementary Figure S3: Representative light microscopy images of organoid lysis. Stepwise images of organoid lysis, starting with enzymatic digestion by collagenase and trypsin, followed by cell lysis with aqueous ammonia solution and addition of IS. Scale bar = 200 µm.

Supplementary Figure S4: Comparisons of direct and logarithm transformed correlations. A Correlation of the organoid uptake ratios and the reported mammalian in vivo brain-to-plasma ratios (Table 3), with the logarithm transformed brain-to-plasma ratios, logarithm transformed organoid uptake ratios and both combined. B Correlation between the BBScore of the in silico model ADMETboost and the reported in vivo brain-to-plasma ratios, logarithm transformed BBB Score, with the logarithm transformed brain-to-plasma ratios and both combined C Correlation of the logBB of the in silico model logBB_Pred and the reported in vivo brain-to-plasma ratios and the with the logarithm transformed brain-to-plasma ratios.

Supplementary Figure S5: Diagnostic plots for performed correlations with and without logarithmic transformation. A For correlation between organoid uptake ratio and reported mammalian in vivo brain-to-plasma ratios. B For BBBScore with the brain-to-plasma ratios and C for logBBB and brain-to-plasma ratios. From left to right: Residual vs. fitted plot; QQ-plot; Scale-Location plot; Crook’s distance or Residual vs Index for Deming Regression.

Supplementary Table 3: Representative quality control results for UPLC-MS/MS drug assays

| drug | Fluoxetine | Octreotide | Olaparib | Paroxetine | Selinexor | Senicapoc | Temsirolimus |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Intraday |  |  |  |  |  |  |  |
| precision [% CV] | 1.76 – 10.5 | 3.42 – 16.2 | 3.27 – 14.3 | 3.04 – 13.6 | 1.94 – 14.2 | 5.58 – 17.4 | 4.20 – 15.3 |
| accuracy [%] | 85.1 – 112.0 | 87.2 – 121.5 | 88.9 – 114.5 | 88.9 – 124.7 | 104.7 – 110.8 | 86.1 – 113.9 | 105.7 – 126.9 |
| Interday |  |  |  |  |  |  |  |
| precision [% CV] | 6.38 – 9.16 | 9.95 – 17.2 | 10.02 – 13.91 | 13.0 – 15.9 | 10.8 | 9.02 – 13.9 | 6.55 – 15.3 |
| accuracy [%] | 94.1 – 110.5 | 99.2 – 108.4 | 99.8 – 101.5 | 100.2 - 118 | 108.4 | 94.1 – 95.7 | 99.0 – 126.9 |
| Specificity | Blank in organoid matrix |  |  |  |  |  |  |
| Digestion stability | 93.0 – 105.5 | 102.1 – 110.8 | 94.4 – 98.3 | 93.4 – 93.6 | 106.2 – 115.4 | 102.6 – 104.2 | 90.5 – 107.6 |
| drug | Ulixertinib | Repotrectinib | Futibatinib | Lorlatinib | Larotrectinib | Selitrectinib |  |
| Intraday |  |  |  |  |  |  |  |
| precision [% CV] | 1.03 – 8.62 | 2.74 – 23.4 | 2.19 – 9.14 | 2.20 – 14.74 | 1.16 – 7.73 | 3.26 – 15.9 |  |
| accuracy [%] | 89.4 – 103.8 | 87.5 – 112.5 | 106.1 – 114.9 | 88.4 – 102.9 | 86.2 – 107.3 | 71.8 – 111.0 |  |
| Interday |  |  |  |  |  |  |  |
| precision [% CV] | 5.88 – 8.15 | 4.66 – 13.4 | 4.57 – 6.88 | 3.52 – 10.13 | 7.19 – 11.3 | 14.0 – 16.0 |  |
| accuracy [%] | 97.1 – 98.5 | 103.3 – 109.8 | 109.7 – 112.1 | 90.54 – 97.6 | 93.5 – 100.8 | 84.6 – 98.4 |  |
| Specificity | Blank in organoid matrix |  |  |  |  |  |  |
| Digestion stability | 101.1 – 104.8 | 104.4 – 124.1 | 97.2 – 105.2 | 95.1 – 101.0 | 125.7 – 144.8 | 97.3 – 115.0 |  |

Supplementary Table S4: Statistical analysis of drug uptake in organoids over time

| drug |  | drug |  |  |
| --- | --- | --- | --- | --- |
| Octreotide |  | Ulixertinib |  |  |
|  | ns | 3000 ng/mL 15 min vs. 1 h | **** <0.0001 |  |
| Olaparib |  | 15 min vs. 2 h | **** <0.0001 |  |
| 50000 ng/mL 1 h vs. 2 h | * 0.047 | 15 min vs. 4 h | **** <0.0001 |  |
| Paroxetine |  | 1 h vs. 2 h | * 0.0249 |  |
| 3000 ng/mL 15 min vs 4 h | * 0.017 | 1 h vs. 4 h | ** 0.002 |  |
| 1000 ng/mL 1 h vs 4 h | ** 0.008 | 1000 ng/mL 15 min vs. 2 h | **** <0.0001 |  |
| 2 h vs 4 h | * 0.017 | 15 min vs. 4 h | * 0.0356 |  |
| Senicapoc |  | 1 h vs. 2 h | * 0.0351 |  |
| 15 min vs. 1 h | **** <0.0001 | 1 h vs. 4 h | **** <0.0001 |  |
| Selinexor |  | 300 ng/mL 15 min vs. 2 h | *** 0.0002 |  |
| 300 ng/mL 2 h vs. 4 h | * 0.023 | 15 min vs. 4 h | **** <0.0001 |  |
| Temsirolimus |  | 2 h vs. 4 h | * 0.0468 |  |
| 2000 ng/mL 15 min vs. 1 h | ** 0.003 | Futibatinib |  |  |
| 15 min vs 4 h | * 0.02 |  | ns |  |
| 666,7 ng/mL 15 min vs. 2 h | * 0.032 | Larotrectinib |  |  |
| 1 h vs 4 h | * 0.024 | 180 ng/mL 2 h vs. 4 h | *** 0.0003 |  |
| 200 ng/mL 15 min vs. 4 h | ** 0.007 | 4 h vs. 6 h | * 0.038 |  |
| 1 h vs 4 h | ** 0.002 | Lorlatinib |  |  |
| Fluoxetine |  |  | ns |  |
| 150 ng/mL 15 min vs. 1 h | * 0.034 | Repotrectinib |  |  |
| 15 min vs. 4 h | **** <0.0001 |  | ns |  |
| 50 ng/mL 15 min vs 1 h | ** 0.0074 | Selitrectinib |  |  |
| 50 ng/mL 15 min vs 2 h | * 0.032 |  | ns |  |
| 50 ng/mL 15 min vs 4 h | * 0.014 |  |  |  |

Statistical analysis for multiple groups was performed with mixed-effect analysis with simple effect between columns (time points) and for single concentrations one-way ANOVA, both with Turkey’s multiple comparisons.

Supplementary Figure S6: Clearance volume of Transwell® experiments of ᴅ-Tyr(Me)Orn for hCMEC/D3. Untreated monolayer results are shown in black and red line depicts clearance volume after VEGF treatment (200 ng/mL VEGF in cell culture medium for 4 h before permeability measurement). Control group (untreated monolayer) were already published50. Please refer to this previous publication for methodical details of Transwell experiments.