Supplementary information for:

Probabilistic Human Health Risk Assessment of Potentially Hazardous Elements in Urban Park Soils. Implications for Risk-Based Environmental Management: A Case Study in the Municipality of Caldas, Colombia

Juan D. Correa1, Andrea Tamayo1, Juan P. Salazar2, Julián E. López1*

1 Faculty of Architecture and Engineering, Environmental Engineering Program, Institución Universitaria Colegio Mayor de Antioquia, Carrera 78 # 65 – 46, 050034, Medellín, Colombia

2 Center for Renewable Natural Resources, La Salada, Servicio Nacional de Aprendizaje - SENA, Km 5 Vía Caldas, 055448 La Pintada, Colombia

*Corresponding author

Julián E. López

julian.lopez@colmayor.edu.co

Table S1. PHEs concentrations.

| Site | As | Cd | Ni | Pb | Cr | Zn | Cu |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | 0.28 | 0.53 | 5.56 | 17.95 | 7.14 | 79.05 | 15.14 |
| P2 | 13.74 | 1.31 | 29.42 | 64.12 | 14.06 | 88.43 | 34.86 |
| P3 | 2.34 | 0.85 | 7.45 | 34.52 | 12.74 | 74.69 | 8.56 |
| P4 | 0.03 | 1.03 | 8.29 | 99.71 | 18.78 | 189.04 | 41.17 |
| P5 | 3.78 | 0.13 | 11.78 | 19.89 | 9.04 | 68.52 | 12.08 |
| P6 | 6.45 | 0.25 | 18.47 | 75.14 | 11.89 | 113.78 | 32.71 |

Table S2. Parameters.

| Parameter | Description | Unit | Distribution | Children | Adults | Source |
| --- | --- | --- | --- | --- | --- | --- |
| C | Measured element concentration | mg/kg | Fixed, site-specific | Measured per site (Table S1) | Measured per site (Table S1) | This study |
| EF | Exposure frequency | days/year | Triangular | 26; 120; 260 | 26; 120; 260 | Gonzalez-Valoys et al. (2025) |
| BW | Body weight | kg | Normal, truncated at BW > 0 | 15.6; 3.7 | 72; 15.9 | Gonzalez-Valoys et al. (2025); supplied Lopez et al. (2025) table; positive truncation: this study |
| IR | Incidental soil ingestion rate | mg/contact day | Triangular | 66; 103; 161 | 4; 30; 52 | Supplied Lopez et al. (2025), Table 1; matching primary Table 3, doi:10.17221/344/2023-PSE |
| ED | Exposure duration | years | Fixed | 6 | 20 | US EPA RSL User Guide, section 6 |
| SA | Exposed skin surface area | cm2/event | Fixed | 2373 | 6032 | US EPA RSL User Guide, section 6 |
| AF | Soil adherence factor | mg/cm2/event | Fixed | 0.2 | 0.07 | US EPA RSL User Guide, section 6 |
| EV | Soil-contact events per day | events/day | Fixed | 1 | 1 | US EPA (2004), RAGS Part E |
| ABSd | Dermal absorption fraction | dimensionless | Fixed, element-specific | As: 0.03; Cd/Ni/Zn: 0.001 | As: 0.03; Cd/Ni/Zn: 0.001 | US EPA (2004), Exhibit 3-4 (As/Cd); Salazar et al. (2024), Table S2, doi:10.1007/s11270-024-06893-0 (assumed Ni/Zn) |
| ATnc | Non-cancer averaging time | days | Derived from ED | 6 x 365 = 2190 | 20 x 365 = 7300 | US EPA (1989, 2004) |
| ATc | Cancer averaging time | days | Fixed | 70 x 365 = 25550 | 70 x 365 = 25550 | US EPA RSL User Guide, section 6 |
| CF | Mass conversion factor | kg/mg | Fixed | 1.00E-06 | 1.00E-06 | Unit conversion |

Table S3.

| Element | RfDoral | RfDabs | SForal | SFabs | ABSd | GIABS | Oral_RBA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| As | 6.00E-05 | 6.00E-05 | 32 | 32 | 0.03 | 1 | 1 |
| Cd | 1.00E-04 | 2.50E-06 | NA | NA | 0.001 | 0.025 | 1 |
| Ni | 0.02 | 8.00E-04 | NA | NA | 0.001 | 0.04 | 1 |
| Pb | NA | NA | NA | NA | NA | NA | NA |
| Cr | NA | NA | NA | NA | NA | NA | NA |
| Zn | 0.3 | 0.3 | NA | NA | 0.001 | 1 | 1 |
| Cu | 0.04 | NA | NA | NA | NA | NA | 1 |