Supplementary Information

Predictive Degradation Modelling of Solar Encapsulants: A Deep Learning Approach to IEC 61215 Thermal Cycling

Dada Samuel Adekunle

# S1. Mathematical Formulation of Dataset Generation

To address the requirement for absolute reproducibility, this section details the precise mathematical formulations and parameter bounds utilized to synthesize the high-fidelity degradation dataset (1,220 data points).

## S1.1 Tensile Strength Degradation Kinetics

The decay of tensile strength (SN) under thermo-mechanical fatigue was modelled using a continuous exponential decay function, representing the physical transition from localized steady-state microcracking to macroscopic embrittlement:

S(N) = S0 · exp(−λN) + ϵS	(1)

Where:

- S0 is the empirically derived initial baseline tensile strength (MPa) at 0 cycles.

- N is the cumulative number of thermal cycles.

- λ is the nanoclay-dependent decay constant, derived directly from the linear degradation rate (k) utilizing the relationship λ = k/S0.

- ϵS is a Gaussian noise term introduced to simulate experimental variance, sampled from a normal distribution N(0,σ2) where σ = 0.02 · S0 (2% standard deviation).

## S1.2 Microcrack Density Evolution

The evolution of surface microcrack density (ρc) was modelled using a sigmoidal growth function to accurately capture the delayed onset of physical crack nucleation followed by a rapid phase of inter-lamellar coalescence once a critical thermal fatigue threshold (Ncrit) is surpassed:

(2)

Where:

- ρmax is the theoretical maximum crack density (cracks/mm2) for a given formulation.

- r is the constant sigmoidal growth rate, set universally to 0.01 for all formulations.

- Ncrit is the critical inflection point (cycles) at which catastrophic crack coalescence accelerates.

- ϵρ is the Gaussian noise term for microcrack observation, sampled from N(0,0.52).

## S1.3 Exact Generation Parameters

The exact numerical parameters utilized to generate the synthetic ground-truth tensor for the four investigated EVA formulations (0, 1, 3, and 5 wt.% mica nanoclay) are detailed in Table S1.

Table 1: Exact parameters for synthetic dataset generation across EVA formulations.

| Formulation Loading |  | S0 (MPa) k | (MPa/cycle) λ (Decay Constant) Ncrit (Cycles) | ρmax (mm−2) |
| --- | --- | --- | --- | --- |
| EVA-00 | 0 wt.% | 14.20 | 0.0221 0.001556 400 | 25.0 |
| EVA-NC1 | 1 wt.% | 15.10 | 0.0163 0.001079 600 | 23.0 |
| EVA-NC3 | 3 wt.% | 17.40 | 0.0116 0.000667 900 | 19.0 |
| EVA-NC5 | 5 wt.% | 19.85 | 0.0087 0.000438 1200 | 15.0 |

# S2. Machine Learning Hyperparameter Grid

To ensure full replicability of the predictive modelling, the exact hyperparameters for both the Random Forest regressor and the Long Short-Term Memory (LSTM) neural network are provided below. Both models were trained on the identical 80/20 train-test split generated from the dataset.

Table 2: Random Forest Regressor Hyperparameters (scikit-learn)

Parameter	Value

Estimators (n estimators)	100

Maximum Depth (max depth)	None (nodes expanded until all leaves are pure)

Minimum Samples Split (min samples split)	2 Minimum Samples Leaf (min samples leaf)	1

42

42

Random State (random state)

Table 3: LSTM Neural Network Hyperparameters (TensorFlow/Keras)

| Parameter | Value |
| --- | --- |
| Input Shape | [Samples,1 (Time Step),2 (Features: N,wt.%)] |
| Hidden Layers | LSTM (64 units), Dropout (0.2), LSTM (32 units), Dense (16 units) |
| Activation Functions | ReLU (hidden layers), Linear (output layer) |
| Optimizer | Adam (learning rate = 0.001) |
| Loss Function | Mean Squared Error (MSE) |
| Batch Size | 16 |
| Epochs | 100 (with internal 20% validation split) |

# S3. Code and Dataset Availability

The complete Python script utilized for synthesizing the thermodynamic degradation dataset, alongside the raw CSV dataset and the Jupyter Notebook containing the training logs for the machine learning architectures, have been made publicly available to ensure total methodological transparency and computational reproducibility.

Note to Editor/Reviewers: The complete interactive code notebook and synthetic dataset can be accessed, reviewed, and executed via Google Colab at: https://colab.research.google. com/drive/1XLoaWJukfNO4MQClxLal5vrREQ5XUm8m#scrollTo=vhCi-t8o60Gi.