2. Materials and methods

2.1. Experiment

A total of 230 human serum samples were collected in this experiment and their GLB values were determined using routine clinical biochemical tests. This work was supported by Youth Innovation Talents Project of Colleges and Universities in Guangdong Province (No. Q18285), and all individual participants provided written informed consent. The study protocol was performed in accordance with relevant laws and institutional guidelines and was approved by local medical institutions and ethics committee. The obtained results were used as reference values in NIR spectroscopy analysis. The statistical analysis of the measured GLB values of 230 samples is given in Table 1.

The spectroscopy instrument was an XDS Rapid ContentTM Liquid Grating Spectrometer (FOSS, Denmark) equipped with a transmission accessory and a 2 mm cuvette. The spectral scanning range was 780-2498 nm with a 2 nm wavelength gap; the detector were Si (780 - 1100 nm) and Pbs (1100 - 2498 nm). The temperature and relative humidity of the laboratory were 25±1°C and 46±1%RH, respectively. Each sample was measured three times, and the mean value of the three measurements was used for modeling.

2.2. Modeling process

Leave-one-out cross validation (LOOCV) is commonly used as the object function for model selection, which aims to make full use of the samples information. In this study, LOOCV was conducted for modeling process, as described below. Only one sample was left out from modeling samples for the prediction, and the other samples were used as calibration set. This process was repeated until the prediction value of every modeling sample was obtained. The measured and predicted values of ith sample in modeling set were denoted as  was the number of modeling samples. For all samples, the mean measured value was denoted as  and the mean predicted value was denoted as . The prediction accuracy was evaluated by the root-mean-square errors of cross validation and the predicted correlation coefficients, and denoted as SECV and RP,CV, respectively. The calculation formulas were as the follows:

,                          (1)

.                 (2)

The model parameters were selected to achieve minimum SECV.

2.3. MW-PLS method

MW-PLS is a time-tested and popular method for screening continuous wavebands. This method uses several continuous wavelengths as a window, the size and position of which can be changed, and the PLS models are established for all possible windows in a predetermined search region of the spectrum. The information waveband was selected according to the minimum SECV. In this study, the search range of the MW-PLS was full spectrum region (780–2498 nm) with 860 wavelengths, and the initial wavelength (I) and number of wavelengths (N) of window as well as the number of PLS factors (F) were set as , , and . The LOOCV for PLS models was performed in each combination of (I, N, F), and the corresponding SECV and RP,CV were calculated. The optimal waveband with minimum SECV was selected to achieve the best prediction accuracy.

2.4. MC-UVE-PLS method

MC-UVE-PLS is a representative method for screening discrete wavelengths. For the method, lots of models are established with randomly selected calibration samples, then the coefficient stability of these models is calculated, and each variable is evaluated with the stability of the corresponding coefficient. 20 In this study, MC-UVE method was performed based on the full spectrum region, and Monte Carlo sampling operation 500 times. The number of variables was determined using the method in reference [20]. MC-UVE-PLS was rerun for 50 times and the best result was recorded for further analysis. The number of PLS factors F was set to be.

2.5. OPWC-PLS method

Based on BLR, the best partner of each wavelength was screened for entire scanning region and a partner wavelength subset (PWS) is determined. Then, a new PWS of all wavelengths in the PWS are also determined according to above obtained correspondence. The same procedure was performed repeatedly until the results converged to optimal partner wavelength combination (OPWC). The specific steps are as follows:

Step 1  Assume that there are N wavelengths in the wavelength screening area , namely, . For any fixed, and , LOOCV was performed based on binary linear regression of wavelength combination. The best partner of  was identified and denoted as  based on minimum. The formula is as follows,

.

The was partner wavelength subset (PWS(1)) of , and its number of wavelengths was denoted by N(1). Theoretically, the best partner  for each wavelength is unique, but several different wavelengths may have the same best partner. If some  was not a best partner of any wavelength, thenPWS(1), and N(1) <N.

Step 2  According to the projection defined above, the partner wavelength subset (PWS(2)) of PWS(1) could be obtained. It will be proved later that PWS converges to stable number of wavelengths after finite projection iterations. Suppose that PWS converges after s-times iterations, N(s) = N(s+1). And the PWS(s) was called optimal partner wavelength combination (OPWC). For OPWC, each wavelength was the best partner of some other wavelength.

The proof of convergence of algorithm

Proof: 1) If , , then the projection  is an one-to-one mapping function defined on ,, i.e. the PWS stop shrinking after this projection.

2) If ,, then is a proper subset of , which is set as, N(1)<N. Next further consider the projection of , i.e.: a) If , ,  then function  is an one-to-one mapping defined on the , , i.e. the PWS stop shrinking after this projection. b) If  then is a proper subset of, which is set as  , N(2)<N(1)<N.

Similarly considered the projection of, i.e.: a) If , , then the function  is an one-to-one mapping defined on the , , i.e. the PWS stop shrinking after this projection. b) If , then  is a proper subset of , which is set as  ,. Because the total number of wavelengths (N) is limited, the number of projections needed is limited.

In this study, the wavelength screening region for GLB spanned the entire scanning region (780–2498 nm), i.e. , with 860 wavelengths. The number of PLS factors F was set to .

The computer algorithms for the three methods discussed above were designed using MATLAB version 7.6.