Supplementary Information

Fast dispersive liquid-liquid microextraction based on temperature switching deep eutectic solvent as a green extractant

Qian Liua, Zhiqiang Lia, Liulin Weia, Xiaomei Chena, Yidong Xua, Jingchan Zhaoa*

a College of Chemistry and Material Science, Northwest University, Xi'an, 710127, China

Corresponding author Email: zhaojc@nwu.edu.cn

# Contents:

Part 1. Preparation and characterization of DES

Scheme S1-1. Synthesis process of DES

Scheme S1-2. FT-IR spectra of OA, LD and DES.

Scheme S1-3. 1H NMR spectra of OA, LD and DES

Part 2. Conditions of HPLC

Part 3. Chemical structural formula of targets

Part 4. Abbreviations

Part 5. Determination of target pesticide in real environmental water samples

Part 1.

1-1 Preparation of DES

The OA and LD were mixed in different molar ratios (1:1 ,2:1 and 3:1) and stirred in vacuo at 25 °C for about 30 minutes to obtain a colorless and homogeneous DES, and the synthesis is shown in Scheme S1-1. The chemical structure of the synthesized DES and two monomers (OA, LD) were confirmed by FT-IR and 1H NMR spectroscopy.

Scheme S1-1. Synthesis process of DES, which be obtained by simple blending of the two monomers in nitrogen atmosphere to homogeneous products

1-2 Characterization of DES

The formation of hydrogen bonds in DES was observed by FT-IR and the occurrence of side reactions was determined by 1H NMR.

1-2-1 FT-IR spectra of OA, LD and DES.

Scheme S2-1 shows the comparison between the FT-IR spectra of OA, LD and DES.

A characteristic broad peak at 937 cm-1 in the infrared spectrum corresponds to the carboxylic acid dimer of OA, and the peak at 3248 cm-1 and 1668 cm-1 corresponds to N-H stretching vibrations and -CO-NH- bond of LD. It is worthwhile mentioning that peak at 937 cm-1 of DES disappeared, confirmed that OA no longer exists in the form of the carboxylic acid dimer. Furthermore, the broad peak of the hydroxyl group of DES shifts to a higher frequency (3279 cm-1) due to hydrogen bonding interactions between the HBA and HBD. All results confirmed the successful synthesis of DES.

1-2-2 1H NMR spectra of OA, LD and DES.

Scheme S2-2 shows the 1H NMR spectra of DES synthesized at a 2:1 ratio of OA and LD, and the signals compared with each individual component and differences of their chemical shift observed. The hydrogen atom on the -CO-NH- group of LD is at 9.09 ppm, and the hydrogen at 7.08 ppm, 3.36 ppm, 2.79 ppm, 2.24 ppm, and 1.18 ppm belong to the benzene ring, -CH2- and -CH3 respectively. Among them, the hydrogen chemical shifts at 1, 3, and 4 are all increased by about 0.1 ppm compared to the LD spectrum. It can be considered that the formation of the -N-H-O- hydrogen bond causes the electron cloud of hydrogen on 1, 3, and 4 to shift, the electron shielding effect is weakened, and the hydrogen chemical shift increases. The hydrogen on the -COOH group in OA is not detected. The hydrogen on -HC=CH- is at 5.35 ppm, the hydrogen on -CH2- is at 2.34 ppm, 2.02 ppm, and 1.17-1.31 ppm, and the hydrogen at 0.88 ppm is -CH₃ of hydrogen. There were no new peaks in the 1H NMR spectrum of DES nor was there a decrease in the total number of peaks, and it is evident that the HBA and HBD synthesized DES through intermolecular bonding but chemical bond.

Scheme S1-2. Infrared spectroscopy of OA, LD and DES, and the broad peak of the hydroxyl group of DES shifts to a higher frequency (3279 cm-1) due to hydrogen bonding interactions between OA and LD, which confirmed the successful synthesis of DES.

Scheme S1-3. 1H NMR spectra of OA, LD and DES, and there were no new peaks in the 1H NMR spectrum of DES nor was there a decrease in the total number of peaks, and it is evident that the OA and LD synthesized DES through intermolecular bonding but chemical bond.

Part 2. Conditions of HPLC

Separation was accomplished with ZORBAX SB–C18 column (150 ×4.6 mm, 5 μm) at 30 °C with a mobile phase comprising of water (solvent A) and methanol (solvent B) at a flow rate of 1.0 mL min-1. The gradient elution procedure is that the initial methanol concentration is 60%, increased to 80% in 20 minutes, then to 90% in 20 minutes, and maintained for 5 minutes. The detection wavelength of the UV detector is set at 230 nm, which can ensure that the response signal of each substance will not be significantly different.

Part 3. Chemical structural formula of targets

Scheme S3-1. Chemical structural formula of targets

Part 4. Abbreviations

DLLME, dispersive liquid-liquid microextraction; DES, deep eutectic solvent; HPLC, high performance liquid chromatography; RSD, relative standard deviation; HBD, hydrogen bond donor; HBA, hydrogen bond acceptor; OA, oleic acid; LD, lidocaine; LOD, limit of detection; LOQ, limit of quantification; EF, enrichment factor; ER%, extraction recovery.

Part 5. Determination of target pesticide in real environmental water samples

Table 3

Determination of target pesticide in real environmental water samples

|  | Spiked (ug L-1) |  | Phoxim |  | Profenofos |  | Triazophos |  | Chlorpyrifos |  | Bifenthrin |  |  | Beta cypermethrin |  |  |  | Etofenprox |  |  | Fenpropathrin |  |  | Fenvalerate |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RR (%) | RSD | RR (%) | RSD | RR (%) | RSD | RR (%) | RSD | RR (%) | RSD | RR (%) |  | RSD |  | RR (%) |  | RSD | RR (%) |  | RSD | RR (%) |  | RSD |
| Ba River |  | 5 | 95 | 3.7 | 107 | 4.5 | 83 | 5.7 | 92 | 7.7 | 89 | 7.9 |  | 95 |  | 3.6 | 87 |  | 9.6 | 91 |  | 7.7 | 99 |  | 7.2 |
|  |  | 10 | 111 | 1.8 | 113 | 4.6 | 98 | 8.8 | 81 | 3.2 | 81 | 11.8 |  | 101 |  | 2.8 | 98 |  | 5.7 | 110 |  | 2.6 | 101 |  | 7.1 |
|  |  | 15 | 103 | 2.2 | 105 | 7.7 | 95 | 7.5 | 85 | 4.6 | 86 | 9.6 |  | 97 |  | 4.9 | 94 |  | 7.1 | 105 |  | 6.1 | 104 |  | 6.3 |
| Wei he River |  | 5 | 97 | 5.1 | 106 | 5.9 | 99 | 2.3 | 87 | 4.1 | 95 | 4.7 |  | 107 |  | 4.7 | 89 |  | 7.6 | 98 |  | 7.5 | 96 |  | 5.1 |
|  |  | 10 | 94 | 1.7 | 88 | 3.1 | 104 | 1.8 | 83 | 6.5 | 90 | 4.8 |  | 105 |  | 5.2 | 94 |  | 5.5 | 106 |  | 7.2 | 105 |  | 4.7 |
|  |  | 15 | 87 | 7.9 | 89 | 8.5 | 109 | 5.5 | 89 | 5.7 | 93 | 6.2 |  | 108 |  | 7.1 | 91 |  | 6.9 | 103 |  | 4.1 | 97 |  | 5.9 |
| JUE River |  | 5 | 90 | 5.5 | 93 | 7.4 | 95 | 7.8 | 97 | 7.9 | 87 | 5.1 |  | 95 |  | 6.5 | 95 |  | 8.1 | 89 |  | 3.7 | 91 |  | 4.6 |
|  |  | 10 | 95 | 6.3 | 95 | 2.6 | 89 | 4.6 | 101 | 5.4 | 89 | 7.3 |  | 89 |  | 4.3 | 91 |  | 8.9 | 87 |  | 8.5 | 89 |  | 5.5 |
|  |  | 15 | 93 | 4.8 | 101 | 3.1 | 91 | 4.1 | 94 | 8.1 | 91 | 7.5 |  | 91 |  | 4.6 | 95 |  | 5.7 | 91 |  | 5.4 | 93 |  | 6.1 |
| Hao River |  | 5 | 89 | 3.7 | 101 | 3.5 | 103 | 6.5 | 89 | 3.6 | 101 | 8.4 |  | 97 |  | 3.1 | 87 |  | 7.9 | 86 |  | 6.7 | 89 |  | 8.9 |
|  |  | 10 | 101 | 6.1 | 98 | 5.9 | 96 | 6.7 | 88 | 5.4 | 104 | 3.1 |  | 96 |  | 3.2 | 97 |  | 5.5 | 94 |  | 6.1 | 97 |  | 7.7 |