Appendix

A. Levelized energy production costs

A practicable framework is applied to estimate the levelized energy production costs for electricity and heat from the sources of coal, oil (heat only), natural gas, wind (onshore and offshore), photovoltaic (PV), and biomass (electricity with non-CHP and CHP, heat from direct combustion). Various raw material price levels are considered. The results are comprehended as representative for global averages at about year 2020. The accounting is entirely documented by equation set (1) of the main text and the first three columns of the following tables. This enables to easily adapt the estimation to varying conditions. Table A1 lists the complete computation for electricity production, Table A2 for heat production. In electricity production with biomass CHP, simultaneous utilization of the produced heat is assumed. Electricity production from coal, natural gas, wind, and biomass is considered to yield profit, optional for PV; heat production is considered for self-consumption (no profit). Brackets are for alternative parameter values; the bracket for coal (Table A1) indicates marginal costs in electricity production. The bottom of the tables shows the results for the CO2 price of 74 $/tCO2 required to balance electricity costs between marginal-cost coal and offshore wind and at the same time, balancing heat costs between natural gas and biomass.

A1. Electricity production

| - Electricity production |  |  | - Coal |  | - Natural gas | - Wind |  | - Photo- voltaic | - Biomass |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Item | - Unit | - Variable |  |  |  | - onshore | - offshore |  | - simple | - CHP |
| - Construction | - $/kW | - Cinst | - 1750 | - 3500 | - 1500 | - 1512 | - 5036 | - 1400 | - 3200 | - 5500 |
| - - self-energy | - % | - eself | - 3 | - 3 | - 2.5 | - 7 | - 7 | - 30 | - 7 | - 7 |
| - - effective | - $/kW | - Ceff | - 1803 | - 3605 | - 1538 | - 1618 | - 5389 | - 1820 | - 3424 | - 5885 |
| - Load ratio | - % | - λ | - 95 | - 95 | - 95 | - 24 | - 38 | - 14 | - 95 | - 95 |
| - Convers. effic. | - % | - ε | - 42 | - 42 | - 42 | - 100 | - 100 | - 100 | - 30 * | - 80 |
| - Full load hours | - h/yr | - Heff | - 3495 | - 3495 | - 3495 | - 2102 | - 3329 | - 1226 | - 2497 | - 6658 |
| - Interest rate | - %/yr | - ι | - 5 | - 5 | - 5 | - 5 | - 5 | - 5 | - 5 | - 5 |
| - - payment | - $/yr |  | - 90 | - 180 | - 77 | - 81 | - 269 | - 91 | - 171 | - 294 |
| - - prod. cost | - ¢/kWh | - int | - 2.6 | - 5.2 | - 2.2 | - 3.8 | - 8.1 | - 7.4 | - 6.9 | - 4.4 |
| - Lifetime | - yr | - τ | - 25 | - 25 | - 25 | - 25 | - 25 | - 25 | - 25 | - 25 |
| - Depreciation | - $/kW/yr |  | - 72 | - 144 | - 62 | - 65 | - 216 | - 73 | - 137 | - 235 |
| - - prod. cost | - ¢/kWh | - inv | - 0.02 | - 0.04 | - 0.02 | - 0.03 | - 0.06 | - 0.06 | - 0.05 | - 0.04 |
| - Decommissioning | - % | - δ | - 9 | - 9 | - 9 | - 9 | - 9 | - 9 | - 9 | - 9 |
| - - prod. cost | - ¢/kWh | - decom | - 0.002 | - 0.004 | - 0.002 | - 0.003 | - 0.006 | - 0.005 | - 0.005 | - 0.003 |
| - Capital cost | - ¢/kWh | - cap | - 2.6 | - 5.2 | - 2.2 | - 3.9 | - 8.2 | - 7.5 | - 6.9 | - 4.5 |
| - O&M fixed | - % | - ο | - 10 | - 10 | - 2.5 | - 2.2 | - 2.5 | - 1.5 | - 3 | - 3 |
| - - prod. cost | - ¢/kWh | - omfix | - 0.002 | - 0.004 | - ~ 0 | - ~ 0 | - 0.002 | - ~ 0 | - 0.002 | - 0.001 |
| - O&M variable | - ¢/kWh | - omvar | - 0.5 | - 0.5 | - 0.5 | - 0 | - 0 | - 0 | - 0.5 | - 0.5 |
| - O&M total | - ¢/kWh | - om | - 0.5 | - 0.5 | - 0.5 | - 0 | - 0.002 | - 0 | - 0.5 | - 0.5 |
| - Fuel - raw mat. price | - $/t | - Pmat | - 42 | - 81 | - 320 | - - | - - | - - | - 75 | - 150 |
| - - energy content | - kWhmat/kg | - Emat | - 5.7 | - 5.7 | - 13.1 | - - | - - | - - | - 4.2 | - 4.2 |
| - - raw mat. energy | - ¢/kWhmat | - Pmat/Emat | - 0.7 | - 1.4 | - 2.5 | - - | - - | - - | - 1.8 | - 3.6 |
| - - prod. cost | - ¢/kWh | - fuel | - 1.8 | - 3.4 | - 5.8 | - 0 | - 0 | - 0 | - 6.0 | - 4.5 |
| - Profit | - % | - π | - 8 | - 8 | - 8 | - 8 | - 8 | - 0 (8) | - 8 | - 8 |
| - - prod. cost | - ¢/kWh | - profit | - 0.4 | - 0.8 | - 0.7 | - 0.3 | - 0.7 | - 0 (0.6) | - 1.2 | - 0.8 |
| - Land | - Pland = 600 $/ha/yr (6 ¢/m2/yr), yield = 10.5 MgDM/ha/yr (4.4 kWhmat/m2/yr): 1.4 ¢/kWhmat |  |  |  |  |  |  |  |  |  |  |
| - - prod. cost | - ¢/kWh | - land | - 0 | - 0 | - 0 | - 0 | - 0 | - 0 | - 0 (4.9) | - 0 (1.8) |
| - Net | - ¢/kWh | - net | - - | - - | - - | - 2.2 | - 5.1 | - 1.5 | - - | - - |
| - Costs (subtotal), w/o emissions | - ¢/kWh | - cost' | - 5.3 (2#) | - 9.9 | - 9.2 | - 6.4 | - 14.0 | - 9.0 (9.6) | - 14.6 (19.5) | - 10.3 (12.1) |

| - Electricity production, continued |  |  | - Coal |  | - Natural gas | - Wind |  | - Photo- voltaic | - Biomass |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Item | - Unit | - Variable |  |  |  | - onshore | - offshore |  | - simple | - CHP |
| - CO2 emission raw material | - gCO2/ kWhmat | - CO2mat | - 325 | - 325 | - 211 | - 30 | - 30 | - 30 | - 367 | - 367 |
| - - w. self-energy | - “ |  | - 335 | - 335 | - 216 | - 32 | - 32 | - 39 | - 393 | - 393 |
| - - w. conv. eff. | - “ |  | - 797 | - 797 | - 515 | - 32 | - 32 | - 39 | - 1309 | - 491 |
| - CO2 self-emissions | - % | - cself (see equ. 1) | - 3 | - 3 | - 3 | - 0 | - 0 | - 0 | - -94 | - -94 |
| - CO2 production emissions | - gCO2/ kWh |  | - 821 | - 821 | - 530 | - 32 | - 32 | - 39 | - 79 | - 29 |
| - CO2 sequestration+, S = 0.6 MgC/ha/yr | - gCO2/ kWh |  | - 0 | - 0 | - 0 | - 0 | - 0 | - 0 | - -179 | - -67 |
| - CO2 price | - $/tCO2 | - PCO2 | - 50 (differentiative, distributive) |  |  |  |  |  |  |  |
| - Emission tax - combustion | - ¢/kWh | - comb | - 4.1 | - 4.1 | - 0 ° | - 0.2 | - 0.2 | - 0.2 | - 0.4 | - 0.1 |
| - - sequestration | - ¢/kWh | - sequ |  |  |  |  |  |  | - -0.9 | - -0.3 |
| - - total | - ¢/kWh | - emis | - 4.1 | - 4.1 | - 0 ° | - 0.2 | - 0.2 | - 0.2 | - -0.5 | - -0.2 |
| - Subsidy (CO2) | - ¢/kWh | - sub | - - | - - | - - | - 0 ° | - -4.1 | - -4.1 | - -4.1 | - -4.1 |
| - Tax (other) | - ¢/kWh | - tax | - 0 | - 0 | - 0 | - 0 | - 0 | - 0 | - 0 | - 0 |
| - Costs (total), w. emissions | - ¢/kWh | - cost | - 9.4 (6.1) | - 14.0 | - 9.2 | - 6.6 | - 10.1 | - 5.1 (5.7) | - 10.0 (14.9) | - 6.0 (7.8) |
| - Coal marginal costs, CO2 price balancing offshore wind costs to coal costs; differentiative and distributive as before |  |  |  |  |  |  |  |  |  |  |
| - CO2 price | - $/tCO2 | - PCO2 | - 74 |  |  |  |  |  |  |  |
| - Costs (total), w. emissions | - ¢/kWh | - cost | - (8.1) | - 16.0 | - 13.2 | - 6.7 | - 8.1 | - 3.2 | - 7.8 | - 3.9 |

Table A1. Electricity production levelized cost estimation by feedstock per produced energy. Convers. effic: energy conversion efficiency; * conversion efficiency in simple biomass electricity production typically lower; ° tax/subsidy exemption according to the discussed regulation (CO2 pricing) model; # marginal costs; + yield 10.5 MgDM/ha/yr. Variable definition details see column 3 of Table A2.

A2. Heat production

| - Heat production from direct combustion |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Item | - Unit | - Variable definiton | - Coal | - Oil | - Natural gas | - Biomass |
| - Construction | - $/kW | - Cinst | - 220 | - 330 | - 120 | - 300 |
| - - self-energy | - % | - eself | - 3 | - 3 | - 2.5 | - 7 |
| - - effective | - $/kW | - Ceff = Cinst ∙ (1 + eself) | - 227 | - 340 | - 123 | - 321 |
| - Load ratio | - % | - λ | - 10 | - 10 | - 10 | - 10 |
| - Conversion efficiency | - % | - ε | - 70 | - 70 | - 70 | - 70 |
| - Full load hours | - h/yr | - Heff = 24 ∙ 365 ∙ ε ∙ λ | - 613 | - 613 | - 613 | - 613 |
| - Interest rate | - %/yr | - ι | - 7 | - 7 | - 7 | - 7 |
| - - payment | - $/yr |  | - 16 | - 24 | - 9 | - 22 |
| - - prod. cost | - ¢/kWh | - int = Ceff ∙ ι / Heff | - 2.6 | - 3.9 | - 1.4 | - 3.7 |
| - Lifetime | - yr | - τ | - 25 | - 25 | - 25 | - 25 |
| - Depreciation | - $/kW/yr | - Ceff / τ | - 9 | - 14 | - 5 | - 13 |
| - - prod. cost | - ¢/kWh | - inv = Ceff / τ / Heff | - 0.01 | - 0.02 | - 0.01 | - 0.02 |
| - Decommissioning | - % | - δ | - 9 | - 9 | - 9 | - 9 |
| - - prod. cost | - ¢/kWh | - decom = (1 + δ) ∙ inv | - 0.02 | - 0.02 | - 0.01 | - 0.02 |
| - Capital cost | - ¢/kWh | - cap = inv + int + decom | - 2.6 | - 3.9 | - 1.4 | - 3.7 |
| - O&M fixed | - % | - ο | - 1 | - 1 | - 1 | - 1 |
| - - prod. cost | - ¢/kWh | - omfix = inv ∙ ο | - ~ 0 | - ~ 0 | - ~ 0 | - ~ 0 |
| - O&M variable | - ¢/kWh | - omvar | - 0.1 | - 0.1 | - 0.1 | - 0.5 |
| - O&M total | - ¢/kWh | - om = omfix + omvar | - 0.1 | - 0.1 | - 0.1 | - 0.5 |
| - Fuel - raw mat. price | - $/t | - Pmat | - 170 | - 556 | - 320 (640) | - 280 |
| - - energy content | - kWhmat/kg | - Emat | - 5.7 | - 12.2 | - 13.1 | - 4.2 |
| - - raw mat. energy | - ¢/kWhmat | - Pmat / Emat | - 3.0 | - 4.5 | - 2.5 | - 6.7 |
| - - prod. cost | - ¢/kWh | - fuel = Pmat / Emat / ε | - 4.3 | - 6.5 | - 3.5 (7.0) | - 9.6 |
| - Profit | - % | - π | - 0 | - 0 | - 0 | - 0 |
| - - prod. cost | - ¢/kWh | - profit = (cap + om + + fuel) ∙ π/(1 – π) | - 0 | - 0 | - 0 | - 0 |
| - Land | - ¢/kWh | - land | - 0 | - 0 | - 0 | - 0 |
| - Net | - ¢/kWh | - net | - 0 | - 0 | - 0 | - 0 |
| - Cost (total), w/o emissions | - ¢/kWh | - cost’ = cap + om + fuel + profit + land + net | - 7.0 | - 10.5 | - 5.0 (8.5) | - 13.8 |
| - CO2 emission raw material | - gCO2/ kWhmat | - CO2mat | - 325 | - 257 | - 211 | - 367 |
| - - w. self-energy | - “ | - CO2mat ∙ (1 + eself) | - 335 | - 265 | - 216 | - 393 |
| - - w. conv. eff. | - “ | - CO2mat ∙ (1 + eself) / ε | - 478 | - 378 | - 309 | - 561 |
| - CO2 self-emissions | - % | - cself, (see equ. 1) | - 3 | - 10 | - 3 | - -94 |
| - CO2 production emissions | - gCO2/ kWh | - CO2mat ∙ (1 + eself) / ε ∙ ∙ (1 + cself) | - 493 | - 416 | - 318 | - 34 |
| - CO2 sequestration, S = 0.6 MgC/ha/yr | - gCO2/ kWh | - (yield = 10.5 MgDM/ha/yr) | - 0 | - 0 | - 0 | - -77 |
| - CO2 price | - $/tCO2 | - PCO2 | - 50 (differentiative, distributive) |  |  |  |
| - Emission tax - combustion | - ¢/kWh | - comb = PCO2 ∙ CO2mat ∙ ∙ (1 + eself) / ε ∙ (1 + cself) | - 2.5 | - 2.1 | - 1.6 | - 0.2 |
| - - sequestration, S = 0.1 MgC/ha/yr) | - ¢/kWh | - sequ = PCO2 ∙ S / (yield ∙ ∙ Emat ∙ (1 + eself)) ∙ ε | - 0 | - 0 | - 0 | - -0.4 |
| - - total | - ¢/kWh | - emis = sequ + comb | - 2.5 | - 2.1 | - 1.6 | - -0.2 |
| - Subsidy (CO2) | - ¢/kWh | - sub | - - | - - | - - | - -4.1 |
| - Tax (other) | - ¢/kWh | - tax | - 0 | - 0 | - 0 | - 0 |
| - Cost (total), w. emissions | - ¢/kWh | - cost = cost’ + emis + + sub + tax | - 9.5 | - 12.6 | - 6.6 (10.1) | - 9.5 |
| - CO2 price balancing biomass costs to natural gas costs; differentiative and distributive as before |  |  |  |  |  |  |
| - CO2 price | - $/tCO2 | - PCO2 | - 74 |  |  |  |
| - Cost (total), w. emissions | - ¢/kWh | - cost | - 10.6 | - 13.6 | - 7.4 | - 7.4 |

Table A2. Levelized heat production cost estimation by feedstock, analogous to Table A1.