Strategic low-cost energy investment opportunities and challenges towards achieving universal access (SDG7) in each African nation

Modelling inputs assumptions

# Power generation technologies.

Supplementary Table 1. Type of power generation technologies.

|  | Fossil and Nuclear | Renewables |
| --- | --- | --- |
| Centralized | Diesel | Geothermal |
|  | Heavy Fuel Oil | Biomass and Waste Combined Heat and Power (CHP) |
|  | Natural Gas: Open Cycle Gas Turbine (OCGT), Combined Cycle Gas Turbine (CCGT) | Hydro: small, medium, large |
|  | Coal | Wind: onshore |
|  | Nuclear | Solar PV (utility-scale) |
|  |  | Solar PV (rooftop) |
|  |  | Solar PV with storage |
|  |  | Concentrated Solar Power (CSP) with storage |
|  |  | CSP without storage |
| Decentralized | Diesel Genset Mini Grid (un-electrified settlements) | Small Hydro Micro Grid (un-electrified settlements) |
|  | Diesel Genset Stand Alone (un-electrified settlements) | Solar PV Micro Grid (un-electrified settlements) |
|  |  | Solar PV 1–5 Stand Alone (0.02 kW, 0.05 kW, 0.1 kW, 0.2 kW, > 0.2 kW; un-electrified settlements) |
|  |  | Wind Micro Grid (un-electrified settlements) |

# Fuel prices.

The fuel prices the years 2015, 2016 and 2017 are based on the IEA, World Energy Outlook (2016), (2017) and (2018) [1]–[3]. The price evolution (2018-2040) is based on the IEA, World Energy Outlook (2020) [4] New Policies scenario. The ratio between the different type of fuels is been calculated based on the IRENA, "Planning and prospects for renewable power: West Africa", 2018 report [5].

Supplementary Table 2. Evolution of fuel prices in each scenario [1]–[5].

|  |  | New Policies |  |
| --- | --- | --- | --- |
| USD/GJ | 2015 | 2030 | 2040 |
| Crude Oil (imports) | 8.33 | 9.66 | 10.81 |
| Biomass (domestic) | 1.6 | 1.6 | 1.6 |
| Biomass (imported) | 1.61 | 1.61 | 1.61 |
| Coal (domestic) | 3.30 | 2.92 | 2.78 |
| Coal (imported) | 4.90 | 4.46 | 4.24 |
| Diesel (domestic) | 10.64 | 10.75 | 12.02 |
| Diesel (imported) | 11.20 | 11.31 | 12.65 |
| Heavy fuel oil (domestic) | 6.27 | 6.32 | 7.07 |
| Heavy fuel oil (imported) | 6.60 | 6.66 | 7.44 |
| Natural gas (domestic) | 7.10 | 7.91 | 8.39 |
| Natural gas (imported) | 8.60 | 9.59 | 10.17 |
| LNG (imports) | 9.2 | 10.24 | 10.86 |
| Uranium (domestic) | 0.20 | 0.14 | 0.14 |
| Uranium (imports) | 0.21 | 0.15 | 0.15 |

The fuel prices in each country are adjusted using the following distance factors for the inland countries. The distance factor is calculated using Geographic Information System (GIS). It expresses the distance between the closest port between the coastal country and the inland one.

Supplementary Table 3. Distance factor used to calculate the fuel prices in inland countries.

| Country | Distance Factor |
| --- | --- |
| Botswana | 1.6 |
| Burkina Faso | 1.7 |
| Burundi | 1.1 |
| Central African Republic | 1.1 |
| Chad | 1.1 |
| Ethiopia | 1.5 |
| Lesotho | 1.3 |
| Malawi | 1.5 |
| Mali | 1.6 |
| Niger | 1.7 |
| Rwanda | 1.1 |
| South Sudan | 1.4 |
| Swaziland | 1.1 |
| Uganda | 1.1 |
| Zambia | 1.8 |
| Zimbabwe | 1.4 |

Supplementary Table 4. Techno-economic parameters for technologies for each scenario [4].

|  | New Policies |  |  |  |  |  | Sustainable Development |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technology | Overnight cost (USD/kW) |  |  | Operational and Maintenance cost (USD/kW) |  |  | Overnight cost (USD/kW) |  |  | Operational and Maintenance cost (USD/kW) |  |  |
|  | 2015 | 2030 | 2040 | 2015 | 2030 | 2040 | 2015 | 2030 | 2040 | 2015 | 2030 | 2040 |
| Diesel centralized | 1200 | 1200 | 1200 | 35 | 35 | 35 | 1200 | 1200 | 1200 | 35 | 35 | 35 |
| Heavy fuel oil | 1467 | 1467 | 1467 | 44 | 44 | 44 | 1467 | 1467 | 1467 | 44 | 44 | 44 |
| OCGT | 400 | 400 | 400 | 20 | 20 | 20 | 400 | 400 | 400 | 20 | 20 | 20 |
| CCGT | 700 | 700 | 700 | 25 | 25 | 25 | 700 | 700 | 700 | 25 | 25 | 25 |
| Supercritical coal | 1600 | 1600 | 1600 | 65 | 60 | 60 | 1600 | 1600 | 1600 | 65 | 60 | 60 |
| Hydro (large scale) | 2100 | 2100 | 2100 | 50 | 50 | 50 | 2100 | 2100 | 2100 | 50 | 50 | 50 |
| Hydro (small scale) | 3300 | 3300 | 3300 | 60 | 60 | 60 | 3300 | 3300 | 3300 | 60 | 60 | 60 |
| Hydro (medium scale) | 2100 | 2100 | 2100 | 50 | 50 | 50 | 2100 | 2100 | 2100 | 50 | 50 | 50 |
| Biomass | 2150 | 2050 | 2050 | 75 | 70 | 70 | 2150 | 2050 | 2050 | 75 | 70 | 70 |
| Nuclear | 4000 | 4000 | 4000 | 170 | 170 | 170 | 4000 | 4000 | 4000 | 170 | 170 | 170 |
| Geothermal | 3050 | 2850 | 2700 | 60 | 55 | 55 | 3050 | 2850 | 2700 | 60 | 55 | 55 |
| Wind onshore | 1950 | 1860 | 1820 | 48 | 48 | 46 | 1950 | 1820 | 1760 | 48 | 46 | 46 |
| Wind offshore | 4440 | 2780 | 2300 | 115 | 80 | 70 | 4440 | 2600 | 2080 | 115 | 75 | 65 |
| Diesel stand-alone (1 kW) (decentralised) | 261 | 261 | 261 | 23 | 23 | 23 | 261 | 261 | 261 | 23 | 23 | 23 |
| Solar PV (centralised-Large scale) | 1600 | 990 | 840 | 24 | 22 | 22 | 1600 | 890 | 720 | 24 | 22 | 20 |
| Solar PV (decentralised-rooftop) | 2200 | 1,360 | 1140 | 32 | 30 | 28 | 2200 | 1220 | 980 | 32 | 30 | 28 |
| Solar PV with battery | 3889 | 2,130 | 1881 | 48 | 46 | 46 | 3889 | 1990 | 1721 | 48 | 46 | 46 |
| Solar CSP | 5050 | 4,000 | 3400 | 200 | 160 | 140 | 5050 | 3650 | 2850 | 200 | 150 | 110 |
| Solar CSP with storage | 6789 | 5,129 | 4513 | 228 | 188 | 168 | 6789 | 4779 | 3963 | 228 | 178 | 138 |
| Small Hydro— Mini Grid | 3000 | 3000 | 3000 | 90 | 90 | 90 | 3000 | 3000 | 3000 | 90 | 90 | 90 |
| Solar PV— Mini Grid | 503 | 503 | 503 | 7 | 4 | 3 | 503 | 461 | 377 | 7 | 4 | 3 |
| Wind — Mini Grid | 2800 | 2613 | 2527 | 56 | 52 | 50 | 2800 | 2613 | 2527 | 56 | 52 | 50 |
| Solar PV—1 Stand Alone (kW<0.02) | 9620 | 5335 | 4285 | 192 | 107 | 86 | 9620 | 5335 | 4285 | 192 | 107 | 86 |
| Solar PV—2 Stand Alone (0.02<kW<0.05) | 8780 | 4869 | 3911 | 176 | 97 | 78 | 8780 | 4869 | 3911 | 176 | 97 | 78 |
| Solar PV—3 Stand Alone (0.05<kW<0.1) | 6380 | 3538 | 2842 | 128 | 71 | 57 | 6380 | 3538 | 2842 | 128 | 71 | 57 |
| Solar PV—4 Stand Alone (0.1<kW<1) | 4470 | 2479 | 1991 | 89 | 50 | 40 | 4470 | 2479 | 1991 | 89 | 50 | 40 |
| Solar PV—5 Stand Alone (kW>1) | 6950 | 3854 | 3096 | 139 | 77 | 62 | 6950 | 3854 | 3096 | 139 | 77 | 62 |
| Inverter | 80 | 80 | 80 | 0 | 0 | 0 | 80 | 80 | 80 | 0 | 0 | 0 |
| Charge controller | 142 | 142 | 142 | 0 | 0 | 0 | 142 | 142 | 142 | 0 | 0 | 0 |

*Note: Investment cost = Overnight cost * (1+discountrate)^constructiontime, Batteries=139 USD/kWh

Supplementary Table 5. Technical parameters for power generation technologies (on-grid and off-grid) [4].

| Technology | Lifetime (years) | Efficiency (%) |  | Load Factor (%) |
| --- | --- | --- | --- | --- |
|  |  | 2015 | 2040 |  |
| Diesel centralized | 25 | 35% | 35% | 80% |
| Diesel stand-alone (1 kW) (decentralised) | 10 | 16% | 16% | 30% |
| Heavy fuel oil | 25 | 35% | 35% | 80% |
| OCGT | 25 | 38% | 46% | 85% |
| CCGT | 30 | 58% | 63% | 85% |
| CCGT - CCS |  |  |  |  |
| Supercritical coal | 30 | 39% | 39% | 85% |
| Coal + CCS |  |  |  |  |
| Hydro (large scale) | 50 * | 100% | 100% | 50% |
| Hydro (small scale) | 30 * | 100% | 100% | 25% |
| Hydro (medium scale) | 50 * | 100% | 100% | 50% |
| Biomass | 40 | 35% | 35% | 50% |
| Nuclear | 60 | 33% | 33% | 85% |
| Geothermal | 25 | 80% | 80% | 85% |
| Wind onshore | 25 | 100% | 100% | Country specific (24%–48%) |
| Wind offshore | 25 | 100% | 100% | Country specific (24%–48%) |
| Solar PV (centralised-Large scale) | 25 | 100% | 100% | Country specific (13%–32%) |
| Solar PV (decentralised-rooftop) | 20 | 100% | 100% | Country specific (13%–32%) |
| Solar PV with battery | 20 | 100% | 100% | Country specific (14%–33%) |
| Solar CSP | 25 | 100% | 100% | Country specific (33%–52%) |
| Solar CSP with storage | 25 | 100% | 100% | Country specific (38%–72%) |
| Diesel Genset—Mini Grid | 15 | 33% | 33% | 70% |
| Small Hydro— Mini Grid | 30 | 100% | 100% | 25% |
| Solar PV— Mini Grid | 20 | 100% | 100% | Country specific (13%–32%) |
| Wind — Mini Grid | 20 | 100% | 100% | Country specific (24%–48%) |
| Solar PV—1 Stand Alone (kW<0.02) | 10 | 28% | 28% | 50% |
| Solar PV—2 Stand Alone (0.02<kW<0.05) | 15 | 100% | 100% | Country specific (13%–32%) |
| Solar PV—3 Stand Alone (0.05<kW<0.1) | 15 | 100% | 100% | Country specific (13%–32%) |
| Solar PV—4 Stand Alone (0.1<kW<1) | 15 | 100% | 100% | Country specific (13%–32%) |
| Solar PV—5 Stand Alone (kW>1) | 15 | 100% | 100% | Country specific (13%–32%) |
| Solar PV (centralised-Large scale) | 15 | 100% | 100% | Country specific (13%–32%) |

*Note: In the model, it is assumed that the lifetime is 80 years. Thus the existing power plants are not retired instead continue their operation.

Supplementary Table 6. Transmission and distribution costs (KTH-Division of Energy Systems, [5])

| Technology—Cost (USD/kW) | 2015 | 2030 | 2040 |
| --- | --- | --- | --- |
| Transmission (grid) | 365 | 365 | 365 |
| Distribution (final electricity demand) | 2502 | 2502 | 2502 |

# Identified fossil fuel reserves in Africa.

Supplementary Table 7. Identified fossil fuel reserves in Africa.

| Reserves | Total Recoverable Coal (mil. short tons) [6] | Crude Oil proved reserves (bil. barrels) [6] | Natural Gas proved reserves (tril. cubic feet) [6] | Uranium (tonnes) [7], [8] |
| --- | --- | --- | --- | --- |
|  | 2017 | 2019 | 2019 | 2019 |
| Algeria | 65.04 | 12.2 | 159.05 | Exploratory |
| Angola | - | 8.38 | 14.91 | - |
| Benin | - | 0.01 | - | - |
| Botswana | 127.72 | - | - | 193500 |
| Cameroon | - | 0.2 | 4.77 | - |
| Central African Republic | 3.31 | - | - | 5700 |
| Chad | - | 1.5 | NA | - |
| Congo (Brazzaville) | - | 1.6 | 3.2 | - |
| Congo (Kinshasa) | 97.00 | 0.18 | 0.04 | 51776 |
| Cote d´Ivoire | - | 0.1 | 1 | - |
| Egypt | 17.67 | 3.3 | 63 | - |
| Equatorial Guinea | - | 1.1 | 5.12 | - |
| Ethiopia | - | 0.00043 | 0.88 | - |
| Gabon | - | 2 | - | Exploratory |
| Ghana | - | 0.66 | 0.8 | - |
| Guinea | - | - | - | 179000 |
| Libya | - | 48.36 | 53.14 | - |
| Malawi | 2.20 | - | - | Exploratory |
| Mali | - | - | - | Exploratory |
| Mauritania | - | 0.02 | 15 | Exploratory |
| Morocco | 15.43 | 0.00068 | 0.05 | Exploratory |
| Mozambique | 1975.34 | - | 100 | - |
| Namibia | - | - | 2.2 | 199068 |
| Niger | 6.61 | 0.15 | NA | 11661 |
| Nigeria | 379.19 | 36.18 | 198.71 | Exploratory |
| Rwanda | - | - | 2 | - |
| South Africa | 34722.77 | 0.02 | 0.03 | 514421 |
| Sudan | - | 5 | 3 | - |
| Swaziland | 158.73 | - | - |  |
| Tanzania | 296.52 | - | 0.23 | 23200 |
| Tunisia | - | 0.43 | 2.3 | - |
| Uganda | - | 2.5 | 0.5 | - |
| Zambia | 49.60 | - | - | 37296 |
| Zimbabwe | 553.36 | - | - | 1720 |

# Renewable energy potential per country in Africa.

Supplementary Table 8. Renewable energy potential per country in Africa.

| Estimates of Renewable Energy Potential | Hydro (MW)[9], [10] | Small hydro (MW) [11] | Solar Thermal (TWh/y)[11] | Solar PV (TWh/y)[11] | Biomass (MW) [12] |
| --- | --- | --- | --- | --- | --- |
| Algeria | n/a | n/a | 26530 | 27904 | n/a |
| Angola | n/a | 861 | 9786 | 13319 | 500 |
| Benin | n/a | 187 | - | 3898 | 761 |
| Botswana | n/a | 1 | 13070 | 13764 | 10 |
| Burkina Faso | - | 38 | - | 7742 | 1075 |
| Burundi | 1700 | 61 | 786 | 888 | n/a |
| Cameroon | 23000 | 615 | 3706 | 10105 | n/a |
| Central African Republic | n/a | 41 | 3471 | 5284 | n/a |
| Chad | - | - | 10284 | 10506 | n/a |
| Congo (Brazzaville) | n/a | 50 | 2 | 6778 | n/a |
| Congo (DRC,Kinshasa) | 100000 | 101 | 12439 | 22862 | 500 |
| Cote d'Ivoire | 1900 | 40.7 | 221 | 10325 | 3260 |
| Djibouti | n/a | - | 852 | 947 | n/a |
| Egypt | 3664 | 52 | 26605 | 32218 | n/a |
| Equatorial Guinea | n/a | 7 | - | 706 | n/a |
| Eritrea | n/a | - | 4349 | 4775 | n/a |
| Ethiopia | 45000 | 1500 | 22959 | 27154 | n/a |
| Gabon | n/a | 6 | 6 | 5402 | n/a |
| Gambia | n/a | 12 | 316 | 474 | 60 |
| Ghana | 2480 | 1245 | 229 | 7644 | 4449 |
| Guinea | n/a | 198 | 467 | 5204 | 1732 |
| Guinea-Bissau | 184 | - | 906 | 1493 | 205 |
| Kenya | 6000 | 3000 | 15399 | 23046 | n/a |
| Lesotho | n/a | 20 | 1122 | 938 | 10 |
| Liberia | 2300 | 65.9 | - | 667 | 1375 |
| Libya | n/a | - | 11823 | 13979 | n/a |
| Malawi | n/a | 150 | 4474 | 5210 | 200 |
| Mali | 1150 | 117 | - | 7906 | 447 |
| Mauritania | n/a | - | 4988 | 7990 | n/a |
| Morocco | n/a | 54 | 15127 | 15155 | n/a |
| Mozambique | n/a | 1000 | 16851 | 22024 | 1000 |
| Namibia | n/a | 108 | 29716 | 26183 | 50 |
| Niger | n/a | - | 8829 | 15669 | 266 |
| Nigeria | >14120 | 735 | 10045 | 32456 | 7291 |
| Rwanda | 500 | 48 | 789 | 892 | n/a |
| Senegal | 1,200 | n/a | 1537 | 7519 | 466 |
| Sierra Leone | - | 1513 | 197 | 1499 | 587 |
| Somalia | n/a | - | 13156 | 25687 | n/a |
| South Africa | n/a | 247 | 43275 | 42243 | 3000 |
| Sudan and South Sudan | 4920 | 27 | 77422 | 87817 | n/a |
| Swaziland | n/a | 16 | 559 | 572 | 200 |
| Tanzania | 3800 | 400 | 31482 | 38804 | 1000 |
| Togo | - | 144 | - | 1257 | 378 |
| Tunisia | n/a | 56 | 2045 | 4645 | n/a |
| Uganda | >4500 | 200 | 8582 | 9470 | n/a |
| Zambia | 6000 | 42 | 15691 | 17894 | 1000 |
| Zimbabwe | n/a | 120 | 11874 | 15864 | 1000 |

# Renewable energy potential per country in Africa.

Supplementary Table 9. Renewable energy potential per country in Africa.

| Estimates of Renewable Energy Potential | Wind (CF 20%) (TWh/y)[11] | Wind (CF 30%) (TWh/y) [11] | Wind (CF 40%) (TWh/y) [11] | Geothermal (MW)[12] |
| --- | --- | --- | --- | --- |
| Algeria | 30155 | 2535.9 | 153.4 | n/a |
| Angola | 202 | - | - | - |
| Benin | 405 | - | - | - |
| Botswana | 9,793 | 303 | - | - |
| Burkina Faso | 4154 | 7.5 | - | - |
| Burundi | - | - | - | n/a |
| Cameroon | 979 | 15.9 | - | - |
| Central African Republic | 79 | - | - | - |
| Chad | 9165 | 1519.4 | 578.3 | - |
| Congo (Brazzaville) | - | - | - | - |
| Congo (DRC,Kinshasa) | 2173 | 41.4 | - | n/a |
| Cote d'Ivoire | 430 | - | - | - |
| Djibouti | 934 | 149.1 | 77.3 | 1000 |
| Egypt | 36601 | 6185 | 529.9 | n/a |
| Equatorial Guinea | - | - | - | - |
| Eritrea | 3154 | 412.4 | 129.1 | n/a |
| Ethiopia | 14838 | 3002.1 | 1981 | 5000 |
| Gabon | - | - | - | - |
| Gambia | 173 | 1.3 | - | - |
| Ghana | 606 | 2.4 | - | - |
| Guinea | 2 | - | - | - |
| Guinea-Bissau | 124 | - | - | - |
| Kenya | 22476 | 4446.4 | 1739.6 | 10000 |
| Lesotho | 599 | 40.1 | 3.7 | - |
| Liberia | - | - | - | - |
| Libya | 21649 | 5149.5 | 1079.5 | n/a |
| Malawi | 1986 | 262.1 | 42.4 | n/a |
| Mali | 1923 | - | - | - |
| Mauritania | 11822 | 2940.5 | 1337.8 | - |
| Morocco | 11297 | 1458.8 | 851 | n/a |
| Mozambique | 10805 | 395.9 | 5.2 | n/a |
| Namibia | 15196 | 497 | 4.9 | - |
| Niger | 14628 | 1262 | 55.8 | - |
| Nigeria | 12867 | 95.3 | - | n/a |
| Rwanda | - | - | - | 700 |
| Senegal | 5454 | 323.6 | 3 | - |
| Sierra Leone | - | - | - | - |
| Somalia | 43539 | 10616.4 | 8893.3 | - |
| South Africa | 41195 | 6076.3 | 1559.1 | n/a |
| Sudan and South Sudan | 61661 | 9837.8 | 2947.1 | 400 |
| Swaziland | 476 | 9.7 | - | - |
| Tanzania | 18456 | 2295.2 | 789.2 | 650 |
| Togo | 79 | - | - | - |
| Tunisia | 6842 | 1244 | 226.5 | n/a |
| Uganda | 815 | 100.7 | 23.8 | 450 |
| Zambia | 13229 | 1145 | 15.6 | n/a |
| Zimbabwe | 12137 | 1000.3 | 47.3 | - |

# Losses in national transmission and distribution network per country in Africa.

The energy conversion technologies are estimated to have 5% losses throughout the modelling period. The distribution losses for the residential areas for the current un-electrified settlements are adjusted using a report published by IRENA [5].

Supplementary Table 10. Losses in national transmission and distribution network per country in Africa [5], [13]–[15].

| Country | Transmission | Distribution losses |  |  | Distribution losses (current un-electrified settlemenents, residential areas) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2015 | 2030 | 2040 | 2015 | 2030 | 2040 |
| Algeria | 5% | 27% | 22% | 20% | 21% | 16% | 14% |
| Angola | 5% | 7% | 5% | 5% | 21% | 16% | 14% |
| Benin | 5% | 14% | 17% | 16% | 19% | 14% | 13% |
| Burkina Faso | 5% | 46% | 46% | 44% | 14% | 14% | 13% |
| Botswana | 4% | 17% | 14% | 13% | 16% | 15% | 14% |
| Burundi | 5% | 11% | 8% | 7% | 7% | 7% | 7% |
| Cameroon | 5% | 21% | 18% | 17% | 7% | 7% | 7% |
| Central African Republic | 5% | 21% | 18% | 17% | 7% | 7% | 7% |
| Chad | 5% | 21% | 18% | 17% | 7% | 7% | 7% |
| Congo (Brazzaville) | 5% | 46% | 43% | 41% | 7% | 7% | 7% |
| Congo (DRC,Kinshasa) | 5% | 11% | 8% | 7% | 19% | 15% | 14% |
| Cote d'Ivoire | 5% | 18% | 14% | 13% | 19% | 14% | 13% |
| Djibouti | 5% | 19% | 17% | 16% | 8% | 8% | 8% |
| Egypt | 3.6% | 9% | 8% | 7% | 7% | 7% | 7% |
| Equatorial Guinea | 5% | 28% | 20% | 18% | 7% | 7% | 7% |
| Eritrea | 5% | 19% | 14% | 13% | 21% | 16% | 14% |
| Ethiopia | 5% | 14% | 12% | 11% | 8% | 8% | 8% |
| Gabon | 5% | 40% | 36% | 34% | 7% | 7% | 7% |
| Gambia | 5% (2015), 4%( 2040) | 18% | 16% | 15% | 21% | 14% | 13% |
| Ghana | 5% | 18% | 18% | 17% | 19% | 14% | 13% |
| Guinea | 5% (2015), 4%( 2040) | 6% | 5% | 5% | 21% | 14% | 13% |
| Guinea-Bissau | 5% | 48% | 52% | 49% | 21% | 14% | 13% |
| Kenya | 5% | 15% | 12% | 9% | 11% | 11% | 11% |
| Lesotho | 5% | 12% | 7% | 7% | 17% | 15% | 14% |
| Liberia | 5% | 32% | 31% | 28% | 21% | 14% | 13% |
| Libya | 5% | 28% | 27% | 25% | 21% | 16% | 14% |
| Malawi | 5% | 18% | 16% | 15% | 20% | 15% | 14% |
| Mali | 5% | 17% | 16% | 15% | 19% | 14% | 13% |
| Mauritania | 5% | 33% | 35% | 33% | 21% | 16% | 14% |
| Morocco | 5% | 12% | 8% | 7% | 21% | 16% | 14% |
| Mozambique | 5% | 12% | 11% | 8% | 24% | 16% | 14% |
| Namibia | 3% | 5% | 6% | 5% | 19% | 15% | 14% |
| Niger | 5% | 22% | 20% | 18% | 17% | 14% | 13% |
| Nigeria | 5% | 12% | 8% | 7% | 18% | 14% | 13% |
| Rwanda | 5% | 30% | 21% | 19% | 6% | 6% | 6% |
| Senegal | 5% | 11% | 8% | 7% | 19% | 14% | 13% |
| Sierra Leone | 5% | 41% | 41% | 37% | 21% | 14% | 13% |
| Somalia | 5% | 47% | 42% | 38% | 21% | 16% | 14% |
| South Africa | 4% | 5% | 4% | 4% | 18% | 15% | 14% |
| South Sudan | 5% (2015), 4%( 2040) | 5% | 5% | 5% | 11% | 11% | 11% |
| Sudan | 5% (2015), 4%( 2040) | 10% | 5% | 4% | 11% | 11% | 11% |
| Swaziland | 5% | 5% | 5% | 5% | 16% | 15% | 14% |
| Tanzania | 5% | 14% | 11% | 9% | 18% | 15% | 14% |
| Togo | 5% | 3% | 8% | 7% | 19% | 14% | 13% |
| Tunisia | 5% | 13% | 11% | 9% | 21% | 16% | 14% |
| Uganda | 5% | 14% | 12% | 11% | 8% | 8% | 8% |
| Zambia | 4% | 6% | 9% | 8% | 23% | 16% | 14% |
| Zimbabwe | 4% | 16% | 11% | 10% | 17% | 15% | 14% |

# National grid-connected renewable energy generation targets.

Supplementary Table 11. National grid-connected renewable energy generation targets.

| Country | Power pool | 2020 | 2024 | 2030 | 2035 |
| --- | --- | --- | --- | --- | --- |
| Algeria | NAPP |  |  | 27% |  |
| Egypt | EAPP | 20% |  |  |  |
| Libya | NAPP | 10% |  |  |  |
| Morocco | NAPP | 42% |  | 52% |  |
| Mauritania | NAPP | 20% |  | 35% |  |
| Tunisia | NAPP |  |  | 30% |  |
| Botswana | SAPP |  |  | 35% |  |
| Cameroon | CAPP |  |  |  | 25% |
| Eritrea | EAPP |  |  | 70% |  |
| Namibia | SAPP |  |  | 70% |  |
| South Africa | SAPP |  |  | 10% |  |
| Rwanda | EAPP |  | 60% | 60% |  |
| Sudan | EAPP |  |  | 20% |  |
| Benin | WAPP | 20% |  | 44% |  |
| Burkina Faso | WAPP | 23% |  | 50% |  |
| Cote d` Ivoire | WAPP |  |  | 16% (excl. large hydro) |  |
| Gambia | WAPP | 35% |  | 48% |  |
| Ghana | WAPP | 10% |  | 20% (excl. large hydro) |  |
| Guinea | CAPP | 25% |  | 30% (excl. large hydro) |  |
| Guinea – Bissau | WAPP | 30% |  | 50% |  |
| Liberia | WAPP | 25% |  | 30% |  |
| Mali | WAPP |  |  | 30% (excl. large hydro) |  |
| Niger | WAPP | 40% |  | 57% |  |
| Nigeria | WAPP | 20% |  | 30% |  |
| Senegal | WAPP | 20% |  | 30% (excl. large hydro) |  |
| Sierra Leone | WAPP | 30% |  | 50% |  |
| Togo | WAPP | 17% |  | 30% |  |

# Emission factors per input fuel.

Supplementary Table 12. Emission factors per input fuel [16].

| Fuel | Emission factor (kg CO2/MMtoe) |
| --- | --- |
| Crude Oil | 1.880 |
| Coal | 2.405 |
| Natural Gas | 1.338 |
| Heavy Fuel Oil | 1.987 |
| Diesel | 1.845 |

# Carbon dioxide price.

Supplementary Table 13. CO2 price ($2019 per tonne).

| Region | Sector | New Policies |  |
| --- | --- | --- | --- |
|  |  | 2025 | 2040 |
| South Africa | Power, industry | 10 | 24 |

*Note: The costs are discounted in the base year.

# Cross-border electricity interconnector projects in Africa.

Supplementary Table 14. Cross-border electricity interconnector projects in Africa.

| From | To | Capacity (MW) | Investment Cost | Status | Year | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Angola | DRC | 600 | 191 mil. USD | PLN | 2019 |  |
| Algeria | Morocco | 240 |  | OPR |  | North Africa Power Transmission Corridor |
| Spain | Morocco | 1400 |  | OPR |  |  |
| Spain | Morocco | 700 | 150 mil. € | PLN | 2026 | Spain shouldering about half the cost. It is due to be commissioned before 2026. Planned addition to an existing interconnector project 1.4 GW. |
| Benin | Niger | 330 | 730 mil. USD | PLN | 2022 | Station: MALANVILLE-ZABORI. Part of the Northcore interconnector |
| Benin | Ghana | 654.6 | 117.6 mil. USD | CON | 2020 | Station: Kara/Bembereke/Parakou (TB)–Yendi (GH) |
| Botswana | South Africa | 800 | - | OPR |  |  |
| Botswana | Zambia | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Botswana | Namibia | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Botswana | Zimbabwe | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Burkina Faso | Niger | 330 | 730 mil. USD | PLN | 2022 | Stations: OUAGADOUGOU-NIYAMEY. Part of the Northcore interconnector |
| Burundi | Rwanda | 17 |  | OPR |  | There are plans to increase the capacity |
| Cameroon | Chad | 1900 | 1362 mi. USD | PLN | 2020 | As option to invest in the following years (2020: 0.475GW, 2021: 0.475GW, 2022: 0.475GW, 2023: 0.475GW) |
| Cote d´Ivoire | Sierra Leone | 110 | 500 mil. USD | PLN | 2020 |  |
| Cote d´Ivoire | Ghana | 655.2 | 90 mil. USD | OPR | 2017 | Station: Riviera - Presea |
| Cote d´Ivoire | Burkina Faso | 327 |  | OPR |  |  |
| Cote d´Ivoire | Liberia | 337.6 | 59.7 mil. USD | CON | 2018 | Station: Man (CI)–Yekepa (LI) |
| DRC | Burundi | 15.5 |  | OPR |  |  |
| DRC | Burundi | 49 |  | CON | 2018 |  |
| DRC | Congo | 60 |  | OPR |  |  |
| DRC | Rwanda | 100 |  | OPR |  |  |
| DRC | Rwanda | 300 |  | CON | 2015 |  |
| DRC | South Africa | 3000 |  | PLN | 2023 | Included partly-2500MW |
| DRC | Zambia | 260 |  | OPR |  |  |
| Egypt | Sudan | 6600 | 6000 mil. USD | CON (400 MW in 2019) |  | North–South Power Transmission Corridor |
| Egypt | Jordan | 300 |  | OPR |  |  |
| Egypt | Saudi Arabia | 3000 |  | CON | 2021 |  |
| Ethiopia | Kenya | 2000 | 1,260 mil. USD | CON | 2019 | Wolayta-Sodo (ETH) and Suswa (KEN) substations; Over 95% of the construction of the Ethio-Kenya interconnection transmission line is completed and work on the project is executed by the respective countries. |
| Ethiopia | Djibouti | 180 |  | OPR |  |  |
| Ethiopia | Sudan | 200 |  | OPR |  |  |
| Ghana | Burkina Faso | 332.2 | 67 mil. USD | CON | 2017; 2019 | The line is operational but only used at 50% capacity due to the unfinished Kumasi –Bolgatanga line. Stations: Han (GH) - Bobo Dioulase (BU). Part of the Hub Intrazonal project. |
| Ghana | Cote d´Ivoire | 327 |  | OPR |  |  |
| Ghana | Togo | 438 |  | OPR |  |  |
| Ghana | Togo | 655.2 | 90 mil. USD | CON | 2017 | Station: VOLTA-DAVIÉ(LOMÉ); Part of the WAPP Coastal Transmission Backbone. The total cost for this project is divided between Ghana, Togo and Benin |
| Ghana | Benin | 655.2 | 90 mil. USD | CON | 2017 | Station: VOLTA-DAVIÉ(LOMÉ); Part of the WAPP Coastal Transmission Backbone |
| Ghana | Mali | 100 | 230 mil. USD | PLN | 2021-2022 |  |
| Gambia | Guinea Bissau | 329.1 | 90.6 mil. USD | CON | 2019 | Station: Soma (GA)–Bissau (GB) .Part of the OMVG project |
| Gambia | Senegal | 340.7 | 36.2 mil. USD | CON | 2019 | Station: Birkelane (SE)–Soma (GA). Part of the OMVG project |
| Guinea | Mali | 321.3 | 117.6 mil. USD | CON | 2020 | Station: Fomi (GU)–Bamako (MA). Part of the Hub Intrazonal project. |
| Guinea | Senegal | 286.3 | 289.8 mil. USD | CON | 2019 | Station: Kaolack (SE)–Linsan (GU). Part of the OMVG project |
| Kenya | Uganda | 103 |  | OPR |  |  |
| Kenya | Uganda | 440 | 380 mil. USD | CON | 2019 |  |
| Kenya | Tanzania | 1 |  | OPR |  |  |
| Kenya | Tanzania | 1300 |  | PLN | 2018 |  |
| Libya | Egypt | 170 |  | OPR | 1998 |  |
| Lesotho | South Africa | 230 |  | OPR |  |  |
| Morocco | Algeria | 480 |  | OPR |  | North Africa Power Transmission Corridor |
| Mali | Cote d´Ivoire | 319.7 | 136.9 mil. USD | CON | 2017 | Station: Segou (MA)– Ferkessedougou (CI). Part of the Hub Intrazonal project. |
| Malawi | Mozambique | 2400 | 125 mil. USD | PLN | 2020 |  |
| Malawi | Tanzania | 180 | 829 mil. USD | PLN | 2022 |  |
| Mozambique | South Africa | 3850 |  | OPR |  | PLN-600 MW |
| Mozambique | Swaziland | 1450 |  | OPR |  |  |
| Mozambique | Zimbabwe | 500 |  | OPR |  | PLN-500 MW |
| Namibia | Botswana | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Namibia | South Africa | 750 |  | OPR |  |  |
| Namibia | Zambia | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Namibia | Zimbabwe | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Niger | Burkina Faso | 358 | 250.4 mil. USD | PLN | 2023: 137MW, 2024: 11MW, 2026: 59MW, 2027: 21MW, 2028: 31MW, 2029: 53MW, 2030: 46 MW | Station: Niamey (NI)–Ouagadougou (BU). Part of the Corridor Nord interconnector |
| Niger | Benin | 30 | 166.6 mil. USD | PLN | 2030 | Station: Zabori (NI)–Bembereke (TB) |
| Nigeria | Benin | 686 |  | OPR |  |  |
| Nigeria | Benin | 494 | 164.6 mil. USD | PLN | 2027: 65MW, 2028: 126MW, 2029: 173MW, 2030: 130MW | Station: Kaindhji (NG)–Kara/Bembereke/Parakou (TB). Part of Dorsale Mediane project |
| Nigeria | Niger | 169 |  | OPR |  |  |
| Nigeria | Niger | 653.1 | 143.1 mil. USD | PLN | 2020 | Station: Birnin Kebbi (NG)–Niamey (NI) |
| Nigeria | Togo | 646.7 | 164.6 mil. USD | CON | 2020 | Station: Kaindhji (NG)–Kara/Bembereke/Parakou (TB) |
| Rwanda | Burundi | 12 |  | OPR |  |  |
| Rwanda | Tanzania | 400 | 33 mil. USD | CON | 2019 |  |
| Senegal | Mauritania | 250 | 207 mil. USD | PLN | 2021 |  |
| South Africa | Swaziland | 1450 |  | OPR |  |  |
| South Africa | Zimbabwe | 600 |  | OPR |  |  |
| Togo | Benin | 345 |  | OPR |  |  |
| Togo | Ghana | 654.6 | 117.6 mil. USD | CON | 2020 | Station: Kara/Bembereke/Parakou (TB)–Yendi (GH) |
| Tanzania | Burundi | 27 |  | CON | 2018 |  |
| Tanzania | Kenya | 2000 | 310 mil. USD | CON | 2020 | Isinia-Singida substation; Part of the ZTK Transmission Interconnector |
| Tanzania | Uganda | 58 |  | OPR |  |  |
| Tanzania | Zambia | 530 | 152.19 mil. USD | CON | 2020 | Part of the ZTK Transmission Interconnector. |
| Tunisia | Algeria | 480 |  | OPR |  |  |
| Uganda | Kenya | 145 |  | OPR |  |  |
| Uganda | Kenya | 300 |  | CON | 2015 |  |
| Uganda | Tanzania | 70 |  | OPR |  |  |
| Uganda | Tanzania | 200 | - | PLN | 2022 |  |
| Uganda | Rwanda | 145 | 58 mil.USD | OPR |  |  |
| Uganda | Rwanda | 300 |  | CON | 2015 |  |
| Zambia | Zimbabwe | 1400 |  | OPR |  |  |
| Zambia | Tanzania | 458 | 172.6 mil. USD | CON | 2020 | Part of the ZTK Transmission Interconnector. 386 MW in 2020, based on the Zambia´s projected electricity generation and in 2025 will be 458MW (max. power transfer). |
| Zambia | Botswana | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Zambia | Namibia | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Guinea Bissau | Guinea | 309.6 |  | CON | 2019 |  |
| Burkina Faso | Mali | 305.8 |  | CON | 2020 |  |
| Liberia | Guinea | 337.6 |  | CON | 2018 |  |
| Mali | Mauritania | 250 |  | PLN | 2021 |  |
| Mali | Sierra Leone | 327 |  | PLN | 2020 |  |
| Mali | Senegal | 329.1 |  | CON | 2020 |  |
| Niger | Togo | 649.7 |  | PLN | 2020 |  |
| Sierra Leone | Guinea | 333.7 |  | CON | 2018 |  |
| Senegal | Gambia | 340.7 |  | CON | 2019 |  |
| Senegal | Guinea | 286.3 |  | CON | 2019 |  |
| Senegal | Mali | 100 |  | OPR |  |  |

# List of crude oil refineries in Africa.

Supplementary Table 15. List of crude oil refineries in Africa [17].

| Country | Name of the plant | Distillation Capacity (tb/d) | Nameplate Capacity (tb/d) | Earliest Year | Status |
| --- | --- | --- | --- | --- | --- |
| Algeria | Adrar | 13 |  | 2007 | OPR |
| Algeria | Algiers | 58 |  | 1964 | OPR |
| Algeria | Arzew | 90 |  | 1973 | OPR |
| Algeria | Skikda | 462 |  | 1993 | OPR |
| Algeria | Hassi Messaoud | 27 |  | 1979 | OPR |
| Angola | Luanda | 65 |  | 1958 | OPR |
| Angola | Luanda expansion |  |  | 2021 | PLN |
| Cameroon | Cape Limboh Limbe | 70 |  | 1981 | OPR |
| Chad | Ndajamena | 20 |  | 2011 | Not operational |
| Congo | Pointe Noire | 21 |  | 1982 | OPR |
| Cote d Ivoire | Abidjan | 84 |  | 1962 | OPR |
| Cote d Ivoire | Société Multinationale des Bitumes (SMB) |  |  |  |  |
| Egypt | Ameriya (Alexandria) | 88 | 81 | 1972 | OPR |
| Egypt | El Mex (Alexandria) | 115 |  | 1958 | OPR |
| Egypt | MIDOR (Alexandria) | 115 |  | 1994 | OPR |
| Egypt | MIDOR (Alexandria) expansion | 45 |  | 2020 | CON |
| Egypt | Assiut | 60 | 47 | 1987 | OPR |
| Egypt | El Suez | 68 |  | 1963 | OPR |
| Egypt | Mostorod | 142 |  | 1982 | OPR |
| Egypt | Nasr El Suez | 146 | 107.55 | 1913 | OPR |
| Egypt | Nasr Wadi Feran | 9 |  | 1990 | OPR |
| Egypt | Tanta | 35 |  | 1982 | OPR |
| Eritrea | Assab oil refinery | 17.5 | 0 | 1960 | Non-operated since 1997 |
| Gabon | Port Gentil | 24 |  | 1967 | OPR |
| Ghana | TEMA | 45 |  | 1963 | OPR |
| Liberia | Monrovia | 15 |  |  | Non-operated |
| Libya | Ras Lanuf plant | 220 |  | 1984 | OPR |
| Libya | Azzawiya and Benghazi | 120 |  | 1974 | OPR |
| Libya | Brega | 10 |  | 1970 | OPR |
| Libya | Sarir | 10 |  | 1989 | OPR |
| Libya | Tobruk | 20 |  | 1985 | OPR |
| Morocco | Mohammedia | 200 | 0 | 1961 | Non operation since 2015 |
| Niger | Zinder, Ganaram | 20 | 12 to 16 | 2011 | OPR |
| Nigeria | Port Harcourt refinery I and II | 210 | 50% | 1965 (I), 1989 (II) | OPR |
| Nigeria | Kaduna | 110 | 50% | 1983 | OPR |
| Nigeria | Warri | 125 | 50% | 1978 | OPR |
| Senegal | M´ Bao (Dakar) | 25 |  | 1963 | OPR |
| Sierra Leone | Freetown | 5 |  | 1970 | Non operated since 1992 |
| South Africa | Caltex | 100 |  | 1966 | OPR |
| South Africa | Durban | 135 | 120 | 1997 | OPR |
| South Africa | Sapref | 180 |  | 1963 | OPR |
| South Africa | Sasolburg | 105 |  | 1971 | OPR |
| Sudan | Khartoum | 100 |  | 2006 | OPR |
| Sudan | Al-Obeid | 10 |  | 1979 | OPR |
| Sudan | Concorp (Alshajara) | 10 |  | 2000 | OPR |
| Sudan | Abu Gabra | 2 |  | 1992 | OPR |
| Sudan | Port Sudan | 21.7 |  | 1964 | Not operational |
| Tunisia | Bizerte | 34 |  | 1963 | OPR |
| Tunisia | La Skhira | 150 |  |  | PLN |
| Zambia | Indeni-Ndola | 24 |  | 1973 | OPR |

*CON: committed plants that are in the implementation phase and will come into operation in the designated year, PLN: planned plants that don’t have a start date but are in the design and planning phase, OPR: operational plants

# List of LNG terminals in Africa.

Supplementary Table 16. List of LNG terminals in Africa [18].

| Country | Project name | Capacity | Year | Status* |
| --- | --- | --- | --- | --- |
| Algeria | Arzew - GL1Z T1-6 | 7.9 MTPA | 1978 | OPR |
| Algeria | Arzew - GL2Z T1-6 | 8.2 MTPA | 1981 | OPR |
| Algeria | Skikda - GL1K Rebuild | 4.5 MTPA | 2013 | OPR |
| Algeria | Arzew - GL3Z | 4.7 MTPA | 2014 | OPR |
| Angola | Angola LNG T1 | 5.2 MTPA | 2014 | OPR |
| Benin | Cotonou (connect to Ghana, Togo) | 1.6 bcm/y |  | PLN |
| Benin | Cotonou extension | 1.1 bcm/y |  | PLN |
| Cameroon | PFLNG Satu | 92 MTPA |  | CON as of March 2018 |
| Cameroon | Cove Point LNG | 92 MTPA |  | CON as of March 2018 |
| Cameroon | Kribi FLNG | 92 MTPA |  | CON as of March 2018 |
| Cameroon | LNG1 | 12 MTPA | 2018 | CON |
| Cameroon | LNG2 | 8 MTPA | 2021 | PLN |
| Cote d´Ivoire | Grand Bassam | 5 MTPA |  | PLN |
| Djibouti | Djibouti FLNG | 3 MTPA | 2021 | CON |
| Egypt | SEGAS LNG T1 | 5 MTPA | 2005 | OPR |
| Egypt | Egyptian LNG T1 | 3.6 MTPA | 2005 | OPR |
| Egypt | Egyptian LNG T2 | 3.6 MTPA | 2005 | OPR |
| Egypt | Ain Sokhna Hoegh (floating) | 4.2 MTPA | 2015 | OPR |
| Egypt | Sumed BW (floating) | 5.7 MTPA | 2017 | OPR |
| Equatorial Guinea | EG LNG T1 | 3.7 MTPA | 2007 | OPR |
| Equatorial Guinea | Fortuna FLNG 1-2 | 4.4 MTPA | 2020-2025 | PLN |
| Ghana | Tema (Golar Tundra) | 1.8 bcm/y | 2017 | CON |
| Ghana | Tema | 2.6 bcm/y |  | CON - OPR in 2020 |
| Ghana | Tema extension | 2 bcm/y |  | CON - OPR in 2020 |
| Ghana | Ghana 1000 | 3.7 bcm/y |  | CON - OPR in 2020 |
| Kenya | Mombasa |  |  | PLN |
| Libya |  |  |  | PLN |
| Mauritania | Greater Tortue FLNG (shared with Senegal) | 2.5 MTPA | 2021 | CON |
| Mozambique | Coral South FLNG | 3.4 MTPA | 2022 | CON - OPR 2022 |
| Mozambique | Mamba LNG | 10 MTPA | 2020-2021 | PLN |
| Mozambique | Coral FLNG (Area 4) | 3.4 MTPA | 2022 | PLN |
| Mozambique | Mozambique LNG (Area 1) | 12 MTPA | 2023-2024 | PLN |
| Morocco | Jorf Lasfar | 2 bcm/y | 2021 | PLN |
| Morocco | Jorf Lasfar extension | 3 bcm/y | 2025 | PLN |
| Namibia | Walvis Bay | 0.6 bcm/y | 2017 | OPR |
| Nigeria | Nigeria LNG T1 | 3.3 MTPA | 2000 | OPR |
| Nigeria | Nigeria LNG T2 | 3.3 MTPA | 2000 | OPR |
| Nigeria | Nigeria LNG T3 | 3 MTPA | 2003 | OPR |
| Nigeria | Nigeria LNG T4 | 4.1 MTPA | 2006 | OPR |
| Nigeria | Nigeria LNG T5 | 4.1 MTPA | 2006 | OPR |
| Nigeria | Nigeria LNG T6 | 4.1 MTPA | 2008 | OPR |
| Nigeria | NLNG T7-8 | 8.6 MTPA | N/A | PLN |
| Republic of Congo | Congo-Brazzaville FLNG | 1.2 MTPA | 2020 | PLN |
| Senegal | Greater Tortue FLNG (shared with Mauritania) | 2.5 MTPA | 2021 | CON |
| South Africa | Saldanha Bay/Richards Bay |  |  | PLN |
| Sudan |  |  |  | PLN |
| Tanzania | Tanzania LNG (T1-3) | 15 MTPA | 2026-2027 | PLN |
| Tanzania | Tanzania LNG (T4) | 5 MTPA | N/A | PLN |
| Togo |  |  |  | PLN |

# List of gas pipeline projects in Africa.

Supplementary Table 17. Gas pipeline projects in Africa [19], [20].

| From | To | Capacity (MW) | Investment Cost | Status | Year | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Algeria | Italy | 8 bcm/year | 2.5 | OPR | 2014 | GALSI project |
| Algeria | Italy | 30.2 bcm/year increased to 33.5 bcm/year by 2012 | 6.25 | OPR | 1983 | Trans-Mediterranean project |
| Algeria | Portugal | 12 bcm/year | 2.3 | OPR | 1996 | Maghreb-Europé project (Pedro Duran Farell) (through Morocco, Spain) |
| Algeria | Spain | 8 bcm/year | 0.90 € | OPR | 2010 | Medgaz project (offshore) |
| Algeria | Spain | 30 bil.cub.litres | 12 | 2015 Com. | 2021 | Trans-Saharan project |
| Algeria | Tunisia | 30.2 bcm/year increased to 33.5 bcm/year by 2012 | 6.25 | OPR | 1983 | Trans-Mediterranean project |
| Egypt | Turkey | 10.3 bcm/year | 1.2 | Out of order since 2012. To re-open in 2019 | 2003 | Arab project (through Israel-Jordan-Syria-Lebanon) |
| Israel | Egypt | 6 bcm/year |  | CON | 2020 | EMG pipeline |
| Cyprus | Egypt |  |  | PLN |  |  |
| Ethiopia | Djibouti | 3 bil. cub. feet/y | 3.2 | CON (Signed) | 2021 | Ethiopia - Djibouti project |
| Libya | Italy | 11 bcm/year |  | OPR | 2004 | Greenstream project |
| Mozambique | South Africa |  | 6 | PLN (not signed) | 2021 | African Renaissance project |
| Nigeria | Algeria | 30 bil.cub.litres | 12 | 2015 Commissioned | 2021 | Trans-Saharan project |
| Nigeria | Benin | 0.2-0.6 mil.cub.feet a day | 0.9 | OPR | 2011 | West African project |
| Nigeria | Cote d´Ivoire | 0.2-0.6 mil.cub.feet a day |  | PLN | 2025 (Estimated) | West African project |
| Nigeria | Ghana | 0.2-0.6 mil.cub.feet a day | 0.9 | OPR | 2011 | West African project |
| Nigeria | Niger | 30 bil.cub.litres | 12 | 2015 Commissioned | 2021 | Trans-Saharan project |
| Nigeria | Togo | 0.2-0.6 mil.cub.feet a day | 0.9 | OPR | 2011 | West African project |
| Tanzania | Uganda | 3 bil.cub.feet/ | 3.55 | CON (2018 the agreement signed) | 2026 | Tanzania – Uganda project (LNG transport) |

# Units

Supplementary Table 18. Units

| Energy |  |  |
| --- | --- | --- |
| PJ | Petajoule | 1015 J |
| GJ | Gigajoule | 109 J |
| toe | Tonne of oil equivalent |  |
| ktoe | Thousand tonnes of oil eq. | 103 toe |
| Mtoe | Million tonnes of oil eq. | 106 toe |
| Gtoe | Giga tonnes of oil eq. | 109 toe |
| Mbl/d | Million barrels per day | 106 bl/d |
| Mt | Million metric tonnes | 106 t |
| MTPA | Million tonnes per annum |  |
| tb/d | Thousand barrels per day |  |
| bcm/y | billion cubic meter per year |  |
| Electricity |  |  |
| GW | Gigawatts | 109 W |
| TWh | terawatt-hours | 1012 W |
| Water |  |  |
| Mm3 | Million cubic meters |  |
| MCM | Million cubic meters |  |
| Prices |  |  |
| $/bbl | $ per barrel of oil |  |
| $/boe | $ per barrel of oil eq. |  |
| Emissions and related |  |  |
| tCO2 | Tonne CO2 |  |
| MtCO2 | Million tonne CO2 |  |
| Monetary units |  |  |
| M$ | Million USD |  |

Supplementary Table 23. Electricity demand (all sectors) projections (GWh).

| GWh | 2015 | 2030 | 2040 |
| --- | --- | --- | --- |
| NAPP |  |  |  |
| Algeria (DZ) | 50153 | 122888 | 162540 |
| Libya (LY) | 15282 | 39541 | 56524 |
| Mauritania (MR) | 932 | 2579 | 3686 |
| Morocco (MA) | 29939 | 70292 | 111106 |
| Tunisia (TN) | 15437 | 40279 | 57845 |
| SAPP |  |  |  |
| Angola (AO) | 8255 | 20785 | 34249 |
| Botswana (BW) | 3495 | 4686 | 5628 |
| Lesotho (LS) | 694 | 3285 | 6732 |
| Malawi (MW) | 1559 | 6160 | 12624 |
| Mozambique (MZ) | 13449 | 11074 | 8975 |
| Namibia (NM) | 3782 | 16977 | 34795 |
| South Africa (ZA) | 191328 | 269642 | 319726 |
| Swaziland (SZ) | 1569 | 6838 | 14015 |
| Zambia (ZM) | 11450 | 18845 | 17746 |
| Zimbabwe (ZW) | 7426 | 3370 | 5103 |
| EAPP |  |  |  |
| Egypt (EG) | 155871 | 380899 | 622061 |
| Burundi (BI) | 214 | 423 | 656 |
| Djibouti (DJ) | 305 | 720 | 1116 |
| Eritrea (ER) | 303 | 621 | 962 |
| Ethiopia (ET) | 7470 | 27363 | 49127 |
| Kenya (KE) | 7963 | 18934 | 31486 |
| Rwanda (RW) | 419 | 1025 | 1589 |
| Somalia (SO) | 315 | 537 | 833 |
| South Sudan (SS) | 529 | 684 | 1060 |
| Sudan (SD) | 11146 | 27798 | 55281 |
| Tanzania (TZ) | 5250 | 16272 | 30176 |
| Uganda (UG) | 1156 | 4708 | 7298 |
| WAPP |  |  |  |
| Benin (BJ) | 1099 | 1891 | 2867 |
| Burkina Faso (BF) | 1200 | 2407 | 3731 |
| Cote d Ivoire (CI) | 6023 | 15187 | 29160 |
| Gambia (GM) | 357 | 368 | 570 |
| Ghana (GH) | 8681 | 22787 | 45650 |
| Guinea (GN) | 966 | 2438 | 3779 |
| Guinea Bissau (GW) | 36 | 133 | 206 |
| Liberia (LR) | 265 | 460 | 714 |
| Mali (ML) | 6876 | 12536 | 19432 |
| Niger (NE) | 975 | 2116 | 3281 |
| Nigeria (NG) | 24847 | 30327 | 37438 |
| Senegal (SNG) | 3466 | 6344 | 8514 |
| Sierra Leone (SL) | 110 | 1366 | 15934 |
| Togo (TG) | 1007 | 2188 | 3476 |
| CAPP |  |  |  |
| Cameroon (CM) | 6107 | 11755 | 18608 |
| Central African Republic (CF) | 161 | 210 | 326 |
| Chad (TD) | 215 | 339 | 526 |
| Republic of the Congo (CG) | 1318 | 1567 | 2633 |
| Democratic Republic of Congo (CD) | 7266 | 12357 | 22698 |
| Equatorial Guinea (GQ) | 1436 | 3403 | 5080 |
| Gabon (GA) | 1852 | 2944 | 3670 |

Supplementary Table 24. Biofuels and waste demand (PJ).

| PJ | 2015 | 2030 | 2040 |
| --- | --- | --- | --- |
| NAPP |  |  |  |
| Algeria (DZ) | 0.3 | 2 | 2 |
| Libya (LY) | 10 | 13 | 15 |
| Mauritania (MR) | 7 | 14 | 18 |
| Morocco (MA) | 56 | 99 | 130 |
| Tunisia (TN) | 36 | 89 | 142 |
| SAPP |  |  |  |
| Angola (AO) | 194 | 245 | 293 |
| Botswana (BW) | 22 | 19 | 14 |
| Lesotho (LS) | 23 | 21 | 28 |
| Malawi (MW) | 29 | 39 | 53 |
| Mozambique (MZ) | 205 | 146 | 196 |
| Namibia (NM) | 8 | 11 | 14 |
| South Africa (ZA) | 247 | 276 | 343 |
| Swaziland (SZ) | 19 | 35 | 48 |
| Zambia (ZM) | 278 | 430 | 614 |
| Zimbabwe (ZW) | 312 | 465 | 573 |
| EAPP |  |  |  |
| Egypt (EG) | 59 | 65 | 76 |
| Burundi (BI) | 41 | 100 | 230 |
| Djibouti (DJ) | 1 | 2 | 4 |
| Eritrea (ER) | 16 | 38 | 88 |
| Ethiopia (ET) | 1392 | 1856 | 2158 |
| Kenya (KE) | 456 | 620 | 744 |
| Rwanda (RW) | 50 | 123 | 284 |
| Somalia (SO) | 78 | 174 | 401 |
| South Sudan (SS) | 6 | 14 | 31 |
| Sudan (SD) | 284 | 373 | 438 |
| Tanzania (TZ) | 589 | 838 | 1051 |
| Uganda (UG) | 462 | 1610 | 3704 |
| WAPP |  |  |  |
| Benin (BJ) | 74 | 102 | 197 |
| Burkina Faso (BF) | 96 | 155 | 134 |
| Cote d Ivoire (CI) | 179 | 180 | 111 |
| Gambia (GM) | 4 | 5 | 5 |
| Ghana (GH) | 114 | 195 | 169 |
| Guinea (GN) | 95 | 156 | 134 |
| Guinea Bissau (GW) | 22 | 38 | 33 |
| Liberia (LR) | 64 | 114 | 99 |
| Mali (ML) | 95 | 99 | 17 |
| Niger (NE) | 90 | 195 | 421 |
| Nigeria (NG) | 4302 | 5805 | 6783 |
| Senegal (SNG) | 47 | 70 | 88 |
| Sierra Leone (SL) | 35 | 50 | 43 |
| Togo (TG) | 61 | 82 | 96 |
| CAPP |  |  |  |
| Cameroon (CM) | 219 | 338 | 400 |
| Central African Republic (CF) | 9 | 9 | 11 |
| Chad (TD) | 52 | 67 | 79 |
| Republic of the Congo (CG) | 50 | 124 | 218 |
| Democratic Republic of Congo (CD) | 837 | 1065 | 1357 |
| Equatorial Guinea (GQ) | 1 | 1 | 1 |
| Gabon (GA) | 159 | 238 | 225 |

Supplementary Table 25. Coal demand (PJ).

| PJ | 2015 | 2030 | 2040 |
| --- | --- | --- | --- |
| NAPP |  |  |  |
| Algeria (DZ) | 2 | 16 | 23 |
| Libya (LY) | 0 | 0 | 0 |
| Mauritania (MR) | 0 | 0 | 0 |
| Morocco (MA) | 1 | 1 | 2 |
| Tunisia (TN) | 0 | 0 | 0 |
| SAPP |  |  |  |
| Angola (AO) | 0 | 0 | 0 |
| Botswana (BW) | 2 | 11 | 20 |
| Lesotho (LS) | 17 | 28 | 40 |
| Malawi (MW) | 2 | 3 | 3 |
| Mozambique (MZ) | 0 | 0 | 0 |
| Namibia (NM) | 0 | 0 | 0 |
| South Africa (ZA) | 683 | 876 | 971 |
| Swaziland (SZ) | 0.3 | 4 | 6 |
| Zambia (ZM) | 4 | 3 | 2 |
| Zimbabwe (ZW) | 12 | 8 | 6 |
| EAPP |  |  |  |
| Egypt (EG) | 8 | 75 | 97 |
| Burundi (BI) | 0 | 0 | 0 |
| Djibouti (DJ) | 0 | 0 | 0 |
| Eritrea (ER) | 0 | 0 | 0 |
| Ethiopia (ET) | 11 | 49 | 84 |
| Kenya (KE) | 21 | 23 | 30 |
| Rwanda (RW) | 0 | 0 | 0 |
| Somalia (SO) | 0 | 0 | 0 |
| South Sudan (SS) | 0 | 0 | 0 |
| Sudan (SD) | 0 | 0 | 0 |
| Tanzania (TZ) | 7 | 139 | 351 |
| Uganda (UG) | 0 | 0 | 0 |
| WAPP |  |  |  |
| Benin (BJ) | 1 | 10 | 42 |
| Burkina Faso (BF) | 0 | 0 | 0 |
| Cote d Ivoire (CI) | 0 | 0 | 0 |
| Gambia (GM) | 0 | 0 | 0 |
| Ghana (GH) | 0 | 0 | 0 |
| Guinea (GN) | 0 | 0 | 0 |
| Guinea Bissau (GW) | 0 | 0 | 0 |
| Liberia (LR) | 0 | 0 | 0 |
| Mali (ML) | 0 | 0 | 0 |
| Niger (NE) | 0 | 0.02 | 0.02 |
| Nigeria (NG) | 1 | 2 | 7 |
| Senegal (SNG) | 13 | 25 | 35 |
| Sierra Leone (SL) | 0 | 0 | 0 |
| Togo (TG) | 0 | 0 | 0 |
| CAPP |  |  |  |
| Cameroon (CM) | 0 | 0 | 0 |
| Central African Republic (CF) | 0 | 0 | 0 |
| Chad (TD) | 0 | 0 | 0 |
| Republic of the Congo (CG) | 0 | 0 | 0 |
| Democratic Republic of Congo (CD) | 0 | 0 | 0 |
| Equatorial Guinea (GQ) | 0 | 0 | 0 |
| Gabon (GA) | 0 | 0 | 0 |

Supplementary Table 26. Natural gas demand (PJ).

| PJ | 2015 | 2030 | 2040 |
| --- | --- | --- | --- |
| NAPP |  |  |  |
| Algeria (DZ) | 617 | 1563 | 2085 |
| Libya (LY) | 65 | 479 | 835 |
| Mauritania (MR) | 0 | 0 | 0 |
| Morocco (MA) | 3 | 8 | 13 |
| Tunisia (TN) | 54 | 259 | 399 |
| SAPP |  |  |  |
| Angola (AO) | 26 | 22 | 33 |
| Botswana (BW) | 0 | 0 | 0 |
| Lesotho (LS) | 0 | 0 | 0 |
| Malawi (MW) | 0 | 0 | 0 |
| Mozambique (MZ) | 6 | 42 | 260 |
| Namibia (NM) | 0 | 0 | 0 |
| South Africa (ZA) | 73 | 96 | 112 |
| Swaziland (SZ) | 0 | 0 | 0 |
| Zambia (ZM) | 0 | 0 | 0 |
| Zimbabwe (ZW) | 0 | 0 | 0 |
| EAPP |  |  |  |
| Egypt (EG) | 438 | 2483 | 4647 |
| Burundi (BI) | 0 | 0 | 0 |
| Djibouti (DJ) | 0 | 0 | 0 |
| Eritrea (ER) | 0 | 0 | 0 |
| Ethiopia (ET) | 0 | 0 | 0 |
| Kenya (KE) | 0 | 0 | 0 |
| Rwanda (RW) | 0 | 0 | 0 |
| Somalia (SO) | 0 | 0 | 0 |
| South Sudan (SS) | 0 | 0 | 0 |
| Sudan (SD) | 0 | 0 | 0 |
| Tanzania (TZ) | 6 | 16 | 36 |
| Uganda (UG) | 0 | 0 | 0 |
| WAPP |  |  |  |
| Benin (BJ) | 0 | 0 | 0 |
| Burkina Faso (BF) | 0 | 0 | 0 |
| Cote d Ivoire (CI) | 10 | 63 | 387 |
| Gambia (GM) | 0 | 0 | 0 |
| Ghana (GH) | 0 | 0 | 0 |
| Guinea (GN) | 0 | 0 | 0 |
| Guinea Bissau (GW) | 0 | 0 | 0 |
| Liberia (LR) | 0 | 0 | 0 |
| Mali (ML) | 0 | 0 | 0 |
| Niger (NE) | 0 | 0 | 0 |
| Nigeria (NG) | 158 | 513 | 1338 |
| Senegal (SNG) | 0 | 0 | 0 |
| Sierra Leone (SL) | 0 | 0 | 0 |
| Togo (TG) | 0 | 0 | 0 |
| CAPP |  |  |  |
| Cameroon (CM) | 0 | 0 | 0 |
| Central African Republic (CF) | 0 | 0 | 0 |
| Chad (TD) | 0 | 0 | 0 |
| Republic of the Congo (CG) | 0 | 0 | 0 |
| Democratic Republic of Congo (CD) | 0 | 0 | 0 |
| Equatorial Guinea (GQ) | 54 | 924 | 8314 |
| Gabon (GA) | 0.1 | 0.2 | 0.2 |

Supplementary Table 27. Oil products demand (PJ).

| PJ | 2015 | 2030 | 2040 |
| --- | --- | --- | --- |
| NAPP |  |  |  |
| Algeria (DZ) | 768 | 1788 | 2334 |
| Libya (LY) | 270 | 513 | 648 |
| Mauritania (MR) | 36 | 68 | 86 |
| Morocco (MA) | 465 | 938 | 1378 |
| Tunisia (TN) | 182 | 192 | 205 |
| SAPP |  |  |  |
| Angola (AO) | 198 | 252 | 265 |
| Botswana (BW) | 44 | 82 | 106 |
| Lesotho (LS) | 9 | 21 | 39 |
| Malawi (MW) | 9 | 55 | 77 |
| Mozambique (MZ) | 54 | 188 | 343 |
| Namibia (NM) | 54 | 102 | 173 |
| South Africa (ZA) | 1075 | 1921 | 2830 |
| Swaziland (SZ) | 12 | 15 | 20 |
| Zambia (ZM) | 51 | 81 | 138 |
| Zimbabwe (ZW) | 51 | 75 | 94 |
| EAPP |  |  |  |
| Egypt (EG) | 1258 | 3471 | 5545 |
| Burundi (BI) | 6 | 20 | 32 |
| Djibouti (DJ) | 5 | 10 | 16 |
| Eritrea (ER) | 4 | 12 | 19 |
| Ethiopia (ET) | 124 | 347 | 578 |
| Kenya (KE) | 167 | 445 | 850 |
| Rwanda (RW) | 7 | 27 | 43 |
| Somalia (SO) | 5 | 12 | 19 |
| South Sudan (SS) | 16 | 25 | 39 |
| Sudan (SD) | 180 | 417 | 778 |
| Tanzania (TZ) | 95 | 178 | 319 |
| Uganda (UG) | 30 | 83 | 130 |
| WAPP |  |  |  |
| Benin (BJ) | 69 | 158 | 268 |
| Burkina Faso (BF) | 0 | 157 | 343 |
| Cote d Ivoire (CI) | 72 | 242 | 519 |
| Gambia (GM) | 4 | 12 | 26 |
| Ghana (GH) | 156 | 307 | 554 |
| Guinea (GN) | 32 | 135 | 296 |
| Guinea Bissau (GW) | 3 | 11 | 23 |
| Liberia (LR) | 9 | 32 | 70 |
| Mali (ML) | 32 | 87 | 191 |
| Niger (NE) | 21 | 74 | 161 |
| Nigeria (NG) | 761 | 2438 | 5526 |
| Senegal (SNG) | 49 | 53 | 47 |
| Sierra Leone (SL) | 14 | 34 | 74 |
| Togo (TG) | 12 | 48 | 114 |
| CAPP |  |  |  |
| Cameroon (CM) | 58 | 81 | 107 |
| Central African Republic (CF) | 3 | 4 | 5 |
| Chad (TD) | 9 | 12 | 16 |
| Republic of the Congo (CG) | 27 | 92 | 166 |
| Democratic Republic of Congo (CD) | 38 | 100 | 267 |
| Equatorial Guinea (GQ) | 21 | 30 | 37 |
| Gabon (GA) | 32 | 33 | 45 |

# Assumptions used for job creation.

There are different types of jobs associated with the energy industry categorized in “direct”, “indirect” and “induced” jobs. “Direct” energy jobs are associated directly to an activity in the value chain of a particular technology. “Indirect” and “Induced” jobs are the ones created by auxiliary activities or activities that are an outcome of enabling energy availability [21]. Different approaches to estimating employment impacts of deploying energy technologies are categorized into bottom-up and top-down approaches or explicitly using the analytical or input-output models [22].

The methodology adopted below by Ram et al. [23].

Different parameters are considered relevant to manufacturing, construction and installation, operations and maintenance, fuel supply associated with electricity generation, decommissioning of power plants and transmission to estimate the energy transition's direct job creation potential. The different types of jobs are:

- Manufacturing Jobs: defined as the number of jobs needed to manufacture a unit of power generation capacity. To build a power plant, the manufacturing of equipment and components will need several weeks, months, or perhaps years. Hence, they amount to relatively temporary employment compared to the entire plant lifetime. They are expressed as jobs-years, or the total number of full-time jobs needed for manufacturing over the plant´s lifetime [24]. These type of jobs can also be held outside of the country since the equipment and components may not build in the country. Specifically, in many countries the renewable energy technologies are not produced locally due to insufficient materials, while other countries have the possibility to export renewable energy equipments and components and consequently generate employment. Suppose they develop locally renewable energy pieces of equipment, the number of jobs increases since it is added to their domestic energy capacity. The local manufacturing factor is considered to account for the degree of import dependence (explained below).

- Construction and installation Jobs: refers to the jobs related to constructing and installing a unit of power generation capacity. It is assumed that the local workforce will undertake the installation and construction of all energy projects. The jobs are expressed as jobs-year over the plant´s lifetime. Most likely, these jobs are considered in the beginning phase of a power plant until its completion. Thus, the construction and installation jobs are annualised over the construction period of a power plant, which is the time required for the construction and installation of unit power plant capacities (in terms of per MW) [23].

- Operation and Maintenance Jobs: account for the jobs related to the operation and maintenance of the operational condition of a power plant over its technical, operational lifetime. The power plants are usually designed to operate for decades, operation and maintenance jobs last for a relatively longer duration and are consequently interpreted as jobs per power generation capacity. These jobs are considered for the lifetime of the respective power plants and are further annualized to get the total number of jobs during the transition period. As and when power plants are decommissioned and new power plants replace the capacity, the O&M jobs continue to exist. A learning factor, which correlates to the productivity increase in operational expenditure (Opex), is adopted to reflect the decline in O&M jobs as technologies and operational processes mature [23].

- Fuel Jobs: refers to all the jobs relevant to fuel supply to power plants (nuclear, fossil and bioenergy). These are expressed as jobs per unit of primary energy, factoring in the different fuel consumption rates for power plants corresponding to the fuel utilized based on conversion efficiencies during the transition period [23].

- Decommissioning Jobs: account for all jobs relevant to the decommissioning of installed power plants at the end of their operational lifetimes, especially if plants are repowered or if certain elements are recycled or reused. These jobs are comparable to construction and installation jobs and are accounted as job-years or the total number of full-time jobs needed for decommissioning over the plant´s lifetime. These jobs are further annualized during the transition period to derive the total number of jobs created [23].

- Transmission Jobs: account for all jobs relevant to power transmission activities. Transmission jobs are expressed in investments made in transmission infrastructure, i.e., jobs per unit investments (in billion euros) [23].

- Jobs Loss: The jobs lost in fossil fuel and nuclear power plants correspond to the decommissioned capacities of conventional power plants during the transition period [23].

Also, the following parameters are considered in the estimation of job creation potential:

- Employment Factors – account for the number of jobs per unit of installed capacity, separated into manufacturing, construction and installation, operation and maintenance, and decommissioning. It is also jobs per unit of primary energy for fuel production and supply and jobs per unit of electricity generated for transmission and distribution. The employment factors in this study are adopted from Rutovitz et al. [25] as shown in Supplementary Table 27.

- Decline Factors – account for reducing jobs as these are expected to reduce as technologies and production of these technologies mature [23].

- Regional Employment Multiplier – the regional employment multipliers of the various regions are adopted to account for the differential labor-intensive economic activities in the different regions across the world. The EFs considered are normalised for OECD countries. The regional employment multipliers account for the additional employment generated in non OECD countries, which needs adjustment for differing stages of economic development. The lower the cost of labour in a country, the greater the number of workers employed to produce a unit of any particular output, be it manufacturing, construction or agriculture. Low average labour costs are closely associated with low gross domestic product (GDP) per capita, a key indicator of economic development. Therefore, the most reasonable indicator is deriving a proxy factor correlated to average labour productivity, measured as GDP (or value added) per worker. The regional employment multipliers are expected to change over the transition period (2015- 2040), as the differences in labour productivity evolve with regional economic growth. The projected change in GDP per capita derived from GDP growth with corresponding population growth is factored to adjust the regional employment multipliers for the different regions over time (Supplementary Table 28) [23].

- Local Manufacturing factor – refers to the share of fuels produced locally in the different regions and is applied only to fossil fuels (oil, gas and coal). Since fossil fuels are exclusive to certain geographical locations, they are produced in some regions and consumed in other regions resulting in the global trade of fossil fuels. To capture the regional production and consumption in the structure of regions, local fuel production factors are applied and import shares and importing regions for all of the fossil fuels are applied across the regions. The local production shares and relative import shares from corresponding import regions are presented for fossil gas in Supplementary Table 30. This approach enables estimatation of jobs related to fossil fuel exports, which is quite predominant in certain regions [23].

Manufacturing EFs, and construction and installation EFs are applied to the newly installed capacities for each year during the transition period from 2020-2040. The operation and maintenance EFs are applied to cumulative installed capacity for every 5-year interval between 2020-2040. The fuel EFs applied to annual electricity from the various power generation technologies which utilize fuel sources. The decommissiong EFs are applied to the annual decommissioned capacities of power generation, fuel production and storage technologies during the entire transition period. The transmission and distribution EFs are applied to the total electricity generated in the different regions during the transition period [23]. The installed capacities, electricity generation, fuel production and supply and decommissioned capacities for the various power generation, fuel production and storage technologies are adopted from the results of the OSeMOSYS Energy System Transition model for Africa.

Supplementary Figure 1. Method for estimating job creating during the energy transition. Abbreviations: Employment Factor (EF), Capital Expenditure (CAPEX) and Operational Expenditure (OPEX).

Supplementary Table 28. Employment factors used in the estimation of jobs created during the energy transition from 2015 to 2030 (Abbreviations: Mfc – Manufacturing, C&I – Construction and Installation, O&M – Operation and Maintenace, DCM – Decommissioning) (Employment Factors are assumed to scale with progress of technologies and overall productivity through the transition period from 2015-2030, therefore, decline factors linked to the capital expenditure and operational expenditure are considered to capture the effects of scaling of technologies and increased levels of automation) [26], [27] [23], [25].

| Technologies | Mfc. [Job-yrs/MW] | C&I [Job-yrs/MW] | O&M [Jobs/MW] | Fuel [Jobs/PJ] | Dcm. [Job-yrs/MW] | Source |
| --- | --- | --- | --- | --- | --- | --- |
| Wind onshore | 4.7 | 3.2 | 0.3 |  | 0.72 | [25] |
| Wind offshore | 15.6 | 8 | 0.2 |  | 2.99 | [25] |
| PV Utility scale | 6.7 | 13 | 0.7 |  | 0.8 | [25] |
| PV rooftop | 6.7 | 26 | 1.4 |  | 1.21 | [28] |
| Biomass | 2.9 | 14 | 1.5 | 29.9 | 0.32 | [25] |
| Hydro Dam | 3.5 | 7.4 | 0.2 |  | 2.22 | [25] |
| Hydro RoR | 8.75 | 18.5 | 0.5 |  | 5.55 | Assuming 2.5 times the values as Hydro Dam |
| Geothermal | 3.9 | 6.8 | 0.4 |  | 0.21 | [25] |
| CSP | 4 | 8 | 0.6 |  | 1.33 | [25] |
| Coal PP (Hard Coal) | 5.4 | 11.2 | 0.14 | 39.7 | 1.65 | [25] |
| Nuclear PP | 1.3 | 11.8 | 0.6 | 0.001 (Jobs/GWh) | 0.95 (Jobs/GWh) | [25] |
| OCGT | 0.93 | 1.3 | 0.14 | 15.1 | 0.21 | [25] |
| CCGT | 0.93 | 1.3 | 0.14 | 15.1 | 0.21 | [25] |
| Steam Turbine | 0.93 | 1.3 | 0.14 |  | 0.21 | Assuming same as Gas Turbine |
| ICE | 0.93 | 1.3 | 0.21 | 15.1 | 0.44 | [25] |
| Battery storage utility-scale | 16.9 | 10.8 | 0.4 |  | 0.8 | [29] |
| Thermal Energy Storage (TES) | 0.07 (Jobs-yrs/MWh) | 0.15 (Jobs-yrs/MWh) | 0.01 (Jobs-yrs/MWh) |  | 0.02 (Jobs-yrs/MWh) | [30] |
| Transmission | Employment factor with regard to investments - 5045 jobs/b€ |  |  |  |  | The Brattle Group, 2011 |

Supplementary Table 29. Regional employment multipliers, based on labour productivity across the different regions used in the estimation of jobs created during the energy transition from 2015 to 2030 (Abbreviations: MENA – Middle East and North Africa, SSA – Sub-Saharan Africa) [23], [25].

| Regions | Regional Employment Multipliers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2025 | 2030 | 2035 | 2040 |
| MENA | 2.26 | 1.94 | 1.66 | 1.51 | 1.37 | 1.32 |
| SSA | 7.49 | 6.42 | 5.51 | 5 | 4.54 | 4.38 |

The regional employment multipliers, which are based on labour productivity factors are derived from ILO [31] and IMF [32].

Supplementary Table 30. Shares of local manufacturing and corresponding shares of imports across the different regions used in the estimation of jobs created during the energy transition from 2015 to 2040 [23], [25].

| Regions | 2015 | 2020 | 2025 | 2030 | 2035 | 2040 |
| --- | --- | --- | --- | --- | --- | --- |
| MENA | 30% | 30% | 40% | 50% | 60% | 70% |
| Sub-Saharan Africa | 30% | 30% | 40% | 50% | 60% | 70% |

Supplementary Table 31. Shares of local production of fossil gas and corresponding shares of imports across the different regions used in the estimation of jobs created during the energy transition from 2015 to 2040 [23], [25].

|  | Production of Fossil Gas |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Shares of Local Production |  |  |  |  |  | Import Shares (2015-2040) |
| Regions | 2015 | 2020 | 2025 | 2030 | 2035 | 2040 | MENA |
| MENA | 100% | 100% | 100% | 100% | 100% | 100% |  |
| Sub-Saharan Africa | 30% | 30% | 40% | 50% | 60% | 70% | 100% |

Supplementary Table 32. Shares of local production of fossil oil and corresponding shares of imports across the different regions used in the estimation of jobs created during the energy transition from 2015 to 2040 [23], [25].

| Production of Fossil Oil |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Shares of Local Production |  |  |  |  |  | Import Shares (2015-2040) |  |  |  |  |
| Regions | 2015 | 2020 | 2025 | 2030 | 2035 | 2040 | Eurasia | MENA | Southeast Asia | North America | South America |
| MENA | 100% | 100% | 100% | 100% | 100% | 100% |  |  |  |  |  |
| Sub-Saharan Africa | 30% | 30% | 40% | 50% | 60% | 70% | 5% | 70% | 5% | 10% | 10% |

Supplementary Table 33. Shares of local production of coal and corresponding shares of imports across the different regions used in the estimation of jobs created during the energy transition from 2015 to 2040 [23], [25].

| Production of Coal |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Shares of Local Production |  |  |  |  |  | Import Shares (2015-2040) |  |  |  |
| Regions | 2015 | 2020 | 2025 | 2030 | 2035 | 2040 | Eurasia | Southeast Asia | North America | South America |
| MENA | 10% | 10% | 10% | 10% | 10% | 10% | 40% | 40% | 10% | 10% |
| Sub-Saharan Africa | 70% | 70% | 70% | 70% | 70% | 70% | 40% | 50% |  | 10% |

# References

[1]	International Energy Agency, World Energy Outlook 2016. 2016. [Online]. Available: https://www.oecd-ilibrary.org/content/publication/weo-2016-en

[2]	International Energy Agency, World Energy Outlook 2017. 2017. [Online]. Available: https://www.oecd-ilibrary.org/content/publication/weo-2017-en

[3]	International Energy Agency, World Energy Outlook 2018. 2018. [Online]. Available: https://www.oecd-ilibrary.org/content/publication/weo-2018-en

[4]	IEA, “World Energy Outlook 2020,” Paris, 2020. [Online]. Available: https://www.iea.org/reports/world-energy-outlook-2020

[5]	International Renewable Energy Agency, Abu Dhabi, “Planning and prospects for renewable power: West Africa.” 2018.

[6]	“U.S. Energy Information Administration (EIA).” https://www.eia.gov/ (accessed Jul. 19, 2019).

[7]	“Welcome - ENERGYDATA.INFO.” https://energydata.info/en/ (accessed Jul. 19, 2019).

[8]	World Nuclear Association, “Uranium in Africa - World Nuclear Association.” https://www.world-nuclear.org/information-library/country-profiles/others/uranium-in-africa.aspx (accessed May 29, 2020).

[9]	IHA, “Hydropower status report 2017 - International hydropower association.” 2017.

[10]	International hydropower association, “AFRICA | International Hydropower Association.” https://www.hydropower.org/country-profiles/africa (accessed Sep. 10, 2019).

[11]	International Renewable Energy Agency, Abhu Dhabi, “Estimating the Renewable Energy Potential in Africa: A GIS-based approach,” p. 73, 2014.

[12]	IRENA, “Analysis of Infrastructure for Renewable Power in Eastern and Southern Africa.” 2015. [Online]. Available: https://www.res4africa.org/wp-content/uploads/2016/06/IRENA_Africa_CEC_infrastructure_2015.pdf

[13]	International Energy Agency, “IEA World Energy Statistics and Balances.” OECD. [Online]. Available: https://www-oecd-ilibrary-org.focus.lib.kth.se/energy/data/iea-world-energy-statistics-and-balances_enestats-data-en

[14]	United Nations Department for Economic and Social Affairs Statistics Division, “UNSD — Energy Balances 2016,” 2018. https://unstats.un.org/unsd/energystats/pubs/balance/ (accessed Oct. 25, 2019).

[15]	IRENA, “West African Power Pool: Planning and Prospects for Renewable Energy.” 2013. [Online]. Available: https://www.irena.org/documentdownloads/publications/wapp.pdf

[16]	“IEA-ETSAP | Energy Supply Technologies Data.” https://iea-etsap.org/index.php/energy-technology-data/energy-supply-technologies-data (accessed Jul. 19, 2019).

[17]	McKinsey&Company, “African refineries.” http://www.mckinseyenergyinsights.com/resources/refinery-reference-desk/african-refineries/ (accessed Sep. 01, 2019).

[18]	International Gas Union, “2018 World LNG Report.” 2018. [Online]. Available: https://www.igu.org/sites/default/files/node-document-field_file/IGU_LNG_2018_0.pdf

[19]	Hydrocarbons Technology, “Hydrocarbons Technology - The leading site for news and procurement in the hydrocarbons industry,” Hydrocarbons Technology. https://www.hydrocarbons-technology.com/ (accessed Sep. 01, 2019).

[20]	NEPAD Transforming Africa, “| AUDA-NEPAD.” https://www.nepad.org/ (accessed Sep. 01, 2019).

[21]	IRENA, “Renewable Energy Jobs: Status, Prospects & Policies,” 2011. Accessed: Oct. 10, 2018. [Online]. Available: https://www.irena.org/-/media/Files/IRENA/Agency/Publication/2012/RenewableEnergyJobs.pdf

[22]	Robert Bacon and Masami Kojima, “Issues in estimating the employment generated by energy sector activities,” The World Bank, Sustainable Energy Department, Jun. 2011. [Online]. Available: https://openknowledge.worldbank.org/bitstream/handle/10986/16969/827320WP0emplo00Box379875B00PUBLIC0.pdf?sequence=1&isAllowed=y

[23]	M. Ram, J. C. Osorio-Aravena, A. Aghahosseini, D. Bogdanov, and C. Breyer, “Job creation during a climate compliant global energy transition across the power, heat, transport, and desalination sectors by 2050,” Energy, vol. 238, p. 121690, Jan. 2022, doi: 10.1016/j.energy.2021.121690.

[24]	IRENA, “Global Renewables Outlook: Energy transformation 2050,” Abu Dhabi, 2020. [Online]. Available: https://www.irena.org/-/media/Files/IRENA/Agency/Publication/2020/Apr/IRENA_Global_Renewables_Outlook_2020.pdf

[25]	Rutovitz, J., Dominish, E., and Downes, J., “Calculating global energy sector jobs: 2015 methodology.,” Institute for Sustainable Futures, Prepared for Greenpeace International by the Institute for Sustainable Futures, University of Technology Sydney., 2015. [Online]. Available: https://opus.lib.uts.edu.au/bitstream/10453/43718/1/Rutovitzetal2015Calculatingglobalenergysectorjobsmethodology.pdf

[26]	D. Bogdanov et al., “Low-cost renewable electricity as the key driver of the global energy transition towards sustainability,” Energy, vol. 227, p. 120467, Jul. 2021, doi: 10.1016/j.energy.2021.120467.

[27]	Ram M. et al., “Global Energy System based on 100% Renewable Energy – Power, Heat, Transport and Desalination Sectors.,” Study by Lappeenranta University of Technology and Energy Watch Group, Lappeenranta, Berlin, Mar. 2019. [Online]. Available: ISBN 978-952-335-339-8

[28]	“Solar Power Europe and E&Y, Solar Photovoltaics Jobs & Value Added in Europe,” Brussels, 2015. [Online]. Available: http://www.solarpowereurope.org/fileadmin/user_upload/documents/Media/Jobs_Growth.pdf

[29]	United States Department of Energy, “US.DOE, U.S. Energy and Employment Report,” Washington, DC, 2017. [Online]. Available: https://www.energy.gov/jobstrategycouncil/downloads/us-energy-employment-report-2017-0.

[30]	X. Py, N. Sadiki, R. Olives, V. Goetz, and Q. Falcoz, “Thermal energy storage for CSP (Concentrating Solar Power),” EPJ Web Conf. 148, 2017.

[31]	“ILO, Summary Report: Labour productivity - International Labour Organization modelled estimates,” Geneva, Switzerland, 2016. Accessed: Oct. 27, 2017. [Online]. Available: https://goo.gl/zdsTok

[32]	International Monetary Fund, “IMF, World Economic Outlook - Real GDP growth.,” Washington, DC, 2017. Accessed: Jun. 05, 2017. [Online]. Available: http://www.imf.org/external/pubs/ft/weo/2017/01/weodata/index.aspx