## Calculations

The gaseous concentration of ethyl acetate, CO2 and H2 (CC, gas, mol/L) in the outflow were calculated based on the ideal gas law according to Eq.1 follows:

CC,gas=XC, gasPatmR*T										(Eq. 1)

With C representing the corresponding compound (ethyl acetate, CO2 and H2) XC, gas the volumetric fraction of compound C in the gas (-), Patm the atmospheric pressure (Pa), R the ideal gas constant (m3 Pa/mol/K) and T the temperature (K). The cumulative mass of compound C (mC,gas, mol) stripped up to each time point (tn, h) was calculated using Eq. 2

mC,gastn=CC,gas tn-1 + CC,gas tn2* Fgas,outtn-1 + Fgas,outtn2* ∆t+ mC,gastn-1	(Eq. 2)

Where the average gaseous concentration of the product between time points tn-1 and tn is calculated from Eq. 1 (mol/L), Fgas,out is the total volumetric gas flow rate leaving the reactor (L/h), Δt the time between two time points (h) and mC,gas(tn-1) the amount of compound C stripped up to the previous time point (mol). Fgas,out was calculated assuming N2 as inert gas and knowing the total volumetric gas flow into the reactor (Fgas,in) and the volumetric fractions of N2 in the corresponding in- and outflows (XN2,in, XN2,out) at time point t using Eq.3.

Fgas,out=XN2,inXN2,out* Fgas,in								(Eq. 3)

The cumulative mass of stripped ethyl acetate after Eq. 3 was divided by the culture volume in the bioreactor and added to the current ethyl acetate concentration in the liquid. The resulting value is an apparent ester concentration at time tn which would be found in the culture medium if no stripping was applied .

## Carbon balance calculation

Carbon balances were calculated according to Eq. 5.

Cbalance=C-mol products formedC-mol glucose consumed 						(Eq. 5)

The compounds included in the calculation were glucose, ethyl acetate, ethanol, acetate, succinate, pyruvate, formate and CO2. Biomass formation was included in the calculation assuming a biomass composition of CH2O0.5N0.2 and an experimentally determined conversion factor of 0.3232 from OD600 to g/L dry weight (data not shown).