# Conventional PCR detection of Brucella DNA

Molecular identification of the genus Brucella was done using two sets of primers: B4 forward (5´-TGG CTC GGT TGC CAA TAT CAA-3´) and B5 reverse (5´-CGC GCT TGC CTT TCA GGT CTG-3), as previously reported (21). The PCR reaction mix comprised of: a final concentration of 0.5µM for each of the primer pairs, 5 µl of the DNA template (4-20ng/ µl) and x1 concentration of the PCR mastermix (AccuPower PCR PreMix Bioneer Corp, Republic of Korea) to a final volume of 25 µl.  After the initial denaturation step of 5 minutes at 95°C in a thermo-cycler (Applied Biosystems SimpliAmp Thermo Cycler, Lite Technologies, Singapore), 35 cycles of denaturation at 94°C for 1minute, annealing at 53°C for 30seconds, extension at 72°C for 1 minute, and final extension steps at 72°C for 10 minutes were performed. Amplification of the target region was confirmed based on the presence of specific bands for Brucella (223 bp). The PCR products (3μl) were analysed on a 1.8% agarose gel pre-stained with GelRed nucleic acid stain (Biotium Inc., USA) run at 6.7 v/cm2 for 30 minutes for electrophoresis detection and visualization with a bioanalytical imaging system (Azure Biosystems Inc., USA).

Results

Out of the 20 samples selected for molecular analysis, 4 samples amplified the target region of interest on molecular testing using conventional PCR, giving an anticipated band of 223 base pairs (Figure 1).

Figure 1: An agarose gel image showing a 223bp amplicon of the NTC – negative template control, samples 1-4 against a 1kbplus ladder (ThermoFisher Scientific, USA).

Six out of the 20 samples tested by real-time PCR amplified with both the bcsp31 and the IS711 genus-specific primers used to detect the presence of Brucella DNA.