# Supplementary information

# Dispersion Scan Frequency Resolved Optical Gating For Evaluation of Pulse Chirp Instability: supplementary material

M. Guesmi, P. Veselá, K. Žídek

Regional Center for Special Optics and Optoelectronic Systems (TOPTEC), Institute of Plasma Physics, Czech Academy of Science v.v.i., Za Slovankou 1782/3, 182 00 Prague 8, Czech Republic

* Author to whom correspondence should be addressed. Email:

# Chirp instability origin

By using a rapid scan in the FROG experiment, we were able to attain the second harmonic generation autocorrelation (SHG-AC) by spectral integration of the FROG traces –see Figure S1. Thus, we were able to investigate THE potential dynamic chirp instability of the pulses. The acquisition time of a single SHG-AC trace was 60 ms. The traces were measured in the sweep mode, ie. continuously moving delay line with velocity 1 mm/s, spectrometer integration time 1 ms. To attain a sufficiently high signal, we increased the repetition rate of the laser to 100 kHz.

In all three studied types of pulses: (i) generated from NOPA; (ii) passing through a 4f-pulse shaper without SLM; (iii) passing through a 4f-pulse shaper with an SLM-adjusted phase; the shape of the SHG-AC traces remains constant throughout a high number of scans (500 scans acquired for each measurement) –see Figure S1(c). This behavior suggests that the instability produced by the SLM is not induced by the temperature fluctuations effect, as these fluctuations would lead to prominent trace-to-trace changes of the pulse shapes with a varying ratio between the central peak and the pulse wings, as was reported by Brühl et al. 1.

Figure S1. SHG AC traces measured (a) NOPA pulses (b) with 4f-pulse shaper without any modulation device (c) with 4f-pulse shaper and SLM-adjusted phase. the single SHG-AC trace take 60 ms to be acquired.

By comparing the AC traces of the three different cases, we remark that the traces of the pulses generated directly from NOPA are more stable than those measured under the presence of the 4f-pulse shaper. The pulse shaper deteriorates mainly the stability of the SHG AC trace amplitude. On the contrary, the shape of the pulses in all three measured cases undergoes only minor changes and the difference between the curves is to a big extent caused by the noise level in the traces, which affects their normalization.

To investigate the instability of the pulse shapes, we fit the set of traces by using a Gaussian distribution. Hence, we can extract the pulse duration for each measured case. In Figure S2, we depict the histogram of the pulse widths extracted from 500 traces. Note that the width is given in the scales of data points, which was affected by the delay line velocity during the experiment (1 ms per measurement, 1 mm/s delay line velocity).

Figure S2.  Histogram showing the distribution of the fitted pulse widths for each set of scans (blue columns). The attained distrubution (blue dots) was fitted with a Gaussian function (red line) to extract the pulse width fluctuation (a) without 4f-pulse shaper (b) with 4f-pulse shaper without any modulation device (c) with 4f-pulse shaper with SLM-adjusted phase.

By comparing the three studied cases, we notice that the traces processed with the pulse shaper (PS) feature longer pulse duration compared to those without PS (see Table S1). Note that all measurements presented in Supporting information were carried out after the optimization of the pulse compression. This implies that the instability has to occur either due to extremely rapid dynamic chirp fluctuations (well below 60 ms timescale) or due to the spatial chirp variation (space-time coupling) induced in the pulse-shaper. For the 4f-pulse shaper without any modulation device, this spatial chirp effect is due to a slight misalignment of the pulse shaper, as it was, for instance, reported by Tanabe et al. 2.

Once we included the SLM, the pulse width increased (Table S1).  We ascribe this effect due to the space-time coupling induced in the SLM3,4. This space-time coupling inevitably arises due to the finite input beam size, the focal distance and the geometric factor of the grating in the 4f-pulse shaper. Some frequency components, when dispersed by the grating, straddle in more than one SLM pixel. After the dispersed frequencies are recombined on the grating into one beam, different parts of the beam are affected by different phase modulation. However, our measurements can also explain by a rapidly fluctuating phase modulation in the SLM.

Table S1. The fitted width and the estimated pulse duration values  for the pulses measured without 4f-pulse shaper (NOPA), with 4f-pulse shaper without any modulation device (PS) and then with SLM-adjusted phase (SLM). Note that due to the missing calibration, the pulse duration is only estimated.

|  | Fitted width [arb.u.] | Pulse width estim. [fs] |
| --- | --- | --- |
| NOPA | 2.89 | 22.7 |
| PS | 3.46 | 27.2 |
| SLM | 3.64 | 28.6 |

# Reference

1. E. BRÜHL, T. BUCKUP, AND M. MOTZKUS. Minimization of 1/fn phase noise in liquid crystal masks for reliable femtosecond pulse shaping.  Opt. Express. 25, 19 (2017).

2. T. Tanabe, H. Tanabe, Y. Teramura, and F. Kannari. Spatiotemporal measurements based on spatial spectral interferometry for ultrashort optical pulses shaped by a Fourier pulse shaper.  J. Opt. Soc. Am. B. 19, 11 (2002).

3. B. J. Sussman, R. Lausten, and A. Stolow. Focusing of light following a 4-f pulse shaper: Considerations for quantum control. PHYSICAL REVIEW A. 77, 043416 (2008).

4. F. Frei, A. Galler, and T. Feurer. Space-time coupling in femtosecond pulse shaping and its effects on coherent control. THE JOURNAL OF CHEMICAL PHYSICS. 130, 034302 (2009).