Protocol for Phytic Acid determination

Reagents required for Phytic Acid estimation

- 0.65 M HCl

- Phytate Standard

Phytic Acid Sodium salt -0.1 gm in 10 ml 0.65 M HCl

- Inorganic Phosphorus Standard

KH2PO4- 0.044 gm in 100 ml of 0.65 M HCl

- Chen’s reagent (Pi estimation reagent)

Solution A: Milli Q water

Solution B: 0.02 M Ammonium molybdate

Solution C: 0.57 M Ascorbic Acid

Solution D: 3M H2SO4

* Inorganic phosphorus estimating reagent (Chen’s reagent) made immediately before use consist of 2 parts of Solution A and 1 part of each other solutions

5)   Wade’s reagent (Phytic acid estimation reagent)

- 2.5 gm 5-sulfosalycyclic acid

- 0.25 gm FeCl3.6H2O

- 150 ml deionized water

Mix all 3 Solutions and adjust pH to 3.05 with NaOH solution and make the final volume to 200 ml

This solution can be store for a month in -4OC condition

To use the wades reagent dilute the concentrate in 1:4 i.e., 1 part wades concentrate + 4 parts of deionized water