Supplemental Information

for

Dynamin Action at the Final Stage of Clathrin-Mediated Endocytosis

Xiaodong Cheng1, Kuangcai Chen1, Bin Dong1, Meek Yang1, Seth L. Filbrun1, Yong Myoung1, Teng-Xiang Huang1, Yan Gu2, Gufeng Wang1, *, Ning Fang1, *

1Department of Chemistry, Georgia State University, Atlanta, GA, USA, 30303.

2Present address: The Bristol-Myers Squibb Company, Devens, MA, USA, 01434.

* To whom correspondence should be addressed.

Dr. Ning Fang (Email: nfang@gsu.edu; Phone: 404-413-5513) and Dr. Gufeng Wang (Email: gufengwang2011@gmail.com).

Table of Contents

I. Microscopy Imaging and Data Analysis 						    	S-3

I.1. Calibration of parallax imaging 	                                     			S-3

I.2. Auto-Chasing in Axial Direction							S-4

I.3. Simulation of Defocused Scattering Imaging					S-4

I.4. Uncertainties of Angle Determination						S-6

I.5. 3D Super Localization Precision 							S-6

II. List of Supplementary Figures and Movies						S-8

III. Supplementary Figures		 							S-10

References 											S-24

I. Microscopy Imaging and Data Analysis

## I.1. Calibration of Parallax Microscopy.

In parallax microscopy, two half-plane images are formed and arranged vertically (upper and lower images in Figure 1).  When the target probe is in the focal plane, the separation distance between the two images is measured as y0. The separation distance between the two images (y) changes when the target probe moves along the z-axis.

To establish the relationship between the movement in the z-axis (z) and y, a set of half-plane images were obtained by scanning a nanoparticle probe along the z-direction. The lateral positions of the two half-plane images were superlocalized using non-linear least squares (NLLS) fitting. Practically, the two half-plane scattering images are ellipse-shaped and were fitted by 2D Gaussian function (Figure S6A):

Ix,y=A+B·exp(-((x-x0)22Sx2+(y-y0)22Sy2))                                                    (S1)

where (x0,y0) is the center position, A is the background level, B is the peak intensity at (x0, y0), Sx and Sy are the standard deviations of the Gaussian distribution along the x- and y-axis, respectively.

To generate a calibration curve of y as a function of z, 20 AuNRs immobilized on a glass slide surface with various orientations were scanned along the z-axis from -1000 nm to 1000 nm with 10 nm scanning steps with an objective scanner. The averaged y from the 20 AuNRs were used to establish the calibration curve with z (Figure S1). The center portion of the curve (within the z range of -750 nm to 750 nm) can be satisfactorily fitted with a linear curve, which can be used to recover the z-position in the sample images.

I.2. Auto-Chasing in Axial Direction.

Based on the established calibration curve, an auto-feedback z adjustment system has been implemented with a closed-loop nanometer-precision piezo stage and a tracking program in ImageJ Manger1,2. When the target probe is in focus, the separation distance between the upper and lower images is measured and assigned as the reference (y0). When the automatic feedback loop system is enabled, the axial movement of the probe would contribute to the change in measured y, and the tracking program would respond by moving the objective scanner accordingly so that the probe is back in focus again. The response time of the stage is usually less than 1 ms.  The axial movement of the probe is obtained from the physical output of the objective scanner.

I.3. Simulation of Defocused Scattering Imaging

In dark field microscopy mode, AuNRs were illuminated by monochromatic, unpolarized light through an oil-immersion dark field condenser and the scattering images were collected by a NA 1.0 objective. The anisotropic object can be defined as a multi-oscillation dipole, and the far-field intensity distribution of a dipole is characterized by a spatial position  and orientation angle which can be described as a unit vector  or an azimuth angle  and polar angle , where . The measured scattering spot can be written as3:

(S4)

where . An asterisk represents the adjoint operator. denotes the conjugate transpose of , and  stand for the real and imaginary components of , and is the parameters of the optical setup.  is a symmetric matrix which contains six non-orthogonal templates given by:

(S5)

where

(S6)

with                                                                       (S7)

where the integrals for ,  and  are the vectoral PSF equation, and denotes the system’s aberrations affected by defocusing.

Using a linear combination of these basis templates (Figure S3), theoretical PSFs as a function of dipole orientation were simulated. Compared to the previous methods that extracted a simplified function between orientation (polar angle and azimuth angle) and scattering intensities by omitting higher-order polynomials, our method is advantageous by retaining all polynomials to decrease the uncertainty of angular determination in pattern matching with the experimental images.

I.4. Uncertainties of Angle Determination

The uncertainties of the azimuth and polar angle are dependent on orientation and background noise. Under conventional cell imaging conditions, the signal-to-noise (S/N) of the cellular regions with relatively high background is ~10. Simulated images with artificial noise (S/N = 10; with 10 interval for both polar and azimuth angles; 50 images per position) were generated to evaluate the angular errors. When the polar angle (θ) is between 10-80°, the precision of the azimuth and polar angle for any combination of these two angles are ~2° (Figure S4 and S5). When the polar angle is very small (i.e., an upright nanorod), the image patterns become analogous to each other so that the azimuth angle cannot be recovered (random  being reported). When the polar angle is very close to 90 (i.e., a nanorod lying flat), the azimuth angles become degenerated so that the azimuth angle only be recovered within the range of 0-180, which is expected because the asymmetry in the PSF disappears when the nanorod is placed flat on a horizontal plane.

I.5. 3D Localization Precision

The z-coordinate for the AuNR position is obtained by the movement of the microscope objective when performing the auto-focusing tracking. On account of anti-foreshortening of the images due to the refractive index difference between the glass (n = 1.515) and cell culture medium (n = 1.33), the z-movement in bifocal parallax imaging may be magnified. The magnification effect is corrected by multiplying all z coordinate by a factor of 0.774. In addition, the z rescaling issue for the refractive index mismatch is related to the distance between the sample and the interface. The relationship between the actual focal plane and the apparent focal plane for n1 = 1.515 (glass) and n2 = 1.34 (water) with imaging focal plane range of 0-3 mm has been established5. The fitting curve can be approximated as a straight line with small deviations above 1.5 m.

As discussed above, the x and y coordinates of AuNRs were resolved by Gaussian fitting their intensity distributions (Figure S6A). Even with utmost care in adjusting the position of the wedge prism in the light path, the scattering intensities of the upper and lower channels are not always the same due to uneven splitting of light. To maintain higher accuracy, only the lateral positions of the brighter image is used for tracking the lateral movement.

To fully evaluate the localization precision of the system in 3D, we collected 500 images of one AuNR when auto-focusing system was engaged. A 3D cluster was obtained (Fig. S6B), showing how much the localized position spread out. Gaussian fitting of the histogram distribution of AuNRs positions in x, y, and z was used to determine the localization precisions in 3D, giving x = 4.9 nm (Figure S6C), y = 6.3 nm (Figure S6D) and z = 14.0 nm (Figure S6E). Note that the worse localization precision in y than that in x is caused by the elongated half PSF in y in our imaging system.

II. List of Supplementary Figures and Movies

Figure S1. Calibration curve of y vs. z obtained from 20 randomly oriented AuNRs.

Figure S2. Another example of defocused images of a AuNR with a polar angle of θ = 60º while azimuth angle φ being stepped with 10 intervals.

Figure S3. The six basic dipole emission templates.

Figure S4. Estimated polar angle error for orientation recovery at S/N = 10.

Figure S5. Estimated azimuth angle error for orientation recovery at S/N = 10.

Figure S6. 3D localization precision of a AuNR.

Figure S7. Dynamin and clathrin fluorescence during endocytosis of the example shown in Figure 3.

Figure S8. A complete 3D trajectory, rotation information, and dynamin fluorescence of an endocytosis event.

Figure S9. More examples of complete endocytosis events.

Figure S10. An example of rotational tracking of a AuNR in Stage a.

Figure S11. Histograms of rotation step sizes at different times during endocytosis.

Figure S12. Endocytosis of AuNRs by A549 cells expressing K44A dynamin mutants with deficient GTPase activity.

Figure S13. An example of an abortive scission event.

Movie S1. An example of overlaid dynamin fluorescence and AuNR scattering images during endocytosis. Green: dynamin fluorescence. Red: Scattering image. Fluorescence images were acquired at 0.5 fps and played at real time. Scattering images were acquired at 50 fps and played at 100 fps. The scale bar is 5 mm.

Movie S2. An example of active rotation in Stage a. Movie was acquired at 50 fps and played at 100 fps. The scale bar is 3 mm.

Movie S3. Scattering images showing the static period and the super twists. Left: original defocused images. Right: reconstructed images using recovered azimuth and polar angles. Movies were collected at 50 fps and played at 10 fps.

Movie S3A: Example 1.

Movie S3B: Example 2.

Movie S3C: Example 3.

III. Supplementary Figures

Figure S1. Calibration curve of y vs. z obtained from 20 randomly oriented AuNRs. The AuNRs were immobilized on a glass slide surface with various orientations and scanned along the z-axis from -1000 nm to 1000 nm with 10 nm steps using a high-precision objective scanner.

Figure S2. Another example of the full plane defocused image patterns of a AuNR with a polar angle of 60 at different azimuth angles with 10 intervals. Scale bar is 1 m.

Figure S3. The six basic dipole emission templates used in simulation.

Figure S4. Estimated polar angle errors for orientation recovery at S/N = 10. (A) Estimated polar errors for orientation with various combinations of the azimuth angle and polar angle at S/N = 10. (B) The cross section along the black line in (A) shows the polar angle errors with various polar angles and a fixed azimuth angle of 90°. (C)  The cross section along the red line in (A) represents the polar angle errors with various azimuth angles and a fixed polar angle of 60°.

Figure S5. Estimated azimuth angle errors for orientation recovery at S/N = 10. (A) Estimated azimuth errors for orientation with various combinations of the azimuth angle and polar angle at S/N = 10. (B) The cross section along the black line in (A) shows the azimuth angle errors with various polar angles and a fixed azimuth angle of 90°. (C) The cross section along the red line in (A) represents the azimuth angle errors with various azimuth angles and a fixed polar angle of 60°.

Figure S6. 3D localization precision of a AuNR. (A) Typical upper and lower half-plane dark-field images of a AuNR with 0.02 s integration time are shown on the left. Scale bar is 5 μm. The lateral positions of the AuNR are determined by 2D elliptical Gaussian fitting (right) the intensity profile. (B) Scatter plot of locations of the same AuNR in 500 frames. The x, y positions are determined using 2D elliptical Gaussian fitting of the particle image intensity profile. The z positions are obtained from feedback of the objective scanner when auto-focusing system was engaged. The localization precision is determined as the standard deviation from 1D Gaussian function fitting the histogram distribution of the AuNR locations in x, y, z, giving x = 4.9 nm, y = 6.3 nm and z = 14.0 nm.

Figure S7. Dynamin and clathrin fluorescence during endocytosis of the example shown in Figure 3. (A) Clathrin channel. (B) Dynamin channel. (C) Focused scattering channel for AuNRs. (D) Overlapped images. (E) Time evolution of clathrin and dynamin fluorescence on the entry spot, clathrin and dynamin fluorescence on the vesicle, the xy-, and the z-displacement of the particle from the entry spot during an endocytosis event. The dashed line indicates the time of fission point.

Figure S8. A complete 3D trajectory, rotation information, and dynamin fluorescence of an endocytosis event. (A) The overlay of the time evolution of the cargo’s xy-, z-displacements, rotational azimuth and polar angles, and dynamin fluorescence in the background, respectively. Labels b, c, d, e and f represent various steps during the endocytosis. (B) Expanded time window near the fission point (orange frame) in (A). (C) Cargo’s defocused image patterns showing the “super twist” at Stag d.

Figure S9. More examples of complete endocytosis events. (A) Example 1, (B) Example 2, (C) Example 3 and (D) Example 4.

Figure S10. An example of rotational tracking of a AuNR in Stage a.

Figure S11. Histograms of rotation step sizes at different times during endocytosis. (A) (C) (E) (G): Azimuth steps. (B) (D) (F) (H) Polar steps. (A) and (B) Stag a: active rotation after initial binding to cell membrane. Azimuth step size: 65± 92º/frame. Polar step size: 13± 16º/frame (Mean ± SD, n = 2000). (C) and (D) Stag b: slow random rotations. Azimuth step size: 5± 8º/frame. Polar step size: 3± 6º/frame (Mean ± SD, n = 990). (E) and (F) Stag e: second random rotations after the super twist. Azimuth step size: 10± 13º/frame. Polar step size: 7± 10º/frame (Mean ± SD, n = 900).  (G) and (H) Stag f, random diffusion after scission. Azimuth step size: 21± 32º/frame. Polar step size: 12± 18º/frame (Mean ± SD, n = 1000).

Figure. S12. Endocytosis of AuNRs by A-549 cells expressing K44A dynamin mutants with deficient GTPase activity. (A) A typical example of a failed endocytosis event in an A-549 cell expressing GFP-Dynamin 2-K44A. The overlay of the time evolution of the cargo’s xy-, z-displacements, rotational azimuth and polar angles, and dynamin fluorescence in the background, respectively. (B) AuNRs internalization efficiency. The internalized AuNRs after 2 hours incubation time were counted for A-549 cells expressing dyn2 (K44A) mutant and wild type A-549 cell, respectively. A-549 cells expressing GFP - Dynamin 2 - K44A: 2.0 ± 1.0 per cell; wild type A 549 cells: 11.4 ± 4.4 per cell.

Figure S13. An example of an abortive scission event.

References

1	Schneider, C. A., Rasband, W. S. & Eliceiri, K. W. NIH Image to ImageJ: 25 years of image analysis. Nat Methods 9, 671 (2012).

2	Edelstein, A., Amodaj, N., Hoover, K., Vale, R. & Stuurman, N. Computer control of microscopes using µManager. Curr Protoc Mol Biol 92, 14.20. 11-14.20. 17 (2010).

3	Aguet, F., Geissbühler, S., Märki, I., Lasser, T. & Unser, M. Super-resolution orientation estimation and localization of fluorescent dipoles using 3-D steerable filters. Opt. Express 17, 6829-6848, doi:10.1364/OE.17.006829 (2009).

4	Huang, B., Wang, W., Bates, M. & Zhuang, X. Three-dimensional super-resolution imaging by stochastic optical reconstruction microscopy. Science 319, 810-813 (2008).

5	Huang, B., Jones, S. A., Brandenburg, B. & Zhuang, X. Whole-cell 3D STORM reveals interactions between cellular structures with nanometer-scale resolution. Nat Methods 5, 1047-1052 (2008).