Table 1. Heat transfer and fluid flow parameters.

| Parameter |  | Formula | Equation No. |
| --- | --- | --- | --- |
| Heat flux (q") |  | V×I4DhL | (1) |
| Heat transfer coefficient (h) |  | q"Tw-Tb | (2) |
| Nusselt number (Nu) |  | hDhk | (3) |
| Friction factor (f) |  | ΔPLD(ρv22) | (4) |
| Reynolds Number (Re) |  | 4mπDhμ | (5) |
| Prandtl number (Pr) |  | μCpk | (6) |

Table 2. Ranges and precisions of tools and working fluid properties.

| Instrument/ sensor | Range | Uncertainty |
| --- | --- | --- |
| Type-T thermocouple | 0-300℃ | ±0.1℃ |
| RTD (PT-100) sensor | 0-200℃ | ±0.1℃ |
| Burkert Flow Meter (Type SE32) | 0.3-10 m/s | ±1% |
| Differential Pressure Transmitter (PX154-025DI) | 0- 6.23 kPa | ±0.75% |
| Power supply | 0 - 260 V | 0.33 V |
|  | 0 - 12 A | 0.04 A |
| Thermal conductivity | 0.2-2 W/m. K | 5% |
| Dynamic viscosity | -150 to +1000℃ | 1% |
| Density | 0-3 g/cm3 | 1% |
| Specific heat | 0.01 ºC to 300 ºC/min | 2% |

Table 3. Heat transfer and fluid flow uncertainties [41].

| No. | Parameter | Uncertainty formulas | Uncertainty values |
| --- | --- | --- | --- |
| 1 | Reynolds number, Re | UReRe=Uρρ2+UVV2+Uμμ2 | ±1.73% |
| 2 | Heat flux, q" | Uqq=UVV2+UII2 | ±1.51% |
| 3 | Heat transfer coefficient, h | Uhh=Uqq2+UTw-TbTw-Tb2 | ±1.52% |
| 4 | Nusselt number, Nu | UNuNu=Uhh2+Ukk2 | ±5.23% |
| 5 | Friction factor, f | Uff=U∆p∆p2+Uρρ2+UVV2 | ±1.60% |

Table 4. Experimental and CFD simulations reports using heated pipes with carbon-based nanofluids.

| Ref. | Study type | Base fluid | Nanoparticles |  | Flow | Observations |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Type | Concentration |  |  |
| [12] | Exp. | H2O | R-GO | 0.05 wt.% | Turbulent | The heat transfer coefficient increased up to 63%. |
| [27] | Exp. & CFD. | H2O | GNPs | 0.025-0.1 wt.% | Turbulent | Nusselt number (Nu) increased up to 83% using 0.1wt.% at 12,320 W/m2 |
| [28] | Exp. | H2O | PGGNPs | 0.025-0.1 wt.% | Turbulent | Heat transfer enhanced about 119% using 0.1wt.% relative to DW. |
| [29] | Exp. | H2O | f-GNPs | 0.02–0.1 wt.% | Turbulent | Using 0.1wt.% increased the overall heat transfer coefficient up to 14.89% and 19.68% at Re = 5,000 and 17,500, respectively. |
| [30] | Exp. | H2O | CGNPs | 0.025, 0.075, 0.1 wt.% | Turbulent | Heat transfer showed an increment about 37.54% using 0.1wt.% versus DW at Re = 15,927. |
| [50] | Exp. | H2O | CNT | 0.15 vol.% | Laminar | Nusselt number (Nu) improved by 18% at Re = 2290 versus DW. |
| [52] | Exp. | H2O | COOH-CNTs | 0.1–0.3 wt.% | Laminar & Turbulent | Thermal performance was up to 44% using 0.3wt.%. |
| [53] | Simulation | H2O | CGNPs | 0.025, 0.075, 0.1 wt.% | Turbulent | ±10% was the average deviation between the CFD and experimental data. |
| Current study | Exp. | H2O | CF-GNPs | 0.025-0.1 wt.% | Turbulent | Heat transfer enhanced about 44.4% versus DW. |