Supplementary Information for

Super-low-dose functional and molecular photoacoustic microscopy Yachao Zhanga,1, Jiangbo Chena,1, Jie Zhangc, Jingyi Zhua, Chao Liua, Hongyan Sunc, Lidai Wanga,b,*

a Department of Biomedical Engineering, City University of Hong Kong, 83 Tat Chee Ave, Kowloon, Hong Kong SAR, China.

b City University of Hong Kong Shenzhen Research Institute, 518057 Shenzhen, China.

c Department of Chemistry and COSADAF (Centre of Super-Diamond and Advanced Films), City University of Hong Kong, 83 Tat Chee Avenue, Kowloon, Hong Kong, China

* Correspondence to:  lidawang@cityu.edu.hk

1 These authors contributed equally to this work

This file includes:

Supplementary Figures:

Supplementary Fig. S1 Representative laser systems used in the SLD-PAM system.

Supplementary Fig. S2 Comparison of T-probe and HS-probe.

Supplementary Fig. S3 Comparison of mouse ear vasculature images from the T-PAM and LD-PAM.

Supplementary Fig. S4 The simulated focal spots at the focal plane of 532 nm from LD-PAM.

Supplementary Fig. S5 Resolution measurement from SLD-PAM.

Supplementary Fig. S6 Depth-encoded photoacoustic images acquired by LD-PAM at 64-nJ pulse energy.

Supplementary Fig. S7 Comparison of in vivo imaging sensitivity of LD-PAM and SLD-PAM.

Supplementary Fig. S8 Comparison of the local image sensitivity from LD-PAM and SLD-PAM.

Supplementary Fig. S9 Comparison of the oxygen saturation (sO2) acquired by SLD-PAM under different laser pulse energies.

Supplementary Fig. S10 Comparison of vasculature images of the mouse ear, eye, and cranium-intact brain acquired by LD-PAM and SLD-PAM with super-low pulse energy.

Supplementary Fig. S11 Comparison of the photoacoustic images of oxygen saturation (sO2), total hemoglobin (HbT) concentration, and deoxyhemoglobin (Deoxy-Hb) concentration using the SLD-PAM.

Supplementary Notes:

Supplementary Note 1 Multi-wavelength low-cost laser systems.

Supplementary Note 2 Mathematics implementation of the spectral-spatial filter.

Supplementary Note 3 Oxygen saturation (sO2) computation via spectral unmixing.

Supplementary Note 4 Spatial resolution and imaging depth measurement.

Supplementary Note 5 Maximum permissible exposure calculations for OR-PAM.

Supplementary Movies:

Supplementary Movie 1 Comparison of photoacoustic image of mouse ear under 1-nJ laser dose from LD-PAM and SLD-PAM. The tracked green solid circle records the corresponding A-line signal at different positions.

Supplementary Figures

Supplementary Fig. S1. Representative laser systems used in the SLD-PAM system. a Dual-wavelength of 532 nm and 558 nm laser source for vascular morphology and oxygen saturation (sO2) imaging. b Single wavelength of 620 nm laser source for the exogenous molecular imaging (Evans blue). c Single wavelength of 675 nm laser source for the deoxyhemoglobin (Deoxy-Hb) imaging. BPF, bandpass filter; CP, coupler; DM, dichroic mirror; HWP, half-wave plate; MR, mirror; NDF, neutral density filter; PBS, polarizing beam splitter; PD, photoelectric detector; PM-SMF, polarization-maintaining single-mode fiber; SRS, stimulated Raman scattering.

Supplementary Fig. S2. Comparison of T-probe and HS-probe. Schematic of (a) the T-probe and (b) the HS-probe. c Experimental sensitivity comparison between T-probe (T-PAM) and HS-probe (LD-PAM). CP, coupler; G, gravity; MR, mirror; UST, ultrasound transducer.

Supplementary Fig. S3 Comparison of mouse ear vasculature images from the T-PAM and LD-PAM. a Mouse-ear vasculature image acquired by T-PAM and (b) its corresponding 3D photoacoustic image. c Mouse-ear vasculature image acquired by LD-PAM and (d) its corresponding 3D photoacoustic image. e The close-up of marked regions in (a) and (c). f The ratio of pixels throughput between the below (LD-PAM) and upper row (T-PAM) images in (e). The wavelength used was 532 nm and the scale bars are 200 μm.

Supplementary Fig. S4 The simulated focal spots at the focal plane of 532 nm from LD-PAM. The airy radius is 3.099 μm for the LD-PAM. The result was simulated using the software of Zemax.

Supplementary Fig. S5 Resolution measurement from SLD-PAM. The resolutions were measured with a wavelength of 532 nm.

Supplementary Fig. S6 Depth-encoded photoacoustic images acquired by LD-PAM at 64-nJ pulse energy. a Illustration of the regions of interest. Photoacoustic imaging of (b) the ear, (c) mouse brain with cranium intact, (d) and (e) eye from different scanning perspectives. The wavelength used was 532 nm and the scale bars are 400 μm.

Supplementary Fig. S7 Comparison of in vivo imaging sensitivity of LD-PAM and SLD-PAM. The SSIM under different power acquired by SLD-PAM and LD-PAM in (a) mouse ear and (b) mouse brain. Comparison of photoacoustic images acquired by LD-PAM under high-power and SLD-PAM under low-power in (c) mouse ear and (d) mouse brain. e The ratio between SLD-PAM and LD-PAM within the energy range shadowed in (a) and (b), and laser dose in LD-PAM is 4 times as much as SLD-PAM. The referenced image for all SSIM calculations is acquired by LD-PAM at a laser dose of 64 nJ. The wavelength used was 532 nm and the scale bars are 400 μm.

Supplementary Fig. S8 Comparison of the local image sensitivity from LD-PAM and SLD-PAM. Comparison of photoacoustic images acquired by LD-PAM and SLD-PAM in (a) mouse ear and (b) mouse brain under the same laser dose. The blue solid circles represent the region of blood vessel signals, and the white rectangle shows the background noise. The signal background ratio (SBR, {max⁡(Isignal)mean(Ibg)}SLD-PAM×{mean(Ibg)max⁡(Isignal)}LD-PAM) is defined by the ratio of maximal signal amplitude and mean background noise. c The ratio of SBR between SLD-PAM and LD-PAM in (a) and (b). The wavelength used was 532 nm and the scale bars are 400 μm.

Supplementary Fig. S9 Comparison of the oxygen saturation (sO2) acquired by SLD-PAM under different laser pulse energies. The arrows show the miscalculated blood vessel is corrected with increased energy. The wavelengths for spectrum unmixing were 532 nm and 558 nm. The scale bars are 400 μm.

Supplementary Fig. S10 Comparison of vasculature images of the mouse ear, eye, and cranium-intact brain acquired by LD-PAM and SLD-PAM with super-low pulse energy. LIC, lesser iris circle; MIC, major iris circle; RV, retinal vessel; RIV, radial iris vessel; The wavelength used was 532 nm and the scale bars are 400 μm. The arrows show the improved details.

Supplementary Fig. S11 Comparison of the photoacoustic images of oxygen saturation (sO2), total hemoglobin (HbT) concentration, and deoxyhemoglobin (Deoxy-Hb) concentration using the SLD-PAM. a SLD-PAM imaging of the mouse ear sO2 using spectral unmixing and the HbT concentration. Wavelength: 532 and 558 nm. b SLD-PAM imaging of the mouse ear showing Deoxy-Hb concentration only. Left: raw image. Middle: with spectral-spatial filter. Right: depth-encoded vasculature image. Wavelength: 675 nm. Pulse energy: 32 nJ. The scale bars are 400 μm.

Supplementary Notes

Supplementary Note 1 Multi-wavelength low-cost laser systems.

For the dual-wavelength optical path, the output laser beam from a pump laser (VPFL-G-20, Spectra-Physics) is split into two paths by a polarization beam splitter (PBS, PBS051, Thorlabs, Inc.), and the energy ratio of the two paths is adjusted with a half-wave plate (HWP, GCL-060633, Daheng Optics). One path transmits in free space, and the other one is coupled into a 60-m polarization-maintaining single-mode fiber (PM-SMF, HB450-SC, Fibercore Limited). Via stimulated Raman scattering (SRS), the 532-nm pulse in the 60-m fiber generates a 558-nm pulse with a 300-ns delay. An HWP is placed before the 60-m PM-SMF to adjust the polarization of the 532-nm light to maximize the SRS efficiency. The 532-nm beam is combined with the delayed 558-nm beam using a dichroic mirror (DM, T550lpxr-UF1, Chroma Technology), and the dual-wavelength is coupled into a 2-m PM-SMF (P1-460Y-FC, Thorlabs).

For the 620-nm and 675-nm lasers, the pump laser beam is coupled into a 20-m PM-SMF (HB450-SC, Fibercore Limited), and the HWP is placed before the 20-m PM-SMF to adjust the polarization of the pump laser to adjust the SRS efficiency. Two different optical filters are used to pass the 620 nm (using #67-083 filter, Edmund Optics) and 675 nm (ET675/20m, Chroma) wavelengths. The filtered laser beam is coupled into a 2-m PM-SMF for photoacoustic imaging.

Supplementary Note 2 Mathematics implementation of the spectral-spatial filter.

Vλ1 and Vλ2 represent the volumetric data acquired at different optical wavelengths of λ1 and λ2, respectively. The normalized volumetric data become

Vλ1=Vλ1/max⁡(Vλ1,Vλ2)Vλ2=Vλ2/max⁡(Vλ1,Vλ2)                     (S-1)

Then the wavelength-dependent volumetric data are jointly processed. We firstly selected the referenced sub-volume Sλ1xR from Vλ1. xR is the spatial coordinate of the reference sub-volume  Sλ1xR, which is used to determine a reference sub-volume Sλ2xR from Vλ2. Sub-volumes Sλ1xi and Sλ2xi that are spatially close to and similar to the reference sub-volumes

Sλ1xRht={xiϵXλ1,Sλ1xi-Sλ1xR22L×L×L}<τmatch}Sλ2xRht={xiϵXλ2,Sλ2xi-Sλ2xR22L×L×L}<τmatch}              (S-2)

where X confines the spatial domain of the volumetric data, i represents matched sub-volume index in a sub-volume set, L is the size of the reference sub-volume, and τmatch is the pre-defined threshold value to search similar sub-volumes of Sλ1xi and Sλ2xi.

Then a 4D data is formed by matrix disjoint union of all similar sub-volumes at all optical wavelengths,

Gj4=∐ Sλ1xiSλ2xi= [Sλ1x1,Sλ1x2,⋯,Sλ1xi,⋯,Sλ2x1,Sλ2x2,⋯,Sλ2xi]xiϵSλ1xRht, or Sλ2xRht (S-3)

where j represent a 4D data Gj4, ∐ is disjoint union operation1.

A collaborative filter, including four separable linear transformation T4 (Wavelet toolbox, MATLAB 2019b, MathWorks, USA), a coefficients shrinkage operator γht, and four inverse transformation T4-1, are written as

Gj4=∐ Sλ1xi Sλ2xi=T4-1(γht(T4Gj4))                  (S-4)

where  Gj4 represents filtered Gj4, and  Sλ1xi and  Sλ2xi are the filtered sub-volumes. T4 includes a 3D biorthogonal spline wavelet (bior1.5) on the cubic, and a 1-D Haar wavelet on the fourth dimension. The coefficients shrinkage is operated by threshold truncation.

Finally, Gj4 is re-assigned to their original positions of both volumetric data. Sub-volume data from different 4D groups or within the same group may be overlapped. Weighting factors WxRλi are used to generate new averaged volumetric data as

Vλ1ht= xR(xiWxRλ1Sλ1xi)xR(xiWxRλ1χxi)Vλ2ht= xR(xiWxRλ2Sλ2xi)xR(xiWxRλ1χxi)WxRλ1= 1σλ12NxRhtWxRλ2= 1σλ22NxRht                          (S-5)

where xRϵX, xiϵSλ1xRht or Sλ2xRht. χxi is an indicator (0 or 1) of the interest domain. σλ1 and σλ2 are the standard deviations of the volumetric data and NxRht is the number of non-zero coefficients of the corresponding 4D data after threshold truncation in the transformation domain.

Supplementary Note 3 Oxygen saturation (sO2) computation via spectral unmixing.

Vλ1 and Vλ2 are the volumetric data collected from the same sample. Vλ1ht and Vλ2ht are filtered volumetric data (Supplementary Note 2). P(λ1) and P(λ2) are maximum amplitude projections (MAP) of the Vλ1ht and Vλ2ht. The molar extinction coefficient matrix is written as

ε=εOxy-Hb(λ1)εDeoxy-Hb(λ1)εOxy-Hb(λ2)εDeoxy-Hb(λ2)                  (S-6)

where εOxy-Hb(λi) and εDeoxy-Hb(λi) are the molar extinction coefficients of Oxy-Hb and Deoxy-Hb under different wavelengths (i = 1 or 2). Therefore, the molar concentrations of Oxy-Hb and Deoxy-Hb can be estimated by solving the following equation,

εCOxy-HbCDeoxy-Hb=P(λ1)/ϕλ1P(λ2)/ϕλ2                    (S-7)

where ϕλ1 and ϕλ2 are optical fluence under different wavelengths. The fluence changes are measured by a fast photodiode. The sO2 can be determined from the following equation.

COxy-HbCDeoxy-Hb=(εTε)-1εTP(λ1)/ϕλ1P(λ2)/ϕλ2sO2=COxy-HbCDeoxy-Hb + COxy-Hb               (S-8)

Supplementary Note 4 Spatial resolution and imaging depth measurement.

Before the experiment, we simulated focal spots at the focal plane of 532 nm in LD-PAM (Supplementary Fig. S4). The lateral resolution of our system (Supplementary Fig. S5) was measured by scanning the stainless-steel blade across the sharp edges. We fitted the amplitude of the photoacoustic signal across the sharp edge to obtain edge spread functions (ESF). Then the ESF function was derived to calculate the line spread functions (LSF). Finally, the full width at half maximum (FWHM) of the LSF was calculated for quantizing the lateral resolution. The axial resolution was measured by scanning a 10-μm-diameter tungsten filament. Then the A-line signals were extracted and the FWHM was calculated after a Hilbert transformation.

To measure maximal imaging depth, we obliquely inserted a segment of a tungsten filament with a diameter of 250 μm into a fresh chicken breast tissue. The maximal imaging depth is determined by the 6-dB signal-to-noise ratio (SNR) compared with the background noise.

Supplementary Note 5 Maximum permissible exposure calculations for OR-PAM.

The lateral resolution of OR-PAM is determined by the diffraction limits of optical spot size. We estimate the numerical aperture (NA) of the optical objective as follows2,

RL,OR=0.51×λNA                        (S-9)

where RL,OR is the experimentally measured lateral resolution (3.24 μm, Supplementary Fig. S5), and λ is the optical wavelength.

The maximum permissible exposure (MPE) is determined by several factors, such as spectral range and exposure duration on the tissue surface3. Certain applications, for example, ophthalmic angiography, have more strict requirements4–6. We calculate MPE on the tissue surface as follows.

The NA of the optical objective estimated from the equation (S-9) is 0.08. Because the wavelengths are between 400 nm and 700 nm, and the exposure duration T is less than 10 seconds, the MPE equation is written as7

MPE=1100×CA×T0.25  (mJ/cm2),               (S-10)

where CA is a wavelength correction factor and is unity for visible wavelength range. The calculated exposure duration T is written as,

T=2×D×(0.51×λ)RL,OR×∆×1PRR=N×1PRR                 (S-11)

where D is an assumed depth of the optical focus and is estimated as 250 μm. ∆ is the scanning step size and is 2.5 μm. PRR is the laser pulse repetition rate and is 4000 Hz in experiments.

According to the equation (S-11), we know the exposure duration is 0.0042 seconds, and there are about N(16.75) adjacent laser pulses overlap on the skin surface. Combing (S-10) and (S-11), the MPE for the single wavelength on the overlapped region is 16.71 mJ/cm2, and the maximal permissible single pulse energy is written,

MPEsingle= MPEN×(π×(D×(0.51×λ)RL,OR)2)                (S-12)

According to the equation of (S-12), maximal permissible single pulse energy MPEsingle is 230 nJ.

For sO2 imaging, two wavelengths, 532 nm and 558 nm are used. Therefore, the PRR is doubled. However, the interval between the two wavelengths is only 300 ns and is a biologically negligible delay. The practical exposure duration is nearly kept unchanged, but because of the doubled pulse energy, the MPEsingle reduces to half of its original value. If the energy of 532 nm and 558 nm is the same, the MPEsingle is estimated as 115 nJ.

References

1.	Maggioni, M., Katkovnik, V., Egiazarian, K. & Foi, A. Nonlocal transform-domain filter for volumetric data denoising and reconstruction. IEEE Trans. Image Process. 22, 119–133 (2013).

2.	Yao, J. & Wang, L. V. Photoacoustic microscopy. Laser Photonics Rev. 7, 758–778 (2013).

3.	Moothanchery, M. et al. High-speed simultaneous multiscale photoacoustic microscopy. J. Biomed. Opt. 24, 1 (2019).

4.	Hu, S., Rao, B., Maslov, K. & Wang, L. V. Label-free photoacoustic ophthalmic angiography. Opt. Lett. Vol. 35, Issue 1, pp. 1-3 35, 1–3 (2010).

5.	Zhang, W. et al. High-resolution, in vivo multimodal photoacoustic microscopy, optical coherence tomography, and fluorescence microscopy imaging of rabbit retinal neovascularization. Light Sci. Appl. 2018 71 7, 1–12 (2018).

6.	Jeon, S. et al. In Vivo Photoacoustic Imaging of Anterior Ocular Vasculature: A Random Sample Consensus Approach. Sci. Reports 2017 71 7, 1–9 (2017).

7.	Razansky, D., Estrada, H., Rebling, J., Subochev, P. & Hofmann, U. A. T. Rapid functional optoacoustic micro-angiography in a burst mode. Opt. Lett. Vol. 45, Issue 9, pp. 2522-2525 45, 2522–2525 (2020).