Table 1: Non-exhaustive presentation of the accessible web-tools and platforms for Multi-Hazard Risk Assessment

| Name /Organisation | Software desktop or standalone application /plugin Qgis / Web portal | Import your files/ Download results | Provided Database /case studies and /or data sets | Hazards (e.g. Cyclones Earthquakes Floods Landslides, Multiple) | Display or visualization/cross over/ indicators/pre-calculation/ calculation | Type of approaches | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OpenQuake Platform, Engine and Tools, GEM Foundation | Desktop computer, OpenQuake engine, GEM’s QGIS plugin. | yes/yes | GEM Building Taxonomy Tools | earthquake | calculation | Seismic hazard and risk calculations; web-based platform that offers an interactive environment in which users can access, manipulate, share and add data, and explore models and tools for integrated assessment of earthquake risk. | Silva et al., 2014; Pagani et al., 2014 ; Silva and Horspool, 2019 |
| risk.EFEHR, EFEHR, hosted at the EUCENTRE | Web access to databases +web services: 1) to access the data available and 2) web services to access the data, models and files stored in the various GitLab repositories, making use of the GitLab API | Non yet, use of own data coming soon /yes | Exposure data and models for European residential; fragility and vulnerability models for classes of European buildings;… | earthquake | interactive map viewers, pre-computed results | Physical damage and loss for residential, commercial and industrial buildings (and their occupants) has been calculated by combining stochastic catalogues obtained from the seismic hazard model, with a ground motion model logic tree, a regional model of site response, and with physical vulnerability and exposure models. The calculations are undertaken with the OpenQuake-engine; Provide exposure data and models; fragility and vulnerability models for classes of European buildings; site response data and models at European scale; earthquake scenario models; | Crowley et al., 2021a ; Crowley et al., 2020b ; Crowley et al., 2018; Danciu et al., 2018 |
| PREVIEW Global Risk Data Platform | Web portal | Non/yes | Yes, wide range of partners | all | visualize, download or extract data on past hazardous events, pre-calculation | pre-calculated scenarios | Giuliani and Peduzzi (2011); https://preview.grid.unep.ch |
| Prevention Web, UNDRR PEOPLES framework | Matlab© programming, Download and used like any other software desktop freely accessible and downloadable: Tool-for-Disaster-Resilience-Quantification-of-Communities (github.com). |  | convert the results into shapefiles or other geographical formats, interactively visualize attribute data and shapes of each community system |  | visualized and downloaded results | loss of functionality on the building portfolio due to the earthquake, landslide, and flood hazards, Resilience-Based Risk (RBR) approach, decision-making tools, visualizing the resilience results; Data on past hazardous events, population, capital stock, economic indicators distribution and risk from natural hazards can be visualized and downloaded. Probabilistic hazard and risk models have been developed for earthquake, tropical cyclone wind and storm surge, tsunami and river flooding worldwide, for volcanic ash in the Asia-Pacific region and for agricultural drought in parts of Africa. | PEOPLES framework; https://www.preventionweb.net/ |
| Capra, CEPREDENAC, UN ISDR, IADB, The World Bank, GFDRR | Several desktop software’s for each hazard | Yes/yes | Latin America specific datasets | earthquake, tsunami, volcano | calculation | Modeling and analysis tool of different types of natural hazard; CAPRA-GIS is a geographic information system developed by ERN-AL, which is oriented to probabilistic risk calculations. + tools exposure and vulnerability curves + for natural hazards (earthquake, tsunami, volcano) | https://ecapra.org/ Gill et al. 2009 |
| Disaster Risk Management Knowledge Centre (DRMKC/JRC) | Risk Data Hub software | Non, use of own data coming soon | Many data sources to compose its own datasets | all natural hazards, the technological and man-made hazards | Display/ Map viewer / cross over | Pre-calculation, make accessible research results, access exposure and vulnerability assessments (useful for risk reduction), as well as historical events catalogue (post-event) to analyses empirical vulnerabilities; decision support system that integrates spatial data along with statistical analysis. | https://drmkc.jrc.ec.europa.eu/risk-data-hub#/ Antofie et al., 2018 ; Antofie et al., 2020 |
| INFORM RISK (DRMKC/JRC) | Web portal | Non/yes | Many data sources | Earthquake, Tsunami, Flood, Cyclone wind, Storm surge, Drought | Display/ Map viewer/ assessment is made through hazard-independent indicators | INFORM index score; Fixed thresholds at the level of categories; information on global distribution of countries by INFORM Risk classes, income group and regions followed through the 10-year trend. The same approach is used for all three dimensions of INFORM Risk: hazard & Exposure, Vulnerability and Lack of Coping Capacity | http://www.inform-index.org/ Marin-Ferrer et al., 2017 |
| Italian Risk MAps (IRMA) is a web platform of the Italian Civil Protection Department (DPC), developed by Eucentre | web platform | Non | seismic risk of the Italian territory, Italian territory at municipality scale | earthquake | Calculation of damage levels (from D1 to D5) of the European Macroseismic Scale (EMS) (Grünthal 1998); Damage –to-risk matrix | IRMA uses OpenQuake (GEM) as a calculation engine and, combining the different exposure databases with all the possible sets of fragility curves, the user can produce maps of conditional damage (i.e. the return period is selected) or unconditional damage (i.e. an observation time window is selected) | Borzi et al. 2018; Dolce et al. 2019a; Faravelli et al. 2019 |
| Aye et al., (2015); prototype web-GIS (Geographic Information System) decision support platform | Not accessible: open-source geospatial software and technologies, particularly the Boundless (formerly OpenGeo) framework and its client side software development kit (SDK) | - | datasets | floods and landslides | Visualization of a new measure (e.g., relocation) in the alternative scenario; Decision Making Module /calculation alternative scenario | risk management strategies through an interactive participation approach, decision support tool; selection of alternatives is based on one of the Multi-criteria Evaluation (MCE) methods; from risk scenario to ranking of alternatives; criteria ( social, economic and environmental criteria, with either qualitative or quantitative type) to evaluate the proposed alternatives | Aye et al., (2015) |
| HAZUS-MH (Multi-Hazard) FEMA | Software desktop, HAZUS-MH uses ESRI software to run its calculations and is an extension for ArcGIS Desktop | Yes/yes | Yes for United States. For the Tsunami Module, users must supply their own hazard data for analysis | earthquake, hurricane, tsunami and flood | Calculation / United States data can be viewed interactively online via Online Hazards Maps | Loss estimation methods and data are obtained from referenced sources tailored to fit the framework of the methodology ; loss estimation method with respect to each hazard; Extensive national databases are embedded within Hazus | FEMA (2004) its last methodological update version, HAZUS-MH 4.2 (FEMA 2018) https://github.com/nhrap-hazus |
| RiskScape 1.0 (no longer supported) RiskScape 2.0 (under development) | 1.0 Desktop application 2.0 open source programming languages and complies with Open Geospatial Consortium (OGC) standards that support an engine for spatial analysis of risk | Yes, customize risk models and the data that feeds into these models | User datasets | earthquake, volcanic ashfall, flood, wind, and tsunami | calculation | calculate consequences to people, buildings, infrastructure | https://niwa.co.nz/natural-hazards/research-projects/riskscape-software King, Andrew & Bell, Rob. (2005); Schmidt et al., 2011 |
| InaSAFE | OpenStreetMap and InaSAFE with QGIS plugin. | Yes/yes | Initially the focus of activity was in Indonesia, adopted for use in many other countries - it is not ‘Indonesia specific’ | flood, tsunami, volcano, earthquakes | Calculation, aggregation feature in InaSAFE to summarise the impact summary results for smaller areas such as administration districts. | An Impact Function (often abbreviated to IF) is software code in InaSAFE that implements a particular algorithm to determine the impact of a hazard on the selected exposure. Running an impact function is done when you have prepared all your input data, defined your analysis extent and wish to now see the impact outputs. | InaSAFE project (http://inasafe.org) |
| RAPID-N | Web portal | Non/yes | Yes, several database | hazardous substances, fires, and explosions at critical chemical infrastructures | Visualization, pre-calculation and calculation. | risk assessment module (main) responsible for hazard damage and probability calculation, consequence modeling, natech risk estimation, and risk mapping tasks; Damage classification of HAZUS; Fragility curves are X‐Y plots relating a hazard parameter (e.g. PGA) to the damage probability of a structure for a certain damage state (U.S. FEMA, 1997). | https://rapidn.jrc.ec.europa.eu/, RAPID-N User's Manual (Girgin, 2011) |
| Thinkhazard! GFDRR and WB | Web portal | Non/non | Yes, published hazard data, provided by a range of private, academic and public organizations | River flood Urban flood Coastal flood Earthquake Landslide Tsunami Volcano | Visualization of maps with Intensity threshold value (High, Medium, Low) for each hazard | Concern only hazard and do not calculate damages or risks; presents likelihood of different natural hazards affecting an area (very low, low, medium and high), provides guidance on how to reduce the impact of these hazards, and where to find more information. | Think Hazard; ThinkHazard! Documentation (gfdrr.github.io) S. Fraser et al. 2016 |
| TURNkey platform | Web portal in development, coming soon | Yes/yes | Yes, European databases | earthquake | Calculation of risk scenario and using Bayesian approaches | Include existing tools like SELENA, Bayesian approaches for RDN; multi-sensor-based earthquake information system Earthquake Early Warning (EEW) Rapid Response to Earthquakes (RRE) | https://earthquake-turnkey.eu/ |
| Rapid Analysis and Spatialisation Of Risk (RASOR). | Software desktop / Web portal | Yes /yes | Data sets of the case study areas in the project ( (Haiti, Indonesia, Italy, Netherlands and Greece) | Landslide, flood, earthquake, coastal flooding from storm surges or tsunmis … | calculation | A scenario-driven query system simulates future scenarios based on existing or assumed conditions and compares them with historical scenarios. Characterizing vulnerability into a multi-hazard context, taking into account direct and indirect damage; selection of suitable libraries of stage-damage curves, including the HAZUS-MH database (www.fema.gov/hazus); damage curves included in the RASOR libraries; damage-frequency curve is drawn for the total losses; calculation of flood risk | Rudari et al., 2015 ; Arrighi et al. 2018 |
| CEDIM ( still under development) | Web portal | No/seem no | Initially for Germany, currently datasets developed in the project for others areas (Indonesia, Chile, Japan, US, …) | mainly earthquakes & tsunamis; | Calculation and Event rapport, shaking map | Damaged Buildings, affected population exopsed to MMI, vulnerability maps show the degree of loss resulting from a hazard as a percentage. Risk is expressed in expected monetary losses from a certain disaster type for a certain time period (so far, only regarding dwelling assets). Risk maps show the spatial allocation of risk, i.e. the expected damage or monetary loss for a certain time period. | Müller et al., 2006 https://www.risklayer-explorer.com/ https://www.cedim.kit.edu/ |

Abbreviations:

Center for Coordination of Natural Disaster Prevention in Central America (CEPREDENAC),

United Nations International Strategy for Disaster Reduction (UN ISDR),

Inter-American Development Bank (IADB)

Spanish Fund for Latin America and the Caribbean (SFLAC),

Department of Foreign Affairs and Trade of the Australian Government, and the Global Facility for Disaster Reduction and Recovery (GFDRR).