Appendix A

Summary of the estimated electrical resistivity, layer thickness, curve types and lithological interpretations from the 13 VES Stations

| VES No | Coordinates | Elevation (m) | Layer Resistivity(Ωm) | Thickness(m) | Depth(m) | Probable Lithology | Curve Type | RMS Error% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 6°27'12.67" | 008 | 5,231 | 0.759 | 0.0 | Top Soil | QH | 5.3 |
|  | 4°25'16.78" |  | 781 | 5.070 | 0.759 | Unsaturated-Sand | _ |  |
|  |  |  | 102 | 7.560 | 5.830 | Sand Aquifer | _ |  |
|  |  |  | 5,926 |  | 13.390 | Bitumen Bearing Bed |  |  |
| 2 | 6°27'11.90" | 008 | 8,301 | 0.888 | 0 | Top Soil | QH | 6.0 |
|  | 4°25'16.58" |  | 2,438 | 3.380 | 0.888 | Unsaturated-Sand | _ |  |
|  |  |  | 739 | 5.860 | 4.270 | Contaminated Sand Aquifer | _ |  |
|  |  |  | 62,332 |  | 10.130 | Bitumen Bearing Bed |  |  |
| 3 | 6°27'12.09" | 007 | 6,187 | 1.130 | 0 | Top Soil | KH | 2.6 |
|  | 4°25'14.99" |  | 12,223 | 2.730 | 1.130 | Unsaturated-Sand | _ |  |
|  |  |  | 4,026 | 5.460 | 3.860 | Contaminated Sand Aquifer | _ |  |
|  |  |  | 353,312 |  | 9.330 | Bitumen Bearing Bed |  |  |
| 4 | 6°27'12.97" | 007 | 15,772 | 0.512 | 0 | Top Soil | QH | 4.8 |
|  | 4°25'15.19" |  | 1,268 | 6.150 | 0.512 | Unsaturated-Sand | _ |  |
|  |  |  | 142 | 8.470 | 6.660 | Sand Aquifer | _ |  |
|  |  |  | 5,476 |  | 15.130 | Bitumen Bearing Bed |  |  |
| 5 | 60 27’ 15.1’’ | 006 | 1,167 | 2.230 | 0 | Top Soil | KQH | 1.7 |
|  | 40 25’ 15.0’’ |  | 2,688 | 2.030 | 2.230 | Unsaturated Sand | _ |  |
|  |  |  | 380 | 5.790 | 4.270 | Contaminated Sand Aquifer | _ |  |
|  |  |  | 53 | 11.190 | 10.060 | Clean Sand Aquifer |  |  |
|  |  |  | 402 |  | 21.250 | Clayey-Sand |  |  |
| 6 | 60 27’ 13.1’’ | 006 | 1,720 | 0.377 | 0 | Top Soil | KQH | 1.9 |
|  | 40 25’ 14.6’’ |  | 9,004 | 0.728 | 0.377 | Unsaturated Sand | _ |  |
|  |  |  | 984 | 7.830 | 1.110 | Contaminated Sand Aquifer | _ |  |
|  |  |  | 40 | 6.990 | 8.940 | Clean Sand Aquifer |  |  |
|  |  |  | 263 |  | 15.930 | Clayey-Sand |  |  |
| 7 | 60 27’ 10.9’’ | 007 | 20,524 | 0.899 | 0 | Top Soil | QQH | 2.7 |
|  | 40 25’ 14.2’’ |  | 7,230 | 1.990 | 0.899 | Unsaturated Sand | _ |  |
|  |  |  | 563 | 8.830 | 2.890 | Contaminated Sand Aquifer | _ |  |
|  |  |  | 235 | 10.370 | 11.720 | Sand Aquifer |  |  |
|  |  |  | 16,748 |  | 22.090 | Bitumen Bearing Bed |  |  |
| 8 | 60 27’ 11.0’’ | 007 | 25,501 | 1.440 | 0 | Top Soil | QHK | 1.8 |
|  | 40 25’ 12.6’’ |  | 5,119 | 0.919 | 1.440 | Unsaturated Sand | _ |  |
|  |  |  | 315 | 4.430 | 2.360 | Contaminated Sand Aquifer | _ |  |
|  |  |  | 3,052 | 7.350 | 6.790 | Bitumen Bearing Bed |  |  |
|  |  |  | 547 |  | 14.140 | Clayey-Sand |  |  |
| 9 | 60 27’ 13.4’’ | 007 | 853 | 0.454 | 0 | Top Soil | KQH | 1.9 |
|  | 40 25’ 13.0’’ |  | 2,498 | 0.945 | 0.454 | Unsaturated Sand | _ |  |
|  |  |  | 903 | 6.830 | 1.400 | Contaminated Sand Aquifer | _ |  |
|  |  |  | 54 | 7.740 | 8.220 | Clean Sand Aquifer |  |  |
|  |  |  | 250 |  | 15.960 | Clayey-Sand |  |  |
| 10 | 60 27’ 15.5’’ | 006 | 1,755 | 0.374 | 0 | Top Soil | QQH | 2.9 |
|  | 40 25’ 13.5’’ |  | 1,469 | 5.930 | 0.374 | Unsaturated Sand | _ |  |
|  |  |  | 248 | 9.740 | 6.300 | Contaminated Sand Aquifer | _ |  |
|  |  |  | 49 | 11.990 | 16.040 | Clean Sand Aquifer |  |  |
|  |  |  | 398 |  | 28.030 | Clayey-Sand |  |  |
| 11 | 60 27’ 15.9’’ | 009 | 2,368 | 1.600 | 0 | Top Soil | QH | 1.7 |
|  | 40 25’ 11.8’’ |  | 1,286 | 6.660 | 1.600 | Contaminated Sand Aquifer | _ |  |
|  |  |  | 32 | 6.170 | 8.260 | Clean Sand Aquifer | _ |  |
|  |  |  | 263 |  | 14.420 | Clayey-Sand |  |  |
| 12 | 60 27’ 13.7’’ | 008 | 1,523 | 5.710 | 0 | Top Soil | QH | 1.9 |
|  | 40 25’ 11.4’’ |  | 215 | 8.800 | 5.710 | Contaminated Sand Aquifer | _ |  |
|  |  |  | 40 | 10.370 | 14.500 | Clean Sand Aquifer | _ |  |
|  |  |  | 1,083 |  | 24.870 | Bitumen Bearing Bed |  |  |
| 13 | 60 27’ 11.9’’ | 007 | 1,749 | 0.580 | 0 | Top Soil | KH | 5.8 |
|  | 40 25’ 11.1’’ |  | 4,611 | 1.190 | 0.580 | Unsaturated Sand | _ |  |
|  |  |  | 80 | 4.950 | 1.770 | Clean Sand Aquifer | _ |  |
|  |  |  | 20,769 |  | 6.720 | Bitumen Bearing Bed |  |  |

Appendix B

Inverted vertical electrical sounding curves and lithological interpretations