Supplementary Note 1: Analytic Model for 3D Rotational Falling Fliers

The aerodynamic behavior of a flier of the sort considered in this paper represents a complex structure-fluid interaction problem. As a simplification, we consider the system in a stable flying state, i.e., with a constant terminal velocity vT and a constant rotating speed ωT. These two key parameters that describe the falling behavior of fliers depend strongly on both the properties of the air (e.g., air density ρ, dynamic viscosity μ, etc.) and the geometry of flier (e.g., 3D configuration, tilted angle of blades, area, etc.):

vT,ωT=fair propertiesflier geometry,                                         (S1.1)

Herein, we use a representative simplified model to parametrically characterize the geometry of the flier, which has several blades (number of n) evenly located at a radius r from the center point (Fig. 2a). Each blade is tilted by an angle β, and has a chord length c, in-plane width b and thickness tm. The density of the flier material is ρm. We assume that the blade width b is small comparing to the radius r (b≪r).

Focusing on the side view of a single blade (Fig. 2a, bottom enlarged subplot), the air flow is of velocity U, which can be decomposed into a vertical component vT and a horizontal component ωTr. This problem now corresponds to a 2D airfoil (side cross-section of blade) with a coming air flow of velocity U and attack angle α, where

U=vT2+ωTr2,                                                      (S1.2)

α=90o-β-arctanωTrvT.                                            (S1.3)

The drag force FD along the direction of airflow and lift force FL perpendicular to the direction of airflow can be expressed as

FD=bc⋅CDb⋅12ρU2,                                                  (S1.4)

FL=bc⋅CLb⋅12ρU2,                                                   (S1.5)

where CD(b) and CL(b) are the drag coefficient and lift coefficient of the airfoil, respectively, which can be obtained by 2D CFD simulations of airfoils (Fig. S3). Decomposing the drag force and the lift force into the rotational and vertical directional forces (Fθ and Fz) yields

Fθ/n=FD⋅ωTrU-FL⋅vTU,                                                   (S1.6)

Fz/n=FD⋅vTU+FL⋅ωTrU.                                                   (S1.7)

The force equilibrium condition is given by

Fθ=0,                                                                  (S1.8)

Fz=W,                                                                 (S1.9)

under the assumption b≪r, where W is the weight of flier.

1. Rotating speed

With the horizontal equilibrium condition Fθ=0, Eq. (S1.6) yields

ωTrvT=CDbCLb=LD,                                                        (S1.10)

where LD is the lift-to-drag coefficient of the airfoil, which depends on the airfoil geometry, Reynolds’ number (Re) and the attack angle (α) of the coming air flow (see Supplementary Figure S5b). For a given airfoil geometry under a certain Reynolds’ number, Eq. (S1.10) yields

ωTrvT=LD90°-β-atanωTrvT.                                      (S1.11)

The value of ωTrvT can be solved by this implicit equation (S1.11) for a given tilt angle β. Moreover, this suggests the scaling law

ωT∝vTr,                                                            (S1.12)

that the rotating speed of flier is proportional to its terminal falling velocity over radius.

2. Terminal velocity

Equation (S1.7) yields

Fz=12ρvT2⋅nbc⋅CDb⋅1+LD32.                                     (S1.13)

Rewriting Eq. (S1.13) with the drag coefficient of flier CD yields

Fz=12ρvT2⋅A⋅CD,                                                      (S1.14)

where CD=CD,b⋅1+Lb32, and A=nbc is the total membrane area of flier. The drag coefficient of the flier is affected by both the drag coefficient and lift-to-drag ratio of its blade.

3. Considering the effect of Reynolds’ number on drag coefficient

The drag coefficient of blade (CD(b)) saturates at high Re, but at small Re it is nearly proportional to 1Re. The simulation results of drag coefficient of a flat airfoil versus Reynolds’ number are presented in Fig. S7a. The CFD simulation results can be fitted by the function

CDb≈G0b+G1bRe,                                                   (S1.15)

where (G0b,G1b)=(0.50, 8.3)  for α=30° and (G0b,G1b)=(1.15,10)  for α=60°. Substitution of Eq. (S1.15) into Eq. (S1.13) gives

Fz=12ρvT2⋅nbc⋅G0b+G1bRe⋅1+LD32.                            (S1.16)

We define η as the area fill factor of flier, according to

η=Aπr2=nbcπr2.                                                      (S1.17)

With Eq. (S1.17), the Eq. (S1.16) can be rewritten as

Fz=12ρvT2⋅πr2η⋅G0b+G1bRe⋅1+LD32.                            (S1.18)

when η≪1. Recall that Re=ρv⋅2r/μ, Eq. (S1.18) yields

Fz=12ρvT2⋅πr2η⋅G0                                               (S1.19)

for large Re, where G0=G0b1+LD3/2. Likewisely, for small Re, Eq. (S1.18) yields

Fz=14μvT⋅πrη⋅G1,                                               (S1.20)

where G1=G1b1+LD3/2. Again, Eqs. (S1.19) and (S1.20) requires the assumption that η≪1.

4. Modification of theory by the extension of fill factor η

The above analysis is based on the assumption that the fill factor η is small (η≪1). To extend the model to practical cases where the fill factor can be larger (e.g., close to 1 in the range of [0,1]), we simulated the simplified flier model with various fill factors (Fig. S13). It was shown that the linear relation between Fz and η is true only for η<0.2. For larger η the theory must be amended by changing the fitting law to Fz∝η. Therefore, the amended model is given by

Fz=12ρvT2⋅πr2η⋅CD.                                                (S1.21)

5. Effect of self-weight and load on terminal falling velocity

The weight of flier consists of two parts, i.e., self-weight (Wself) and load (Wload), as given by

W=Wself+Wload=ρmtmgπr2η+Wload.                             (S1.22)

Substitution of Eqs. (S1.9) and (S1.22) into (S1.21) yields

ρmtmgπr2η+Wload=12ρvT2⋅πr2η⋅CD.                                 (S1.23)

For macrofliers, the terminal velocity is given by

vT=2ρG0ρmtmgη+Wloadπr2η12.                                         (S1.24)

For microfliers, the terminal falling velocity is

vT=4μG1ρmtgrη+Wloadπrη.                                           (S1.25)

Supplementary Note 2: Effect of Porosity on Terminal Velocity

The idea of utilizing porosity to reduce the terminal falling velocity arises from structures in nature (Fig. S8a). For example, feathers consist of micro-fibers, between which there are void spaces. The aerodynamic properties of feathers are still very good even with these void spaces.  The benefits of the void spaces are in significant reductions in the overall weight. Another example is dandelion seeds, with similar micro-fiber structures but even more void spaces, with capabilities for exceptionally low terminal velocities. Thus, we explored the possibility of adding porosity into the fliers (Fig. S8b&c) to reduce these velocities. The porosity (i.e., p) is defined by

p=Avoid/A0,                                                        (S2.1)

where Avoid is the area of voids and A0 is the area of a void-free flier. The area with voids present is

A=A0-Avoid=A0(1-p),                                          (S2.2)

We consider a representative 2D flat airfoil, for which the CFD simulations reveal that the porosity leads to an increase in the drag coefficient CDb by increasing G0b and G1b, recall Eq. (S1.2), as shown in Fig. Sd&e, and can be fitted analytically by

G0bpG0bp=0≈1-0.95p21-p,                                              (S2.3)

G1bpG1bp=0≈1-0.3p1-p.                                                 (S2.4)

Eqs. (S2.3) and (S2.4) indicate that G1b increases faster than G0b with porosity. This behavior can be explained by the flow fields near the porous airfoil, as shown in Fig. S9. At low Re, the boundary layer surrounding the airfoil, which can be seen as a virtual airfoil, is not affected by the porosity on the flat airfoil. The drag force acting on the airfoil, is therefore not significantly decreased. However, at large Re, the thickness of boundary layer is comparable to the size of void, such that the drag force decreases similarly to the area as porosity increases.

Therefore, per Eqs. (2) and (3), the knock-down factor of porosity on vT for a microflier is given by

vTpvTp=0=1-p1-0.3p,                                                 (S2.5)

and for a macroflier, the terminal velocity knock-down factor is

vTpvTp=0=1-p1-0.95p2,                                                 (S2.6)

as validated by the falling tests as shown in Fig. S10.

Supplementary Note 3: Effect of Attack Angle on the Drag Coefficient of 2D Flat Airfoil

The tilt angle β affects the terminal velocity through the attack angle (α). A larger β indicates a smaller G0 and G1 (Fig. S7) due to its smaller attack angle (α=90°-β-atanωTξvT, for the point on the flier at radius ξ∈[0,r]), and therefore leads to a larger vT. For a large flier, the lift-to-drag ratio (LD) of the blade can also increase significantly with small attack angle.  Therefore, a small tilt angle can increase the flier drag coefficient through LD, per Eq. (1.13), that CD=CDb⋅1+LD32. However, the lift-to-drag coefficient for a microflier is very small such that only the drag coefficient of blade plays an important role.  Therefore, optimizing the tilt angle is not helpful for microscale fliers.

The 2D (two-dimensional) airfoil is the cross section of a blade for a 3D flier. Figure S7a shows the CFD simulated (see details in Methods and Fig. S3b) drag coefficient of a 2D flat airfoil (i.e., CD(b)) versus Reynolds number, where the subscript ‘(b)’ denotes the ‘blade’. The drag coefficient of the airfoil, as defined by

CDb=FDA⋅12ρv2,                                                        (S3.1)

where A is the area of the airfoil (per unit depth). CDb can be empirically given by

CDb≈G0b+G1bRe,                                                   (S3.2)

where G0b and G1b are the fitting parameters. Notably, Eq. (S1.2) indicates that CDb is dominated by G0b at high Re, but is dominated by G1b at low Re. In other words, G0b and G1b stand for the viscous and inertia effects of the flow, respectively (Fig. S7b). To investigate the effect of attack angle (α) on CDb, we first study the effect on G0b and G1b, as presented in Fig. S7c&d, which can be analytically fitted by

G0b≈0.435sinα+1.02sin2α,                                   (S3.3)

G1b≈7.38+3.50sin2α .                                       (S3.4)

Substitution of Eqs. (S1.3) and (S1.4) into Eq. (S1.2) gives the drag coefficient as a function of α

CDb≈0.435sinα+1.02sin2α+1Re7.38+3.50sin2α.              (S3.5)

Supplementary Note 4: Stability analysis of rotating-falling fliers (spinners)

The stability of the rotating-falling state of the flier is analyzed by simplifying the flier as a rigid body and considering the influence of air flow. Generally, the rotational movement for a rigid body can be described as

ω=ω1e1+ω2e2+ω3e3,                                              (S4.1)

where ω is the rotating velocity of the rigid body, e1, e2 and e3 are the three-dimensional unit vectors of the rotating coordinate along with the rigid body, and ω1, ω2 and ω3 are the rotating speed components. Euler’s rotation equations take the general form:

T1=I1ω1-I2-I3ω2ω3

T2=I2ω2-I3-I1ω3ω1,                                          (S4.2)

T3=I3ω3-I1-I2ω1ω2

where I1, I2 and I3 are assumed to be the principal moments of inertia, and T=T1,T2,T3 is the torque on the body. Consider the flier as a rigid body free falling in the air (air density ρ, dynamic viscosity μ) at a terminal velocity vT, and rotating with an angular speed ωT along the z direction, where the rotational movement is given as

ω=ωTe3.                                                            (S4.3)

If a perturbation is applied on the flier and causes a slight tilt angle on 1 and 2 directions (Fig. 2f) as Λ1 and Λ2, respectively, then the angular velocity becomes

ω=Λ1e1+Λ2e2+ω3e3,                                              (S4.4)

where Λ1 and Λ2 are both assumed to be small. Euler’s equations (S4.2) take the form

T1=I1Λ1-I2-I3Λ2ω3

T2=I2Λ2-I3-I1ω3Λ1.                                          (S4.5)

T3=I3ωT-I1-I2Λ1Λ2

Consider the torque of the air flow on the flier body as

T=T(Λ,Λ).                                                            (S4.6)

For small perturbations off the balanced state as

δT=∂T∂ΛδΛ+∂T∂ΛδΛ.                                                      (S4.7)

The dependence of torque T on angle perturbation δΛ is given by

∂T∂ΛδΛ=-Wd⋅δΛ,                                                      (S4.8)

where d is the distance between the center of pressure and center of gravity. The dependence of torque on angular speed perturbation is

∂T∂ΛδΛ=-SdfDdvδv⋅ξdS,                                                (S4.9)

in which dS is the unit area of the flier locates at radius ξ, and fD is the drag force per unit area acting on the flier as given by

fDv=12ρv2⋅1η⋅G0+G1Re,                                              (S4.10)

The velocity perturbation δv is

δv=ξrδΛ.                                                          (S4.11)

Substitution of Eq. (S4.10) and (S4.11) into equation (S4.9) gives

∂T∂ΛδΛ=-Sddv12ρv2⋅1η⋅G0+G1ReξrδΛ⋅ξdS.                              (S4.12)

Under the assumption that the area distribution of microflier is uniform at radius r, Eq. (S4.12) yields

∂T∂ΛδΛ=-π8μr2ηRe2G0+G1ReδΛ ,                                      (S4.13)

Substitution of equations (S4.8) and (S4.13) into (S4.5) and (S4.7), yields the control equations

I1Λ1-I2-I3Λ2ωT+π8μr2η⋅Re2G0+G1ReΛ1+WdΛ1=0,              (S4.14)

I2Λ2-I3-I1Λ1ωT+π8μr2η⋅Re2G0+G1ReΛ2+WdΛ2=0,              (S4.15)

Rewriting them in matrix form and normalized as

Λ1Λ2+β0ω1-ω2aβ0Λ1Λ2+γ00aγΛ1Λ2=0,                           (S4.16)

where β0=π8μr2η⋅Re2G0+G1Re1I1, ω1=I3-I2I1ωt, ω2=I3-I1I2ωT, a=I1I2 and γ=WdI1. For the extreme case that the flier is highly symmetric and a=1, Eq. (S4.16) becomes

Λ1Λ2+β0ω0-ω0β0Λ1Λ2+γ00γΛ1Λ2=0,                           (S4.17)

where ω0=ω1=ω2. Equation (S3.17) can by diagonalized by the transformation

α1α2=i-i11Λ1Λ2,                                                     (S4.18)

that

α1α2+β0-ω0i00β0+ω0iα1α2+γ00γα1α2=0.                      (S4.19)

This is a decoupled oscillation equation with damping, of which the 4 eigenvalues are

λ1,2=-12β0-ω0i±β0-ω0i22-γ ,                                     (S4.20)

λ3,4=-12β0+ω0i±β0+ω0i22-γ ,                                     (S4.21)

which can be normalized as

λ1,2=-β021-ω0β0i±1-ω0β0i2-4γβ02 ,                               (S4.22)

λ3,4=-β021+ω0β0i±1+ω0β0i2-4γβ02 .                               (S4.23)

The stability of the solution (i.e., Λ→0 when t→+∞) requires that all real part of the eigenvalues are negative. Define Γ as the stability factor as

Γ≡min-2Realλ1,2,3,4β0=minReal1±ω0β0i±1±ω0β0i2-4γβ02 .     (S4.24)

The stability condition requires Γ>0.

The solution of Eq. (S4.17) is expressed as

Λ(t)=i=14Cieλit.                                               (S4.25)

The amplitude of oscillation (Λamp) decays with time as

Λamp∝Λ0exp-β02Γt,                                               (S4.26)

where Λ0 is the amplitude of the initial perturbation. Therefore, a larger Γ means the amplitude decays faster that the flier can recover to its balanced stable state faster.