MAGNETIC RESONNANCE IMAGING METHODS

Specific information on image acquisition in the different samples

T1 weighted structural scans were acquired at Siemens, Philips and GE scanners at the various sites.

| Sample | Scanner | Field strength (Tesla) | Sequence parameters |
| --- | --- | --- | --- |
| BASE-II | Tim Trio Siemens | 3.0 | TR: 2500 ms, TE: 4.77 ms, TI: 1100 ms, flip angle: 7°, slice thickness: 1.0 mm, FoV 256×256 mm, 176 slices |
| Betula | Discovery GE | 3.0 | TR: 8.19 ms, TE: 3.2 ms, TI: 450 ms, flip angle: 12°, slice thickness: 1 mm, FOV 250×250 mm, 180 slices |
| Cam-CAN | Tim Trio Siemens | 3.0 | TR: 2250 ms, TE: 2.98 ms, TI: 900 ms, flip angle: 9°, slice thickness 1 mm, FOV 256×240 mm, 192 slices |
| LCBC | Avanto Siemens | 1.5 | TR: 2400 ms, TE: 3.61 ms, TI: 1000 ms, flip angle: 8°, slice thickness: 1.2 mm, FoV: 240×240 m, 160 slices, iPat = 2 |
|  | Avanto Siemens | 1.5 | TR: 2400 ms, TE = 3.79 ms, TI = 1000 ms, flip angle = 8, slice thickness: 1.2 mm, FoV: 240 x 240 mm, 160 slices |
|  | Skyra Siemens | 3.0 | TR: 2300 ms, TE: 2.98 ms, TI: 850 ms, flip angle: 8°, slice thickness: 1 mm, FoV: 256×256 mm, 176 slices |
|  | Prisma Siemens | 3.0 | TR: 2400 ms, TE: 2.22 ms, TI: 1000 ms, flip angle: 8°, slice thickness: 0.8 mm, FoV: 240×256 mm, 208 slices, iPat = 2 |
| UB | Tim Trio Siemens | 3.0 | TR: 2300 ms, TE: 2.98, TI: 900 ms, slice thickness 1 mm, flip angle: 9°, FoV 256×256 mm, 240 slices |
| WH-II | Verio Siemens | 3.0 | TR: 2530 ms, TE: 1.79/3.65/5.51/7.37 ms, TI: 1380 ms, flip angle: 7°, slice thickness: 1.0 mm, FOV: 256×256 mm |
| HCP | Connectome Skyra Siemens* | 3.0 | TR: 2400 ms, TE: 2.14 ms, TI: 1000 ms, flip angle: 8°, slice thickness: 0.7 mm, FOV: 224 mm, 256 slices, GRAPPA = 2 |
| UKB | Skyra Siemens | 3.0 | TR: 2000 ms, TI: 880 ms, slice thickness: 1 mm, FoV: 208×256 mm, 256 slices, iPAT=2 |

Supplementary Table MR acquisition parameters

TR: Repetition time, TE: Echo time, TI: Inversion time, FoV: Field of View, iPat: in-plane acceleration, GRAPPA: GRAPPA acceleration factor. *Customized

Lifebrain MRI scanning and processing

MRI data originated from 11 different scanners (table above), and were processed with FreeSurfer (https://surfer.nmr.mgh.harvard.edu/) (1-4). To avoid introducing possible site-specific biases, quality control measures were imposed and no manual editing was done. We previously have reported that across-scanner consistencies in estimated hippocampal volumes for the scanners used in Lifebrain are high (5).

HCP MRI scanning and processing

Imaging data were collected and processed by the human connectome project (https://www.humanconnectome.org/study/hcp-young-adult) as described in (6). Imaging data were collected a customized Siemens 3T “Connectome Skyra” housed at Washington University in St. Louis, using a standard 32-channel Siemens receive head coil and a “body” transmission coil designed by Siemens specifically for the smaller space available using the special gradients of the WU-Minn and MGH-UCLA Connectome scanners. Anatomical T1-weighted magnetization-prepared rapid gradient echo (MPRAGE) images were obtained in the sagittal plane at 0.7mm isotropic resolution, and T2 weighted FLAIR images were acquired at identical resolution in the sagittal plane. Images were processed using a custom combination of tools from FSL and FreeSurfer (7-9).

UKB MRI scanning and processing

Imaging data were collected and processed by the UK Biobank (https://www.ukbiobank.ac.uk) as described in (10), using the FreeSurfer 6.0 software package. Imaging data were collected using 3.0 T Siemens Skyra (32-channel head coil). Anatomical T1-weighted magnetization-prepared rapid gradient echo (MPRAGE) images were obtained in the sagittal plane at 1mm isotropic resolution, and T2 weighted FLAIR images were acquired at 1.05x1x1mm resolution in the sagittal plane.

References

1.	A. M. Dale, B. Fischl, M. I. Sereno, Cortical surface-based analysis. I. Segmentation and surface reconstruction. Neuroimage 9, 179-194 (1999).

2.	B. Fischl et al., Whole brain segmentation: automated labeling of neuroanatomical structures in the human brain. Neuron 33, 341-355 (2002).

3.	M. Reuter, N. J. Schmansky, H. D. Rosas, B. Fischl, Within-subject template estimation for unbiased longitudinal image analysis. Neuroimage 61, 1402-1418 (2012).

4.	J. Jovicich et al., Brain morphometry reproducibility in multi-center 3T MRI studies: a comparison of cross-sectional and longitudinal segmentations. Neuroimage 83, 472-484 (2013).

5.	A. M. Fjell et al., Continuity and Discontinuity in Human Cortical Development and Change From Embryonic Stages to Old Age. Cereb Cortex 29, 3879-3890 (2019).

6.	M. F. Glasser et al., The minimal preprocessing pipelines for the Human Connectome Project. Neuroimage 80, 105-124 (2013).

7.	B. Fischl, FreeSurfer. Neuroimage 62, 774-781 (2012).

8.	M. Jenkinson, P. Bannister, M. Brady, S. Smith, Improved optimization for the robust and accurate linear registration and motion correction of brain images. Neuroimage 17, 825-841 (2002).

9.	M. Jenkinson, C. F. Beckmann, T. E. Behrens, M. W. Woolrich, S. M. Smith, Fsl. Neuroimage 62, 782-790 (2012).

10.	F. Alfaro-Almagro et al., Image processing and Quality Control for the first 10,000 brain imaging datasets from UK Biobank. Neuroimage 166, 400-424 (2018).