# Appendix

Calculation tables.

|  |  |  |  |  |  | Internal Combustion Engine vehicles |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII | XIII |
| Veh. class | Subclass | Number registered in Scotland | Chassis type (ICE) | Weight range |  | Reference weight (ICE) | Nr. of axles per vehicle | Ref. weight / axle | Average annual km, per vehicle | Standard 80kN axles per axle (ICE) | RWP | RWIF |
| Data source or method: | (Scottish Government 2020b) | (Scottish Government 2020b) | (UK Government 2010) | (Scottish Government 2020b) |  | Calculated (VI - V)/2 | (UK Government 2010) | Calc-ulated VII/VIII | (UK Department of Transport 2019) | Calculated (VII/80)^4 | Calculated XI x VIII | Calculated XII x X x III |
|  |  | nr | % | from kg | to kg | kg | nr. | kN | km | nr. | Std. axles / vehicle | million std. axle – km / year |
| Buses & Coaches |  | 14,154 | Two axle rigid | 14,000 | 24,800 | 17000 | 2 | 83 | 45,471 | 1.18030 | 2.36 | 1,519 |
| Cars | Petrol | 1,544,505 |  | 1,000 | 2,600 | 1,800 | 2 | 9 | 14,571 | 0.00015 | 0.000297 | 7 |
| Cars | Diesel | 973,215 |  | 1,000 | 2,600 | 1,800 | 2 | 9 | 14,571 | 0.00015 | 0.000297 | 4 |
| Motor-cycles |  | 71,666 |  |  |  | 200 | 2 | 1 | 4,490 | negligible | negligible | <0.1 |
| HGV | 3.5t - 7.5t | 7,986 | Two axle rigid | 3,500 | 7,500 | 5,500 | 2 | 27 | 72,127 | 0.01293 | 0.0259 | 15 |
| HGV | 7.5t - 12t | 1,275 | Two axle rigid | 7,500 | 12,000 | 9,750 | 2 | 48 | 72,127 | 0.12771 | 0.255 | 23 |
| HGV | 12t - 16t | 1,238 | Two axle rigid | 12,000 | 16,000 | 14,000 | 2 | 69 | 72,127 | 0.54289 | 1.09 | 97 |
| HGV | 16t - 20t | 4,656 | Three axle rigid | 16,000 | 20,000 | 18,000 | 3 | 59 | 17,259 | 0.29304 | 0.879 | 71 |
| HGV | 20t - 24t | 678 | Three axle rigid | 20,000 | 24,000 | 22,000 | 3 | 72 | 17,259 | 0.65392 | 1.96 | 23 |
| HGV | 24t - 28t* | 5,367 | Three axle rigid | 24,000 | 27,000 | 25,500 | 3 | 83 | 17,259 | 1.18030 | 3.54 | 328 |
| HGV | 28t - 32t | 4,446 | Four axle rigid | 28,000 | 32,000 | 30,000 | 4 | 74 | 45,693 | 0.71542 | 2.86 | 581 |
| HGV | 32t - 38t | 637 | Four axle articulated | 32,000 | 38,000 | 35,000 | 4 | 86 | 173,886 | 1.32540 | 5.30 | 587 |
| HGV | 38t - 40t | 3,958 | Five axle articulated | 38,000 | 40,000 | 39,000 | 5 | 77 | 125,999 | 0.83694 | 4.18 | 2,087 |
| HGV | 40t - 44t | 6,596 | Six axle articulated | 40,000 | 44,000 | 42,000 | 6 | 69 | 125,999 | 0.54289 | 3.26 | 2,707 |
| LGV | Petrol | 9,842 |  | 2,500 | 3,500 | 3000 | 2 | 15 | 26,262 | 0.00114 | 0.00229 | 592 |
| LGV | Diesel | 296,495 |  | 2,500 | 3,500 | 3000 | 2 | 15 | 26,262 | 0.00114 | 0.00229 | 18 |
|  |  |  |  |  |  | Total Standard axle -km per year, as Road Wear Impact Factor |  |  |  |  |  | 8,068 |
| All HGVs |  | 36,837 |  |  |  |  |  |  |  |  |  | 6,520 |
| All Cars |  | 2,524,000 |  |  |  |  |  |  |  |  |  | 11 |
| All LGVs |  | 308,000 |  |  |  |  |  |  |  |  |  | 18 |

Table A1	Calculation of the Road Wear Impact Factor for the current scenario of an Internal Combustion Engine based national vehicle fleet.  Where ‘Calculated’ is shown at Data Source or Method, Roman numerals in the formula refer to the preceding columns as numbered in the top row.    * Max weight set at 27,000 kg, the most allowable for a 3 axle vehicle with ZEV powertrain.

| Battery Electric and Fuel Cell Electric Vehicles |  | Battery drivetrain weight formula |  |  |  |  |  |  | Fuel cell drive train weight formula |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BEV weight = 1.0744 x ICE weight + 430.17 |  |  |  |  |  |  | HFCEV weight = 1.014 x ICE weight |  |  |  |  |  |  |
| I | II | XIII | XIV | XV | XVI | XVII | XVIII | XIX | XX | XXI | XXII | XXIII | XXIV | XXV | XXIV |
| Vehicle class | Sub-class | Reference weight | Nr. of axles per vehicle | Ref. weight / axle | Average annual km, per vehicle | Standard 80kN axles per axle | RWP | RWIF | Reference weight | Nr. of axles per vehicle | Ref. weight / axle | Average annual km, per vehicle | Standard 80kN axles per axle | RWP | RWIF |
| Data source / method: | (Scottish Government 2020b) | From formula above | (UK Government 2010) | Calc: XIII/XIV | (UK Department of Transport 2019) | Calc. (XV/80)^4 | Calc. XVII x XIV | Calc. XVIII x XVI x III | From formula above | (UK Government 2010) | Calc: XX/XIX | (UK Department of Transport 2019) | Calc: (XXII/80)^4 | Calc. XXIV x XXI | Calc: XXV x XXIII x III |
|  |  | kg | nr. | kN | km | nr. | Std. axles / vehicle | million std. axle – km / year | kg | nr. | kN | km | nr. | Std. axles / vehicle | million std. axle – km / year |
| Buses & Coaches |  | 18695 | 2 | 92 | 45,471 | 1.726 | 3.45 | 2,222 | 17238 | 2 | 85 | 45,471 | 1.248 | 2.50 | 1,606 |
| Cars | Petrol | 2364 | 2 | 12 | 14,571 | 0.0004 | 0.0008 | 20 | 1825 | 2 | 9 | 14,571 | 0.00016 | 0.0003 | 7 |
| Cars | Diesel | 2364 | 2 | 12 | 14,571 | 0.0004 | 0.0008 | 13 | 1825 | 2 | 9 | 14,571 | 0.00016 | 0.0003 | 4 |
| Motor-cycles |  | 645 | 2 | 3 | 4,490 | neg. | neg. | <0.1 | 203 | 2 | 1 | 4,490 | neg. | neg. | <0.1 |
| HGV | 3.5t - 7.5t | 6339 | 2 | 31 | 72,127 | 0.023 | 0.0456 | 26 | 5577 | 2 | 27 | 72,127 | 0.014 | 0.0273 | 16 |
| HGV | 7.5t - 12t | 10906 | 2 | 53 | 72,127 | 0.200 | 0.400 | 37 | 9887 | 2 | 48 | 72,127 | 0.135 | 0.270 | 25 |
| HGV | 12t - 16t | 15472 | 2 | 76 | 72,127 | 0.810 | 1.62 | 145 | 14196 | 2 | 70 | 72,127 | 0.574 | 1.15 | 102 |
| HGV | 16t - 20t | 19769 | 3 | 65 | 17,259 | 0.426 | 1.279 | 103 | 18252 | 3 | 60 | 17,259 | 0.310 | 0.929 | 75 |
| HGV | 20t - 24t | 24067 | 3 | 79 | 17,259 | 0.937 | 2.81 | 33 | 22308 | 3 | 73 | 17,259 | 0.691 | 2.07 | 24 |
| HGV | 24t - 28t* | 27000 | 3 | 88 | 17,788 | 1.48 | 4.45 | 425 | 25857 | 3 | 85 | 17,259 | 1.24 | 3.74 | 347 |
| HGV | 28t - 32t | 32000 | 4 | 78 | 46,638 | 0.926 | 3.70 | 768 | 30420 | 4 | 75 | 45,693 | 0.756 | 3.03 | 615 |
| HGV | 32t - 38t | 38000 | 4 | 93 | 174,042 | 1.84 | 7.37 | 817 | 35490 | 4 | 87 | 173,886 | 1.40 | 5.60 | 621 |
| HGV | 38t - 40t | 40000 | 5 | 78 | 133,344 | 0.926 | 4.63 | 2,444 | 39546 | 5 | 78 | 125,999 | 0.885 | 4.42 | 2,206 |
| HGV | 40t - 44t | 44000 | 6 | 72 | 130,452 | 0.654 | 3.92 | 3,376 | 42588 | 6 | 70 | 125,999 | 0.57393 | 3.44 | 2,862 |
| LGV | Petrol | 3500 | 2 | 17 | 27,413 | 0.002 | 0.004 | 1.1 | 3042 | 2 | 15 | 26,262 | 0.00121 | 0.0024 | 0.6 |
| LGV | Diesel | 3500 | 2 | 17 | 27,413 | 0.002 | 0.004 | 34 | 3042 | 2 | 15 | 26,262 | 0.00121 | 0.0024 | 19 |
|  |  | Total Standard axle -km per year, as Road Wear Impact Factor |  |  |  |  |  | 10,463 |  |  |  |  |  |  | 8,530 |
| All HGVs |  |  |  |  |  |  |  | 8,173 |  |  |  |  |  |  | 6,893 |
| All Cars |  |  |  |  |  |  |  | 32 |  |  |  |  |  |  | 12 |
| All LGVs |  |  |  |  |  |  |  | 36 |  |  |  |  |  |  | 19 |

Table A2	Calculation of the Road Wear Impact Factor for the all BEV and all HFCEV scenarios.  Where ‘Calculated’ (or ‘calc’) is shown at Data Source or Method, the formula is given, with Roman numerals in the formula refer to the preceding columns as numbered in the top row.