# On the cost competitiveness of blue and green hydrogen

Falko Ueckerdt, Philipp C. Verpoort, Rahul Anantharaman, Christian Bauer, Fiona Beck, Thomas Longden, Simon Roussanaly

## Supplementary information

### Intersection of FSCPs

In Fig. 3, it becomes apparent that the three FSCPs (Fossil→Blue H₂, Fossil→Green H₂, Blue H₂→Green H₂) intersect in one single point. This may be perceived as a coincidence at first, however this is in fact a fundamental requirement of the interrelationship of the FSCPs. We provide a brief mathematical proof of this assertion.

Let X≠Y ϵ {b,g,f} be two of the three fuels (blue H₂, green H₂, fossil), and let

FSCPY→X= costX - costYghgiY - ghgiX

be the FSCP for switching from fuel Y to fuel X, which depends on the cost and GHG intensity of the respective fuels. We can use this to write

costg=costf+FSCPf→g *(ghgif - ghgig) ,

costb=costf+FSCPf→b *(ghgif - ghgib) ,

and hence

costg-costb=FSCPf→b *ghgib - FSCPf→g *ghgig + (FSCPf→g-FSCPf→b )*ghgif ,

and substitute this into the definition of FSCPb→g, which yields

FSCPb→g=FSCPf→b *ghgib - FSCPf→g *ghgig + (FSCPf→g-FSCPf→b )*ghgifghgib - ghgig

=FSCPf→b *ghgib - FSCPf→b *ghgigghgib - ghgig+FSCPf→b *ghgig - FSCPf→g *ghgig + (FSCPf→g-FSCPf→b )*ghgifghgib - ghgig

=FSCPf→b+(FSCPf→g-FSCPf→b )*ghgif-ghgigghgib-ghgig

This equation reveals that when FSCPf→b and FSCPf→g coincide, this results in an equality of all three FSCPs:

FSCPb→g=FSCPf→b=FSCPf→b

Moreover, assuming that ghgig<ghgib<ghgif, we find that for FSCPf→b≠FSCPf→b it is either

FSCPb→g>FSCPf→b>FSCPf→b

or

FSCPb→g<FSCPf→b<FSCPf→b.

The case of ghgib<ghgig<ghgif is readily obtained from the above by exchanging b and g.

### Input data

This is a summary of all input data used.

| Name | Unit | 2025 | 2050 | Comment/Source |
| --- | --- | --- | --- | --- |
| Natural gas price (per LHV) | EUR/GJ | 6.0 +- 0.2 | 6.0 + 0.5 - 2.0 | 6 €/GJ = 21.6 €/MWh, as used in the IEA GHG report. Reflects European wholesale market prices before the current energy crisis. |
| Base Emissions of NG (per LHV; suppy-chain and combustion) direct gwp100 | gCO2eq/MJ | 56.0 +- 0.28 | 56.0 +- 0.28 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection 'Life-cycle emissions of blue hydrogen' in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base Emissions of NG (per LHV; suppy-chain and combustion) direct gwp20 | gCO2eq/MJ | 56.0 +- 0.28 | 56.0 +- 0.28 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base Emissions of NG (per LHV; suppy-chain and combustion) elec gwp100 | gCO2eq/MJ | 0 | 0 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base Emissions of NG (per LHV; suppy-chain and combustion) elec gwp20 | gCO2eq/MJ | 0 | 0 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base Emissions of NG (per LHV; suppy-chain and combustion) scco2 gwp100 | gCO2eq/MJ | 8.3 +- 0.4 | 4.2 +- 3.1 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base Emissions of NG (per LHV; suppy-chain and combustion) scco2 gwp20 | gCO2eq/MJ | 9.3 +- 0.5 | 4.7 +- 3.5 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base Emissions of NG (per LHV; suppy-chain and combustion) cts gwp100 | gCO2eq/MJ | 0 | 0 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base Emissions of NG (per LHV; suppy-chain and combustion) cts gwp20 | gCO2eq/MJ | 0 | 0 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base Emissions of NG (per LHV; suppy-chain and combustion) other gwp100 | gCO2eq/MJ | 0.7 +- 0.04 | 0.7 +- 0.04 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base Emissions of NG (per LHV; suppy-chain and combustion) other gwp20 | gCO2eq/MJ | 0.7 +- 0.04 | 0.7 +- 0.04 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Amount of methane leakage in production chain (relative to total amount used) | percent | 1 | 0.1 | Based on today's range of leakage rates and the ambition of the oil and gas industry to lower it significantly along with substantial uncertainty on achieving this reduction. |
| GHG emissions due to methane leakage per CH4 emission rate (per LHV) gwp100 | gCO2eq/MJ | 606.06 | 606.06 | see LCA data methods |
| GHG emissions due to methane leakage per CH4 emission rate (per LHV) gwp20 | gCO2eq/MJ | 1727.273 | 1727.273 | see LCA data methods |
| Blue CAPEX smr-ccs-56% | EUR/kW | 673.0 +- 67.3 | 593.7 +- 59.4 | SMR data from IEAGHG report, Sec. 8.1.3, Table 2, Sheet 84. Assuming a 0.5% yearly learning rate, i.e. 0.995**25 = 88% of cost in 2050. ATR data provided by Sintef (see methods). Assuming uncertainty of 10%. |
| Blue CAPEX atr-ccs-93% | EUR/kW | 1200.0 +- 120.0 | 700.0 +- 70.0 | SMR data from IEAGHG report, Sec. 8.1.3, Table 2, Sheet 84. Assuming a 0.5% yearly learning rate, i.e. 0.995**25 = 88% of cost in 2050. ATR data provided by Sintef (see methods). Assuming uncertainty of 10%. |
| Fixed Operation and Maintenance Cost smr-ccs-56% | EUR/kW | 29.0 +- 2.9 | 29.0 +- 2.9 | SMR data from IEAGHG report, p. 18, Table 4. ATR data provided by Sintef (see methods). Assuming uncertainty of 10%. |
| Fixed Operation and Maintenance Cost atr-ccs-93% | EUR/kW | 32.0 +- 3.2 | 21.0 +- 2.1 | SMR data from IEAGHG report, p. 18, Table 4. ATR data provided by Sintef (see methods). Assuming uncertainty of 10%. |
| Variable Operation and Maintenance Cost (w/o fuel cost) smr-ccs-56% | EUR/MWh | 0.209 | 0.209 | SMR data from IEAGHG report, p. 18, Table 4. ATR data provided by Sintef (see methods). Assuming uncertainty of 10%. |
| Variable Operation and Maintenance Cost (w/o fuel cost) atr-ccs-93% | EUR/MWh | 1.908 +- 0.19 | 0.252 +- 0.025 | SMR data from IEAGHG report, p. 18, Table 4 ATR data provided by Sintef (see methods). Assuming uncertainty of 10%. |
| Full load hours of blue-hydrogen plant | h | 8322 | 8322 | IEAGHG SMR data from IEAGHG report, p. 18, Table 4. Assuming same for ATR. |
| Captured emissions for transport and storage smr-ccs-56% | kgCO2/Nm3_H2 | 0.466 | 0.466 | SMR data from IEAGHG, p. 16, Table 2 (Plant Performance Summary), Specific CO2 Captured. ATR data estimated from NG consumption and capture-rate of facility. |
| Captured emissions for transport and storage atr-ccs-93% | kgCO2/Nm3_H2 | 0.8004 | 0.8004 | SMR data from IEAGHG, p. 16, Table 2 (Plant Performance Summary), Specific CO2 Captured. ATR data estimated from NG consumption and capture-rate of facility. |
| Carbon transport and storage cost | EUR/tCO2 | 15.0 +- 5.0 | 10.0 + 10.0 - 5.0 | Own estimates. |
| Base emissions of blue hydrogen (per LHV) direct smr-ccs-56% gwp100 | gCO2eq/MJ | 33.5 +- 1.7 | 33.5 +- 1.7 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) direct smr-ccs-56% gwp20 | gCO2eq/MJ | 33.5 +- 1.7 | 33.5 +- 1.7 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) direct atr-ccs-93% gwp100 | gCO2eq/MJ | 4.9 +- 0.2 | 4.9 +- 0.2 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) direct atr-ccs-93% gwp20 | gCO2eq/MJ | 4.9 +- 0.2 | 4.9 +- 0.2 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) elec smr-ccs-56% gwp100 | gCO2eq/MJ | 0.03 +- 0.002 | 0.03 +- 0.002 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) elec smr-ccs-56% gwp20 | gCO2eq/MJ | 0.04 +- 0.002 | 0.04 +- 0.002 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) elec atr-ccs-93% gwp100 | gCO2eq/MJ | 0.2 +- 0.01 | 0.2 +- 0.01 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) elec atr-ccs-93% gwp20 | gCO2eq/MJ | 0.2 +- 0.01 | 0.2 +- 0.01 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) scco2 smr-ccs-56% gwp100 | gCO2eq/MJ | 10.7 +- 0.5 | 5.4 +- 0.3 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) scco2 smr-ccs-56% gwp20 | gCO2eq/MJ | 10.7 +- 0.5 | 5.4 +- 0.3 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) scco2 atr-ccs-93% gwp100 low supply-chain CO2 | gCO2eq/MJ | 10.9 +- 0.5 | 1.09 +- 0.1 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) scco2 atr-ccs-93% gwp20 low supply-chain CO2 | gCO2eq/MJ | 10.9 +- 0.5 | 1.09 +- 0.1 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) scco2 atr-ccs-93% gwp100 | gCO2eq/MJ | 10.9 +- 0.5 | 5.5 +- 4.1 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) scco2 atr-ccs-93% gwp20 | gCO2eq/MJ | 10.9 +- 0.5 | 5.5 +- 4.1 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) cts smr-ccs-56% gwp100 | gCO2eq/MJ | 0.2 +- 0.01 | 0.2 +- 0.01 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) cts smr-ccs-56% gwp20 | gCO2eq/MJ | 0.2 +- 0.01 | 0.2 +- 0.01 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) cts atr-ccs-93% gwp100 | gCO2eq/MJ | 0.3 +- 0.01 | 0.3 +- 0.01 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) cts atr-ccs-93% gwp20 | gCO2eq/MJ | 0.3 +- 0.01 | 0.3 +- 0.01 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) other smr-ccs-56% gwp100 | gCO2eq/MJ | 1.0 +- 0.05 | 1.0 +- 0.05 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) other smr-ccs-56% gwp20 | gCO2eq/MJ | 1.3 +- 0.07 | 1.3 +- 0.07 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) other atr-ccs-93% gwp100 | gCO2eq/MJ | 1.0 +- 0.05 | 1.0 +- 0.05 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Base emissions of blue hydrogen (per LHV) other atr-ccs-93% gwp20 | gCO2eq/MJ | 1.3 +- 0.07 | 1.3 +- 0.07 | Most contributions to the LCA emissions stay mostly constant, as outlined in subsection ‘Life-cycle emissions of blue hydrogen’ in the Methods section. Supply-chain CO2 emissions are reduced to 87.5% between 2035 and 2050. The uncertainty is assumed to be 5% throughout, except for the uncertainty of supply-chain CO2 emissions in 2050, which are 50% +- 37.5% of those in 2035. |
| Methan leakage emissions per CH4 emission rate (per LHV) smr-ccs-56% gwp100 | gCO2eq/MJ | 707 | 707 | LCA analysis, see methods. |
| Methan leakage emissions per CH4 emission rate (per LHV) smr-ccs-56% gwp20 | gCO2eq/MJ | 2010 | 2010 | LCA analysis, see methods. |
| Methan leakage emissions per CH4 emission rate (per LHV) atr-ccs-93% gwp100 | gCO2eq/MJ | 710 | 710 | LCA analysis, see methods. |
| Methan leakage emissions per CH4 emission rate (per LHV) atr-ccs-93% gwp20 | gCO2eq/MJ | 2032 | 2032 | LCA analysis, see methods. |
| Fuel efficiency (natural gas) smr-ccs-56% | percent | 73.51 | 73.51 | SMR data from IEAGHG report, p.16, Table 2 (Plant Performance Summary), total energy of inlet & outlet stream. ATR data provided by Sintef. |
| Fuel efficiency (natural gas) atr-ccs-93% | percent | 73.53 | 73.53 | SMR data from IEAGHG report, p.16, Table 2 (Plant Performance Summary), total energy of inlet & outlet stream. ATR data provided by Sintef. |
| Fuel efficiency (grid electricity) smr-ccs-56% |  | -200 | -200 | SMR data from IEAGHG report, p.16, Table 4 (Plant Performance Summary), total energy of inlet & outlet stream. ATR data provided by Sintef. |
| Fuel efficiency (grid electricity) atr-ccs-93% |  | 25 | 25 | SMR data from IEAGHG report, p.16, Table 4 (Plant Performance Summary), total energy of inlet & outlet stream. ATR data provided by Sintef. |
| Green CAPEX | USD/kW | 900 +- 200 | 200 + 100 - 70 | Own assumptions based on various sources. |
| Fixed Operation and Maintenance Cost | percent | 3 | 3 | AEMO Integrated System Plan assumptions. |
| Variable Operation and Maintenance Cost (w/o fuel cost) | EUR/MWh | 0.4 +- 0.1 | 0.4 +- 0.1 | Assuming USD 0.01-0.02/kg H2 from IEA for water consumption. |
| Electricity price RE | USD/MWh | 45.0 +- 10.0 | 25.0 +- 10.0 | This is the wholesale electricity price (without taxes and duties) that an electrolyser operator pays for renewable electricity input. Note that this is lower than the average electricity price in a system, because the electrolyser is operated at a low capacity factor (50%) and thus benefits from negative- and low-price hours. Own estimate based on average electricity prices in global scenarios57, further solar PV cost reductions56, and modeled electricity supply curves of potential hydrogen export countries10. |
| Electricity price fossil | USD/MWh | 85.0 +- 10.0 | 70.0 + 5.0 - 10.0 | Own assumption based on recent peak price market data in the EU. |
| CI of electrolysis gwp100 | kgCO2eq/kgH2 | 0.12 +- 0.02 | 0.06 +- 0.01 | Based on LCA analysis, see method |
| CI of electrolysis gwp20 | kgCO2eq/kgH2 | 0.14 +- 0.02 | 0.07 +- 0.01 | Based on LCA analysis, see method |
| CI of electricity RE gwp100 | kgCO2eq/kWh | 0.027 +- 0.010 | 0.016 +- 0.005 | Based on LCA analysis, see method Fossil is assuming natural gas as a source for power generation. |
| CI of electricity RE gwp20 | kgCO2eq/kWh | 0.033 +- 0.010 | 0.020 +- 0.005 | Based on LCA analysis, see method Fossil is assuming natural gas as a source for power generation. |
| CI of electricity fossil gwp100 | kgCO2eq/kWh | 0.400 +- 0.05 | 0.400 +- 0.05 | Based on LCA analysis, see method Fossil is assuming natural gas as a source for power generation. |
| CI of electricity fossil gwp20 | kgCO2eq/kWh | 0.650 +- 0.05 | 0.650 +- 0.05 | Based on LCA analysis, see method Fossil is assuming natural gas as a source for power generation. |
| Operational capacity factor | percent | 50 | 50 | Feasible OCF for RE-only operation mode. |
| Elec. efficiency | percent | 65 | 70 | Own estimates based on various sources. |
| Share of renewable and grey electricity. 100% means RE only. | percent | 80 | 100 | Own assumptions. |
| lifetime |  | 20 | 20 | Stack lifetime is 95kh according to IEA Future of Hydrogen (2019) report. With 50% OCF that is ~21 years. |
| Interest rate | percent | 8 | 8 | Assuming a typical WACC value. |
| Transp. cost | EUR/MWh | 20 | 10 | Numbers encouraged by research results shared by Acatech and the ESYS project. |
| GHGI of energy used in H2 transportation | percent | 3 | 3 | Rödl et al59 present recompression every 100 km, 0.02 kWh(Strom)/kg hydrogen>>, which gives 0.4 kWh/kg H2. Using a compressor with 40% efficiency, this results in 1kWh_LHV-H2 per kg_H2. This means a 3% surplus on the GHGI. |