A novel VOC breath tracer method to evaluate indoor respiratory exposures in the near- and far-fields

Hooman Parhizkar1,2, Mark Fretz1,2,3, Aurélie Laguerre4, Jason Stenson1,2, Richard L. Corsi5, Kevin G. Van Den Wymelenberg1,2,3*, Elliott Gall4

1 - Institute for Health and the Built Environment, University of Oregon, Portland, OR, United States, 97209

2 - Energy Studies in Buildings Laboratory, University of Oregon, Eugene, OR, United States, 97403

3 - Biology and the Built Environment Center, University of Oregon, Eugene, OR, United States, 97403

4 - Department of Mechanical and Materials Engineering, Portland State University, Portland, OR, United States, 97201

5 - College of Engineering, UC Davis, Davis, CA, United States, 95616

*Corresponding Author: Kevin G. Van Den Wymelenberg, kevinvdw@uoregon.edu, (541) 346-5647, Biology and the Built Environment Center, University of Oregon, Eugene, OR, United States, 97403, Energy Studies in Buildings Laboratory, University of Oregon, Eugene, OR, United States, 97403, Institute for Health and the Built Environment, University of Oregon, Portland, OR, United States, 97209

Supplemental figure 1 Evaluating the impact of distance on bioaerosol exposure in a typical indoor environment

Supplemental table 1 Magnifiers and effect size values of 2.5, 5, and 7.5 ft normalized by volume averaged concentrations in 5 minutes time step

| 2.5 ft |  |  | 5 ft |  |  | 7.5 ft |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intervals | Magnifier | Effect size | ±EU2 | Magnifier | Effect size | ±EU2 | Magnifier | Effect size | ±EU2 |
| Min_0_60 | 1.2123 | 0.32 (s) | 0.3496 | 1.0799 | 0.14 (N) | 0.3265 | 1.0516 | 0.1 (N) | 0.3094 |
| Min_0_5 | 2.7476 | 0.57(M) | 4.6822 | 1.7211 | -0.08(N) | 3.4974 | 0.7354 | -0.58(M) | 1.8898 |
| Min_5_10 | 0.8995 | -0.26(S) | 0.507 | 0.6893 | -0.95(L) | 0.51 | 0.6587 | -1.12(L) | 0.4337 |
| Min_10_15 | 1.3637 | 1.96(L) | 0.4228 | 0.949 | -0.42(S) | 0.3454 | 0.878 | -0.93(L) | 0.3048 |
| Min_15_20 | 1.4471 | 3.65(L) | 0.3764 | 1.021 | 0.09(N) | 0.3187 | 0.9731 | -0.28(S) | 0.2789 |
| Min_20_25 | 1.2559 | 2.00(L) | 0.3088 | 1.1754 | 1.32(L) | 0.2948 | 1.1446 | 1.07(L) | 0.2637 |
| Min_25_30 | 1.1251 | 1.39(L) | 0.2301 | 1.0277 | 0.23(S) | 0.2242 | 1.1122 | 1.08(L) | 0.2173 |
| Min_30_35 | 1.1901 | 2.13(L) | 0.251 | 1.162 | 1.78(L) | 0.2595 | 1.1689 | 2.12(L) | 0.2292 |
| Min_35_40 | 1.1774 | 2.00(L) | 0.2356 | 1.1293 | 1.35(L) | 0.2131 | 1.1219 | 1.41(L) | 0.2147 |
| Min_40_45 | 1.1904 | 2.54(L) | 0.2036 | 1.1633 | 2.27(L) | 0.2108 | 1.1343 | 1.88(L) | 0.1938 |
| Min_45_50 | 1.1518 | 2.39(L) | 0.1856 | 1.1082 | 1.71(L) | 0.1826 | 1.0647 | 1.01(L) | 0.1761 |
| Min_50_55 | 1.2103 | 2.76(L) | 0.2546 | 1.1452 | 2.01(L) | 0.217 | 1.0662 | 0.92(L) | 0.202 |
| Min_55_601 | 1.1839 | 2.39(L) | 0.2559 | 1.1121 | 1.49(L) | 0.213 | 1.0775 | 1.03(L) | 0.1793 |

* N = negligible effect size, S = small effect size, M = medium effect size, L = large effect size

1 = Steady state period

2 =Expanded uncertainty

Supplemental figure 2. Comparison of the concentrations of breath tracers for each distance to volume integrated background with expanded uncertainties.

APPENDIX A

The implications of magnifiers in a real-world case study

The purpose of this section is to compare the results of near -field and far-field magnifiers for the present study and two recent relevant studies.1,2

In one study 2 the team measured near-field and far-field CO2 concentrations to estimate magnifiers in patient rooms within a healthcare environment having 8-11 ACH. The study reported background (far-field) CO2 levels of 580 ppm (mean across 7 patients) and reported near-field mean CO2 levels of 605 ppm, thus 25 ppm higher than background, which equates to a near-field magnifier of 4.3%2

A second study reports on bioaerosols emitted from individuals that were diagnosed with COVID-19 within a space having similar environmental conditions as the present study.1 Near-field and far-field terms were used to report the concentration of SARS-CoV-2 viral RNA in rom aerosols, room CO2, and room particles in the range of 0.3-25 µm at 1.2 m (4 ft) and 3.5 m (11 ft). We translate their data into near-/far-field ratios to provide a comparison with the near-field magnifiers reported in Figure 3 of the present study. The near-/far-field ratios from the previous study (Supplemental table 3) ranged from ~8-12% for CO2 and particles (1-2.5um), which correspond reasonably well with the near-field magnifiers of the present study (Figure 3) where the concentration of targeted VOCs in the near-field (2.5 ft) was ~10% higher than far-field (7.5ft) during steady-state periods.

Supplemental table 2 Comparison of spatial parameters between the present and the controlled study with participants diagnosed with COVID-191.

| Key variables | Breath tracer (present study) | Parhizkar, et al (2022) |
| --- | --- | --- |
| Volume (m3) | 27 | 28.04 |
| Air exchange rate (1/h) | ~ 3 ACH | ~0 |
| Duration (minutes) | 60 | 60 |
| Near- field distance (ft) | 2.5 | 4 |
| Far- field distance (ft) | 7.5 | 11 |
| Near/far fields ratio | 3 | 2.75 |
| Number of participants in the room for each trial | 1 | 1 |

Supplemental table 3 Comparison of near- field and far-field in a recent controlled study on participants that were diagnosed with COVID-19.1

| Variable | Near-field | Far-field | Near-field / Far-field |
| --- | --- | --- | --- |
| CO2 (ppm) | 937.724 | 862.4149 | 1.0836 |
| Particles, 0.3 µm - 1µm | 16454.36 | 15959.25 | 1.0305 |
| Particles, 1 µm - 2.5 µm | 559.5155 | 493.7172 | 1.1249 |
| Particles, 2.5 µm - 3µm | 37.61776 | 31.71535 | 1.1702 |
| Particles, 3 µm - 5 µm | 50.8684 | 45.12782 | 1.1196 |
| Particles, 5 µm - 10 µm | 26.47086 | 27.24334 | N/A |
| Particles, 10 µm - 25 µm | 8.85414 | 8.961011 | N/A |

Reference

1. 	Parhizkar H, Dietz L, Olsen-Martinez A, et al. Quantifying environmental mitigation of aerosol viral load in a controlled chamber with participants diagnosed with COVID-19. Clin Infect Dis. January 2022. January 6, 2022. https://academic.oup.com/cid/advance-article/doi/10.1093/cid/ciac006/6498295. Accessed January 17, 2022.

2. 	Gall ET, Laguerre A, Noelck M, Van Meurs A, Austin JP, Foster BA. Near-field airborne particle concentrations in young children undergoing high-flow nasal cannula therapy: a pilot study. J Hosp Infect. 2021;113:14–21.