Table S1: Analytical procedure,  Accuracy and Precision data of Standard Reference Materials of (SRM 1643f)

0.5 ml of conc. HNO3 was added to the 10 ml of acid preserved water samples and heated at 1050C for ~2 hour and 30 mins, till a colourless solution was obtained. The resulting solution was cooled and diluted to 10 ml with ultrapure water (18.2 ms/cm). Then the solution was filtered through Axiva 0.2µ PTFE filter paper and stored at 4°C in 15 ml polypropylene tubes until further analysis in Inductively-coupled plasma optical emission spectrometer (ICP-OES) (Thermo Fisher iCAP 7400 ICP-OES) (APHA 2017). The study involved the estimation of concentrations of the following 10 PTEs: Al, Cd, Co, Cr, Cu, Fe, Mn, Ni, Pb and Zn. Plasticwares and glasswares used in the analysis of toxic elements were washed with 10% v/v HNO3 solution followed by ultrapure Milli-Q water and air dried. Calibration standard solutions were prepared by using ICP Multi-elemental Standard Solution IV and IX (Merck, Germany). The quality of the analytical process in ICP-OES was compared with the analysis of certified standard reference material. Certified standard reference material of toxic element in water (SRM 1643f) from National Institute of Standard and Technology (NIST) was used in triplicate to ensure quality control, precision and accuracy of the analyses. Analytical accuracy, expressed as recovery, was 94.9 – 104.4 %, and precision was <2.5% relative standard deviation for all estimated toxic elements (Table S1). Concentrations of PTEs were expressed as µg/l. Table S1.

| Element | Certified Value (µg/l) | Experimental Value (µg/l) | SD | Percentage Recovery | Precision | Detection Limit (µg/l) |
| --- | --- | --- | --- | --- | --- | --- |
| Al | 132.5 | 137.53 | 1.30 | 103.8 | 0.98 | 0.12 |
| Cd | 5.89 | 6.04 | 0.11 | 102.5 | 1.80 | 0.07 |
| Co | 25.03 | 24.31 | 0.53 | 97.1 | 2.11 | 0.51 |
| Cr | 18.50 | 18.12 | 0.33 | 97.9 | 1.76 | 0.21 |
| Cu | 21.66 | 20.70 | 0.31 | 95.6 | 1.42 | 0.39 |
| Fe | 93.44 | 90.28 | 1.23 | 96.6 | 1.32 | 0.25 |
| Mn | 37.14 | 38.77 | 0.81 | 104.4 | 2.18 | 0.07 |
| Ni | 59.2 | 56.20 | 1.45 | 94.9 | 2.45 | 0.36 |
| Pb | 18.48 | 18.05 | 0.05 | 97.7 | 0.27 | 1.06 |
| Zn | 74.4 | 76.53 | 0.25 | 102.9 | 0.34 | 0.19 |

APHA (2017). Standard Methods for the Examination of Water and Wastewater. American Public Health Association, eds. Baird, B. R., Eaton, A. D., & Rice, E. W. Washington, DC, 23, 1600.

Table S2: Detailed calculation of WQI, risk assessment and HQ

Different weights were assigned to each of the chemical parameters 1 (minimum impact on water quality) to 5 (maximum impact on water quality) depending on their ability to affect human health. PTEs like Cd, Cr, Mn and Pb were assigned the maximum weight 5. The physicochemical parameters like pH, hardness, TDS, DO, COD and PTEs like Al and Fe were assigned a weightage of 4, whereas physicochemical parameters like EC, salinity and alkalinity were assigned a weightage of 3. Cu was assigned a weight of 2 while Co, Ni and Zn were assigned the minimum weight of 1 due to their low impact on water quality (Şener et al., 2017; Gao et al., 2019).The relative weight (Wr) was calculated from the following mathematical expression (Eq. I):

Wr=wri=1nwr						Eq. (I)

where wr is the assigned weight of each parameter and n is the number of parameters.

A quality rating (QR) of individual parameter was estimated by dividing respective concentration in individual water samples by its acceptable value as per Indian standards for drinking and surface water (IS 10500 : 2012 and IS 2296 : 1982) (ISI, 1991; BIS, 2012), and WHO standards for drinking water (WHO, 2008, 2011).multiplied by 100 (Eq. II).

QR= CiAiX 100  						Eq. (II)

where Ci is the concentration of examined parameters in water sample and Ai is acceptable values of examined parameters as per Indian standards of drinking and surface water quality (IS 10500 : 2012 and IS 2296 : 1982) and WHO standards for drinking water.

The water quality index (WQI) was calculated as the summation of products of relative weight and quality rating of individual examined parameters (Eq. III) (Ramakrishnaiah et al., 2009; Sener et al., 2017).

WQI= i=1nWri X Qri						Eq. (III)

where Wri and Qri are the individual relative weight and quality rating of examined parameters respectively. The calculated WQI values were differentiated into five categories of water quality status (Yidana and Yidana, 2010; Şener et al., 2017) as follows: <50= Excellent; 50–100= Good; 100–200= Poor; 200–300= Very poor ; >300= Very severe and unsuitable for drinking purpose.

Risk Assessment

The doses of PTEs exposure rate through direct consumption or ingestion and dermal absorption were estimated according to modified USEPA, (2004), using the following equation (IV) and (V) :

MDIingestion= EC X IR X ER X EDBW X MT						Eq. (IV)

MDIdermal= EC X Sa X AF X DAF X Ed X ER X ED X CFBW X MT				Eq. (V)

where EC is the concentration of PTEs (mg/l); IR is the rate of ingestion (l/day) (for child: 0.78, adult: 2.5); ER is the exposure rate (day/year) (for child and adult: 365); ED is the duration of exposure (year) (for child: 6, adult: 70); BW is the individual body weight (kg) (for child: 15, adult: 70); MT is the mean time of exposure (day) (for child: 2190 d; for adults: 25,550 d); SA is the skin area in contact (cm2) (for child and adult: 5700), AF is the adherence factor (mg/cm2) (for child and adult: 0.07); DAF is the dermal absorption coefficient (0.001 cm/h for Al, Cd, Cu, Fe and Mn; 0.002 cm/h for Cr; 0.004 cm/h for Co, Pb and Ni, and 0.0006 cm/h for Zn; Ed is the exposure duration in a day (h/day) (for child: 1 h/day; adult: 0.58 h/day); and CF is the conversion factor (L/cm3) (10−3) (USEPA, 1989, 1991, 2002, 2004, 2011; Wu et al., 2009; Li and Zhang, 2010; Mitra et al., 2018b; Singh et al., 2018; Saleem et al., 2019).

Toxicological profile of the studied elements indicates all PTEs have toxic carcinogenic or non-carcinogenic human health effects (Luo et al., 2012). In the study, the potential degree of non-carcinogenic risk to human population due to ingestion of PTE contaminated water was evaluated as hazard quotients (HQ) by comparing the exposures of PTEs through oral or dermal pathways with the reference dose (RfD) following equation Eq. (VI) (USEPA, 2004):

HQingestion/ dermal=MDIingestion/dermalRfDingestion/dermal					Eq. (VI)

where RfDingestion and RfDdermal are reference dose (µg/kg/day) for oral and dermal exposure respectively.The RfDingestion and RfDdermal values for respective PTEs used to determine HQ for adults and child are as follows: for RfDingestion Al :1000; Cd:0.5; Co: 0.3; Cr: 3; Cu: 40; Fe: 300; Mn: 20; Ni: 20; Zn: 300; Pb :1.4; while for RfDdermal Al : 200; Cd: 0.005; Co: 0.06;  Cr: 0.015; Cu:12; Fe: 45; Mn: 0.8; Ni: 5.4; Zn: 60; Pb: 0.42 (USEPA, 2006; Mitra et al., 2018b; Saleem et al., 2019).

Hazard index (HI) was evaluated as summation of non carcinogenic risks caused due to exposures from PTEs though oral and dermal routes. However, HI was calculated using the following equation Eq. (VII):

HI= HQingestion+ HQdermal					Eq. (VII)

If the value of HQ and HI is less than unity, then there is no harmful effect on human health. However, if the HQ and HI values exceed unity, then there might be concern for harmful non carcinogenic effect on human health (USEPA, 2004; Gao et al., 2019).

Table S3: Seasonal physicochemical properties of water of river Hooghly (n = 3 for each site and season)

| Sampling Locations | Season | Physico-chemical Properties |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | pH | EC (µS/cm2) | TDS (mg/l) | Salinity | Hardness (mg/l) | COD (mg/l) | DO (mg/l) | Alkalinity (mg/l) |
| S1 | Summer | 7.45 ± 0.05 | 318 ± 4.5 | 568.2 ± 27.6 | 0.04 ± 0.005 | 138.6 ± 2.6 | 28.1 ± 2.33 | 4.33 ± 0.05 | 340 ± 5 |
|  | Monsoon | 7.34 ± 0.1 | 156 ± 3.8 | 274.6 ± 23.2 | 0.11 ± 0.009 | 74.25 ± 3.8 | 36± 1.8 | 3.8 ± 0.05 | 180 ± 2.6 |
|  | Winter | 7.56 ± 0.13 | 175 ± 5.0 | 503.36 ± 25.2 | 0.09 ± 0.005 | 77.9 ± 2.3 | 33.3± 2.47 | 3.7 ± 0.1 | 155 ± 1.9 |
| S2 | Summer | 7.52 ± 0.07 | 377 ± 5.2 | 624 ± 21.6 | 0.04 ± 0.005 | 143.55 ± 2.4 | 32.2 ± 2.1 | 4 ± 0.1 | 272 ± 2.33 |
|  | Monsoon | 7.31 ± 0.09 | 221 ± 6.7 | 444.3 ± 26.1 | 0.11 ± 0.005 | 84.15 ± 3.5 | 40.2 ± 2.11 | 4.9 ± 0.05 | 126.1 ± 2.9 |
|  | Winter | 7.45 ± 0.1 | 240 ± 5.7 | 675.2 ± 23.8 | 0.08 ± 0.008 | 88.3 ± 3.1 | 41.4 ± 1.54 | 4.6 ± 0.1 | 133.2 ± 2.5 |
| S3 | Summer | 7.62 ± 0.09 | 513 ± 3.5 | 573 ± 28.2 | 0.09 ± 0.006 | 173.25 ± 4.3 | 24.2 ± 2.3 | 3.9 ± 0.1 | 136.5 ± 5.5 |
|  | Monsoon | 8.0 ± 0.06 | 312 ± 3.9 | 606.4 ± 35.7 | 0.10 ± 0.006 | 99 ± 5.3 | 20.5 ± 3.13 | 5.1 ± 0.1 | 162 ± 4.3 |
|  | Winter | 7.9 ± 0.09 | 353 ± 4.0 | 620.4 ± 31.0 | 0.11 ± 0.005 | 103.9 ± 5.7 | 36 ± 2.56 | 4 ± 0.05 | 190 ± 3.2 |
| S4 | Summer | 7.63 ± 0.08 | 348 ± 10.1 | 676.2 ± 21.8 | 0.08 ± 0.007 | 80.6 ± 5.2 | 24.3 ± 1.63 | 3.33 ± 0.2 | 204 ± 6 |
|  | Monsoon | 7.84 ± 0.07 | 227 ± 2.1 | 488.1 ± 28.7 | 0.09 ± 0.005 | 113.85 ± 5.5 | 32.4 ± 2.9 | 3.8 ± 0.05 | 180 ± 5.1 |
|  | Winter | 7.74 ± 0.06 | 257 ± 10.6 | 522.0 ± 26.1 | 0.10 ± 0.005 | 119.5 ± 6.2 | 37.35 ± 2.1 | 4.15 ± 0.1 | 170 ± 2.5 |
| S5 | Summer | 7.51 ± 0.06 | 502 ± 12.5 | 695.4 ± 39.7 | 0.09 ± 0.005 | 193.05 ± 8.7 | 28.3 ± 1.47 | 3.1 ± 0.1 | 391 ± 4 |
|  | Monsoon | 7.71 ± 0.05 | 195 ± 4.2 | 546.5 ± 32.1 | 0.12 ± 0.008 | 69.3 ± 6.2 | 28.1 ± 5.6 | 5 ± 0.05 | 324 ± 8.7 |
|  | Winter | 7.73 ± 0.07 | 220 ± 13.0 | 754.4 ± 35.7 | 0.12 ± 0.009 | 72.7 ± 5.5 | 27.9 ± 2.38 | 3.1 ±0.1 | 225 ± 3.2 |
| S6 | Summer | 7.63 ± 0.14 | 367 ± 8.0 | 834.0 ± 25.6 | 0.07 ± 0.005 | 158.4 ± 6.4 | 32.2 ± 2.56 | 4.66 ± 0.1 | 170 ± 4 |
|  | Monsoon | 7.9 ± 0.07 | 187 ± 3.8 | 464.8 ± 27.3 | 0.12 ± 0.005 | 59.4 ± 5.5 | 36.8 ± 3.9 | 4.9 ± 0.1 | 144 ± 6.4 |
|  | Winter | 8 ± 0.05 | 210 ± 8.5 | 714.4 ± 30.7 | 0.11 ± 0.005 | 62.4 ± 5.0 | 39.375 ± 2.73 | 4.37 ± 0.05 | 160 ± 1.9 |
| S7 | Summer | 7.22 ± 0.16 | 308 ± 11.0 | 710.7 ± 37.4 | 0.04 ± 0.005 | 257.4 ± 4.3 | 52.1 ± 2.33 | 4.33 ± 0.05 | 204 ± 7 |
|  | Monsoon | 7.69 ± 0.05 | 122 ± 3.1 | 775.9 ± 45.6 | 0.15 ± 0.005 | 34.65 ± 3.6 | 40.1 ± 2.40 | 4.8 ± 0.1 | 198 ± 2.5 |
|  | Winter | 7.65 ± 0.08 | 135 ± 11.5 | 822.4 ± 41.1 | 0.08 ± 0.006 | 92.3 ± 4.9 | 47.475 ± 1.96 | 5.27 ± 0.05 | 170 ± 2.4 |
| S8 | Summer | 7.55 ± 0.09 | 490 ± 7.8 | 748.5 ± 29.9 | 0.11 ± 0.01 | 237.6 ± 3.8 | 16.3 ± 1.86 | 3.66 ± 0.05 | 306 ± 8 |
|  | Monsoon | 7.75 ± 0.07 | 259 ± 4.1 | 843.2 ± 29.6 | 0.14 ± 0.009 | 19.8 ± 4.7 | 30.2 ± 2.07 | 4 ± 0.15 | 216 ± 3.2 |
|  | Winter | 7.46 ± 0.06 | 300 ± 8.3 | 657.8 ± 32.9 | 0.15 ± 0.009 | 129.7 ± 3.5 | 39.375 ± 2.17 | 4.37 ± 0.05 | 183 ± 2.3 |
| S9 | Summer | 7.37 ± 0.05 | 1358 ± 22.0 | 941.3 ± 32.8 | 0.5 ± 0.05 | 391.05 ± 11.6 | 24.1 ± 2.1 | 4 ± 0.1 | 544 ± 12.5 |
|  | Monsoon | 7.48 ± 0.09 | 290 ± 3.1 | 934.1 ± 34.9 | 0.18 ± 0.05 | 74.25 ± 10.2 | 30.1 ± 1.8 | 5 ± 0.1 | 378 ± 1.9 |
|  | Winter | 7.31 ± 0.14 | 300 ± 22.5 | 720.5 ± 36.0 | 0.2 ± 0.02 | 111.4 ± 15.4 | 49.5 ± 2.5 | 5. ± 0.15 | 412 ± 8.33 |
| S10 | Summer | 7.47 ± 0.18 | 612 ± 5.0 | 1100.1 ± 49.8 | 0.18 ± 0.06 | 218.8 ± 13.5 | 22.3 ± 2.47 | 4.53 ± 0.05 | 558 ± 14.55 |
|  | Monsoon | 7.56 ± 0.08 | 312 ± 3.8 | 772.7 ± 45.5 | 0.2 ± 0.06 | 84.15 ± 10.7 | 22.9 ± 2.1 | 4 ± 0.1 | 342 ± 7.33 |
|  | Winter | 7.29 ± 0.05 | 330 ± 5.5 | 947.0 ± 47.3 | 0.22 ± 0.03 | 126.2 ± 12.5 | 39.375 ± 3.89 | 4.37 ±0.05 | 373 ± 7.2 |
| S11 | Summer | 7.39 ± 0.12 | 750 ± 8.50 | 1196.4 ± 32.8 | 3.67 ± 0.09 | 608.85 ± 13.2 | 28.7 ± 1.54 | 3.2 ± 0.5 | 629 ± 11.5 |
|  | Monsoon | 7.64 ± 0.05 | 315 ± 6.7 | 730.0 ± 28.9 | 0.42 ± 0.09 | 118.8 ± 11.8 | 26.1 ± 2.47 | 3.9 ± 0.2 | 558 ± 11.5 |
|  | Winter | 7.71 ± 0.04 | 350 ± 9.0 | 1022.4 ± 31.1 | 1.5 ± 0.1 | 178.2 ± 9.2 | 36 ± 2.11 | 4 ± 0.05 | 490 ± 11.4 |
| S12 | Summer | 7.68 ± 0.08 | 1055 ± 10.5 | 1335.3 ± 19.5 | 8.96 ± 0.2 | 1089 ± 11.4 | 24.5 ± 2.56 | 4.66 ± 0.1 | 680 ± 10.5 |
|  | Monsoon | 7.48 ± 0.18 | 759 ± 5.1 | 1242.4 ± 33.1 | 3.83 ± 0.3 | 495 ± 18.8 | 28.5 ± 1.54 | 4 ± 0.1 | 612 ± 13.4 |
|  | Winter | 7.58 ± 0.08 | 850 ± 11.0 | 1346.8 ± 27.3 | 3.30 ± 0.2 | 459.4 ± 11.4 | 39.375 ± 3.13 | 4.37 ± 0.1 | 525 ± 5.5 |
| S13 | Summer | 7.72 ± 0.13 | 1038 ± 6.4 | 1539.2 ± 49.9 | 9.37 ± 0.25 | 1336.5 ± 15.1 | 35.99 ± 2.1 | 4 ± 0.1 | 731 ± 14.33 |
|  | Monsoon | 7.53 ± 0.12 | 732 ± 4.3 | 1399.8 ± 58.1 | 3.75 ± 0.25 | 396 ± 10.2 | 32.7 ± 2.56 | 4 ± 0.1 | 684 ± 14.33 |
|  | Winter | 7.56 ± 0.09 | 820 ± 6.9 | 1678.1 ± 59.9 | 4.2 ± 0.15 | 449.7 ± 9.1 | 38.025 ± 2.99 | 4.22 ± 0.05 | 623 ± 4.33 |
| S14 | Summer | 7.80 ± 0.10 | 1435 ± 8.5 | 1459.6 ± 43.0 | 18.74 ± 0.30 | 1435.5 ± 15.2 | 36.99 ± 2.38 | 4.4 ± 0.15 | 867 ± 21.33 |
|  | Monsoon | 7.64 ± 0.08 | 812 ± 5.3 | 1421.9 ± 46.9 | 3.71 ± 0.1 | 445.5 ± 13.7 | 30.3 ± 2.1 | 5 ± 0.05 | 666 ± 8.5 |
|  | Winter | 7.71 ± 0.07 | 920 ± 9.0 | 2016.2 ± 40.8 | 4.1 ± 0.09 | 444.8 ± 11.2 | 34.31 ± 5.6 | 3.81 ± 0.05 | 679 ± 11.2 |
| S15 | Summer | 7.63 ± 0.09 | 3120 ± 11.3 | 1707.6 ± 50.8 | 26.80 ± 0.5 | 1878 ± 18.3 | 32.1 ± 2.73 | 4.9 ± 0.1 | 714 ± 17.33 |
|  | Monsoon | 7.76 ± 0.05 | 2750 ± 13.1 | 1407.4 ± 47.3 | 18.0 ± 0.5 | 1336.5 ± 14.3 | 32.2 ± 2.38 | 3.7 ± .1 | 720 ± 7.3 |
|  | Winter | 7.32 ± 0.05 | 2900 ± 11.8 | 1204.4 ± 48.2 | 19.8 ± 0.4 | 1620.3 ± 16.5 | 32.65 ± 3.9 | 3.6 ± 0.1 | 755 ± 14.5 |
| S16 | Summer | 7.57 ± 0.06 | 3070 ± 6.5 | 1895.3 ± 20.5 | 22.81 ± 0.8 | 1782 ± 14.8 | 44.2 ± 1.96 | 3.8 ± 0.1 | 952 ± 21.5 |
|  | Monsoon | 7.82 ± 0.02 | 2130 ± 4.8 | 1305.9 ± 25.3 | 12.30 ± 0.8 | 1485 ± 12.3 | 42.1 ± 2.40 | 4.4 ± 0.05 | 774 ± 8 |
|  | Winter | 7.19 ± 0.07 | 2200 ± 7.0 | 1449.4 ± 23.6 | 13.53 ± 0.7 | 1650.4 ± 14.9 | 32.62 ± 2.40 | 3.6 ± 0.1 | 800 ± 13.2 |
| S17 | Summer | 7.65 ± 0.07 | 2420 ± 3.4 | 2250.0 ± 46.5 | 26.07 ± 0.9 | 1881 ± 10.6 | 40.3 ± 2.17 | 4.1 ± 0.05 | 1173 ± 13.33 |
|  | Monsoon | 7.48 ± 0.14 | 3100 ± 5.7 | 1668.3 ± 50.5 | 19.8 ± 0.9 | 1287.2 ± 15.5 | 28.1 ± 2.70 | 4.8 ± 0.05 | 1044.3 ± 12.5 |
|  | Winter | 7.76 ± 0.09 | 3200 ± 3.9 | 1722 ± 56.1 | 21.78 ± 0.3 | 1583.7 ± 13.1 | 30.9 ± 2.70 | 3.43 ± 0.15 | 970.4 ± 11.4 |
| S18 | Summer | 7.45 ± 0.05 | 2900 ± 12.1 | 2080 ± 22.8 | 19.39 ± 0.4 | 1732.5 ± 16.4 | 20.3 ± 2.56 | 4.66 ± 0.1 | 986 ± 14.33 |
|  | Monsoon | 7.23 ± 0.05 | 2700 ± 7.9 | 1637.3 ± 76.3 | 18.82 ± 0.4 | 1435.5 ± 11.3 | 22.2 ± 2.1 | 4.9 ± 0.1 | 846.2 ± 17.33 |
|  | Winter | 7.53 ± 0.08 | 3050 ± 12.6 | 2023.681.2 | 20.7 ± 0.7 | 1467.8 ± 16.9 | 36.0 ± 2.5 | 4.0 ± 0.1 | 1085.5 ± 10.1 |

Table S4: Seasonal distribution of PTE in river Hooghly (n = 3 for each site and season)

| Sampling Locations | Season | Al | Cd | Co | Cr | Cu | Fe | Mn | Ni | Pb | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1 | Summer | 6900.45 ± 832.8 | 4.2±0.15 | 17.5±1.5 | 39.7±2.9 | 53±1.9 | 4434.9 ± 350.7 | 272.0 ± 12.9 | 40.8 ± 3.1 | 12.4 ± 2.2 | 51.5 ± 2.9 |
|  | Monsoon | 7687.1 ± 479.3 | 2.7 ± 0.6 | 11.0 ± 0.3 | 14.6 ± 1.9 | 24.0 ± 2.3 | 5014.6 ± 126.1 | 106.3 ± 11.4 | 31.07 ± 5.6 | 9.9 ± 1.9 | 39.8 ± 2.5 |
|  | Winter | 8583.0 ± 1126.2 | 5.4± 0.8 | 17.0 ± 1.0 | 36.5 ± 3.7 | 39.6 ± 2.3 | 8055.2 ± 689.5 | 157.8 ± 5.4 | 38.6 ± 2.22 | 12.3 ± 0.7 | 61.8 ± 3.6 |
| S2 | Summer | 5401.7 ± 546.1 | 3.7 ± 0.5 | 14.7 ±1.7 | 22.66 ± 1.9 | 34.6 ± 3.5 | 5589.03 ± 582.6 | 173.8 ± 5.6 | 35.6 ± 3.7 | 9.7 ± 0.8 | 50.1 ± 4.7 |
|  | Monsoon | 5881.8 ± 501.3 | 2.9 ± 0.2 | 10.6 ± 1.42 | 10.6 ± 0.9 | 25.9 ± 1.4 | 4597.9 ± 510.2 | 300.7 ± 4.6 | 29.8 ± 3.6 | 7.1 ± 0.7 | 37.4 ± 3.0 |
|  | Winter | 7856.1 ± 640.8 | 4.3 ± 0.3 | 14.4 ± 1.2 | 18.86 ± 1.1 | 33.6 ± 2.7 | 6546.8 ± 534.0 | 130.9 ± 10.7 | 32.7 ± 2.7 | 11.7 ± 0.9 | 56.1 ± 4.6 |
| S3 | Summer | 6767.6 ± 244.7 | 4.4 ± 0.3 | 15.8 ±2.0 | 17.3 ± 2.1 | 35.5 ± 3.6 | 9317.2 ± 703.3 | 150.7 ± 8.3 | 32.7 ± 3.1 | 11.0 ± 0.1 | 52.7± 0.6 |
|  | Monsoon | 10851.7 ± 681.0 | 4.2 ± 0.9 | 11.5 ± 11.0 | 12.02 ± 0.1 | 27.7 ± 2.3 | 5020.5 ± 116.2 | 160.1 ± 5.7 | 23.3 ± 2.1 | 9.4 ± 1.2 | 42.2 ± 0.5 |
|  | Winter | 8958.4 ± 271.7 | 5.2 ± 0.6 | 16.9 ± 1.3 | 16.0 ± 1.3 | 37.8 ± 0.5 | 7380.7 ± 90.8 | 147.6 ± 1.8 | 36.9 ± 0.4 | 13.2 ± 0.2 | 63.3 ± 0.8 |
| S4 | Summer | 7088.7 ± 502.8 | 4.7 ± 0.9 | 17.9 ± 1.6 | 23.42 ± 4.1 | 37.0 ± 2.1 | 6386.4 ± 1334.4 | 126.43 ± 9.8 | 32.6 ± 4.3 | 21.2 ± 3.0 | 50.8 ± 9.3 |
|  | Monsoon | 6499.4 ± 399.4 | 7.1 ± 0.2 | 13.2 ± 2.4 | 18.7 ± 0.7 | 29.8 ± 3.0 | 4642.5 ± 285.3 | 86.1 ± 9.6 | 28.2 ± 1.6 | 9.8 ± 2.6 | 40.6± 7.4 |
|  | Winter | 7361.3 ± 425.3 | 4.9 ± 0.4 | 16.3 ± 1.3 | 18.0 ± 0.2 | 58.8 ± 3.3 | 6797.0 ± 1050.6 | 137.4 ± 8.8 | 33.5 ± 3.6 | 30.7 ± 2.0 | 60.9 ± 11.1 |
| S5 | Summer | 9439.5 ± 1121.4 | 8.5 ± 0.5 | 28.7 ± 1.8 | 33.1 ± 2.9 | 41.8 ± 3.5 | 7535.7 ± 600.9 | 129.1 ± 7.6 | 37.8 ± 3.4 | 20.2 ± 2.5 | 66.4 ± 3.3 |
|  | Monsoon | 7887.6 ± 822.9 | 4.6 ± 0.3 | 23.2 ± 1.5 | 24.3 ± 2.5 | 50.0 ± 1.8 | 4628.6 ± 204.7 | 109.51 ± 9.0 | 51.6 ± 5.0 | 12.4 ± 2.0 | 53.1 ± 2.7 |
|  | Winter | 8688.2 ± 1070.9 | 9.9 ± 1.0 | 32.8 ± 3.4 | 18.1 ± 1.5 | 38.0 ± 3.1 | 7442.9 ± 764.6 | 159.3 ± 8.0 | 39.8 ± 2.0 | 15.6 ± 1.3 | 79.6 ± 4.0 |
| S6 | Summer | 11001.9±597.7 | 8.5 ± 0.7 | 49.4 ± 6.3 | 53.4 ± 4.5 | 58.2 ± 2.7 | 13579.6 ± 917.0 | 224.9 ± 13.7 | 63.6 ± 4.8 | 20.4 ± 1.1 | 98.0 ± 5.5 |
|  | Monsoon | 7037.6 ± 374.3 | 5.9 ± 0.2 | 27.7 ± 2.3 | 20.8 ± 2.2 | 27.4 ± 1.0 | 8797 ± 467.89 | 195.9 ± 11.0 | 39.0 ± 1.8 | 16.3 ± 0.9 | 78.4 ± 4.4 |
|  | Winter | 10298.4 ± 813.4 | 10.9 ± 0.8 | 55.1 ± 3.1 | 61.2 ± 3.4 | 43.8 ± 4.5 | 9362.2 ± 739.4 | 293.9 ± 16.5 | 73.5 ± 4.1 | 24.5 ± 1.4 | 117.6 ± 6.6 |
| S7 | Summer | 9516.2 ± 1488.4 | 8.1 ± 1.0 | 19.7± 1.3 | 34.15 ± 2.7 | 35.0 ± 2.8 | 7595.5 ± 990.7 | 198.4 ± 16.1 | 54.1 ± 6.8 | 15.7 ± 0.48 | 75.4 ± 2.3 |
|  | Monsoon | 6059.9 ± 361.2 | 2.6 ± 0.3 | 19.8 ± 0.6 | 22.0 ± 0.7 | 24.72 ± 2.1 | 5943.1 ± 1062.7 | 84.0 ± 1.5 | 71.6 ± 3.6 | 13.3 ± 1.5 | 590 ± 3.4 |
|  | Winter | 8310.3 ± 252.1 | 8.9 ± 0.3 | 29.68 ± 0.9 | 33.0 ± 1.4 | 73.5 ± 4.1 | 7914.6 ± 240.1 | 226.1 ± 6.9 | 56.5 ± 1.7 | 18.8 ± 0.8 | 90.4 ± 2.7 |
| S8 | Summer | 9928.8 ± 667.5 | 5.7 ± 1.1 | 45.66 ± 4.3 | 29.33 ± 2.7 | 54.65 ± 2.4 | 9797 ± 548.9 | 245.9 ± 11.0 | 54.6 ± 4.6 | 16.33 ± 0.9 | 78.4 ± 4.4 |
|  | Monsoon | 8075.4 ± 537.3 | 5.1 ± 0.3 | 17.5 ± 1.4 | 17.7 ± 0.8 | 39.19 ± 2.2 | 7637.6 ± 739.72 | 102.7 ± 6.3 | 39.2 ± 2.2 | 14.06 ± 1.7 | 62.7 ± 3.5 |
|  | Winter | 9481.1 ± 785.0 | 7.7 ± 0.4 | 26.2 ± 2.0 | 28.0 ± 1.6 | 39.6 ± 1.2 | 11423.1 ± 945.8 | 235.1 ± 13.2 | 107.3 ± 5.3 | 19.59 ± 1.1 | 94.0 ± 5.3 |
| S9 | Summer | 10548.0 ± 1360.9 | 5.7 ± 1.1 | 17.7 ± 2.0 | 22.5 ± 2.47 | 39.6 ± 2.7 | 8455.7 ± 1209.9 | 168.8 ± 14.1 | 42.3 ± 6.0 | 12.98 ± 1.1 | 62.3 ± 5.3 |
|  | Monsoon | 7477.3 ± 636.3 | 4.01 ± 0.3 | 13.7 ± 12 | 14.8 ± 1.3 | 31.2±2.6 | 6231.2 ± 530.2 | 124.6 ± 10.6 | 31.2 ± 2.6 | 11.4 ± 1.2 | 49.2 ± 3.2 |
|  | Winter | 11216.2 ± 954.5 | 6.0 ± 0.5 | 20.0 ± 1.7 | 22.2 ± 1.9 | 46.7 ± 4.0 | 9346.8 ± 795.4 | 186.9 ± 15.9 | 58.8 ± 3.3 | 15.4 ± 1.3 | 74.77 ± 6.4 |
| S10 | Summer | 14633.5 ± 369.7 | 9.9 ± 1.5 | 31.7 ± 3.0 | 37.1 ± 2.4 | 72.6 ± 6.3 | 17556.4 ± 641.0 | 334.5 ± 11.0 | 55.8 ± 3.4 | 29.8 ± 1.5 | 59.6 ± 3.0 |
|  | Monsoon | 13600.6 ± 1410.8 | 6.4 ± 0.8 | 24.5 ± 1.2 | 27.3 ± 1.4 | 57.2 ± 2.8 | 15111.8 ± 1567.6 | 286.2 ± 14.3 | 22.3 ± 2.4 | 25.5 ± 3.8 | 48.0 ± 2.8 |
|  | Winter | 16287.3 ± 1017.8 | 11.0 ± 0.5 | 36.8 ± 1.8 | 40.9 ± 2.0 | 85.9 ± 4.3 | 21467.7 ± 1069.9 | 155.5 ± 0.8 | 40.2 ± 2.2 | 35.8 ± 1.8 | 71.6 ± 3.6 |
| S11 | Summer | 13957.2 ± 929.5 | 7.6 ± 1.2 | 23.7 ± 3.8 | 28.8 ± 4.2 | 35.1 ± 2.7 | 9147.6 ± 595.2 | 129.6 ± 0.7 | 34.1 ± 2.8 | 12.47 ± 1.6 | 51.8 ± 0.3 |
|  | Monsoon | 9876.4 ± 1816.9 | 5.0 ± 0.03 | 16.7 ± 0.1 | 18.2 ± 0.6 | 26.6 ± 1.1 | 10973.7 ± 2018.8 | 107.0 ± 6.3 | 25.9 ± 0.1 | 9.3 ± 1.1 | 42.5 ± 1.6 |
|  | Winter | 6146.6 ± 730.7 | 7.8 ± 0.6 | 25.7 ± 1.1 | 27.8 ± 0.1 | 38.9 ± 0.2 | 7777.1 ± 41.1 | 422.7 ± 15.7 | 46.7 ± 3.9 | 12.9 ± 0.1 | 62.2 ± 0.3 |
| S12 | Summer | 15488.5 ± 862.8 | 14.1 ± 1.8 | 47.2 ± 7.5 | 54.1 ± 5.6 | 76.29 ± 6.1 | 13925.5 ± 3063.6 | 218.5 ± 9.2 | 63.0 ± 12.5 | 23.21 ± 5.1 | 71.9 ± 15.8 |
|  | Monsoon | 16988.6 ± 577.4 | 8.7 ± 0.7 | 30.5 ± 3.4 | 22.22 ± 2.5 | 61.04 ± 3.3 | 9157.21 ± 481.1 | 256.1 ± 10.8 | 55.7 ± 12.2 | 20.23 ± 1.3 | 57.56 ± 12.7 |
|  | Winter | 11162.1 ± 1090.8 | 16.1 ± 3.5 | 45.6 ±3.0 | 59.7 ± 13.1 | 83.5 ± 18.4 | 13710.6 ± 1412.0 | 334.2 ± 7.5 | 91.9 ± 4.4 | 14.51 ± 2.3 | 86.3 ± 19.0 |
| S13 | Summer | 14548.4 ± 871.9 | 11.4 ± 0.7 | 26.9 ± 2.6 | 45.2 ± 5.4 | 60.4 ± 5.6 | 12154.8 ± 1229.9 | 243.1 ± 4.6 | 29.4 ± 4.1 | 19.7 ± 1.3 | 64.41 ± 2.0 |
|  | Monsoon | 7330.9 ± 1884.8 | 7.8 ± 0.5 | 18.81 ± 1.2 | 41.4 ± 3.8 | 47.3 ± 3.1 | 6109.1 ± 157.4 | 189.1 ± 12.3 | 47.3 ± 3.1 | 15.1 ± 0.9 | 48.9 ± 3.2 |
|  | Winter | 7801.8 ± 1524.7 | 18.2 ± 0.4 | 22.0 ± 2.0 | 50.7 ± 3.3 | 70.9 ± 4.6 | 9519.1 ± 1241.9 | 280.4 ± 15.4 | 70.9 ± 4.6 | 23.6 ± 1.5 | 73.3 ± 4.8 |
| S14 | Summer | 10607.9 ± 1223.4 | 6.8 ± 1.2 | 35.5 ± 3.8 | 25.5 ± 3.6 | 39.9 ± 3.8 | 8719.7 ± 1008.9 | 166.4 ± 6.3 | 40.3 ± 2.6 | 13.4 ± 0.9 | 64.4 ± 4.2 |
|  | Monsoon | 9490.2 ± 963.9 | 4.1 ± 0.3 | 30.73 ± 2.4 | 33.8 ± 2.2 | 31.5 ± 2.8 | 10544.6 ± 1071 | 127.8 ± 9.4 | 34.9 ± 4.0 | 10.7 ± 0.7 | 52.2 ± 4.5 |
|  | Winter | 10629.9 ± 693.2 | 13.7 ± 0.9 | 34.9 ± 4.4 | 23.01 ± 1.5 | 48.3 ± 3.1 | 9663.6 ± 630.21 | 193.3 ± 12.6 | 48.3 ± 3.1 | 16.11 ± 1.0 | 77.31 ± 5.04 |
| S15 | Summer | 13837.3 ± 305.8 | 11.3 ± 3.4 | 23.1 ± 3.0 | 36.4 ± 3.1 | 74.6 ± 3.6 | 11930.8 ± 872.1 | 285.3 ± 12.4 | 50.2 ± 4.1 | 21.1 ± 2.6 | 101.2 ± 12.7 |
|  | Monsoon | 6501.7 ± 517.9 | 6.7 ± 0.7 | 13.81 ± 0.9 | 15.0 ± 0.4 | 55.4 ± 5.1 | 5418.1 ± 431.5 | 217.6 ± 27.3 | 50.6 ± 6.3 | 16.9 ± 2.1 | 77.6 ± 7.6 |
|  | Winter | 11985.3 ± 541.3 | 6.2 ± 0.4 | 20.14 ± 2.9 | 38.8 ± 4.9 | 81.6 ± 10.2 | 13316.9 ± 601.4 | 339.7 ± 18.9 | 43.0 ± 0.5 | 19.6 ± 2.0 | 121.4 ± 15.2 |
| S16 | Summer | 8519.8 ± 449.9 | 12.5 ± 4.4 | 33.7 ± 3.8 | 24.6 ± 2.6 | 44.2 ± 3.38 | 7832.0 ± 1156.9 | 139.9 ± 7.8 | 35.6 ± 0.7 | 11.9 ± 0.2 | 57.0 ± 1.0 |
|  | Monsoon | 7518.8 ± 1110.6 | 5.0 ± 0.5 | 23.3 ± 2.9 | 25.2 ± 2.6 | 31.9 ± 4.1 | 6265.6 ± 925.5 | 197.4 ± 4.3 | 27.5 ± 2.0 | 8.8 ± 0.98 | 43.9 ± 3.4 |
|  | Winter | 10216.8 ± 798.5 | 10.5 ± 1.3 | 30.1 ± 1.02 | 22.4 ± 3.31 | 47.0 ± 6.9 | 14389.9 ± 1124.7 | 188.0 ± 27.8 | 75.9 ± 9.5 | 14.2 ± 0.3 | 68.4 ± 1.2 |
| S17 | Summer | 7206.4 ± 183.9 | 5.7 ± 0.9 | 57.00 ± 5.1 | 31.85 ± 3.6 | 48.3 ± 2.78 | 6459.46 ± 741.7 | 122.52 ± 4.1 | 24.9 ± 2.61 | 7.85 ± 0.3 | 37.7 ± 1.3 |
|  | Monsoon | 4710.8 ± 572.3 | 5.0 ± 0.8 | 13.76 ± 1.7 | 15.9 ± 1.8 | 24.5 ± 0.8 | 5234.2 ± 635.9 | 94.0 ± 5.0 | 18.2 ± 1.2 | 6.3 ± 0.2 | 30.2 ± 1.0 |
|  | Winter | 14439.8 ± 1362.7 | 6.0 ± 0.9 | 61.6 ± 0.8 | 33.4 ± 1.1 | 36.8 ± 1.2 | 9398.4 ± 1388.3 | 147.0 ± 4.9 | 42.8 ± 0.8 | 10.1 ± 1.3 | 45.2 ± 1.5 |
| S18 | Summer | 8167.9 ± 786.5 | 8.5 ± 0.9 | 41.5 ± 2.2 | 51.4 ± 5.1 | 65.4 ± 2.5 | 7547.2 ± 968.5 | 200.9 ± 3.1 | 44.5 ± 2.2 | 11.9 ± 0.1 | 57.4 ± 0.7 |
|  | Monsoon | 9334.0 ± 556.9 | 9.33 ± 0.9 | 20.0 ± 0.7 | 23.3 ± 2.0 | 50.2 ± 0.6 | 10371.1 ± 618.7 | 125.3 ± 18.5 | 28.01 ± 1.5 | 9.23 ± 0.7 | 43.9 ± 3.1 |
|  | Winter | 8690.3 ± 484.9 | 9.03 ± 0.3 | 57.0 ± 6.1 | 68.4 ± 0.8 | 75.3 ± 0.9 | 7351.35 ± 248.2 | 207.8 ± 12.9 | 28.3 ± 0.9 | 14.3 ± 0.2 | 68.8 ± 0.8 |

Table S5: Pearson’s Correlation analysis between toxic elements in water of river Hooghly (n = 162)

|  |  |  |  |  | Correlations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Al | Cd | Co | Cr | Cu | Fe | Mn | Ni | Pb | Zn |
| Al | 1 |  |  |  |  |  |  |  |  |  |
| Cd | .422** | 1 |  |  |  |  |  |  |  |  |
| Co | .389** | .440** | 1 |  |  |  |  |  |  |  |
| Cr | .337** | .592** | .656** | 1 |  |  |  |  |  |  |
| Cu | .507** | .624** | .423** | .664** | 1 |  |  |  |  |  |
| Fe | .737** | .504** | .377** | .407** | .594** | 1 |  |  |  |  |
| Mn | .267** | .404** | .217** | .453** | .537** | .368** | 1 |  |  |  |
| Ni | .197* | .466** | .302** | .383** | .342** | .336** | .401** | 1 |  |  |
| Pb | .514** | .432** | .226** | .380** | .629** | .633** | .367** | .345** | 1 |  |
| Zn | .285** | .450** | .337** | .478** | .532** | .413** | .514** | .621** | .524** | 1 |
| **. Correlation is significant at the 0.01 level (2-tailed). |  |  |  |  |  |  |  |  |  |  |
| *. Correlation is significant at the 0.05 level (2-tailed). |  |  |  |  |  |  |  |  |  |  |

Table S6: ANOVA co-efficients of toxic elements between sampling locations in different seasons

|  | Summer |  |  | Monsoon |  | Winter |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Elements | F | Sig. | F |  |  | Sig. | F |  | Sig. |
| Al | 42.894 | 0.0005 | 41.265 |  |  | 0.0005 | 24.47 |  | 0.0005 |
| Cd | 10.814 | 0.0005 | 38.517 |  |  | 0.0005 | 40.145 |  | 0.0005 |
| Co | 39.574 | 0.0005 | 43.843 |  |  | 0.0005 | 99.096 |  | 0.0005 |
| Cr | 28.566 | 0.0005 | 51.724 |  |  | 0.0005 | 53.64 |  | 0.0005 |
| Cu | 47.324 | 0.0005 | 71.917 |  |  | 0.0005 | 29.767 |  | 0.0005 |
| Fe | 26.222 | 0.0005 | 37.4 |  |  | 0.0005 | 56.131 |  | 0.0005 |
| Mn | 122.452 | 0.0005 | 110.771 |  |  | 0.0005 | 44.529 |  | 0.0005 |
| Ni | 17.087 | 0.0005 | 31.547 |  |  | 0.0005 | 103.536 |  | 0.0005 |
| Pb | 26.917 | 0.0005 | 25.953 |  |  | 0.0005 | 83.796 |  | 0.0005 |
| Zn | 22.028 | 0.0005 | 21.649 |  |  | 0.0005 | 22.424 |  | 0.0005 |
| Degree of Freedom |  | Between Groups |  |  | Within Groups |  |  | Total |  |
|  |  | 17 |  |  |  | 36 |  | 53 |  |

Table S7: ANOVA co-efficients of toxic elements in river Hooghly considering three seasons

| Elements | F |  | Sig. |  |
| --- | --- | --- | --- | --- |
| Al | 5.3 |  | 0.0005 |  |
| Cd | 18.039 |  | 0.0005 |  |
| Co | 17.786 |  | 0.0005 |  |
| Cr | 20.549 |  | 0.0005 |  |
| Cu | 18.324 |  | 0.0005 |  |
| Fe | 9.533 |  | 0.0005 |  |
| Mn | 9.443 |  | 0.0005 |  |
| Ni | 15.007 |  | 0.0005 |  |
| Pb | 11.979 |  | 0.0005 |  |
| Zn | 31.129 |  | 0.0005 |  |
| Degree of Freedom |  | Between Groups |  | 2 |
|  |  | Within Groups |  | 159 |
|  |  | Total |  | 161 |

Table S8 : Relative Weight (Wr) of studied water quality parameters

|  | Parameters | Acceptable limits as per Indian Standards (ISI, 191; BIS, 2012) and WHO, (2008) | Assigned Weight (wr) | Relative weight (Wr) |
| --- | --- | --- | --- | --- |
| Physico-chemical Properties | pH | 6.5 - 8.5 | 4 | 0.0645 |
|  | Hardness | 200 | 4 | 0.0645 |
|  | TDS | 1000 | 4 | 0.0645 |
|  | DO | 5 | 4 | 0.0645 |
|  | COD | 10 | 4 | 0.0645 |
|  | EC | 1500 | 3 | 0.0484 |
|  | Salinity | 0.1 | 3 | 0.0484 |
|  | Alkalinity | 200 | 3 | 0.0484 |
| PTE | Cu | 2000 | 2 | 0.0323 |
|  | Co | 40 | 1 | 0.0161 |
|  | Ni | 20 | 1 | 0.0161 |
|  | Zn | 3000 | 1 | 0.0161 |
|  | Cd | 3 | 5 | 0.0806 |
|  | Cr | 50 | 5 | 0.0806 |
|  | Mn | 500 | 5 | 0.0806 |
|  | Pb | 10 | 5 | 0.0806 |
|  | Al | 200 | 4 | 0.0645 |
|  | Fe | 300 | 4 | 0.0645 |
|  |  |  | ∑wr =62 | ∑Wr = 1 |