<Appendix A Supplementary Material>

# Source apportionment and risk assessment of heavy metals in urban soils from a Central China city by using Positive matrix factorization model coupled with Monte Carlo simulation

Lianfeng Shen1  Jin Zeng1  Erhui Guo1  Yongxia Cheng2  Limei Li1  Cunyang Niu1  Yuan Yuan1  Li Ma1

Li Ma

malizhk@henau.edu.cn

1  College of Forestry, Henan Agricultural University, Zhengzhou, 450002, China

2  Key Laboratory for Monitor and Remediation of Heavy Metal Polluted Soils of Henan Province, Jiyuan, 459000, China

Supplementary Tables

Table S1 Different pollution risk index and their grades

Table S2 Uncertain concentrations (mg/kg) of heavy metals in urban soils of Zhengzhou city

Table S3 Parameters for the exposure risk calculations of heavy metals in soil with Mote Carlo simulator

Table S4 The reference doses (RfD) for noncarcinogenic metals and slope factors (SF) for carcinogenic metals

Table S5 The mean values of health risk for heavy metals from different exposure routes of people in urban soils

Table S6 Summary statistics for 5th, 25th, 50th, 75th and 95th percentile non-carcinogenic and carcinogenic health risk based on Monte Carlo simulation using Crystal Ball (vs. 11.1.2.4)

Supplementary Figures

Fig. S1 The wind rose map of the mean concentration / the corresponding local average background values for eight heavy metals in the study area

Fig. S2 Proportions of heavy metals exceeding background values in all sampling sites

Fig. S3 Spatial distribution of the concentrations of soil heavy metals in study area

Fig. S4 Fitting coefficients (r2) between soil heavy metals observed concentrations and predicted concentrations by PMF model

Fig. S5 Factors profiles and source contributions of heavy metals from PMF model.

Fig. S6 Factor contributions of soil heavy metals from PMF model

Fig. S7 Exceedance percentage of acceptable threshold (1E-06) and unacceptable threshold (1E-04) for TCR and the CR of As, Ni, Co and Cd

Table S1 Different pollution risk index and their grades

| Class | Geo-accumulation index (Igeo) | Pollution status | Nemerow composite index (Ps) | Pollution status | Ecological risk index (RI) | Potential ecological hazard |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Igeo≤0 | Uncontaminated | Ps＜0.7 | Safety | RI＜150 | Low |
| 1 | 0<Igeo≤1 | Uncontaminated to moderately uncontaminated | 0.7≤Ps＜1 | Precaution | 150≤RI＜300 | Moderate |
| 2 | 1<Igeo≤2 | Moderately contaminated | 1≤Ps＜2 | Slightly polluted | 300≤RI＜600 | High |
| 3 | 2<Igeo≤3 | Moderately to heavily contaminated | 2≤Ps＜3 | Moderately polluted | RI≥600 | Significantly high |
| 4 | 3<Igeo≤4 | Heavily contaminated | Ps≥3 | Seriously polluted | / | / |
| 5 | 4<Igeo≤5 | Heavily to extremely contaminated | / |  | / | / |
| 6 | Igeo>5 | Extremely contaminated | / |  | / | / |
| Reference | (Müller, 1969) |  | (Fredj et al., 2014) |  | (Tian et al., 2020) |  |

Table S2 Uncertain concentrations (mg/kg) of heavy metals in urban soils of Zhengzhou city

| Heavy metals | Probabilistic distribution | Parameters (mean, SD) | Reference |
| --- | --- | --- | --- |
| As | Normal | N(10.65, 0.18) | This study |
| Cd | Lognormal | LN(0.19, 0.01) | This study |
| Co | Lognormal | LN(7.92, 0.13) | This study |
| Cu | Lognormal | LN(15.28, 0.56) | This study |
| Mn | Lognormal | LN(374.51, 6.59) | This study |
| Ni | Normal | N(17.59, 0.28) | This study |
| Pb | Lognormal | LN(37.16, 1.41) | This study |
| Zn | Lognormal | LN(52.36, 2.31) | This study |

(Here, the software Crystal Ball (11.1.2.4, Oracle, USA) was used to fit the probabilistic distribution of the uncertain concentrations of heavy metals.)

Table S3 Parameters for the exposure risk calculations of heavy metals in soil with Mote Carlo simulator

| Parameter | Description | Unit | Type | Children | Adult | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| EF | exposure frequency | day/year | Triangular | TRI (180, 345, 365) |  | (Huang et al., 2021) |
| Ring | ingestion rate of soil | mg/day | Triangular | TRI (66, 103, 161) | TRI (4, 30, 52) | (Yang et al., 2019) |
| SL | skin adherence factor | mg/cm2 | Lognormal | LN (0.65, 1.2) | LN (0.49, 0.54) | (Chen et al., 2019) |
| Rinh | Inhalation rate | m3/day | Point | 7.5 | 14.5 | (Han et al., 2020) |
| ED | exposure duration | years | Point | 6 | 24 | (Han et al., 2020) |
| PEF | particle emission factor | m3/kg | Point | 1.36×109 | 1.36×109 | (Huang et al., 2021) |
| BW | average body weight | kg | Point | 15 | 55.9 | (Han et al., 2020) |
| AT | average time of exposure to contaminated soils | day | Point | 365×ED（non-carcinogenic） 365×70（carcinogenic） |  | (Han et al., 2020) |
| SA | exposed skin area | m2 | Triangular | TRI(0.076, 0.086, 0.382) | TRI(0.076, 0.153, 0.382) | (Huang et al., 2018) |
| ABF | dermal adsorption factor | - | - | 0.001（non-carcinogenic） 0.01（carcinogenic） |  | (Huang et al., 2021) |

Table S4 The reference doses (RfD) for noncarcinogenic metals and slope factors (SF) for carcinogenic metals

| Heavy metals | RfD (mg/kg·d) |  |  |  | SF (kg/d·mg) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RfDing | RfDinh | RfDderm |  | SFing | SFinh | SFderm |
| As | 3.00E-04a | 1.23E-04a | 1.23E-04a |  | 1.50E+00a | 1.51E+01a | 3.66E+00a |
| Cd | 1.00E-03a | 1.00E-05a | 1.00E-05a |  | 6.10E+00a | 6.30E+00a | 3.80E-01b |
| Co | 2.00E-02c | 1.60E-03c | 5.71E-06c |  | N/A | N/A | 9.80E+00c |
| Cu | 4.00E-02a | 4.02E-02a | 1.20E-02a |  | N/A | N/A | N/A |
| Mn | 4.60E-01a | 1.43E-05a | 1.84E-03a |  | N/A | N/A | N/A |
| Ni | 2.00E-02a | 2.06E-02a | 5.40E-03a |  | 1.70E+00d | 9.00E-01d | 4.25E+00d |
| Pb | 3.50E-03a | 3.52E-03a | 5.25E-04a |  | 8.50E-03a | N/A | N/A |
| Zn | 3.00E-01a | 3.00E-01a | 6.00E-02a |  | N/A | N/A | N/A |

N/A, Data not available, a (Huang et al., 2021), b(Wang et al., 2017), c(Han et al., 2020), d(Tong et al., 2020)

Table S5 The mean values of health risk for heavy metals from different exposure routes of people in urban soils

| Risk | Crowd category | Approach | Heavy metals |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | As | Cd | Co | Cu | Mn | Ni | Pb | Zn |
| Non-carcinogenic | Children | Ingestion | 2.12E-01 | 1.13E-03 | 2.36E-03 | 2.29E-03 | 4.84E-03 | 5.23E-03 | 6.34E-02 | 1.04E-03 |
|  |  | Inhalation | 2.59E-05 | 5.67E-06 | 1.48E-06 | 1.14E-07 | 7.81E-03 | 2.55E-07 | 3.16E-06 | 5.22E-08 |
|  |  | Dermal contact | 5.54E-07 | 1.22E-07 | 8.82E-06 | 8.15E-09 | 1.26E-06 | 2.07E-08 | 4.67E-07 | 5.84E-09 |
|  | Adult | Ingestion | 1.48E-02 | 7.91E-05 | 1.64E-04 | 1.59E-04 | 3.39E-04 | 3.68E-04 | 4.38E-03 | 7.27E-05 |
|  |  | Inhalation | 1.34E-06 | 2.94E-07 | 7.66E-08 | 5.90E-09 | 4.06E-04 | 1.32E-08 | 1.64E-07 | 2.70E-09 |
|  |  | Dermal contact | 1.27E-07 | 2.76E-08 | 2.00E-06 | 1.87E-09 | 2.93E-07 | 4.74E-09 | 1.01E-07 | 1.27E-09 |
| Carcinogenic | Children | Ingestion | 8.17E-06 | 5.90E-07 | - | - | - | 1.53E-05 | 1.62E-07 | - |
|  |  | Inhalation | 4.13E-09 | 3.06E-11 | - | - | - | 4.05E-10 | - | - |
|  |  | Dermal contact | 2.14E-05 | 3.98E-08 | 4.23E-05 | - | - | 4.07E-05 | - | - |
|  | Adult | Ingestion | 2.29E-06 | 1.66E-07 | - | - | - | 4.29E-06 | 4.47E-08 | - |
|  |  | Inhalation | 8.56E-10 | 6.35E-12 | - | - | - | 8.41E-11 | - | - |
|  |  | Dermal contact | 1.95E-05 | 3.60E-08 | 3.84E-05 | - | - | 3.73E-05 | - | - |

Table S6 Summary statistics for 5th, 25th, 50th, 75th and 95th percentile non-carcinogenic and carcinogenic health risk based on Monte Carlo simulation using Crystal Ball (vs. 11.1.2.4)

| Risk | Heavy metals | Children |  |  |  |  |  | Adult |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5th | 25th | 50th | 75th | 95th |  | 5th | 25th | 50th | 75th | 95th |
| HQ | As | 1.36E-01 | 1.77E-01 | 2.09E-01 | 2.43E-01 | 2.98E-01 |  | 5.84E-03 | 1.08E-02 | 1.47E-02 | 1.86E-02 | 2.44E-02 |
|  | Cd | 7.34E-04 | 9.45E-04 | 1.12E-03 | 1.31E-03 | 1.60E-03 |  | 3.18E-05 | 5.79E-05 | 7.88E-05 | 1.00E-04 | 1.31E-04 |
|  | Co | 1.54E-03 | 1.98E-03 | 2.33E-03 | 2.74E-03 | 3.30E-03 |  | 6.79E-05 | 1.23E-04 | 1.66E-04 | 2.10E-04 | 2.74E-04 |
|  | Cu | 1.49E-03 | 1.91E-03 | 2.25E-03 | 2.64E-03 | 3.23E-03 |  | 6.32E-05 | 1.16E-04 | 1.57E-04 | 1.99E-04 | 2.63E-04 |
|  | Mn | 9.23E-03 | 1.13E-02 | 1.29E-02 | 1.42E-02 | 1.58E-02 |  | 5.00E-04 | 6.28E-04 | 7.39E-04 | 8.53E-04 | 1.01E-03 |
|  | Ni | 3.41E-03 | 4.37E-03 | 5.17E-03 | 6.03E-03 | 7.36E-03 |  | 1.46E-04 | 2.67E-04 | 3.63E-04 | 4.61E-04 | 5.97E-04 |
|  | Pb | 4.11E-02 | 5.26E-02 | 6.21E-02 | 7.29E-02 | 8.90E-02 |  | 1.81E-03 | 3.14E-03 | 4.33E-03 | 5.53E-03 | 7.25E-03 |
|  | Zn | 6.70E-04 | 8.68E-04 | 1.03E-03 | 1.19E-03 | 1.46E-03 |  | 2.82E-05 | 5.27E-05 | 7.19E-05 | 9.13E-05 | 1.21E-04 |
| HI | Total | 2.19E-01 | 2.63E-01 | 2.97E-01 | 3.33E-01 | 3.89E-01 |  | 1.15E-02 | 1.66E-02 | 2.06E-02 | 2.47E-02 | 3.07E-02 |
| CR | As | 8.17E-06 | 1.20E-05 | 1.76E-05 | 3.04E-05 | 8.41E-05 |  | 4.56E-06 | 8.67E-06 | 1.45E-05 | 2.59E-05 | 6.22E-05 |
|  | Cd | 4.01E-07 | 5.18E-07 | 6.16E-07 | 7.26E-07 | 8.99E-07 |  | 8.80E-08 | 1.48E-07 | 1.97E-07 | 2.48E-07 | 3.32E-07 |
|  | Co | 2.30E-06 | 7.81E-06 | 1.89E-05 | 4.47E-05 | 1.56E-04 |  | 5.04E-06 | 1.29E-05 | 2.46E-05 | 4.73E-05 | 1.24E-04 |
|  | Ni | 1.54E-05 | 2.27E-05 | 3.33E-05 | 5.82E-05 | 1.63E-04 |  | 8.62E-06 | 1.68E-05 | 2.81E-05 | 4.92E-05 | 1.18E-04 |
|  | Pb | 1.05E-07 | 1.34E-07 | 1.58E-07 | 1.86E-07 | 2.27E-07 |  | 1.85E-08 | 3.21E-08 | 4.41E-08 | 5.64E-08 | 7.40E-08 |
| TCR | Total | 4.14E-05 | 6.39E-05 | 9.37E-05 | 1.47E-04 | 3.26E-04 |  | 3.58E-05 | 5.80E-05 | 8.43E-05 | 1.26E-04 | 2.28E-04 |

Fig. S1 The wind rose map of the mean concentration / the corresponding local average background values for eight heavy metals in the study area

Fig. S2 Proportions of heavy metals exceeding background values in all sampling sites

Fig. S3 Spatial distribution of the concentrations of soil heavy metals in study area

Fig. S4 Fitting coefficients (r2) between soil heavy metals observed concentrations and predicted concentrations by PMF model

Fig. S5 Factors profiles and source contributions of heavy metals from PMF model

Fig. S6 Factor contributions of soil heavy metals from PMF model

Fig. S7 Exceedance percentage of acceptable threshold (1E-06) and unacceptable threshold (1E-04) for TCR and the CR of As, Ni, Co and Cd

## References

Chen, R.H., Chen, H.Y., Song, L.T., Yao, Z.P., Meng, F.S., Teng, Y.G., 2019. Characterization and source apportionment of heavy metals in the sediments of Lake Tai (China) and its surrounding soils. Sci Total Environ 694.

Fredj, F.B., Wali, A., Khadhraoui, M., Han, J., Funamizu, N., Ksibi, M., Isoda, H., 2014. Risk assessment of heavy metal toxicity of soil irrigated with treated wastewater using heat shock proteins stress responses: case of El Hajeb, Sfax, Tunisia. Environ Sci Pollut R 21, 4716-4726.

Han, Q., Wang, M., Cao, J., Gui, C., Liu, Y., He, X., He, Y., Liu, Y., 2020. Health risk assessment and bioaccessibilities of heavy metals for children in soil and dust from urban parks and schools of Jiaozuo, China. Ecotoxicol Environ Saf 191, 110157.

Huang, J., Wu, Y., Sun, J., Li, X., Geng, X., Zhao, M., Sun, T., Fan, Z., 2021. Health risk assessment of heavy metal(loid)s in park soils of the largest megacity in China by using Monte Carlo simulation coupled with Positive matrix factorization model. J Hazard Mater 415, 125629.

Huang, J.H., Guo, S.T., Zeng, G.M., Li, F., Gu, Y.L., Shi, Y.H., Shi, L.X., Liu, W.C., Peng, S.Y., 2018. A new exploration of health risk assessment quantification from sources of soil heavy metals under different land use. Environ Pollut 243, 49-58.

Müller, G., 1969. Index of geoaccumulation in sediments of the Rhine Rive. Geojournal 2, 108-118.

Tian, K., Wu, Q., Liu, P., Hu, W., Huang, B., Shi, B., Zhou, Y., Kwon, B.O., Choi, K., Ryu, J., Seong Khim, J., Wang, T., 2020. Ecological risk assessment of heavy metals in sediments and water from the coastal areas of the Bohai Sea and the Yellow Sea. Environ Int 136, 105512.

Tong, S., Li, H., Wang, L., Tudi, M., Yang, L., 2020. Concentration, spatial distribution, contamination degree and human health risk assessment of heavy metals in urban soils across China between 2003 and 2019-A systematic review. Int J Environ Res Public Health 17.

Wang, G., Liu, H.Q., Gong, Y., Wei, Y., Miao, A.J., Yang, L.Y., Zhong, H., 2017. Risk assessment of metals in urban soils from a typical industrial city, Suzhou, Eastern China. Int J Environ Res Public Health 14.

Yang, S.Y., Zhao, J., Chang, S.X., Collins, C., Xu, J.M., Liu, X.M., 2019. Status assessment and probabilistic health risk modeling of metals accumulation in agriculture soils across China: A synthesis. Environ Int 128, 165-174.