Supplemental Material

A predictive in vitro risk assessment platform for pro-arrhythmic toxicity using human 3D cardiac microtissues

Celinda M. Kofron*1, Tae Yun Kim*2, Fabiola Munarin1, Arvin H. Soepriatna1, Rajeev J. Kant1, Ulrike Mende2, Bum-Rak Choi§2, Kareen L.K. Coulombe§1

1Center for Biomedical Engineering, School of Engineering, Brown University, Providence, RI

2Cardiovascular Research Center, Cardiovascular Institute, Rhode Island Hospital and Alpert Medical School of Brown University, Providence, RI

* Contributed equally to this study.

§ Contributed equally to this study, co-corresponding authors.

Supplemental Figure 1. Automated analysis pipeline from fluorescence signals obtained during optical mapping. (A) A greyscale snap shot of fluorescence from microtissues at 3.2x magnification (also shown in Fig 1E) during optical mapping. Individual microtissues are numbered 1 - 4. (B) Sample membrane voltage (Vm) traces recorded from the corresponding microtissues in panel A. Note that electrical stimulation did not evoke APs in microtissue #4 (where the fluorescence trace shows a flat line). Fast Fourier transform (FFT) was used to distinguish responsive microtissues from non-responsive microtissues lacking APs. (C) FFTmax image. The microtissue (#4) without APs is automatically removed in FFTmax (circle). (D) Automated thresholding using Otsu’s thresholding. (E) Blob coloring algorithm to detect individual microtissues. Signals within the same microtissue are averaged to acquire high signal-to-noise ratio and fidelity of data analysis. (F) Automated analysis (of sample trace #3, black) uses the first derivative (dF/dt, red trace) for detecting AP upstroke to calculate duration to 80% recovery (APD80, see Figure 1G) or uses the maximum peak of the second derivative (d2F/dt2, blue trace) for assessing the end of maximum repolarization rate to calculate AP duration APDMxR.1  APDMxR is useful when motion artifacts or other interference on fluorescence recordings such as fluctuation in water level is suspected to elevate fluorescence level during repolarization and reproducible APD80 measurement with low variation is not as reliable. (G) Sample output shows APD80 statistics from the 3 excitable individual microtissues.

Supplemental Figure 2. Quantile-Quantile plot of APD data to normal distribution. APD80 metric shows a nearly normal distribution for all data points except at the extremities when compared to the reference line (red) calculated from a normal distribution function for  (A) all microtissues from 7 molds from 3 batches and (B) a single batch of 35 microtissues.

Supplemental Figure 3. Early after depolarizations in response to hERG channel blocker E4031 and isoproterenol in hiPSC-CMLP with 5% hCF microtissues. (A-C) Representative Vm traces from control (DMSO vehicle, A), 2 µM E4031 exposure (B), and 2 µM E4031 plus 100 nM isoproterenol (ISO, C).  APD80 are prolonged under 2 µM E4031 and additional beta-adrenergic stimulation triggers early afterdepolarizations (EADs, red arrows). ISO alone does not evoke EADs (see Supplemental Figure S4A). (D, E) Scatter plot (D) and cumulative distribution of APD80 (E) show dose-dependent increase of APD80 and increased variation with E4031 (n=59-69 microtissues from 2 molds per group). Values shown by lines in (D) are means ± SD. *P < 0.05 vs. 0 μM. (F) Quantification of EAD incidence before (0 mM) and after (2 µM) E4031 in the presence of 100 nM ISO.

Supplemental Figure 4. Physiological responses to isoproterenol (ISO), 4-AP, and BayK8644 in hiPSC-CMLP with 5% hCF microtissues.  Representative traces of (top) and cumulative probability plots for APD80 (bottom) show relative changes in response to indicated drugs before and after acute exposure. (A) Beta-adrenergic stimulation with 100 nM ISO shortened APD80 (red) (166.6 ± 17.7 vs. 120.1 ± 10.5 after ISO, P = 8.9110-19, n=29 microtissues). No early afterdepolarization (EAD) was observed. (B) Blockade of the transient outward potassium current (Ito) with 4-Aminopyridine (4-AP) prolonged APD80 (red) (169.3 ± 14.7 vs. 288.7 ± 33.9 ms, P = 2.6210-26, n=35 microtissues). (C) L-type Ca2+ channel stimulation with 300 nM BayK8644 prolonged APD80 (red) (199.5 ± 20.3 vs. 225.8 ± 14.6 ms, 1.1510-9, n=35 microtissues).

Supplemental Figure 5. Tracking individual microtissue dose responses of Ranolazine and BPA. Dose response curves for APDMxR with increasing concentrations of (A) ranolazine (n=34 microtissues; shown in heatmap of Figure 5B) and (B) BPA (n=34 microtissues; heatmap shown in Figure 6B). The lines depict dose-dependent APDMxR changes of individual microtissues. The analysis of individual microtissues before and after drug exposure at different concentrations using paired t-tests increases statistical power.

Supplemental Table 1. P-values from paired t-tests of data depicted in Figure 5C for ranolazine (top); Figure 6B for one experiment with BPA (up to 1 μM, bottom). Red font indicates P<0.05.

Supplemental Figure 6. APDtri metric as an indicator of hERG channel blockade. The relationship between APD prolongation and APD triangulation (APDtri) can be used to infer hERG channel blockade. We tested 3 major ion channel modulators that can prolong APD during the early phase (Ito, phase 1; with 4-AP), plateau phase (ICa, phase 2; with BayK8644), and repolarization (IKr, phase 3; with E4031) of the AP. Since IKr impacts the later phase of AP repolarization (phase 3), IKr blockade delays phase 3 repolarization to increase APDtri. (A) Computer modeling to study the impact of IKr blockade on APDtri using the O’Hara human myocyte model.2 The conductances of INa, Ito, ICa, IKr, IKs were varied to have a Gaussian distribution of APDs in 10,000 cells (original parameters in O’Hara and Rudy2 ± 20%). APDtri changes under IKr block were investigated. Increase of IKr blockade (from black to blue, green, orange and red) increases APDtri as APDMxR increases (plot, left) and steepens the slope of APDtri versus APDMxR (bar graph, right). (B-D) Experimental results show that the increase of APDtri is unique to IKr blockade. (B) The slope of APDtri versus APDMxR steepened in response to 1µM E4031, in agreement with the results from the computer modeling in panel A. The slope significantly increases between 0 and 1 µM E4031 (right, p = 0.011). (C) 0.5 mM 4-AP (Ito blockade) did not steepen the slope of APDtri versus APDMxR despite APD prolongation (p = 0.199). (D) 300 nM BayK8644 (ICa agonist) did not steepen APDtri versus APDMxR despite APD prolongation (p = 0.539).

Dose-dependent mean differences of AP metrics (Ranolazine)

| Ranolazine | 2 M | 10 M | 100 M |
| --- | --- | --- | --- |
| Rise Time | -0.35093 | 0.03604 | 7.77581 |
| APD30 | 4.39627 | 16.6689 | 53.5831 |
| APD50 | -3.46581 | 14.7128 | 101.51 |
| APD80 | 3.43512 | 64.7965 | 219.988 |
| APDMxR | -10.5228 | 6.64536 | 307.389 |
| APDtri | -3.5293 | 2.56326 | 198.626 |

Dose-dependent mean differences of AP metrics (BPA)

| BPA | 1 nM | 10 nM | 100 nM |
| --- | --- | --- | --- |
| Rise Time | -0.12072 | -1.53018 | -2.27589 |
| APD30 | 19.687 | -10.3786 | -16.5818 |
| APD50 | 16.8487 | -15.1815 | -21.6624 |
| APD80 | 22.9907 | 1.23495 | 0.76422 |
| APDMxR | 10.2523 | -19.7025 | -31.285 |
| APDtri | -6.57858 | -4.48536 | -9.56911 |

Supplemental Table 2.  Mean dose-response difference of AP metrics. Red and blue fonts indicate that AP metrics increased and decreased, respectively, by the paired t-test in Figure 7.

Movie S1. Beating human cardiac microtissue comprised of hiPSC-CMs and 5% hCFs imaged after 1 day in 3D culture while paced at 1 Hz.

Movie S2. Optical mapping of a human cardiac microtissue (left) and the corresponding voltage trace (right) under control conditions (DMSO as vehicle) while paced at 0.5 Hz.

Movie S3. Optical mapping of 2 human cardiac microtissues (left) and the corresponding voltage traces (right) under 2 μM E4031 showing APD prolongation and distinct EADs while paced at 0.5 Hz.

Supplemental Material References:

1.	Efimov IR, Huang DT, Rendt JM, Salama G. Optical mapping of repolarization and refractoriness from intact hearts. Circulation 1994;90:1469-1480.

2.	O'Hara T, Virag L, Varro A, Rudy Y. Simulation of the undiseased human cardiac ventricular action potential: model formulation and experimental validation. PLoS Comput Biol 2011;7:e1002061.