Supplementary Table 1.  U.S. Army Corps of Engineers (USACE; 2000) Building Depth-Damage Function of One-Story, No-Basement Structure:

| Depth (ft.) | Mean of Damage as a Percent of Building Value | Standard Deviation of Damage |
| --- | --- | --- |
| −2 | 0 | 0 |
| −1 | 2.5 | 2.7 |
| 0 | 13.4 | 2 |
| 1 | 23.3 | 1.6 |
| 2 | 32.1 | 1.6 |
| 3 | 40.1 | 1.8 |
| 4 | 47.1 | 1.9 |
| 5 | 53.2 | 2 |
| 6 | 58.6 | 2.1 |
| 7 | 63.2 | 2.2 |
| 8 | 67.2 | 2.3 |
| 9 | 70.5 | 2.4 |
| 10 | 73.2 | 2.7 |
| 11 | 75.4 | 3 |
| 12 | 77.2 | 3.3 |
| 13 | 78.5 | 3.7 |
| 14 | 79.5 | 4.1 |
| 15 | 80.2 | 4.5 |
| 16 | 80.7 | 4.9 |

Supplementary Figure 1.  Depth-damage function curve from U.S. Army Corps of Engineers (2000) that expresses percentage of residential building damage as a function of flood depth (in feet); negative values of flood depth refer to floods that extend vertically below the first-floor height.

Python Programming Script for apportioning flood loss between individual and National Flood Insurance Program:

import numpy as np

import random

import scipy.stats

### Flood hazard parameter

def a_u_pointwise(return_periods, flood_depths):

excedence_prob = [round(np.log(-np.log(1-1/i)),2) for i in return_periods]

slope, intercept, _, _, _ = scipy.stats.linregress(excedence_prob, flood_depths)

slope_a = round(-slope,4)

int_u = round(intercept,4)

if int_u>0:

return_periods.insert(0,2)

x = -0.1

flood_depths.insert(0,x)

excedence_prob = [round(np.log(-np.log(1-1/i)),2) for i in return_periods]

slope, intercept, _, _, _ = scipy.stats.linregress(excedence_prob, flood_depths)

slope_a = round(-slope,4)

int_u = round(intercept,4)

while int_u>0:

x = round(x - 0.1,2)

flood_depths [0] = x

slope, intercept, _, _, _ = scipy.stats.linregress(excedence_prob, flood_depths)

slope_a = round(-slope,4)

int_u = round(intercept,4)

return [slope_a,int_u]

########################

### Monte Carlo simulation

def get_probability():

return (random.uniform(0,1))

def inv_flood_depth(x,u,a):

return u - a * np.log(-(np.log(x)))

def monte_carlo(u, a, ffh, building_value, coverage, deductible, num_samples=50000):

sum_loss_build = 0

sum_loss_owner = 0

sum_loss_nfip = 0

for i in range(num_samples):

p = get_probability()

Di = inv_flood_depth(p,u,a)

Dsi = Di - ffh

### Loss estimation using USACE 200 Structure DDF

if Dsi <-2:     #inflation point of DDF is -2

build_loss = 0

else:

build_loss = ((0.0015*(Dsi **3)-0.3373*(Dsi **2) +9.0339*Dsi +15.413)/100) * building_value     #DDF for inflation point -2

# build_loss = ((0.0092*(Dsi **3) - 0.5342 * (Dsi **2) +10.404*Dsi + 13.418)/100) * building_value   #DDF for inflation point -1

# build_loss = ((0.0092*(Dsi **3)-0.5362*(Dsi **2) +10.419*Dsi +13.39)/100) * building_value   #DDF for inflation point 0

if build_loss <0:

build_loss = 0

###############

### Loss allocation

if build_loss <coverage:

deduct_owner = [deductible if build_loss >deductible else build_loss][0]

deduct_nfip = build_loss  - deduct_owner

elif build_loss >coverage:

deduct_owner = deductible + (build_loss -coverage)

deduct_nfip = build_loss - deduct_owner

###############

sum_loss_build += build_loss

sum_loss_owner += deduct_owner

sum_loss_nfip += deduct_nfip

aal_total = int(sum_loss_build /num_samples)

aal_owner = int(sum_loss_owner /num_samples)

aal_nfip = int(sum_loss_nfip /num_samples)

owner_portion = round((aal_owner/aal_total),2)

return [aal_total, aal_owner, aal_nfip, owner_portion]

########################

### case study

return_period = [10,50,100,500]

flood_depths= [2.3,2.8,3.1,3.6]

ffh = 3

building_value = 166446

coverage = 150000

deductible = 1500

a,u = a_u_pointwise(return_period,flood_depths)

aals = monte_carlo(u, a, ffh, building_value, coverage, deductible)

print("AAl Total: ", aals[0])

print("Owner AAL: ", aals[1])

print("NFIP AAL: ", aals[2])

print('Homeowner proportion: ', aals[3])